CPC Guide - Input/Output port addressessValid I/O Port addressess

PORT

| &7Fxx | Gate array |
| --- | --- |
| &BCxx | CRTC: select register |
| &BDxx | CRTC: register data |
| &BFxx | CRTC: data |
| &DFxx | Expansion rom select |
| &EFxx | Centronics data |
| &F4xx | 8255: port A data (PSG) |
| &F5xx | 8255: port B data (Cassette, Printer, Misc sensing) |
| &F6xx | 8255: port C data (PSG,Keyboard scanning,Cassette) |
| &F7xx | 8255: control |
| &F8xx | Expansion Bus |
| &F9xx | Expansion Bus |
| &FAxx | Expansion Bus |
| &FA7E | FDC: Disk drive motor control |
| &FBxx | Expansion Bus |
| &FB7E | FDC: main status register |
| &FB7F | FDC: data register |
| &FFxx | NOT USED |
| &FADC | Serial: SIO Channel A data |
| &FADD | Serial: SIO Channel A Control |
| &FADE | Serial: SIO Channel B data |
| &FADF | Serial: SIO Channel B control |
| &FBDC | Serial: 8253 Load Counter 0 |
| &FBDD | Serial: 8253 Load Counter 1 |
| &FBDE | Serial: 8253 Load Counter 2 |
| &FBDF | Serial: 8253 Write Mode Word |

For those port addressess in the expansion bus range: Ports &F8xx to &FBxx xx byte as specified below:

| &00 - &7B | DO NOT USE |
| --- | --- |
| &7C - &7F | Disc Interface |
| &80 - &BB | DO NOT USE |
| &BC - &BF | Reserved for future use |
| &C0 - &DB | DO NOT USE |
| &DC - &DF | For communications interfaces |
| &E0 - &FE | Available for user expansion interfaces |
| &FF | Bus Reset interfaces |

CPC Guide - The Gate ArrayGate Array

Introduction

The gate array is a specially designed chip exclusively for use in the Amstrad CPC. It is for this reason not much is known about it, however from extensive research I have managed to compile the information in this document. It was designed by Amstrad.

It is responsible for controlling screen mode, colour palette, ram configuration and rom configuration.

The "new" ASIC Gate Array (in the new Plus machines) is described here. In standard mode on the Plus machines (that is when the new ASIC Gate Array has not been enabled), it acts as a standard gate array, allowing programs on the CPC to work on the Plus machines without modification.

However when the new ASIC gate array is enabled, it allows the user to use some extra enhancements.

The Gate Array is described here, as it would be on a standard CPC.

Display

The CRTC reads data from the CPC memory and passes it to the gate array which interprets the data depending on the current mode selected, and then adds colour depending on the current palette settings. This data is converted into red, green and blue components which are sent to the monitor.

The gate array recieves some signals from the CRTC to help it to build the display.

DISPTMG

when low tells the gate array to interpret the data as pixels, and when high tells the gate array to ignore the data and put border colour.

HSYNC

The gate array will only allow a mode change when a HSYNC occurs.

The new mode is stored until the HSYNC is received and then the mode is changed, this has probably been built into the hardware because there might be some graphics distortion if the mode change was visible.

This is also passed to the monitor to maintain a steady image.

VSYNC

This is passed to the monitor to maintain a steady image.

Interrupts

The interrupts originate from the gate array. Every 1/300th of a second the gate array will send the interrupt pulse to the z80 and wait for an acknowledgement. If the z80 has interrupts enabled, they will be processed almost immediatly, otherwise the interrupt is held.

Controlling the Gate Array

The gate array is controlled by sending values to port &7Fxx. The function which is to be performed is dependant on the settings of bits 7 and 6; the remaining bits are data used by the appropiate function.

All these registers are write only and cannot be read.

A brief description of the functions are described below with their settings for bit 7 and 6.

Bit 7	Bit 6	Function

0	0	Select which pen is to have its colour changed.

0	1	Select colour for currently selected pen. (from hardware colour palette)

1	0	Select screen mode and rom configuration.

1	1	Select ram configuration (bank switching).

Pen selection

When bit 7 and bit 6 are set to 0, the remaining bits determine which pen is to have its colour changed. Bits 3 to 0 define which pen is to be selected. When bit 4 is set to 1, the value contained in bits 3-0 is ignored and the border is selected.

The pen remains selected until another is selected.

Each mode has a fixed number of pens. Mode 0 has 16 pens, mode 1 has 4 pens and mode 2 has 2 pens.

Summary

Bit 7: }	Gate Array function	(0)

Bit 6: }				(0)

Bit 5:		Not used

Bit 4:		Border selection	1=Select border, 0=use pen value contained in bits 3-0.

Bit 3: }				Pen number

Bit 2: }

Bit 1: }

Bit 0: }

Colour selection

Once the pen has been selected the colour can then be changed. Bits 4 to 0 hold the colour number (from the hardware pallette).

The hardware colour number is different to the colour range used by the firmware, so a conversion chart is provided for the corresponding firmware/hardware colour values and the corresponding colour name.

Even though there is provision for 32 colours, only 27 are possible. The remaining colours are duplicates of those already in the colour palette.

Note: The firmware keeps track of the colours it is using. Every VSYNC (assuming interrupts are enabled) the firmware sets the colours. This enables the user to have flashing colours. If the user selects a new colour using the gate array, the new colour will flash temporarily and then return to it's original colour.

This is due to the firmware re- setting the colour. When using the firmware, use it's routines to select the colour, and the colour will remain.

Summary

Bit 7: }	Gate Array function	(0)

Bit 6: }				(1)

Bit 5:		Not used.

Bit 4: }	Colour number (from hardware colour palette)

Bit 3: }

Bit 2: }

Bit 1: }

Bit 0: }

Select screen mode and rom configuration

This is a general purpose register responsible for the screen mode and the rom configuration.

Screen mode selection

The function of bits 1 and 0 is to define the screen mode. The settings for bits 1 and 0 and the corresponding screen mode are given in the table below.

From the combinations possible, we can see that 4 modes can be defined, although the Amstrad only has 3. Mode 3 is similar to mode 0, because it has the same resolution, but it is limited to only 4 colours. This mode may be different on different versions of the Gate Array and on different CPC models.

The screen mode is changed when a HSYNC occurs.

Bit 1	Bit 0	Screen mode

0	0	Mode 0,160x200, 16 colours.

0	1	Mode 1,320x200, 4 colours.

1	0	Mode 2,640x200, 2 colours

1	1	Mode 3,160x200, 4 colours.

Rom configuration selection

(See the memory map for more information)

Bit 2 is used to enable or disable the lower rom. The lower rom occupies memory addressess &0000-&3fff.

When it is enabled, reading from &0000-&3FFF will return data in the rom. When a value is written to &0000-&3FFF, it will be written to the ram at the same address as the rom.

When it is disabled, data read from &0000-&3FFF will return the data in the ram.

Similarly, bit 3 controls enabling or disabling of the upper rom.

The upper rom resides in memory addressess &C000-&FFFF. The upper rom can be basic or any other extra expansion roms which may be plugged into an expansion board. The upper rom is selected by writing the rom number to port &DFxx. (See upper rom selection for more information).

When the upper rom is enabled, reading from &c000-&ffff, will return data in the rom. When data is written to &c000-&FFFF, it will be written to the ram at the same address as the rom.

When the upper rom is disabled, and data is read from &c000-&ffff the data

returned will be the data in the ram.

Bit 4 controls the CPU interrupts. A CPU interrupt occurs every 300th of a second. When an interrupt occurs, the CPU stops executing the current program, and executes an interrupt program. When the interrupt program has been completed, it returns to the current program. (See interrupts for more

information).

Bit 4 is used to control the timing of interrupts.

When this bit is set to 1, any pending interrupts are cleared and the top bit of the scanline counter is cleared (this will suspend the next interrupt for a few scanlines).

When this bit is set to 0, the next interrupt will continue as normal.

See this document for more information.

Summary

Bit 7: }	Gate Array function	(1)

Bit 6: }				(0)

Bit 5:		Not used.

Bit 4:		CPU Interrupt enable/disable

1:disable next interrupt, 0:enable next interrupt

Bit 3:		Upper rom enable/disable. (Expansion roms)

1:disable upper rom, 0:enable upper rom

Bit 2:		Lower rom enable/disable. (Operating System rom)

1:disable lower rom, 0:enable lower rom

Bit 1: }	Screen Mode

Bit 0: }

Select ram configuration

This register is used to select the ram configuration.

The ram is split into banks of 64K. Each of these banks is split into blocks of 16K. Using this register, the user can select which bank they wish to use and how it is 'mixed' with the main ram.

The CPU is capable of accessing a maximum of 64K at one time. To access extra memory, blocks in the main ram (bank 1) can be exchanged with blocks in the extra ram. The number of blocks exchanged and the the blocks they change place with changes depending on the ram configuration used.

Ram configurations

Bits 2-0 select which ram configuration is to be used. When configuration 0 is selected (regardless of the bank selected), the CPU will only access the main ram. When configuration 2 is selected the CPU will only access the bank selected.

In the table above, 0-3 are the blocks in the main ram (bank 1), and 4-7 are the blocks of the selected bank, as they would normally be arranged shown below. Their new position, is the position they would take if that configuration was used.

It is not adviseable to use configuration 2 when the firmware is running, or the computer will crash and you will lose valuable data.

NOTE	Normally the CPC runs in ram configuration 0 (AMSDOS) and configuration 1  (CP/M).

The CRTC can only display graphics from the main ram. The Amstrad hardware was designed, so that it could not use the additional memory to store graphics.

Normal arrangement of blocks:

(Without any ram configuration selected).

Main Ram        Selected bank

(bank 1)

&FFFF   +---------------+---------------+

|               |               |

|       3       |       7       |

|               |               |

&C000   +---------------+---------------+

|               |               |

|       2       |       6       |

|               |               |

&8000   +---------------+---------------+

|               |               |

|       1       |       5       |

|               |               |

&4000   +---------------+---------------+

|               |               |

|       0       |       4       |

|               |               |

&0000   +---------------+---------------+

Bit 2   Bit 1   Bit 0   Configuration number

--------------------------------------------

0       0       0       0

0       0       1       1

0       1       0       2

0       1       1       3

1       0       0       4

1       0       1       5

1       1       0       6

1       1       1       7

Organization            Block accessed at memory address

&0000-&3FFF     &4000-&7fff     &8000-&bfff    &c000-&ffff

0          0               1               2               3

1          0               1               2               7

2          4               5               6               7

3          0               3               2               7

4          0               4               2               3

5          0               5               2               3

6          0               6               2               3

7          0               7               2               3

Bank selections

Bits 5-3 select the bank required as shown in the table below.

Bit 5	Bit 4	Bit 3	Bank Selected

0	0	0	Bank 2

0	0	1	Bank 3

0	1	0	Bank 4

0	1	1	Bank 5

1	0	0	Bank 6

1	0	1	Bank 7

1	1	0	Bank 8

1	1	1	Bank 9

NOTE:

If you have 64k, and you attempt to use a bank and ram configuration, there will be no effect. i.e. the arrangement of the 64K will not change.

If you only have 128k, regardless of the bank you choose, bank 2 will be selected.

If you have more than 128K, you will be able to choose the bank and the ram configuration and it will arrange the ram correctly.

Summary

Bit 7:}	Gate Array Function	(1)

Bit 6:}				(1)

Bit 5:}	Bank

Bit 4:}

Bit 3:}

Bit 2:}	Ram configuration

Bit 1:}

Bit 0:}

Palette conversion chart

Firmware colour       Colour          Hardware palette    Quick

number                                    number        Reference

0                  Black                   20          &54

1                  Blue                    4           &44

2                  Bright Blue             21          &55

3                  Red                     28          &5C

4                  Magenta                 24          &58

5                  Mauve                   29          &5D

6                  Bright Red              12          &4C

7                  Purple                  5           &45

8                  Bright Magenta          13          &4D

9                  Green                   22          &56

10                  Cyan                    6           &46

11                  Sky Blue                23          &57

12                  Yellow                  30          &5E

13                  White                   0           &40

14                  Pastel Blue             31          &5F

15                  Orange                  14          &4E

16                  Pink                    7           &47

17                  Pastel Magenta          15          &4F

18                  Bright Green            18          &52

19                  Sea Green               2           &42

20                  Bright Cyan             19          &53

21                  Lime                    26          &5A

22                  Pastel green            25          &59

23                  Pastel Cyan             27          &5B

24                  Bright Yellow           10          &4A

25                  Pastel Yellow           3           &43

26                  Bright White            11          &4B

This chart also gives a quick reference guide for programming the colours. The number is the colour number which can be sent directly, once the pen has been selected, to get the colour wanted.

Pallette R,G,B definitions for emulators

There are 27 colours which are generated from red, green and blue mixed in different quantities. There are 3 levels of red, 3 levels of green and 3 levels of blue, and these can be thought of as off/no colour, half-on/half-colour, and on/full-colour.

To display a CPC image you will need to use a analogue monitor with a composite sync.

This table shows the relationship between hardware colour number, colour name and RGB mixing. (* indicates a colour which is a copy of a documented colour in the hardware colour palette)

Hardware	Colour name		Red	Green	Blue

colour                                  (%)    (%)   (%)

number

0		White			50	50	50

1		*White			50	50	50

2		Sea Green		0	100	50

3		Pastel yellow		100	100	50

4		Blue			0	0	50

5		Purple			100	0	50

6		Cyan			0	50	50

7		Pink			100	50	50

8		*Purple		100	0	50

9		*Pastel yellow	100	100	50

10		Bright yellow		100	100	0

11		Bright White		100	100	100

12		Bright red		100	0	0

13		Bright magenta	100	0	100

14		Orange			100	50	0

15		Pastel magenta	100	50	100

16		*Blue			0	0	50

17		*Sea green		0	100	50

18		Bright green		0	100	0

19		Bright cyan		0	100	100

20		Black			0	0	0

21		Bright Blue		0	0	100

22		Green			0	50	0

23		Sky Blue		0	50	100

24		Magenta		50	0	50

25		Pastel Green		50	100	50

26		Lime			50	100	0

27		Pastel Cyan		50	100	100

28		Red			50	0	0

29		Mauve			50	0	100

30		Yellow			50	50	0

31		Pastel blue		50	50	100

RGB assignments for the software colours

This is simply a sidenote to illustrate a pattern in the RGB assignments of the software colours and to show how their value is calculated.

Firmware Colour	Colour name		R	G	B

Number						(%)   (%)    (%)

0			Black			0	0	0

1			Blue			0	0	50

2			Bright Blue		0	0	100

3			Red			50	0	0

4			Magenta		50	0	50

5			Mauve			50	0	100

6			Bright Red		100	0	0

7			Purple			100	0	50

8			Bright Magenta	100	0	100

9			Green			0	50	0

10			Cyan			0	50	50

11			Sky Blue		0	50	100

12			Yellow			50	50	0

13			White			50	50	50

14			Pastel Blue		50	50	100

15			Orange			100	50	0

16			Pink			100	50	50

17			Pastel Magenta	100	50	100

18			Bright Green		0	100	0

19			Sea Green		0	100	50

20			Bright Cyan		0	100	100

21			Lime			50	100	0

22			Pastel Green		50	100	50

23			Pastel Cyan		50	100	100

24			Bright Yellow		100	100	0

25			Pastel Yellow		100	100	50

26			Bright White		100	100	100

To calculate the colour value:

Red

0% => do not add anything

50% => add 3

100% => add 6

Green

0% => do not add anything

50% => add 9

100% => add 18

Blue

0% => do not add anything

50% => add 1

100% => add 2

Green Screen Colours

On a green screen (where all colours are shades of green), the colours (in the software/firmware colours), are in order of increasing intensity. So that black is very dark, and white is bright green, and colour 13 is a medium green. (Thanks to Mark Rison for suggesting I add this).

CPC Guide - Expansion ROM SelectionSelecting an Expansion ROM

To select an expansion rom, simply send the number of the expansion rom to port &DFxx. (An expansion rom may have a number from 0-255). Where a value of 0 will select BASIC, and a value of 7 will select AMSDOS.

However this process only selects the expansion rom. It does not allow the user to access any routines in the rom. To allow routines to be accessed the user must enable the upper rom using the Gate Array. (See programming the gate array to see how this is done).

A programming example is given to show how to select and disable an expansion rom.

Note port &DFxx is a write only port, reading will return a nonsense value.

Programming Examples

a) Selecting an expansion rom

To select rom 7. (AMSDOS)

ld bc,&7f00     ;Gate Array

ld a,%10000100  ;enable upper rom, disable lower rom, mode 0

out (c),a       ;send it

ld bc,&DF00     ;expansion rom select port

ld a,7          ;expansion rom wanted

out (c),a       ;select it

ret

b) Disabling an expansion rom

ld bc,&7f00     ;Gate Array

ld a,%10001100  ;upper and lower rom disabled, mode 0

out (c),a       ;send it

ret

CPC Guide - CRTC6845Cathode Ray Tube Controller (CRTC6845)

Programming the CRTC

The CRTC has 18 registers (0-17) which are used to control the screen, screen  cursor, and to sense a light pen/gun.

To access these registers, the CRTC has 3 control ports. One port to select a register, one port to write data into the register and one port to read data from the register.

Port address    Read/Write      Function/Use

&BCxx           W               Select CRTC register

&BDxx           W               Write data to selected CRTC register

&BFxx           R               Read data from selected CRTC register

NOTE: Due to differences in CRTC's, some of the information described here may not work. Where differences occur (that are known), they will be listed.

If you intend to use special effects, ensure they work with all CRTC types, or you specify which CRTC types the effect works on.

Thankyou to David Long (dave@kechb.demon.co.uk) for the following information.

The CRTC's can be destinguished by the manufacturers identification number printed on the top.

HD6845 (Hitachi) - Type 0

UM6845 (UMC) - Type 1 (but some can act like Type 0)

MC6845 (Motorola) - Type 2

Selecting a register

A register is selected by sending the register number (in the range 0-17) to port &BCxx. This register will remain selected until another is selected. This means you can write to the same register many times, without having to re-select the register each time.

Writing to a register

Once a register has been selected using the method above, the user can write data into it. Data can be written into the register by sending the data to port &BDxx.

If the selected register cannot be written to, there will be no effect.

Reading a register

Once a register has been selected, it can be read from. The data in the register can be read from port &BFxx.

If the register cannot be read, a value of &FF will be returned.

NOTE: On certain CRTC types this is not possible.

CRTC Status Register

Over to Mark Rison:

Here's a translation of what `Le Livre de l'Amstrad' says about the status

register of the 6845:

" In addition to the 18 registers described above, the S version of the 6845 has an extra register called the status register. This read-only register only has 3 active bits:

b7: 1 if the CRTC is accessible

b6: 0 if R16 and R17 have been read by the processor, 1 if the light-pen   signal has been detected

b5: Blanking indicator. 0 if the spot is being displayed, 1 if it is in   flyback

This register not being very useful except for light pens, it will no longer be discussed. "

And some comments from Pierre Guerrier:

"So this register contains one bit (if you are reading it, then the CRTC is accessible) one redundant bit (b5 is on the PIO also) and one flip-flop for firmwares too stupid to remember if they have sensed the light pen in the frame...

As it uses only the upper 3 bits, and the pointer register is only 5 bits, and they have the same position (same value of RS pin), it's probably two parts of one byte with different I/O directions.

The information seems to be relevant only to the HD6845 S and not other chips."

Register Functions - and normal values

Register        Read/   Register Title              Normal values

Number          Write                             PAL   SECAM   NTSC

0               W       Horizontal Total          63    63      63

1               W       Horizontal Displayed      40    40      40

2               W       Horizontal Sync Position  46    46      46

3               W       Sync Width                &8E   &8E     &8E

4               W       Vertical Total            38    38      31

5               W       Vertical Total Adjust     0     0       6

6               W       Vertical Displayed        25    25      25

7               W       Vertical Sync Position    30    30      27

8               W       Interlace and Skew        0     0       0

9               W       Maximum Raster Address    7     7       7

*10             W       Cursor Start Raster       X     X       X

*11             W       Cursor End Raster         X     X       X

12              R/W     Start Address (High)      X     X       X

13              R/W     Start Address (Low)       X     X       X

*14             R/W     Cursor (High)             X     X       X

*15             R/W     Cursor (Low)              X     X       X

16              R       Light Pen (High)          X     X       X

17              R       Light Pen (Low)           X     X       X

* - These registers are not used on the CPC.

R - Read only

W - Write only

R/W - Both Read and Write

X indicates the values of these registers may vary during normal operation.

Register 10,11,14 and 15 are not used on the CPC. The CPC doesnt use the hardware cursor of the CRTC. It uses a software cursor. I will not document these registers because they are not used.

Register 16 and 17 are used to sense the Light pen position. On CRTC2 it is not possible to read the CRTC registers, so I dont think it is possible to use a Light Pen on this CRTC. NOTE:

Experimenting with the registers can produce interesting effects. You will not do any harm by doing this. Just switch the computer off and then on, and everything will be ok again.

Register sizes

This defines the number of bits used in each register. It is mainly of use to emulator authors.

Register	Name	 			7 6 5 4 3 2 1 0

AR		Address Register		. . . x x x x x

0		Horizontal Total		x x x x x x x x

1		Horizontal Displayed	x x x x x x x x

2		Horizontal Sync Pos.	x x x x x x x x

3		Sync Width			. . . . x x x x

x x x x x x x x ('S')

4		Vertical Total		. x x x x x x x

5		Vertical Total Adjust	. . . x x x x x

6		Vertical Displayed		. x x x x x x x

7		Vertical Sync Pos		. x x x x x x x

8		Interlace and Skew		x x x x . . x x ('S')

. . . . . . x x

9		Max Raster Address		. . . x x x x x

10		Cursor Start raster		. x x x x x x x

11		Cursor Stop raster		. . . x x x x x

12		Start Address (H)		. . x x x x x x (can read on 'S' only)

13		Start Address (L)		x x x x x x x x

14		Cursor (H)			. . x x x x x x

15		Cursor (L)			x x x x x x x x

16		Light Pen (H)			. . x x x x x x

17		Light Pen (L)			x x x x x x x x

Note:

AR is the address register, or the register of the CRTC to use.

"." means this bit is not used, and x means the bit is used.

Register usage

Certain registers can be used for various things. Here is a list of registers and their common usage.

Register 0:	Horizontal splitting (changing of screen address/mode accross line).

Register 1	Width of screen

Register 2:	a) Horizontal positioning of screen

b) Screen distorting (when changed as beam scans screen)

Register 3:	Smooth left-right hardware scroll

Register 4:	Rupture/Vertical Splitting (allowing screen address to be changed on any line

Register 5:	Very smooth vertical scrolling

Register 6:	Height of screen

Register 7:	Setting up the rupture/splitting and vertical position of screen

Register 8:	-

Register 9:	-

Register 10:	NOT USED

Register 11:	NOT USED

Register 12:	Setting screen address + Overscan + Hardware scrolling

Register 13:	Hardware scrolling

Register 14:	NOT USED

Register 15:	NOT USED

Register 16:	Light pen address

Register 17:	Light pen address

Register 1 - Horizontal Displayed

This register is used to define the width of the screen. The width is calculated in mode 1 character sized steps.

i.e. the number of characters accross the screen in mode 1, is the same as the number programmed into this register.

Therefore, the width of the screen may be increased or decreased, to get the desired screen width.

If the value programmed is increased, the width of the screen will increase, if the value programmed is decreased, the width of the screen will decrease.

The normal width of the screen is 40 characters. The width of the monitor screen is 50 characters.

(The number of bytes displayed on a line=value programmed to register 1*2)

Register 2 - Horizontal Sync Position

This register defines the horizontal position of the screen within the monitor screen.

If the programmed value is increased, the screen will shift to the left, if the programmed value is decreased, the screen will shift to the right.

So, using this register, the user can put the screen to any horizontal position they want.

Register 5 - Vertical total adjust

This register allows the vertical position of the screen to be positioned to any pixel offset from the current vertical position defined in register 7.

If the value is increased, the picture will shift down by a pixel line, if it is decreased the picture will shift up by one pixel line.

This register used in conjunction with register 12 and 13, can be used to get perfect pixel by pixel vertical scrolling.

Register 6 - Vertical displayed

This register defines the height of the screen in character sized units.

If the value programmed is increased, the height of the screen will increase, if the value programmed is decreased, the height of the screen will decrease.

The normal value programmed is 25. The height of the monitor screen is 35 characters.

NOTE:

We can find the height of the screen in pixel lines, by multiplying this by the value in register 9 plus 1.

Height of screen = register 6 value*(register 9 value+1)

Register 7 - Vertical sync position

This register defines the vertical position of the screen within the monitor screen.

If the programmed value is increased, the screen will shift upwards, if the programmed value is decreased, the screen will shift downwards.

Therefore, using this register and register 2, it is possible to position the picture to any position on the monitor screen.

Register 12 - Start Address (High)

This register specifys where the screen will start in memory and how much memory is to be used to display the screen. Together with register 13, it allows the user to scroll the screen.

Bit 5 and Bit 4 determine which 16k block the screen is located, in the following way:

Bit 5	Bit 4	Memory block

0	0	&0000-&3FFF

0	1	&4000-&8000

1	0	&8000-&C000

1	1	&C000-&FFFF

In this way, the screen can be positioned almost anywhere in memory.

The screen address is only changed when a VSYNC occurs. So, if you attempt to change the screen address, as the electron beam is drawing the screen, only one screen will be shown.

(To change the screen address more than once a special technic called "rupture" or "hardware splitting" is needed. This special effect was developed by many coders individually (credits go to Logon System, NWC, Paul Shirley (Mission Genocide).)

The CRTC cannot access the extra memory, so it will only display screens which are located in memory blocks 0-3 only, regardless of any ram configuration. (i.e. it will only display screens in the first bank of 64k). If you change the ram configuration, so that a different 16k block is switched into the area which holds the screen there will be no effect on the picture.

Bit 3 and 2 determine how much memory the CRTC can use for the screen. The CRTC is capable of displaying 16k or 32k screens. Bits 3 and 2 are defined in the following way:

Bit 3	Bit 2	Screen memory

0	0	16k

1	0	16k

0	1	16k

1	1	32k

When the CRTC is accessing 32k, you can expand the screen size so that you can have a picture which covers the borders! (overscan)

When 32k is selected, the first and second screens occupy the following range of addressess:

First screen	Second screen

&0000-&3FFF	&4000-&7FFF

&4000-&7FFF	&8000-&BFFF

&8000-&BFFF	&C000-&FFFF

&C000-&FFFF	&0000-&3FFF

Bit 1 and 0 are used with register 13 to do hardware scrolling. Register 13 contains the lower 8 bits of the offset and bit 1 and 0 contain the upper 2 bits of the offset. The offset is 10 bits and defines the position of the screen within the 16k block specified in bits 5 and 4.

So, by changing the value contained in register 13, and bits 1 and 0 of register 12, the screen can be scrolled. This effect is called hardware scrolling.

Summary:

Bit 7:	Not used.

Bit 6:	Not used.

Bit 5: }	Memory block of start of screen

Bit 4: }

Bit 3: }	Screen memory

Bit 2: }

Bit 1: }	Offset (upper 2 bits of offset value - lower 8 bits defined by register 13).

Bit 0: }

Register 13 - Start Address (Low)

This register defines the lower 8 bits of the offset within the selected 16k

block. This register is used in conjunction with register 12 to perform hardware

scrolling.

[code examples cut]

CPC Guide - Chips - INTEL 8255PPIINTEL 8255 Interface chip documents

The 8255 is a general purpose input output chip. It is connected to the PSG, keyboard and cassette recorder.

Programming the 8255

The 8255 has 3 8-bit ports (A,B and C), each of which can have a different I/O Status (i.e. input or output). Port C can also be programmed to operate in two halves. (As two separate 4-bit ports).

There are 3 basic modes of operation under which the ports can function.

Mode 0 - Basic Input/Output

Mode 1 - Strobed Input/Output **NOT USED BY CPC**

Mode 2 - Bi-Directional Bus **NOT USED BY CPC**

In some of these modes port C is used as a control/status port for port A or B. It can be used to confirm when data transfer may take place, and reflect any other flags. The 8255 PPI is therefore supplied with the added option for the user to set or reset any individual bits in port C.

The I/O status, mode of operation and bit setting is defined by the 8255 PPI control byte. (The control byte is accessed using port &F7xx.(8255 Control port))

The ports may be accessed separately by the CPU.

Port A is accessed using port &F4xx.

Port B is accessed using port &F5xx.

Port C is accessed using port &F6xx.

NOTE:

Mode 0 operation:

Mode 0 operation does not require other ports to be used to control data transfer. Data may only be sent to a port if the port is defined as output, and data may only be read from a port when the port is set to input.

Using the 8255 Ports

When writing to a port:

1) Make sure the port is operating as output. (This is done by sending a control byte to port &F7xx).

2) Write the data to the port.

3) Return the port to it's original I/O Status.

When reading from a port:

1) Make sure the port is operating as input. (This is done by sending a control byte to port &F7xx).

2) Read the data from the port.

3) Return the port to it's original I/O Status.

8255 PPI Control Byte

This byte controls the Input/Output (I/O) status and operating mode of each port.

It also has a secondary function which allows the user to set or reset a particular bit of port C.

To send a control byte to the 8255, simply sent the byte to port &F7xx. (NOTE: The control byte can only be written, it cannot be read.)

The function to be executed by the 8255 is dependant on the state of bit 7.

If bit 7 is 1, the remaining bits are used to define the I/O status, and operating mode of each port.

If bit 7 is 0, the remaining bits are used to determine what bit is set (set to 1) or reset (set to 0) of port C.

Configuration control byte

When bit 7 is set to 1, the command byte operates in the following way:

Bit 7: 		Control byte function (1=Configuration control byte)

Bit 6: }	Operating Mode (Port A and Port C upper)

Bit 5: }

Bit 4:		Port A I/O Status

0: Output, 1: Input

Bit 3:		Port C (upper) I/O Status

0: Output, 1: Input

Bit 2:		Operating Mode selection (Port B and Port C lower)

Bit 1:		Port B I/O Status

0: Output, 1: Input

Bit 0:		Port C (lower) I/O Status

0: Output, 1: Input

Port A and Port C (upper) Operating mode

Bit 6	Bit 5	Mode selection

0	0	Mode 0

0	1	Mode 1

1	0	Mode 2

1	1	Mode 2

Port B and Port C (lower) operating mode

Bit 2		Mode Selection

0		Mode 0

1		Mode 1

Bit Set/Reset control byte

When bit 7 is set to 0, the control byte operates in the following way:

Bit 7:	Control byte function (0=Bit Set/Reset control byte)

Bit 6: }	Not used. (send 0)

Bit 5: }

Bit 4: }

Bit 3: }	Bit to change.

Bit 2: }

Bit 1: }

Bit 0:		Bit operation

1: Set bit (make bit=1), 0: Reset bit (make bit=0)

Table to show how bit to change is coded:

Bit 3	Bit 2	Bit 1	Bit selected

0	0	0	Bit 0

0	0	1	Bit 1

0	1	0	Bit 2

0	1	1	Bit 3

1	0	0	Bit 4

1	0	1	Bit 5

1	1	0	Bit 6

1	1	1	Bit 7

Port Usage

For each port, the port address (at which the port can be accessed) it's normal I/O Status (unless changed by the user), it's operating mode (unless changed by the user) are given.

For more information on a particular function of each port, see the appropiate part of the CPC Guide.

NOTE: If you are using the firmware, always return the operating modes and I/O status of the ports used to their status below, otherwise the firmware may crash. (The firmware expects the operating modes to be the same as given below).

Port A (Port address:&F4xx) (I/O Status: Output) (Operating mode: 0)

Bit 7: }	PSG data

Bit 6: }

Bit 5: }

Bit 4: }

Bit 3: }

Bit 2: }

Bit 1: }

Bit 0: }

Port B (Port address: &F5xx) (I/O Status: Input) (Operating mode: 0)

Bit 7:		Cassette read data

Bit 6:		Printer configuration

1: Not-ready,  0: Ready

Bit 5:		Expansion peripheral detect

1: Expansion peripheral present, 0: Not present

Bit 4:		Screen refresh frequency (defined by hardware)

1: 50Hz    0: 60Hz

Bit 3: }	Computer name on power-up.

Bit 2: }

Bit 1: }

Bit 0:		VSYNC status (Frame flyback status)

1: VSYNC occured, 0: no VSYNC

Table showing Computer name on power-up:

Bit 3	Bit 2	Bit 1	Computer name

0	0	0	Isp

0	0	1	Triumph

0	1	0	Saisho

0	1	1	Solavox

1	0	0	Awa

1	0	1	Schneider

1	1	0	Orion

1	1	1	Amstrad

Port C (Port address: &F6xx) (I/O Status: Output) (Operating mode: 0)

Bit 7: }	PSG control

Bit 6: }

Bit 5:		Cassette write data

Bit 4:		Cassette motor control

1: Turn motor on, 0: Turn motor off

Bit 3: }	Keyboard line to be scanned (0-10)

Bit 2: }

Bit 1: }

Bit 0: }

[code examples cut]

CPC Guide - AY-3-8912 PSGAY-3-8912 PSG Documentation

Controlling the PSG

The PSG has three independant tone channels and one noise channel. These can be mixed to produce a variety of sounds.

Just listen to a Soundtrakker or Protracker/Digitraker music to see what the PSG is capable of.

To control the tone, volume, noise and enveloping of the sound the PSG has 15 registers.

These registers are accessed by bits 7 and 6 of port &F6xx and data written to/read from port &F4xx of the 8255 PPI.

The PSG also has a secondary function and that is to scan the keyboard.

** Data written to ports &F6xx and &F4xx remain until new data is programmed. **

Register selection

Before reading or writing to the PSG, the appropiate register must be selected.

This is done by putting the register number (0-14) into port &F4xx, and setting bits 7 and 6 of port &F6xx to 1. The register will now be selected and the user can now read or write a value to it. Finally, the PSG must be put into an inactive state by setting bit 7 and 6 to 0 of port &F6xx.

This is necessary, otherwise if the register select command was still in operation, and a byte was sent to port &F4xx, it would use this and change the data in the last register selected. (see below)

Writing to a PSG register

To write data to the PSG, the user must put the data in port &F4xx, and then set bit 7 to 1 and bit 6 to 0 of port &F6xx. The data will be written into the register. Finally, the PSG must be put into an inactive state by setting bit 7 and 6 to 0 of port &F6xx.

Reading from a PSG register

To read data from the PSG, the user must put port &F4xx into input mode (See the section on the 8255 PPI to find out how to do this), then the user must set bit 7 to 0 and bit 6 to 1 of port &F6xx. The data in the PSG register will be supplied to port &F4xx, and this can now be read by the user. When it has been read, the PSG must be put in an inactive state, by setting bits 7 and 6 to 0 of port &F6xx, and port &F4xx must be returned to output mode.

Note: It is necessary to change port &F4xx to input mode otherwise the data will not be read from the PSG register, and it is also necessary to return port &F4xx to output mode, otherwise when the 'select register' or 'write to register' commands are specified they will not work.

In between sets of data sent to control the PSG, the PSG should be set to inactive, otherwise unexpected sounds could be generated becuause the function would still be active.

Summary:

Port &F6xx

| Bit 7 | Bit 6 | Function |
| --- | --- | --- |
| 0 | 0 | INACTIVE The PSG is put into inactive state. Previous function cancelled |
| 0 | 1 | READ from PSG register. The user wants to read the value contained in the currently selected PSG register. The value contained in the currently selected register is supplied to port &F4xx. |
| 1 | 0 | WRITE to PSG register. The user wants to write a value to the currently selected register. The value written to port &F4xx is the value which is to be written into the currently selected register. |
| 1 | 1 | SPECIFY PSG register. The user wants to select which register is to be read from/written to. The value witten to port &F4xx contains the register number, where register number must be in the range 0-14. The register selected will remain active, until another is selected. |

Register Functions

This is a list of the PSG registers. They are described in more detail in the next section.

Register        Function

0            Channel A tone period {fine tune}

1            Channel A tone period {coarse tune}

2            Channel B tone period {fine tune}

3            Channel B tone period {coarse tune}

4            Channel C tone period {fine tune}

5            Channel C tone period {coarse tune}

6            Noise generator

7            Mixer control

8            Channel A volume/Hardware envelope enable

9            Channel B volume/Hardware envelope enable

10            Channel C volume/Hardware envelope enable

11            Hardware envelope period {fine tune}

12            Hardware envelope period {coarse tune}

13            Hardware envelope shape

14            Port A data (Used for reading the keyboard)

Register Functions

[sound registers cut]

Register 9

Bit 6      Port A I/O Control

This bit determines in which state (e.g. input or output) the port is to be operated. If this bit is set to 1, port A will be used for output. If this bit is set to 0, port A will be used for input.

Bit 7:  Not used.

Bit 6:  Port A control.

Bit 5:  Channel C enable/disable noise.

Bit 4:  Channel B enable/disable noise.

Bit 3:  Channel A enable/disable noise.

Bit 2:  Channel C enable/disable tone.

Bit 1:  Channel B enable/disable tone.

Bit 0:  Channel A enable/disable tone.

Register 14 - Port A

This register holds any data which is written to/read from port A of the PSG.

In the CPC, port A is used for a special function. It is used for scanning the keyboard. It must only be read, writing values to this port may have an unpredicted result.

To read a value from this port, it must be put into input mode by setting bit 6 of register 7 (mixer control) of the PSG to 0. Usually, this is not necessary as port A always operates as input, unless it has been changed by the user.

When this is done, port &F4xx of the 8255 must be programmed to operate in input mode. The data may then be read from register 14. Remember to program port &F4xx to operate in output mode again.

To write a value to this port, it must be put into output mode by setting bit 6 of register 7 (mixer control) of the PSG to 1. The user can then write the byte into the register.

To see how the keyboard is controlled via this port, please see the section on reading the keyboard.

Summary:

Bit 7:  } Port A data

Bit 6:  }

Bit 5:  }

Bit 4:  }

Bit 3:  }

Bit 2:  }

Bit 1:  }

Bit 0:  }

CPC Guide - Reading the keyboard and Joysticks

Scanning the Keybord and Joysicks

The keyboard and joysticks are scanned using the PSG and the 8255 PPI. It is necessary that you understand how to use these before you continue to read this part.

The keys on the keyboard and the joysticks are arranged in an 8 by 10 matrix. The matrix consists of 10 bytes. (each of which is called a line). Each of these lines has a bit set aside to a particular key. (Therefore allowing 80 possible keys to be read)

When the keyboard is read, a particular bit is 0 or 1 depending on whether the key has been pressed. If the value is 0, the key has been pressed, if the value is 1, the key has not been pressed.

The joysticks are also scanned via the matrix. Joystick 0 occupies it's own space in the matrix, but joystick 1 occupies space already used for the keyboard.

It is entirely possible to simulate the state of joystick 1 by pressing the appropiate keys on the keyboard.

Keyboard clash

There is a peculiarity when scanning the keyboard. If you press three keys in a 2 by 2 block together, you will get the other key in the block. i.e. if you press J,F and B you will get N, and similarly if you press J,K and L you will get H.

NOTE: This effect also occurs at the edges. i.e. if you press V,X and 4 you will get 6. And if you press Down and left on the joystick and right arrow key is pressed, you will get Down arrow produced.

Another thing, this affect is not confined to keys in a 2 by 2 block together. i.e. if you press W and S and L you will get I. and if you press Q,A and P you will get : .

This effect doesn't occur if you press two keys in a vertical next to each other and two in a horizontal next to each other. i.e. if you press W and S and J, you will not get K.

These effects cause the keyboard clash which effect many two player games. It doesn't usually effect one player games, so as long as the keys do not clash to give a key wanted, (i.e. pressing Q,A and P gives :, but since most games only use Q,A,O,P and SPACE for control, it doesn't matter that : also occurs.), there is no need to worry about keyboard clash.

The only solution is to choose keys which will not clash.

The keyboard matrix

The position of each key in the keyboard matrix is shown in the diagram below.

Bit                          Keyboard Line

0     1     2     3     4     5     6     7     8     9

--+--------------------------------------------------------------------

7 |  f.    f0    Ctrl  >     <     Space V     X     Z     Del

|                    .     ,

--+--------------------------------------------------------------------

6 |  Enter f2    `     ?     M     N     B     C     Caps  Spare

|              \     /                             Lock

--+--------------------------------------------------------------------

5 |  f3    f1    Shift *     K     J     F     D     A     Joy 0

|                    :                                   Fire 1

--+--------------------------------------------------------------------

4 |  f6    f5    f4    +     L     H     G     S     Tab   Joy 0

|                    ;                                   Fire 2

--+--------------------------------------------------------------------

3 |  f9    f8    }     P     I     Y     T     W     Q     Joy 0

|              ]                                         Right

--+--------------------------------------------------------------------

2 |  Curs  f7    Ret-  |     O     U     R     E     Esc   Joy 0

|  Down         urn  @                                   Left

--+--------------------------------------------------------------------

1 |  Curs  Copy  {     =     )     '     %     #     "     Joy 0

|  Right       [     -     9     7     5     3     2     Down

--+--------------------------------------------------------------------

0 |  Curs  Curs  Clr   £     _     (     &     $     !     Joy 0

|  Up    Left        ^     0     8     6     4     1     Up

--+--------------------------------------------------------------------

Note:

1) Joystick 1 is mapped onto line 6. When checking Line 6 becomes:

Bit 7: Spare

Bit 6: Spare

Bit 5: Joy 1 Fire 1

Bit 4: Joy 1 Fire 2

Bit 3: Joy 1 Right

Bit 2: Joy 1 Left

Bit 1: Joy 1 Down

Bit 0: Joy 1 Up

2) Bit 6 on the joystick lines (line 9 and 6), may be used as the middle button on an AMX compatible mouse.

3) Those keys prefixed with an f are located on the keypad. (464 users)

4) For CPC464 users, ENTER refers to the small Enter key, and RETURN refers to the large enter key.

5) The hardware does allow keyboard lines 11,12,13 and 14 to be selected. After testing on a real CPC, it is found that these never change, they always return &FF. 6) Finally, DON'T FORGET THE DREADED KEYBOARD CLASH with combinations of keys!

Reading the keyboard and Joysticks

As mentioned before the keyboard and joysticks are scanned using the PSG and the 8255 PPI.

The PSG is used to read the keyboard line (using port A of the PSG; register 14) and port &F6xx of the 8255 PPI.

The keyboard line (in the range 0-10) is put into port &F6xx of the PPI. The data on the keyboard line will now be present in register 14 of the PSG.

Before the data can be read we must make sure Port A of the PSG is acting in input mode.

This is done by setting bit 6 of the mixer register (of the PSG) to 0. Normally, port A is programmed for input unless it has been changed by the user. If it has not been changed it is not necessary to put this into the keyboard scanning routine.

Once this is done, we can select register 14 and read the value from it. See programming the PSG for more information.

(Note: when you are sending control information to the PSG using port &F6xx it is important to change bits 6 and 7 only, otherwise the wrong keyboard line will be read).

This process is repeated until all keyboard lines are read. This allows you the flexability to read one,two or all of the keyboard lines.

When all reading has been done, port &F4xx of the 8255 PPI must be returned to output mode (so that the PSG can be used), and the PSG must be sent inactive.

[code example cut]

CPC Guide - Controlling the PrinterPrinter Control

The printer is controlled using port &EFxx and port &F5xx of the 8255 PPI.

The ready status of the printer is controlled by checking bit 6 of port &F5xx. If the printer is on-line, this bit will be set to 1, if it is offline this bit will be set to 0.

Data may be sent to the printer by using port &EFxx. (This port cannot be read.)

The user may only send 7 bit values. The 8th bit is used as the strobe. This is like a flag telling the printer that data is ready, it is a peak, as shown in the diagram below:

The strobe looks like this:

+---+

|   |

_______|   |_________

The user must send a 0 on the strobe bit, then a 1 and then a 0. The printer will then take the byte and print it. (At all times the data that you want the printer to take and print, should be present on bits 7-0.)

NOTE:

When the computer is switched on or reset, the strobe bit is set to 0.

Summary

Bit 7   Centronics Strobe

1: Activate strobe

Bit 6   Data bit 6

Bit 5   Data bit 5

Bit 4   Data bit 4

Bit 3   Data bit 3

Bit 2   Data bit 2

Bit 1   Data bit 1

Bit 0   Data bit 0

[code example cut]

CPC Guide - Z80 Interrupt handling on the CPCZ80 Interrupt handling on the CPC

General Interrupt information

This document explains how the interrupts are handled in the CPC.

The Z80 has the provision for 2 interrupt types; Maskable interrupts, and Non-Maskable Interrupts (NMI). When the device wants to cause a maskable interrupt a low should be applied to INT pin, for non-maskable interrupts a low to NMI pin.

Maskable Interrupts

This interrupt type can be enabled or disabled using a Z80 opcode (EI will enable maskable interrupts and DI will disable maskable interrupts).

There are 3 modes of maskable interrupt depending on what the user wants. The mode is also specified with a Z80 opcode (IM0, IM1 or IM2). Each mode is slightly different in the way that it performs the interrupt.

See this document for more information about how each maskable interrupt works.

This interrupt is used in the CPC. The rate of interrupts is fixed, and is controlled by the Gate Array.

Non-Maskable Interrupts NMI

This type of interrupt cannot be disabled and it has the highest priority. This type is not actually used in the CPC and so it is not supported, but the signal can be found on the expansion port if the user wishes to use it. (The Multiface 2 uses the NMI interrupt to freeze the current program).

When the Z80 recieves a NMI interrupt it will do the following:

Push current program counter onto the stack

Jump to &0066 in memory.

In all cases the Z80 will send an acknowledgement when it is ready to continue with the interrupt. (In Interrupt Mode 2, the device can use the acknowledgement to put the interrupt vector onto the z80 bus).

So for maskable interrupts, the process is:

Apply low to INT pin

Z80 will finish the current command, and then depending on the interrupt mode,

it may put the return address onto the stack.

When the z80 is ready, it will acknowledge the interrupt by M1 and IORQ being

low

Interrupts in the CPC

The following information may not be exactly correct, but this is my current understanding

The section above described how the z80 handles interrupts, this section will attempt to describe how interrupts are performed in the CPC.

In the CPC, the Gate Array is responsible for generating interrupt requests, and ensuring that interrupts do not occur to close together (especially if interrupts have been disabled and are enabled again just before another interrupt request).

The Gate Array contains a 6-bit counter. This is incremented after each HSYNC signal from the CRTC. So in a way it is a scanline counter. The counter is used to determine when interrupts will occur.

Basically, when the counter reaches 52, the Gate Array will send the INT request to the Z80, and then wait for the z80 to acknowledge. (If the z80 has interrupts enabled, it will respond immediatly, otherwise there might be sometime before it does). The gate array will continue to count scanlines and increment the counter during this time.

So each interrupt effectively splits the screen into 52 scanline tall blocks. And there are (39*8) total lines, so this gives 6 interrupt regions or "interrupt blocks".

To ensure the interrupts are at the same place every screen refresh, the gate array will synchronise with the VSYNC signal from the CRTC.

It is possible that the current interrupt request is not acknowledged, because the Z80 has disabled interrupts. The gate array will continue to wait until there is acknowledgement. However, since the counter still continues, it is possible that the interrupts are enabled just before a new interrupt request is made. In this case, the interrupts would be very close together and could cause problems (especially if interrupt handling routine takes a while). So, the gate array clears the top bit of the counter. This will effectively ensure that the new interrupt which is about to occur is suspended for a short amount of time so

the interrupt handling routine has time to execute. The top bit of the counter can also be cleared by using a bit in one of the gate array registers, so you can control the position of an interrupt if you require.

If you had some code which used some time, and it's operation would be spoiled by an interrupt, then you could use this function of the gate array register to suspend the interrupt for a few lines until your program is able to finish.

Notes

The Gate Array uses the VSYNC to synchronise interrupts, so that they occur at the same time for each screen refresh.

It waits for the start of the VSYNC, and now it will be synchronised, it will then wait for another 2 lines (to give time for software to recognise the VSYNC) before the first interrupt is requested.

The Gate Array will clear the scanline counter at this time.

When the Gate Array sends a INT request to the z80 it will wait for it to be acknowledged. (This may be almost immediate if interrupts are enabled, or a long time if interrupts are disabled).

During this time, the INT pin of the z80 is low, and the Gate Array will continue to count lines.

If interrupts are acknowledged, the gate array will clear the interrupt by putting INT high.

If interrupts are disabled, it will wait until interrupts are enabled (as soon as they are enabled, the z80 will notice the int request and acknowledge it).

If interrupts are enabled, the z80 will start to prepare for the interrupt and then send acknowledgement to the gate array which will clear the interrupt.

The Gate Array has an "interrupt disable bit". In reality, this performs two functions. It will clear any pending interrupt, and it will also clear the top bit of the counter (this has the effect of making sure that two interrupts do not occur close together).

When interrupts are enabled

If interrupts are enabled and a interrupt request is sent, and the counter>52 when the acknowledge comes back, then the top bit of the scanline counter is cleared.

(This is done to ensure the next interrupt is not to close to the one being executed).

A interrupt will be sent from the Gate Array when the scanline counter is to equal to 52 and the counter will then be cleared.

Quote from the firmware manual, Appendix 12.1:

"The processor interrupt pin is driven by a flip-flop in the VGA. This FF is set during every vertical flyback and every 52 scan lines thereafter until the next VF. The interrupt is arranged to occur approx. 2 scans (125 us) into the 8 scan (500 us) VF signal. The interrupt latch is cleared by the processor acknowledging the interrupt, or explicitly, using a software command. The top bit of the divide by 52 counter is also cleared when the processor acknowledges an interrupt occurring after this counter has overflowed. This allows the

interrupt system to be expanded."

CPC Guide - Interrupts revisitedInterrupts Revisited - Interrupts 2

Article supplied by Brice Rive

The main reason for the interrupt delay mechanism is to allow for discrimination between Gate Array interrupts and external interrupts (from the extension port):

The mechanism implemented by Locomotive to distinguish between GA Ints and external Ints is as follows:

When the Z80 is in enabled interrupts and notices a low level on the INT line, it disables interrupts and jumps to address 0x0038. At the same time, it puts lines IORQ and M1 to 0. This is recognized as an interrupt acknoledge by the GA, which in turn sets its output INT line back to 1.

The interrupt routine starts with:

0038    JP B941

B941    DI

EX AF,AF'

JR C,B978

EXX

LD A,C

SCF

--->   EI

EX AF,AF'

DI

...

The Z80, shortly after being interrupted will briefly re-enable interrupts (EI).

In the case of a GA Int, this has no effect because the GA has already reset its INT line to 1. In the case of an external Int however, the INT line will still be at 0, and the code will be re-interrupted. This situation will be detected by the fact that AF' CARRY flag was set, and the interrupt routine will jump to B978 where the external interrupt handling code is located.

So the key to this mechanism is that, when the Z80 executes EI, the INT line is at 1 for a GA Int and at 0 for an external Int. This means that external interrupts must be long enough, and that the GA interrupt line must be low by thsat time.

However, without the interrupt delay mechanism, there could be a situation where the GA's scan counter reaches 52 again just when the Z80 executes EI (if the Z80 ignored the first interrupt long enough by running in interrupt disabled mode). This would be wrongly interpreted as an external interrupt and ... BOUM!

To prevent that, when the GA detects the interrupt acknowledge from the Z80, it also resets the high bit of its scan counter.

This is a kludgy but efficient way to avoid the race condition described above.

CPC Guide - Graphics formatGraphics

Mode 0

In mode 0, there are 2 pixels per byte. Each pixel can be one of 16 colours.  (The pixel is 4 times the size of a mode 2 pixel).

The pixels are numbered p1 and p2. p1 is the left pixel, p2 is the right pixel. p1 and p2 contain a number in the range 0..15.

Screen Byte

+--+--+--+--+--+--+--+--+

|7.|6 |5.|4 |3.|2 |1.|0 |

|..|  |..|  |..|  |..|  |

+--+--+--+--+--+--+--+--+

^     ^     ^     ^

|     |     |     |

|  ___|     |     |

|  |  ______|     |

|__|__|___        |

|  |  |        |

__|__|__|________|

|  |  |  |

|  |  |  |

+--+--+--+--+

|3 |2 |1 |0 |

|  |  |  |  |

+--+--+--+--+

p1

Byte

+--+--+--+--+--+--+--+--+

|7 |6,|5 |4,|3 |2,|1 |0,|

|  |,,|  |,,|  |,,|  |,,|

+--+--+--+--+--+--+--+--+

^     ^     ^     ^

|     |     |     |

|  ___|     |     |

|  |  ______|     |

|__|__|___        |

|  |  |        |

__|__|__|________|

|  |  |  |

|  |  |  |

+--+--+--+--+

|3 |2 |1 |0 |

|  |  |  |  |

+--+--+--+--+

p2

Mode 1

In mode 1, there are 4 pixels per byte. Each pixel can be one of 4 colours. (The pixel is 2 times the size of a mode 2 pixel).

The pixels are numbered p1, p2, p3 and p4. p1 is the leftmost pixel, and p4 is the rightmost pixel. p1,p2,p3,p4 contain a number in the range 0..3.

Note: In this mode the pixels are square.

Screen Byte

+--+--+--+--+--+--+--+--+

|7.|6 |5 |4 |3.|2 |1 |0 |

|..|  |  |  |..|  |  |  |

+--+--+--+--+--+--+--+--+

^           ^

|           |

|           |

|__         |

|        |

__|________|

|  |

+--+--+ p1

|1 |0 |

|  |  |

+--+--+

Screen Byte

+--+--+--+--+--+--+--+--+

|7 |6,|5 |4 |3 |2,|1 |0 |

|  |,,|  |  |  |,,|  |  |

+--+--+--+--+--+--+--+--+

^           ^

|           |

|           |

|__         |

|        |

__|________|

|  |

+--+--+ p2

|1 |0 |

|  |  |

+--+--+

Screen Byte

+--+--+--+--+--+--+--+--+

|7 |6 |5-|4 |3 |2 |1-|0 |

|  |  |--|  |  |  |--|  |

+--+--+--+--+--+--+--+--+

^           ^

|           |

|           |

|__         |

|        |

__|________|

|  |

+--+--+ p3

|1 |0 |

|  |  |

+--+--+

Screen Byte

+--+--+--+--+--+--+--+--+

|7 |6 |5 |4*|3 |2 |1 |0*|

|  |  |  |**|  |  |  |**|

+--+--+--+--+--+--+--+--+

^           ^

|           |

|           |

|__         |

|        |

__|________|

|  |

+--+--+ p4

|1 |0 |

|  |  |

+--+--+

Mode 2

In mode 2, there are 8 pixels per byte. Each pixel can be one of 2 colours. Each bit of the byte is a pixel. If the bit value is 0, then pen 0 is used, if the bit value is 1, then pen 1 is used.

The pixels are numbered p1, p2, p3, p4, p5, p6, p7 and p8. p1 is the leftmost pixel and p8 is the rightmost pixel. p1,p2,p3,p4,p5,p6,p7,p8 contain 0 or 1.

CPC Guide - Chips - Floppy Disc ControllerFloppy Disk Controller

FDC ports

The Floppy disk controller (NEC uPD765-A or INTEL 8272), is controlled using two ports.

&FB7E - Main Status Register. Used for data transfer. See below.

&FB7F - Data Register.

When transfer is from CPU to FDC, you poke the values for the FDC to this port.

When transfer is from FDC to CPU, the data returned by the FDC can be read from this port.

There is also one other port (&FA7E). This port controls the Disk Drive Motor.

The Drive motor MUST be switched on, before any commands are sent to the FDC.

FDC pins

_______ _______

RESET =|1      U     40|= Vcc

/RD   =|               |= /RW/SEEK

/WR   =|               |= LCT/DIR

/CS   =|               |= FR/STP

A0    =|               |= HDL

DB0   =|               |= RDY

DB1   =|               |= WP/TS

DB2   =|8            33|= FLT/TR0

DB3   =|               |= PS0

DB4   =|               |= PS1

DB5   =|   uPD765-A    |= WDA

DB6   =|               |= US0

DB7   =|               |= US1

DRQ   =|               |= HD

/DACK  =|               |= MFM

TC    =|16           25|= WE

IDX   =|               |= VCO

INT   =|               |= RD

CLK   =|               |= RDW

GND   =|20___________21|= WCK

Disk Drive Motor Control

The disc drive motor is controlled by OUTing values to port &FA7E.

Any odd value (bit 0 = 1), will switch the drive motor ON.

Any even value (bit 0 = 0), will switch the drive motor OFF.

This is a single bit port. The value of bit 0 is important, all other bits are ignored.

When the drive is switched ON, the disk starts to pick up speed. A little time is needed before the disc is rotating at FULL speed. Many drives are different, and the time needed before the motor has picked up full speed will be different.

If a command is sent to the FDC when the drive has not picked up FULL speed, then there will be lots of errors.

Some drives will take less time for their motors to be rotating at full speed. The long pause is needed for compatibility with all drives.

So, as soon as the drive has been switched off, pause before any commands are sent.

The drive motor can be switched off immediatly, there is no need to wait for it to slow down. Although, it might make more reliable reading and writing if some time is allowed for it to slow down. But, I have not noticed any difference.

Bit 7           ** ignored **

Bit 6           ** ignored **

Bit 5           ** ignored **

Bit 4           ** ignored **

Bit 3           ** ignored **

Bit 2           ** ignored **

Bit 1           ** ignored **

Bit 0           Disk drive motor control

1: ON, 0: OFF

FDC Main Status Register

The main status register is used to control data transfer between the FDC and the CPU.

The Main Status Register is a byte, with each bit having a particular function.  The function of each bit is shown below.

The Main Status Register can be read from port &FB7E. (This port is INPUT only)

Bit 7   Data flow flag

1: FDC is ready for data transfer.

Bit 6   Data Direction flag

1: FDC --> CPU, 0: CPU --> FDC

The data direction for different phases is shown below:

Command phase:  CPU->FDC        CPU sends command bytes to FDC.

Execution phase:

Read data, read deleted data, read a track:

FDC->CPU        FDC sends sector contents to CPU.

Write data, write deleted data, scan equal,

scan low or equal, scan high or equal:

CPU->FDC        CPU sends sector contents to FDC.

Format track

CPU->FDC        CPU sends sector ID information

to FDC.

Result phase:   FDC->CPU        FDC sends result bytes to CPU.

Bit 5   Execution phase flag

1: FDC is performing execution phase of command

This bit is set to 1 after the FDC has read all command

bytes. It will be set to 0 when the execution phase has

been completed or if an error is received.

Bit 4   FDC Busy flag

1: FDC is busy executing a command.

0: FDC is in standby mode.

This flag is set to 1 after the FDC has read the first command

byte, and it is cleared as soon as all result bytes have

been read.

Bit 3   FDD 3 Busy (Disk Drive 3) ** NOT CONNECTED ON CPC **

Bit 2   FDD 2 Busy (Disk Drive 2) ** NOT CONNECTED ON CPC **

Bit 1   FDD 1 Busy (Disk Drive 1/Drive B)

1: FDD 1 is performing a seek (head is moving)

Bit 0   FDD 0 Busy (Disk Drive 0/Drive A)

1: FDD 0 is performing a seek (head is moving)

FDC Commands

READ DATA

Read sectors from current track. There is an option to skip sectors with deleted data address marks.

Command phase

MT MFM SK  0  0  1  1  0

0  0  0  0  0  S  DS1 DS0

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

<------- EOT --------->

<------- GPL --------->

<------- DTL --------->

Execution Phase

The FDC sends the sector data to the CPU.  The amount of data depends on N.

Result phase

<------- ST0 --------->

<------- ST1 --------->

<------- ST2 --------->

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

READ DELETED DATA

Read sectors from current track. Read them if they contain deleted data.

Command phase

MT MFM SK  0  1  1  0  0

0  0  0  0  0  S DS1 DS0

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

<------- EOT --------->

<------- GPL --------->

<------- DTL --------->

Execution phase

CPU must read sector data from FDC.

Result phase

<------- ST0 --------->

<------- ST1 --------->

<------- ST2 --------->

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

WRITE DATA

Write sectors to current track. Mark sectors with data address mark.

Command phase

MT MFM  0  0  0  1  0  1

0  0  0  0  0  S DS1 DS0

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

<------- EOT --------->

<------- GPL --------->

<------- DTL --------->

Execution phase

CPU must write sector data to FDC.

Result phase

<------- ST0 --------->

<------- ST1 --------->

<------- ST2 --------->

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

WRITE DELETED DATA

Write sectors to current track. Mark sectors with deleted data address mark.

Command phase

MT MFM  0  0  1  0  0  1

0  0  0  0  0  S DS1 DS0

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

<------- EOT --------->

<------- GPL --------->

<------- DTL --------->

Execution phase

CPU must write sector data to FDC.

Result phase

<------- ST0 --------->

<------- ST1 --------->

<------- ST2 --------->

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

READ A TRACK

This command will read EOT number of sectors from the current track. It will start by reading the first sector after the index pulse, and then read the data from each sector in turn. The sectors are read in the order they are found on the track (the order is specified in the format command). If the number of sectors to read is greater than the number of sectors stored on the track, then the read is repeated and the sectors are read in order again starting from the first sector. (data wraps)

Command phase

0 MFM SK  0  0  0  1  0

0  0  0  0  0  S DS1 DS0

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

<------- EOT --------->      number of sectors to read

<------- GPL --------->

<------- DTL --------->

Execution phase

CPU must read sector data from FDC.

Result phase

<------- ST0 --------->

<------- ST1 --------->

<------- ST2 --------->

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

READ SECTOR ID

Read sector ID from current position on current track. Repeat this command to read the sector ID for each sector on the track in the order that they were specified when the track was formatted. If you continue this command, the IDs will repeat.

I do not know of any way to make sure that the Fdc will start reading with the first sector id actually on the track.

Command phase

0 MFM  0  0  1  0  1  0

0  0  0  0  0  S DS1 DS0

Execution phase

Internal execution phase. The FDC will read the ID for the current sector.

Result phase

<------- ST0 --------->

<------- ST1 --------->

<------- ST2 --------->

<-------- C ----------> } sector ID

<-------- H ----------> }

<-------- R ----------> }

<-------- N ----------> }

FORMAT A TRACK

This command operates on the current track. This command writes the ID fields and data fields to initialise the track for storing data.

Command phase

0 MFM  0  0  1  1  0  1                        Command

0  0  0  0  0  S DS1 DS0

<-------- N --------->          Physical Sector size

<-------- SC -------->          Number of sectors per track

<------- GPL -------->          Formatting Gap length

(usually &52)

<-------- D --------->          Filler byte (byte to write into sector)

(usually &e5)

Execution phase

The CPU must send the sector ID information for each sector. The sector ID is 4 bytes long and contains C,H,R,N values. The total number of bytes to transfer is

number of sectors*4

For DATA format the following data would be transfered:

&00,&00,&c1,&02,

&00,&00,&c6,&02,

&00,&00,&c2,&02,

&00,&00,&c7,&02,

&00,&00,&c3,&02,

&00,&00,&c8,&02,

&00,&00,&c4,&02,

&00,&00,&c9,&02,

&00,&00,&c5,&02

The fdc will create each of the sectors and fill them with the filler byte.

Result phase

<------- ST0 --------->

<------- ST1 --------->

<------- ST2 --------->

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

SCAN EQUAL

Command phase

MT MFM SK  1  0  0  0  1

0  0  0  0  0  S DS1 DS0

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

<------- EOT --------->

<------- GPL --------->

<------- DTL --------->

Execution phase

CPU must write sector data to FDC to be compared against the sector specified in the command.

Result phase

<------- ST0 --------->

<------- ST1 --------->

<------- ST2 --------->

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

SCAN LOW OR EQUAL

Command phase

MT MFM SK  1  1  0  0  1

0  0  0  0  0  S  DS1 DS0

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

<------- EOT --------->

<------- GPL --------->

<------- DTL --------->

Execution phase

CPU must write sector data to FDC to be compared against the sector specified in the command.

Result phase

<------- ST0 --------->

<------- ST1 --------->

<------- ST2 --------->

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

SCAN HIGH OR EQUAL

Command phase

MT MFM SK  1  1  1  0  1

0  0  0  0  0  S  DS1 DS0

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

<------- EOT --------->

<------- GPL --------->

<------- DTL --------->

Execution phase

CPU must write sector data to FDC to be compared against the sector specified in the command.

Result phase

<------- ST0 --------->

<------- ST1 --------->

<------- ST2 --------->

<-------- C ---------->

<-------- H ---------->

<-------- R ---------->

<-------- N ---------->

RECALIBRATE

Move the drive head to track 0.

Command phase

0  0  0  0  0  1  1  1

0  0  0  0  0  0  DS1 DS0

Execution phase

SENSE INTERRUPT STATUS

Return Status Register 0, and the current position of the head.

Command phase

0  0  0  0  1  0  0  0

Result phase

<-------- ST0 -------->

<-------- CTK -------->

SPECIFY

Specify physical drive operation times.

Command phase

0  0  0  0  0  0  1  1

<--SRT---><--- HUT -->

<---------HLT-----> ND

SENSE DRIVE STATUS

Return status information from selected drive.

Command phase

0  0  0  0  0  1  0  0

0  0  0  0  0  S DS1 DS0

Result

<------- ST3 -------->

SEEK

(Move read/write head)

Command phase

0  0  0  0  1  1  1  1

0  0  0  0  0  S DS1 DS0

<------ DTK --------->

Execution

The FDC will move the head. A sense interrupt status command must be sent to monitor the progress of this command. The sense interrupt status register is used to check if the seek command has been completed, and if the head is over the track we wanted.

There is not any data transfer between CPU and FDC.

INVALID

(Not recognised command) If a bit pattern is specified which doesnt match one of the commands above then it is invalid.

Command phase

?  ?  ?  ?  ?  ?  ?  ?

Result

<-------- ST 0 ------->

Key

| CTK | Physical track head is over, this changes when a SEEK or RECALIBRATE command is sent. (0..255) |
| --- | --- |
| DTK | Destination track (physical track we want to move to) (0..255) |
| SK | READ DATA command: Skip a sector if it is has deleted data mark READ DELETED DATA command: Skip a sector if is has a data mark |
| Skip Bit | Read data If skip=0 and the FDC finds a sector with a Deleted Data address mark, then Control Mark is set to 1 and stops read command after reading the data from the sector into memory. If skip=1 and the FDC finds a sector with deleted data address mark, then it will ignore the sector (not transfer any data) and read the next sector. Read deleted data If skip=0 and the FDc finds a data address mark, then control mark is set to 1 and it reads the sector data in and ends the command. If skip=1 then it skips the sector with data address mark and reads the next deleted data sector. |
| MT | 1: Multi-track - In this mode, the FDC will automatically change side if needed. 0: Not multi-track - (not used much on the CPC) |
| MFM | 1: Use double density (MFM), 0: use single density (FM) (not supported on CPC) |
| DS1/DS0 | DS1 DS0 Drive 0 0 0/A 0 1 1/B 1 0 2 0 1 3 On the CPC, DS1 bit is ignored. |
| S | Physical side (0=top, 1=bottom) |
|  | C,H,R,N are the sector ID information. These values are stored before each sector on the track. They identify the sector and describe some information about it. |
| C | Track value. This values is compared with the sector ID information on the disc. They may not be the same as the physical track and side. In some copyprotections these values are totally different from the physical track and side. |
| H | Side value. This values is compared with the sector ID information on the disc. They may not be the same as the physical track and side. In some copyprotections these values are totally different from the physical track and side. |
| R | Sector ID for first sector to read DATA FORMAT &c1 SYSTEM FORMAT &41 IBM FORMAT &01 |
| EOT | Sector ID for last sector to read. IF R=EOT then the FDC will only transfer 1 sector of data. IF R<>EOT then the FDC will start with the sector ID specified and after each sector read it will increment this value until it is the same as EOT. |
| N | Sector size 0 - 128 bytes 1 - 256 bytes 2 - 512 bytes 3 - 1024 bytes 4 - 2048 bytes 5 - 4096 bytes 6 - 8192 bytes A track can only hold about 6.5k of data. Copyprotections which use a sector size of 6 only use &1800 bytes maximum (The remaining data transfered is garbage). These proove very difficult to copy by software alone. |
| GPL | Gap Length for reading/writing data. |
| SRT | Step Rate Time (in ms). After this time, the drive head will have moved to the next track or previous track. |
| HUT | Head unload time - time before head is removed from disk surface. This doesnt apply to 3" or 3.5" drives, and so therefore doesnt apply on CPC. |
| HLT | Head load time - time before head is removed from disk surface. This doesnt apply to 3" or 3.5" drives, and so therefore doesnt apply on CPC. |
| ND | DMA mode 1=Do not use DMA control, 0=Use DMA control (not supported on CPC). This needs DMA support chips to control data transfer.) |
| ST[01234] | ST0 = FDC Status Register 0 ST1 = FDC Status Register 1 ST2 = FDC Status Register 2 ST3 = FDC Status Register 3 (drive status) |
| DTL | Data length to read. (Only makes sense if N=0. Then DTL is the number of bytes to read. DTL bytes will be returned by READ DATA and other similar commands. If N<>0 (say 2), then DTL must be &ff). |

Peculiarities

We can assume that when a READ DATA, READ DELETED DATA, WRITE DATA, WRITE DELETED DATA,READ A TRACK, READ SECTOR ID, FORMAT TRACK, SEEK, RECALIBRATE and the SCAN commands will clear ST0, ST1 and ST2 before execution. ST0=ST1=ST2=0.

SENSE INTERRUPT STATUS will not clear ST0 before execution. It will return the current value of ST0. So when a SEEK is performed, it will hold ST0 bits saying when the seek has ended. If SENSE INTERRUPT STATUS is sent the first time, it will return the true value of ST0. But if it is sent for the second time, then ST0 will return &80. Subsequently it will continue to do this (if SENSE INTERRUPT STATUS) is repeatidly sent, until a command is executed which will change ST0.  ST0 bit 7 and 6 reflect if there were any errors.

Bit_7=Bit_6=ST0_Bit_3=1 when there is not a disk in the drive. (The drive will be not-ready).

Bit_7=0, Bit_6=1,ST1_Bit_7=1 after a READ DATA/WRITE DATA operation (assuming the sector data is not corrupted).

FDC status registers

Status Register 0

Bit 7   Error code (see below)

Bit 6

Bit 5   Seek end

When the FDC has finished a SEEK, then this bit is set.

Bit 4   Equipment Check

If the FDC detects a fault from the FDD then this bit is set.

Bit 3   Not Ready

When the FDC tried to do a read or write command, the drive was not

ready, then this bit is set. Also, if a read command is issued to

side 2 of a single sided drive then this bit is set.

Indicates that the drive motor is not running to full speed.

A small amount of time is needed after a "start motor" has been

issued before the motor is at full speed and the disk drive is

ready for the commands.

Bit 2   Head address

Currently selected side of drive being used (0=top, 1=bottom)

Bit 1   Unit Select 1   } drive number being used

Bit 0   Unit select 0   }

Error code

7       6       Error

0       0       Command was executed correctly. No error. Command was

finished properly.

0       1       Command was started but not completed properly.

1       0       Invalid (not recognised) command was sent to FDC, and was

not started.

1       1       Command was stopped before completion, because READY state

changed from drive. (Drive was suddenly not ready, so the

FDC gave up the rest of the command).

Status Register 1

Bit 7   End of Cylinder

When the FDC tries to get a sector beyond the end of a track

this bit is set. (On the CPC this bit is always 1 - do not

worry because the data will still be sent to the CPU).

Bit 6   Not used (This bit is 0)

Bit 5   Data Error

When the FDC is reading the sectors from the track, it checks

the data to make sure it is correct. This bit means that the

check failed for the ID field or the Data field of a sector.

This bit is 1, if ST2 bit 5=1.

Bit 4   Overrun

The CPU is taking to long to get the data the FDC is sending.

(All FDC routines invloving reading or writing must run with

interrupts DISABLED, otherwise the data will NOT be read into

memory correctly.

Interrupt loaders work, by disabling the interrupts whilst the

loading is done, and then re-enabling the interrupts again.)

Bit 3    Not used (This bit is 0)

Bit 2   No data

i) During a READ command, if the FDC cannot find the sector

specified in the command, then this bit is set.

ii) During a READ ID command, if the FDC cannot read the ID field

without an error, this bit is set.

This means that the track has not been formatted.

iii) During a READ TRACK command, if the starting sector cannot

be found this bit is set.

Bit 1   Not Writeable

If the FDC wants to do a write to the disk, and the disk is

WRITE PROTECTED, then this bit is set.

Bit 0   Missing Address Mark

This indicates that the FDC has found the sector ID, but it

cannot find the data for the sector. (Missing Address Mark

in Data Field will also be set)

Status register 2 (ST2)

Bit 7   Not used. (This bit is 0)

Bit 6   Control mark.

When performing a read command. This bit indicates that a sector

has been found which contains DELETED DATA.

Bit 5   Data Error in Data Field

When reading the sector data, the FDC has found that the CRC

check doesnt match, which means the sector data could be corrupted.

Bit 4   Wrong Cylinder/Wrong Track value

When reading the sector, the C value specified in the command doesnt

equal the C value stored for the sector on the disk.

Bit 3   Scan Equal Hit

When the FDC is executing a SCAN command, this indicates that the

condition "equal" is true.

Bit 2   Scan Not satisfied

When the FDC is executing a SCAN command, this indicates that no

sector has been found which matches the condition.

Bit 1   Bad Cylinder

Bit 0   Missing Address Mark in Data Field

When the FDC is reading a track, this indicates that the FDC cannot

detect if the sector contains DATA or DELETED DATA. The mark which

identifies which one the sector contains cannot be found.

Status Register 3 (ST3)

This status register will always reflect the state of the drive hardware.

Bit 7   Fault

This is a signal from the FDD. It indicates if there is a fault

with the drive.

Bit 6   Disk Write protected

This is a signal from the FDD. This indicates if the disk in

the selected drive is write protected (1), or write enabled (0).

Bit 5   Drive Ready

1: Drive Ready, 0: drive not ready.

This is a signal from the FDD. This indicates the disk drive motor

is on, and the disk is rotating at full speed, and the drive is

ready to recieve commands.

Bit 4   Disk drive head is positioned over track 0

1: Drive head is over track 0, 0: Drive is on another track.

This is a signal from the FDD. This indicates that the read/write head

is positioned over track 0 of the disc.

Bit 3   Two side

This is a signal from the FDD. This probably indicates that the drive

is double sided.

Bit 2   Head address

This is a signal from the FDD. This indicates the current side being

used.

Bit 1   Unit select 1  } Drive number being used

Bit 0   Unit select 0  }

When a track is formatted, the FDC initialises a data-structure on the disc. For each sector there is an ID field followed by a Data Field. Both of these are followed by a CRC checksum which is used by the FDC to check the data is not corrupted. The ID field contains the C,H,R and N for the sector and the Data Field contains the data in the sector. The FDC can mark the sector data with a special byte to show it contains DELETED DATA. This does not mean that the data has been deleted, or it is stored in a different way, but it is a way of showing some difference between sectors which do not have this mark.

Programming Examples

1) switch off drive motor,

LD A,0                          ; even = drive off

LD BC,&FA7E                     ; disk drive motor control port

OUT (C),A                       ; drive off

2) switch on drive motor and wait for it to pick up speed

LD A,1                          ; odd = drive on

LD BC,&FA7E                     ; disk drive motor control port

OUT (C),A                       ; disk drive motor on

LD B,5

.pause LD HL,0000

.pause2 DEC HL

LD A,H

OR L

JR NZ,pause2

DJNZ pause                      ; wait a little while, and make sure drive

; is rotating at full speed.

; now our drive routines will be compatible

; with ALL drives.

3) Sending command byte to the FDC

; Send a command byte to the FDC

;

; IN:

;  A = command byte to send to FDC

.send_command_byte

LD BC,&FB7E             ;; FDC Main status register

PUSH AF

.wait   IN A,(C)                ; get main status register

ADD A,A

JR NC,wait              ; is fdc ready for data?

ADD A,A

JR NC,wait              ; cpu->fdc?

POP AF

INC C                   ; BC = &fb7f (data register)

OUT (C),A               ; send command byte

LD A,5                  ; small pause to allow data register to pick up

; byte

.DELAY  DEC A

JR NZ,DELAY

RET

4) Getting result data from FDC

; Get result data from FDC.

;

; IN:

; HL = 7 byte buffer to store result phase data

.result_phase

LD BC,&FB7E

.l1     IN A,(C)                ; check data register is ready to send data

; back to us

CP &C0                  ; FDC ready to send data and FDC->CPU?

JP C,l1

INC C                   ; BC=&FB7F (data register)

IN A,(C)                ; get result byte

LD (HL),A               ; store in our buffer

INC HL                  ; increment buffer position

DEC C                   ; BC=&FB7F (main status register)

LD A,5                  ; small pause to wait for next byte from data

; register

.delay  DEC A

JR NZ,delay

IN A,(C)                ; is fdc busy? (is there more data?)

AND 16

JR NZ,l1

RET

5) Execution phase: read data from FDC

; This is used to send data to the CPU. (FDC->CPU)

;

; IN:

; HL = address of buffer to put data in

.execution_read

LD BC,&FB7E

JP r2

.r1     INC C                           ; &FB7F=data register

IN A,(C)                        ; read byte

LD (HL),A                       ; store in buffer

INC HL                          ; increment position in buffer

DEC C                           ; &FB7E=main status register

.r2     IN A,(C)                        ; FDC ready for data?

JP P,r2

AND 32                          ; are we in the execution phase?

JP NZ,r1

RET

6) Execution phase: write data to FDC

; This routine is used to send data to the FDC. (CPU->FDC)

;

; IN:

; HL = address of buffer to write out

.execution_write

LD BC,&FB7E                     ; main status register

JP w2

.w1     INC C                           ; &FB7F=data register

LD A,(HL)                       ; get byte from buffer

OUT (C),A                       ; send to FDC

INC HL                          ; increment buffer position

DEC C                           ; &FB7E=main status register

.w2     IN A,(C)                        ; FDC ready for data?

JP P,w2

AND 32                          ; are we still in execution phase?

JP NZ,w1

RET

7)Furthur examples

8)Disabling disk reading/writing

This is actually a bit weird. It is used in new Speedlock Tape protections to prevent those tape-to-disc routines from saving the main file to disk.

The following makes the fdc a bit confused. The fdc can be reset so that it can save to disc again, but the code to do this is a bit lengthy, and often too much to fit into a tape-to-disc transfer program.

ld bc,&fb7f

ld a,3

out (c),a