The CPCEMU Amstrad CPC Firmware Guide

Originally by Bob Taylor and Thomas Defoe, 1992

Electronic version scanned and formatted by David Cantrell, 1994

Introduction

Computer programming is one of  the  most satisfying hobbies as it

is a rare opportunity to invent, develop,  and test your own ideas and

see them come to fruition.  Competent  programming is a skill which is

not easy to master, but once learnt,  it will give hours of pleasure -

there are many people  who  spend  hours  inventing new coding tricks,

solving complex problems, or just trying out an idea.

Fortunately, when Amstrad developed  the  CPC  and PLUS computers,

they let the user access many  of  the computer's routines and use the

Firmware in their own programs.  Experienced programmers will no doubt

write faster routines which are  more  efficient, or have some special

feature, yet these extra facilities can easily be patched in using the

Firmware Jumpblock.

For many years,  Amstrad  produced  the  definitive  guide  to the

insides of the CPC but sale  of  this  was stopped almost 3 years ago.

[Note - 3 yrs ago means  1989  here].   Since then the Firmware Manual

has been regarded as an antique  by  those who are fortunate enough to

own a copy.   Nevertheless,  the  original  guide  had some omissions,

notably the abscence of information  on  the  system variables and the

Z80 microprocessor inside every CPC or Plus.

This guide is not intended  to  explain  how to program in machine

code, but we hope that it  will  supply the information needed to make

the most of the Amstrad's capabilities when writing your own programs.

Bob Taylor and Thomas Defoe, 1992

This Conversion ...

In August 1994, I phoned  the  original  authors, asking whether I

could distribute an electronic  version  of  their  firmware guide, to

complement the Amstrad  emulator  CPCEMU,  which  has  been written by

Marco Vieth for IBM compatible PCs.  They replied that I could, and so

I am making this available to  everyone  who wants it.  Of course, Bob

Taylor and Thomas Defoe still own  the  copyright.  They are no longer

distributing the paper version, so won't  lose out by the existence of

an electronic version.

To get the text into a  computer, I used a Hewlett-Packard Scanjet

flatbed scanner, and Omnipage Professional  OCR  software.  (or was it

Deskscan? - I forget now).  Of course,  the OCR software fucked up the

formatting.   Additionally,  it  couldn't  recognize  ampersands  (&),

converted 'm's to 'rn', lots of 'rn's to 'm', IX to I~Y, and all sorts

of other silliness.

It's taken me 5 months to get  it  sorted out, doing a few pages a

week.  It's VERY boring!  I used PROTEXT  version 6 to reformat it, as

it's the only word-processor  with  powerful enough search-replace and

ruler setting facilities, yet  is  still  easy  to  use.   Mind you, I

suppose I think that  because  I'm  used  to  it!  It was particularly

useful for the list of Z80 instructions  towards the end of this file.

(This section was 3 roughly instructions per line, with a single space

between each field after  being  scanned.   The  original was arranged

with all instructions alphabetically in to  columns.  This one has all

instructions arranged in one column, alphabetically.  Protext did most

of the reformatting/sorting automagically)

I've tried to reproduce the layout of the original book as closely

as possible, but there are a few changes:

In the plain ASCII version of this, I can't put in bold and italic

text, different fonts, and point sizes.

With a limit of 70 characters  per  line for most of the document,

I've put in more line-wraps than the original.

I've indented some  parts  of  the  document  differently.  In the

original, things were made clearer by  using  boldface, but I can't so

have had to change the layout slightly.

This document is available  in  two  forms - FIRMWARE.TXT contains

the plain ASCII version.   FIRMWARE.PTX  contains the PROTEXT version.

This includes the ruler lines, soft spaces and returns, etc...  If you

want to format it yourself (you want  to do what?!?!?), write to me at

the snail-mail address given in  the CPCEMU documentation.  Of course,

to do this, you'll need PROTEXT 6 yourself.

Of course, I disclaim any  responsibility  for  any errors in this

document.  I have NOT checked every single letter in it, including the

memory addresses and values given  in  various places, although I have

corrected a few of the scanners  HUGE errors.  When using the original

paper-based version of the Firmware Guide,  I had no problems, but the

scanner has probably put in a few errors.

David Cantrell, December 1994

*** ADVERT FOR ARNOR *** ADVERT FOR ARNOR *** ADVERT FOR ARNOR ***

Protext version 6 is available for PCs running MSDOS, from:

Arnor Ltd,

611 Lincoln Road

Peterborough

PE1 3HA

England

Phone +44 1733 68909

*** END OF ADVERT *** END OF ADVERT *** END OF ADVERT ***

Use of memory by the Operating System

The following list of  memory  addresses  and  their uses has been

compiled over a number of  years,  mainly from personal investigation.

It does not claim to be definitive,  since no accurate source seems to

be available to the average computer user, and so may be inaccurate or

deficient at certain points; also,  some  of  the areas described have

uses additional to those listed.  We have tried to make it as accurate

as possible, to enable  others  to  use  to  the full those facilities

which present themselves via this information.

¨ Addresses and values are present  in  memory with the low byte

first.  The Z80 processor represents  all 16-bit values in the

order lo-byte hi-byte.

¨ The term 'above' means higher in memory.

¨ Areas with numbers of  bytes  of  either  &00  or &FF given in

brackets, may be safe to use for machine code routines etc, as

may the tape area, and the  Sound  ENT  and ENT areas if these

are unused.

¨ The first column given is  the  address  (for the 6128) of the

memory being considered,  while  the  second  column gives the

equivalent 464 address -  unfortunately  the  464 differs from

the 6128 for most addresses, so if one address is omitted, the

system variable is not available for that machine.

¨ The next column gives the  size allocated in bytes.  Addresses

on the right hand  side  enclosed  in  brackets  are of System

Variables which hold the address of the bytes being explained.

With addresses or values anywhere in the text, the value shown

is for the 6128; a value in italics is for the 464 only.

Overview of the CPC's memory

In the following tables, the following symbols are used:

<>  means 'not the value or bit which follows'

*   means 'this applies to all machine with a disk drive fitted'

b0  means bit 0

b1  means bit 1

...

b15 means bit 15

HB  means 'most significant byte, hi-byte'

LB  means 'least significant byte, lo-byte'

When addresses are given in the  comments, they apply to the 6128.

When the 464's address is different, it  is given in brackets, such as

at the comment for &B763.

Please note that this section of the  guide  has been set out with all

the addresses in the  leftmost  column  in  the  correct order for the

6128.  However, a Conversion Chart specifically for the 464 is printed

in Appendix C.

The System Variables

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&0000 ¦ &0000 ¦ &40  ¦ Restart block:

&0000 ¦ &0000 ¦      ¦ RST0: complete machine reset

&0008 ¦ &0008 ¦      ¦ RST 1: LOWJUMP: in-line two byte address:

¦       ¦      ¦ b0 to b13=address; b14=Low ROM disabled;

¦       ¦      ¦ b15=Upper ROM disabled

&000B ¦ &000B ¦      ¦ LOW PCHL: HL has address as RST 1

&000E ¦ &000E ¦      ¦ 'JMP BC': BC has address to jump to

&0010 ¦ &0010 ¦      ¦ RST 2: SIDE CALL: inline two byte address:

¦       ¦      ¦ b0 to b13=address-&C000; bl4 to b15=offset

¦       ¦      ¦ to required ROM (used between sequenced

¦       ¦      ¦ Foreground ROMs)

&0013 ¦ &0013 ¦      ¦ SIDE PCHL: HL has address as RST 2

&0016 ¦ &0016 ¦      ¦ 'JMP DE': DE has address to jump to

&0018 ¦ &0018 ¦      ¦ RST 3: FAR CALL: inline three byte address

¦       ¦      ¦ block: bytes 1 and 2 hold the address;

¦       ¦      ¦ byte 3 holds the ROM select address

&001B ¦ &001B ¦      ¦ FAR PCHL: as RST 3, but HL has address;

¦       ¦      ¦ C has ROM select

&001E ¦ &001E ¦      ¦ 'JMP HL': HL has address to jump to

&0020 ¦ &0020 ¦      ¦ RST 4: RAM LAM: LD A,(HL) from RAM with

¦       ¦      ¦ ROMs disabled

&0023 ¦ &0023 ¦      ¦ FAR CALL: as RST 3, but HL has address of

¦       ¦      ¦ three byte address block

&0028 ¦ &0028 ¦      ¦ RST 5: FIRM JUMP: inline two byte address

¦       ¦      ¦ to jump to

&0030 ¦ &0030 ¦      ¦ RST 6: User restart; default to RST 0

&0038 ¦ &0038 ¦      ¦ RST 7: Interrupt entry (KB/Time etc)

&003B ¦ &003B ¦      ¦ External interrupt (default to RET)

&0040 ¦ &0040 ¦ &130 ¦ ROM lower foreground area: BASIC input

¦       ¦      ¦ area (tokenised)

&016F ¦ &016F ¦      ¦ end of BASIC input

&0170 ¦ &0170 ¦      ¦ BASIC working area for program, variables,

¦       ¦      ¦ etc...

&0170 ¦ &0170 ¦      ¦ Program area

¦       ¦      ¦ Variables and DEF FNs area

¦       ¦      ¦ Arrays area

¦       ¦      ¦ Free space

¦       ¦      ¦   end of free space

¦       ¦      ¦ Strings area

¦       ¦      ¦   end of Strings area (=HIMEM)

¦       ¦      ¦ Space for user machine code routines

¦       ¦      ¦   end of user space, byte before user

¦       ¦      ¦   defined graphics area

¦       ¦ n*8  ¦ User defined graphics area

¦       ¦      ¦   end of UDG area

¦       ¦      ¦ ROM Upper reserved area, expandible during

¦       ¦      ¦   KL ROM WALK, including:

¦       ¦ r*4  ¦ ROM chaining blocks (arranged as follows):

&A6FC ¦ &A6FC ¦ 4    ¦ AMSDOS chain,ing block:

&A6FC ¦ &A6FC ¦ 2    ¦ address of next ROM block in chain (or

¦       ¦      ¦   &0000 if the last in chain)

&A6FE ¦ &A6FE ¦ 1    ¦ ROM Select address

&A6FF ¦ &A6FF ¦ 1    ¦ &00

----------------------------------------------------------------

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&A700 ¦ &A700 ¦ &500 ¦ AMSDOS reserved area

¦       ¦      ¦   This area is moved down if any ROMs have

¦       ¦      ¦   numbers greater than eight (6128 only)

&A700 ¦ &A700 ¦ 1    ¦ Current drive number (0=A; 1=B)

&A701 ¦ &A701 ¦ 1    ¦ Current USER number

&A702 ¦ &A702 ¦ 1    ¦ flag?

&A703 ¦ &A703 ¦ 2    ¦ address?

&A705 ¦ &A705 ¦ 1    ¦ flag?

&A706 ¦ &A706 ¦ 2    ¦ address?

------+-------+------+------------------------------------------

&A708 ¦ &A708 ¦ 1    ¦ OPENIN flag (&FF=closed; <>&FF=opened)

&A709 ¦ &A709 ¦ &20  ¦ Copy of current or last Disc Directory

¦       ¦      ¦   entry for OPENIN/LOAD:

&A709 ¦ &A709 ¦ 1    ¦ USER number

&A70A ¦ &A70A ¦ 8    ¦ filename (padded with spaces)

&A712 ¦ &A712 ¦ 3    ¦ file extension (BAS, BIN, BAK, etc)

¦       ¦      ¦   including:

&A712 ¦ &A712 ¦ 1    ¦ b7 set = Read Only

&A713 ¦ &A713 ¦ 1    ¦ b7 set = System (ie not listed by CAT or

¦       ¦      ¦   DIR)

&A715 ¦ &A715 ¦ 1    ¦ 16K block sequence nurnber for this

¦       ¦      ¦   directory entry (0 for first block; if

¦       ¦      ¦   <>0 part of a larger file)

&A716 ¦ &A716 ¦ 2    ¦ unused

&A718 ¦ &A718 ¦ 1    ¦ length of this block in 128 byte records

&A719 ¦ &A719 ¦ 16   ¦ sequence of Disc Block nurnbers containing

¦       ¦      ¦   file - &00 as end marker

&A729 ¦ &A729 ¦ 1    ¦ number of 128 byte records loaded so far;

¦       ¦      ¦   before loading proper: &00 for ASCII (ie

¦       ¦      ¦   nothing loaded yet); &01 for BIN or BAS

¦       ¦      ¦   files (ie header record loaded)

&A72A ¦ &A72A ¦ 1    ¦

&A72B ¦ &A72B ¦ 1    ¦

------+-------+------+------------------------------------------

&A72C ¦ &A72C ¦ 1    ¦ OPENOUT flag (&FF=closed; <>&FF=opened)

&A72D ¦ &A72D ¦ &20  ¦ Copy of current or last Disc Directory

¦       ¦      ¦   entry for OPENOUT/SAVE:

&A72D ¦ &A72D ¦ 1    ¦ USER number

&A72E ¦ &A72E ¦ 8    ¦ filename (padded with spaces)

&A736 ¦ &A736 ¦ 3    ¦ file extension ($$$ while open; correct

¦       ¦      ¦   extension when finished)

&A739 ¦ &A739 ¦ 1    ¦ flag (&00=open; &FF=closed, ie finished)

&A73A ¦ &A73A ¦ 1    ¦

&A73B ¦ &A73B ¦ 1    ¦ flag (&00=open; &FF=closed)

&A73C ¦ &A73C ¦ 1    ¦ number of 128 byte records saved so far

&A73D ¦ &A73D ¦ 16   ¦ sequence of Disc Block numbers containing

¦       ¦      ¦   file - &00 as end marker

&A74D ¦ &A74D ¦ 1    ¦ number of 128 byte records saved so far

&A74E ¦ &A74E ¦ 1    ¦

&A74F ¦ &A74F ¦ 1    ¦

------+-------+------+------------------------------------------

&A750 ¦ &A750 ¦ 1    ¦ flag (&00=OPENIN; &01=In Char;

¦       ¦      ¦   &02=In Direct (whole file))

&A751 ¦ &A751 ¦ 2    ¦ address of 2K buffer for ASCII, or of

¦       ¦      ¦   start of current/last block if BIN or

¦       ¦      ¦   BAS file

&A753 ¦ &A753 ¦ 2    ¦ address of next byte to read for ASCII, or

¦       ¦      ¦   of 2K buffer for BAS or BIN file

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&A755 ¦ &A755 ¦ &45  ¦ first &45 bytes of BAS/BIN file (extended

¦       ¦      ¦   header) or of extended header made for

¦       ¦      ¦   ASCII file

&A755 ¦ &A755 ¦ 1    ¦ USER number

&A756 ¦ &A756 ¦ 8    ¦ filename (padded)

&A75E ¦ &A75E ¦ 3    ¦ extension

&A761 ¦ &A761 ¦ 6    ¦ unused

&A767 ¦ &A767 ¦ 1    ¦ file type (&00=BASIC; &01=protected BASIC;

¦       ¦      ¦ &02=Binary; &16=ASCII)

&A768 ¦ &A768 ¦ 2    ¦ unused

&A76A ¦ &A76A ¦ 2    ¦ address to load file into (=actual

¦       ¦      ¦   destination), or buffer for an ASCII

¦       ¦      ¦   file

&A76C ¦ &A76C ¦ 1    ¦ unused for disc

&A76D ¦ &A76D ¦ 2    ¦ length of file in bytes (&0000 for ASCII

¦       ¦      ¦   files)

&A76F ¦ &A76F ¦ 2    ¦ execution address fora BIN file

&A770 ¦ &A770 ¦ &25  ¦ unused

&A795 ¦ &A795 ¦ 3    ¦ length of actual file in bytes (as &A76D)

¦       ¦      ¦   -BAS and BIN only

&A798 ¦ &A798 ¦ 2    ¦ simple checksum of first 67 bytes of

¦       ¦      ¦   header (LB first) - BAS and BIN only

------+-------+------+------------------------------------------

&A79A ¦ &A79A ¦ 1    ¦ flag (&00=OPENOUT; &01=Out Char;

¦       ¦      ¦   &02=Out Direct(whole file))

&A79B ¦ &A79B ¦ 2    ¦ address of 2K block if an ASCII file, or

¦       ¦      ¦   of current/last block saved if a BAS

¦       ¦      ¦   or BIN file

&A79D ¦ &A79D ¦ 2    ¦ address of next byte to write for ASCII

¦       ¦      ¦   files, or of 2K buffer for BAS and BIN

¦       ¦      ¦   files

------+-------+------+------------------------------------------

&A79F ¦ &A79F ¦ &45  ¦ first &45 bytes of BAS/BIN file (ie

¦       ¦      ¦   extended header)

&A79F ¦ &A79F ¦ 1    ¦ USER number

&A7A0 ¦ &A7A0 ¦ 8    ¦ filename (padded)

&A7A8 ¦ &A7A8 ¦ 3    ¦ extension

&A7AB ¦ &A7AB ¦ 1    ¦ flag (&00=Open)

&A7AC ¦ &A7AC ¦ 1    ¦

&A7AD ¦ &A7AD ¦ 1    ¦ flag (&00=Open)

&A7AE ¦ &A7AE ¦ 3    ¦ unused

&A7B1 ¦ &A7Bl ¦ 1    ¦ file type (&00=BASIC; &01=protected BASIC;

¦       ¦      ¦   &02=Binary; &16=ASCII)

&A7B2 ¦ &A7B2 ¦ 2    ¦ unused

&A7B4 ¦ &A7B4 ¦ 2    ¦ address to save file from (for BAS or BIN

¦       ¦      ¦   files), or of buffer for ASCII files

&A7B6 ¦ &A7B6 ¦ 1    ¦ unused for disc

&A7B7 ¦ &A7B7 ¦ 2    ¦ length of file in bytes

&A7B9 ¦ &A7B9 ¦ 2    ¦ execution address for BIN files

&A7BB ¦ &A7BB ¦ &25  ¦ unused

&A7DF ¦ &A7DF ¦ 3    ¦ length of actual file in bytes (as at

¦       ¦      ¦   &A7B7) - BAS and BIN only

&A7E2 ¦ &A7E2 ¦ 2    ¦ simple checksum of first 67 bytes of

¦       ¦      ¦   header (LB first) - BAS and BIN only

&A7E4 ¦ &A7E4 ¦ &80  ¦ buffer area for records sent to or loaded

¦       ¦      ¦   from Disc, or used in forming filename

¦       ¦      ¦   and extension

&A864 ¦ &A864 ¦ 14*3 ¦ Tape Jumpblock is stored here by AMSDOS

¦       ¦      ¦   - is moved to &BC77 etc after |TAPE

&A88B ¦ &A88B ¦ 3    ¦ far address used by AMSDOS RST 3s at &BC77

¦       ¦      ¦   etc (&CD30,&07)

---------------------+------------------------------------------

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&A890 ¦ &A890 ¦ &19  ¦ Drive A Extended Disc Parameter Block

¦       ¦      ¦   (XDPB):

&A890 ¦ &A890 ¦ 2    ¦ number of 128 byte records per track

&A892 ¦ &A892 ¦ 1    ¦ log2(Block size)-7 (&03=1024 bytes;

¦       ¦      ¦   &04=2048 bytes)

&A893 ¦ &A893 ¦ 1    ¦ (Block size)/128-1 (&07=1024 bytes;

¦       ¦      ¦   &0F=2048 bytes)

&A894 ¦ &A894 ¦ 1    ¦ (Block size)/1024 (if total of blocks

¦       ¦      ¦   <256, else /2048)-1

&A895 ¦ &A895 ¦ 2    ¦ number of blocks per disc side (excluding

¦       ¦      ¦   reserved tracks)

&A897 ¦ &A897 ¦ 2    ¦ number of (directory entries)-1

&A899 ¦ &A899 ¦ 2    ¦ bit signiflcant value of number of blocks

¦       ¦      ¦   for directory (&0080=1; &00C0=2)

&A89B ¦ &A89B ¦ 2    ¦ number of bits in checksum

¦       ¦      ¦   =((&A894)+ 1)/4

&A89D ¦ &A89D ¦ 2    ¦ number of reserved tracks (&00=Data;

¦       ¦      ¦   &01=IBM; &02=System)

&A89F ¦ &A89F ¦ 1    ¦ number of first sector (&01=IBM;

¦       ¦      ¦   &41=System; &C1=Data)

&A8A0 ¦ &A8A0 ¦ 1    ¦ number of sectors per track (Data=9;

¦       ¦      ¦   System=9; IBM=8)

&A8A1 ¦ &A8Al ¦ 1    ¦ gap length (Read/Write)

&A8A2 ¦ &A8A2 ¦ 1    ¦ gap length (Format)

&A8A3 ¦ &A8A3 ¦ 1    ¦ format filler byte (&E5)

&A8A4 ¦ &A8A4 ¦ 1    ¦ log2(sector size)-7 (&02=512; &03=1024)

&A8A5 ¦ &A8A5 ¦ 1    ¦ records per sector

&A8A6 ¦ &A8A6 ¦ 1    ¦ current track (not for use)

&A8A7 ¦ &A8A7 ¦ 1    ¦ 0=not aligned (not for use)

&A8A8 ¦ &A8A8 ¦ 1    ¦ Auto select flag (&00=Auto select;

¦       ¦      ¦   &FF= don't alter)

------+-------+------+------------------------------------------

&A8A9 ¦ &A8A9 ¦      ¦

&A8B9 ¦ &A8B9 ¦      ¦

&A8D0 ¦ &A8D0 ¦ &19  ¦ Drive B Extended Disc Parameter Block

¦       ¦      ¦   (arranged as at &A890)

&A8E9 ¦ &A8E9 ¦      ¦ (&17 bytes of &FF)

&A8F9 ¦ &A8F9 ¦      ¦

&A900 ¦ &A900 ¦      ¦ (&12 bytes of &00)

&A910 ¦ &A910 ¦      ¦

------+-------+------+------------------------------------------

&A918 ¦ &A918 ¦ 2    ¦ address of area for reading directory

¦       ¦      ¦   entries for Drive A

&A91A ¦ &A91A ¦ 2    ¦ address of Drive A XDPB

&A91C ¦ &A91C ¦ 2    ¦ address of the byte after the end of

¦       ¦      ¦   Drive A XDPB

&A91E ¦ &A91E ¦ 2    ¦

&A920 ¦ &A920 ¦      ¦ (8bytes of &00)

&A928 ¦ &A928 ¦ 2    ¦ address of area for reading directory

¦       ¦      ¦   entries for Drive B

&A92A ¦ &A92A ¦ 2    ¦ address of Drive B XDPB

&A92C ¦ &A92C ¦ 2    ¦ address of the byte after the end of

¦       ¦      ¦   Drive B XDPB

&A92E ¦ &A92E ¦ 2    ¦

&A930 ¦ &A930 ¦ &80  ¦ block of directory entries, including last

¦       ¦      ¦   file loaded

&A9B0 ¦ &A9B0 ¦ &200 ¦ buffer for loading; usually contains last

¦       ¦      ¦   sector loaded

&ABB0 ¦ &ABB0 ¦      ¦ (&50 bytes of &00)

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&AC00 ¦ &AC00 ¦      ¦ Start of BASIC Operating System reserved

¦       ¦      ¦   area:

&AC00 ¦ &AC00 ¦ 1    ¦ program line redundant spaces flag

¦       ¦      ¦   (0=keep extra spaces; <>0=remove)

¦ &AC01 ¦ 9*3  ¦ groups of 3 RET bytes (&C9) called by the

¦       ¦      ¦   Upper ROM

&AC01 ¦ &AC1C ¦ 1    ¦ AUTO flag (0=off; <>0=on)

&AC02 ¦ &AC1D ¦ 2    ¦ number of the next line (6128) or of the

¦       ¦      ¦ current line (464) for AUTO

&AC04 ¦ &AC1F ¦ 2    ¦ step distance for AUTO

&AC06 ¦ &AC21 ¦ 1    ¦

&AC07 ¦ &AC22 ¦ 1    ¦

&AC08 ¦       ¦ 1    ¦

¦ &AC23 ¦ 1    ¦

&AC09 ¦ &AC24 ¦ 1    ¦ WIDTH (&84=132)

&AC0A ¦ &AC25 ¦      ¦

&AC0B ¦       ¦      ¦

&AC0C ¦ &AC26 ¦ 1    ¦ FOR/NEXT flag (0=NEXT not yet used;

¦       ¦      ¦   <>0=used)

&ACOD ¦ &AC27 ¦ 5    ¦ FOR start value (real).  Only 2 bytes are

¦       ¦      ¦   used if % or DEFINT variable

&AC12 ¦ &AC2C ¦ 2    ¦ address of ': ' or of the end of program

¦       ¦      ¦   line byte after a NEXT command

&AC14 ¦ &AC2E ¦ 2    ¦ address of LB of the line number

¦       ¦      ¦   containing WEND

&AC16 ¦ &AC30 ¦ 1    ¦ WHILE/WEND flag (&41=WEND not yet used;

¦       ¦      ¦   &04=used)

&AC17 ¦ &AC31 ¦      ¦

&AC18 ¦ &AC32 ¦ 2    ¦

&AClA ¦ &AC34 ¦ 2    ¦

&AC1C ¦ &AC36 ¦ 2    ¦ address of location holding ROM routine

¦       ¦      ¦   address for KB event block

------+-------+------+------------------------------------------

&AC1E ¦ &AC38 ¦ &0C  ¦ Event Block for ON SQ(l):

&AC1E ¦ &AC38 ¦ 2    ¦ chain address to next event block; &0000

¦       ¦      ¦   if last in chain, but &FFFF if unused

&AC20 ¦ &AC3A ¦ 1    ¦ count

&AC21 ¦ &AC3B ¦ 1    ¦ class: Far address, highest (ON SQ)

¦       ¦      ¦   priority, Normal & Synchronous event

&AC22 ¦ &AC3C ¦ 2    ¦ routine address (in BASIC ROM)

&AC24 ¦ &AC3E ¦ 1    ¦ ROM Select number (&FD ie ROM 0 enabled,

¦       ¦      ¦   Lower ROM disabled)

&AC25 ¦ &AC3F ¦ 1    ¦ (first byte of user field)

&AC26 ¦ &AC40 ¦ 2    ¦ address of the end of program line byte or

¦       ¦      ¦   ':' after 'ON SQ(x) GOSUB line number'

¦       ¦      ¦   statement

&AC28 ¦ &AC42 ¦ 2    ¦ address of the end of program line byte of

¦       ¦      ¦   the line before the GOSUB routine

&AC2A ¦ &AC44 ¦ &0C  ¦ Event block for ON SQ(2), arranged as

¦       ¦      ¦   &AC1E onwards - second ON SQ priority

&AC36 ¦ &AC50 ¦ &0C  ¦ Event block for ON SQ(4), arranged as

¦       ¦      ¦   &AC1E onwards - lowest ON SQ priority

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&AC42 ¦ &AC5C ¦ &12  ¦ Ticker and Event Block for AFTER/EVERY

¦       ¦      ¦   Timer 0

&AC42 ¦ &AC5C ¦ 2    ¦ chain address to next event block (usually

¦       ¦      ¦   to another timer or &00FF)

&AC44 ¦ &AC5E ¦ 2    ¦ 'count down' count

&AC46 ¦ &AC60 ¦ 2    ¦  recharge count (for EVERY only - &0000 if

¦       ¦      ¦   AFTER)

&AC48 ¦ &AC62 ¦ 2    ¦ chain address to next ticker block

&AC4A ¦ &AC64 ¦ 1    ¦ count

&AC4B ¦ &AC65 ¦ 1    ¦ class: Far address, lowest (timer)

¦       ¦      ¦   priority, Normal and Synchronous event

&AC4C ¦ &AC66 ¦ 2    ¦ Routine address (in BASIC ROM)

&AC4E ¦ &AC68 ¦ 1    ¦ ROM Select No (&FD ie ROM 0 enabled, Lower

¦       ¦      ¦   ROM disabled)

&AC4F ¦ &AC69 ¦ 1    ¦ (first byte of user field)

&AC50 ¦ &AC6A ¦ 2    ¦ address of the end of prograrn line byte

¦       ¦      ¦   or ':' after statement in use when the

¦       ¦      ¦   timer 'timed-out'

&AC52 ¦ &AC6C ¦ 2    ¦ address of tbe end of prograrn line byte

¦       ¦      ¦   of tbe line before the GOSUB routine

&AC54 ¦ &AC6E ¦ &12  ¦ Ticker and Event Block for AFTER/EVERY

¦       ¦      ¦   Timer 1 (3rd Timer priority) arranged as

¦       ¦      ¦   at &AC42

&AC66 ¦ &AC80 ¦ &12  ¦ Ticker and Event Block for AFTER/EVERY

¦       ¦      ¦   Timer 2 (2nd Timer priority) arranged as

¦       ¦      ¦   at &AC42

&AC78 ¦ &AC92 ¦ &12  ¦ Ticker and Event Block for AFTER/EVERY

¦       ¦      ¦   Timer 3 (highest priority) arranged as

¦       ¦      ¦   at &AC42

------+-------+------+------------------------------------------

&AC8A ¦ &ACA4 ¦ &100 ¦ BASIC input area for lines (as typed in

¦       ¦      ¦   and not tokenised) or for INPUT

&AD8C ¦ &ADA6 ¦ 2    ¦ address of line number LB in line

¦       ¦      ¦   containing error

&AD8E ¦ &ADA8 ¦ 2    ¦ address of byte before statement

¦       ¦      ¦   containing error - ie of ':' or of

¦       ¦      ¦   HB of Line No

&AD90 ¦ &ADAA ¦ 1    ¦ ERR (Error No)

&AD91 ¦       ¦ 1    ¦ DERR (Disc Error No)

&AD92 ¦ &ADAB ¦ 2    ¦ as &AD8E if error is in a program (ie not

¦       ¦      ¦   if in Direct Command Mode)

&AD94 ¦ &ADAD ¦ 2    ¦ as &AD8C if error is in a program (ie not

¦       ¦      ¦   if in Direct Command Mode)

&AD96 ¦ &ADAF ¦ 2    ¦ address of the length LB of line specified

¦       ¦      ¦   by the 'ON ERROR GOTO' command

&AD98 ¦ &ADB1 ¦ 1    ¦

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&AD99 ¦ &ADB2 ¦ &09  ¦ Current SOUND parameter block (see

¦       ¦      ¦   Firmware Jump &BCAA)

&AD99 ¦ &ADB2 ¦ 1    ¦ channel andrendezvous status

&AD9A ¦ &ADB3 ¦ 1    ¦ amplitude envelope (ENV) number

&AD9B ¦ &ADB4 ¦ 1    ¦ tone envelope (ENT) number

&AD9C ¦ &ADB5 ¦ 2    ¦ tone period

&AD9E ¦ &ADB7 ¦ 1    ¦ noise period

&AD9F ¦ &ADB8 ¦ 1    ¦ initial amplitude

&ADA0 ¦ &ADB9 ¦ 2    ¦ duration, or envelope repeat count

&ADA2 ¦ &ADBB ¦ &10  ¦ Current Amplitude or Tone Envelope

¦       ¦      ¦   parameter bloc (see &BCBC or &BCBF)

&ADA2 ¦ &ADBB ¦ 1    ¦ number of sections    (+&80 for a negative

¦       ¦      ¦   ENT number, ie the envelope is run until

¦       ¦      ¦   end of sound

&ADA3 ¦ &ADBC ¦ 3    ¦ first section of the envelope:

&ADA3 ¦ &ADBC ¦ 1    ¦ step count (if <&80) otherwise envelope

¦       ¦      ¦   shape (not tone envelope)

&ADA4 ¦ &ADBD ¦ 1    ¦ step size (if step count<&80) otherwise

¦       ¦      ¦   envelope period (not tone envelope)

&ADA5 ¦ &ADBE ¦ 1    ¦ pause time (if step count<&80) otherwise

¦       ¦      ¦   envelope period (not tone envelope)

&ADA6 ¦ &ADBF ¦ 3    ¦ second section of the envelope, as &ADA3

&ADA9 ¦ &ADC2 ¦ 3    ¦ third section of the envelope, as &ADA3

&ADAC ¦ &ADC5 ¦ 3    ¦ fourth section of the envelope, as &ADA3

&ADAF ¦ &ADC8 ¦ 3    ¦ fifth section of the envelope, as &ADA3

&ADB2 ¦ &ADCB ¦ 5    ¦

&ADB7 ¦ &ADD0 ¦ &36  ¦

&ADEB ¦ &AE04 ¦ 2    ¦

&ADED ¦ &AE06 ¦ 6    ¦

¦       ¦      ¦

&ADF3 ¦ &AE0C ¦ 26*1 ¦ table of DEFINT (&02), DEFSTR (&03) or

¦       ¦      ¦   DEFREAL default (&05), for variables 'a'

¦       ¦      ¦   to 'z'

&AE0D ¦ &AE26 ¦      ¦

&AE0E ¦ &AE27 ¦ 2    ¦

&AE10 ¦ &AE29 ¦ 2    ¦

&AE12 ¦ &AE2B ¦ 2    ¦

&AE14 ¦ &AE2D ¦ 1    ¦

&AE15 ¦ &AE2E ¦ 2    ¦ address of line number LB of last BASIC

¦       ¦      ¦   line (or &FFFF)

&AE17 ¦ &AE30 ¦ 2    ¦ address of byte before next DATA item (eg

¦       ¦      ¦   comma or space)

&AE19 ¦ &AE32 ¦ 2    ¦ address of next space on GOSUB etc stack,

¦       ¦      ¦   (see also &B06F)

&AE1B ¦ &AE34 ¦ 2    ¦ address of byte before current statement

¦       ¦      ¦   (&003F if in Direct Command mode)

&AE1D ¦ &AE36 ¦ 2    ¦ address of line nurnber LB of line of

¦       ¦      ¦   current statement (&0000 if in Direct

¦       ¦      ¦   Command mode)

&AE1F ¦ &AE38 ¦ 1    ¦ trace flag (0=TROFF; <>0=TRON)

&AE20 ¦ &AE39 ¦ 1    ¦ flag used with Trace (&00 if in Direct

¦       ¦      ¦   Command mode; &01 if in a program)

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&AE21 ¦ &AE3A ¦      ¦

&AE22 ¦ &AE3B ¦ 2    ¦

&AE24 ¦ &AE3D ¦ 2    ¦

&AE26 ¦ &AE3F ¦ 2    ¦ address to load cassette file to

&AE28 ¦ &AE41 ¦      ¦

&AE29 ¦ &AE42 ¦ 1    ¦ file type from cassette header

&AE2A ¦ &AE43 ¦ 2    ¦ file length from cassette header

&AE2C ¦ &AE45 ¦ 1    ¦ program protection flag (<>0 hides program

¦       ¦      ¦   as if protected)

&AE2D ¦ &AE46 ¦ 17   ¦ buffer used to form binary or hexadecimal

¦       ¦      ¦   numbers before printing etc

&AE3A ¦ &AE53 ¦ 5    ¦ start of buffer used to form hexadecimal

¦       ¦      ¦   numbers before printing etc

&AE3A ¦ &AE53 ¦ 1    ¦ Key Number used with INKEY (providing the

¦       ¦      ¦   Key Number is written as a decimal)

&AE3E ¦ &AE57 ¦ 1    ¦ last byte (usually &00 or &20) of the

¦       ¦      ¦   formed binary or hexadecimal number

&AE43 ¦ &AE5D ¦ 13   ¦ buffer used to form decimal numbers before

¦       ¦      ¦   printing etc

&AE4E ¦ &AE68 ¦ 1    ¦ last byte (usually &00 or &20) of the

¦       ¦      ¦   formed decimal number

¦ &AE6B ¦ 3    ¦

&AE51 ¦       ¦ 1    ¦

&AE52 ¦ &AE6E ¦ 2    ¦

&AE54 ¦       ¦ 1    ¦

¦ &AE70 ¦ 2    ¦ temporary store for address after

¦       ¦      ¦   using (&AE68)

¦       ¦      ¦

&AE55 ¦ &AE72 ¦ 2    ¦ address of last used ROM or RSX JUMP

¦       ¦      ¦   instruction in its Jump Block

&AE57 ¦ &AE74 ¦ 1    ¦ ROM Select number if address above is in

¦       ¦      ¦   ROM

&AE58 ¦ &AE75 ¦ 2    ¦ BASIC Parser position, moved on to ': ',

¦       ¦      ¦   or the end of program line byte after a

¦       ¦      ¦   CALL or an RSX

&AE5A ¦ &AE77 ¦ 2    ¦ the resetting address for machine Stack

¦       ¦      ¦   Pointer after a CALL or an RSX

&AE5C ¦ &AE79 ¦ 2    ¦ ZONE value

&AE5D ¦       ¦ 1    ¦

¦ &AE7A ¦ 1    ¦

------+-------+------+------------------------------------------

&AE5E ¦ &AE7B ¦ 2    ¦ HIMEM (set by MEMORY)

¦ &AE7D ¦ 2    ¦ address of the byte before the UDG area

¦       ¦      ¦   (the end of the user M/C routine area or

¦       ¦      ¦   the Strings area) if the UDG area is

¦       ¦      ¦   still present, otherwise the highest

¦       ¦      ¦   byte of Program etc area

&AE60 ¦ 2     ¦      ¦ address of highest byte of free RAM (ie

¦       ¦      ¦   last byte of UDG area)

&AE62 ¦ &AE7F ¦ 2    ¦ address of start of ROM lower reserved

¦       ¦      ¦   area (used for tokenised lines)

&AE64 ¦ &AE81 ¦ 2    ¦ address of end of ROM lower reserved area

¦       ¦      ¦   (byte before Program area)

&AE66 ¦ &AE83 ¦ 2    ¦ as &AE68

&AE68 ¦ &AE85 ¦ 2    ¦ address of start of Variables and DEF FNs

¦       ¦      ¦   area

&AE6A ¦ &AE87 ¦ 2    ¦ address of start of Arrays area (where

¦       ¦      ¦   next Variable or DEF FN entry is placed)

&AE6C ¦ &AE89 ¦ 2    ¦ address of start of free space (where

¦       ¦      ¦   next Array entry is placed)

&AE6E ¦       ¦ 1    ¦

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&AE70 ¦ &AE8C ¦ &1FF ¦ GOSUB, FOR and WHILE stack.  Entries are

¦       ¦      ¦   added above any existing ones in use

¦       ¦      ¦   (mixed as encountered) at address given

¦       ¦      ¦   by &B06F and must be used up in the

¦       ¦      ¦   opposite order.  Completed entries are

¦       ¦      ¦   not deleted, just overwitten by the

¦       ¦      ¦   next new entry:

------+-------+------+------------------------------------------

¦       ¦      ¦ GOSUB (84 max capacity):

¦       ¦ 1    ¦ (byte of &00)

¦       ¦ 2    ¦ address of end of program line byte or ':'

¦       ¦      ¦   after GOSUB statement (the point to

¦       ¦      ¦   RETURN to)

¦       ¦ 2    ¦ address of line number HB of line

¦       ¦      ¦   containing GOSUB

¦       ¦ 1    ¦ byte of &06, ie the length of the GOSUB

¦       ¦      ¦   entry

------+-------+------+------------------------------------------

¦       ¦      ¦ FOR (21 max for Real FORs,31 max for

¦       ¦      ¦   Integer FORs):

¦       ¦ 2    ¦ address of current value of control

¦       ¦      ¦   variable (in Variables area)

¦       ¦ 5    ¦ value of limit (ie the TO value) - there

¦       ¦      ¦   are two bytes only for Integer FORs

¦       ¦ 5    ¦ value of STEP - two bytes for Integer FORs

¦       ¦ 1    ¦ sign byte (&00 for positive; &01 for

¦       ¦      ¦   negative)

¦       ¦ 2    ¦ address of the end of program line byte,

¦       ¦      ¦   or ':' after the FOR statement (ie the

¦       ¦      ¦   address for the NEXT loop to restart at)

¦       ¦ 2    ¦ address of line number LB of line

¦       ¦      ¦   containing FOR

¦       ¦ 2    ¦ address of byte after NEXT statement (ie

¦       ¦      ¦   the address to continue at when the

¦       ¦      ¦   limit is exceeded)

¦       ¦ 2    ¦ address of byte after NEXT statement again

¦       ¦ 1    ¦ length byte (&16 for Real FORs; &10 for

¦       ¦      ¦   Integer FORs)

------+-------+------+------------------------------------------

¦       ¦      ¦ WHILE (66 max capacity):

¦       ¦ 2    ¦ address of line number LB of line

¦       ¦      ¦   containing WHILE

¦       ¦ 2    ¦ address of the end of program line byte or

¦       ¦      ¦   ':' after WEND statement (ie the address

¦       ¦      ¦   to continue at when the condition is

¦       ¦      ¦   false)

¦       ¦ 2    ¦ address of condition after the WHILE

¦       ¦      ¦   command

¦       ¦ 1    ¦ length byte of &07 - end of WHILE entry

¦       ¦      ¦   proper but:

¦       ¦ +5   ¦ value of condition (0 or -1 as a floating

¦       ¦      ¦   point value) only while the WHILE entry

¦       ¦      ¦   is the last on the stack

&B06F ¦ &B08B ¦ 2    ¦ address of the next space on the GOSUB etc

¦       ¦      ¦   stack (see also &AE19)

------+-------+------+------------------------------------------

NB: The free space on the stack is also used as a workspace by

various routines for values and addresses and for Variable

names

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B071 ¦ &B08D ¦ 2    ¦ address of end of free space (the byte

¦       ¦      ¦   before the Strings area)

&B073 ¦ &B08F ¦ 2    ¦ address of end of Strings area (=HIMEM)

&B075 ¦       ¦ 1    ¦

¦ &B091 ¦ 1    ¦

¦ &B092 ¦ 2    ¦

&B076 ¦ &B094 ¦ 2    ¦

&B078 ¦ &B096 ¦ 2    ¦ address of the highest byte of free RAM

¦       ¦      ¦   disregarding UDGs (usually &A6FB)

&B07A ¦ &B098 ¦ 2    ¦

&B07C ¦ &B09A ¦ 2    ¦ address for the next entry in the String

¦       ¦      ¦   Concatenation area

&B07E ¦ &B09C ¦ 10*3 ¦ concatenation area holding descriptors of

¦       ¦      ¦   strings being used

&B09C ¦ &B0BA ¦ 1    ¦ length of last String used

&B09D ¦ &B0BB ¦ 2    ¦ address of last String used

¦ &B0BD ¦ 2    ¦

¦ &BOBF ¦ 2    ¦

&B09F ¦ &BOC1 ¦ 1    ¦ type byte used with the Virtual

¦       ¦      ¦   Accumulator (&02=Integer; &03=String;

¦       ¦      ¦   &05=Real)

&B0A0 ¦ &B0C2 ¦ 5    ¦ Virtual Accurnulator used by the maths

¦       ¦      ¦   routines (two bytes for an Integer

¦       ¦      ¦   value; three bytes for a String

¦       ¦      ¦   Descriptor; five bytes for a Real

¦       ¦      ¦   value)

&B0A0 ¦ &B0C2 ¦ 2    ¦

&B0A2 ¦ &B0C4 ¦ 1    ¦

&B0A3 ¦ &B0C5 ¦ 2    ¦

&B0A5 ¦ &B0C7 ¦ &5B  ¦ (&39 bytes on 464) bytes of &FF

&B100 ¦ &B8E4 ¦ 2    ¦ &07, &C6

&B102 ¦ &B8E6 ¦ 2    ¦ &65, &89

&B104 ¦ &B8E8 ¦ 5    ¦

&B109 ¦ &B8ED ¦ 5    ¦

&B10E ¦ &B8F2 ¦ 5    ¦

&B113 ¦ &B8F7 ¦ 1    ¦ DEG/RAD flag (&00=RAD; &FF=DEG)

&B114 ¦ &B8DC ¦ 1    ¦

&B115 ¦ &B8DD ¦ 1    ¦

&B116 ¦ &B8DE ¦ 1    ¦

&B117 ¦ &B8DF ¦ 1    ¦

------+-------+------+------------------------------------------

&B118 ¦ &B800 ¦ &D2  ¦ Area used for Cassette handling:

&B118 ¦ &B800 ¦ 1    ¦ cassette handling messages flag

¦       ¦      ¦   (0=enabled; <>O=disabled)

&B119 ¦ &B801 ¦ 1    ¦

&B11A ¦ &B802 ¦ 1    ¦ file IN flag (&00=closed; &02=IN file;

¦       ¦      ¦   &03=opened; &05=IN char)

&B11B ¦ &B803 ¦ 2    ¦ address of 2K buffer for directories

&B11D ¦ &B805 ¦ 2    ¦ address of 2K buffer for loading blocks of

¦       ¦      ¦   files - often as &B11B

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B11F ¦ &B807 ¦ &40  ¦ IN Channel header

&B11F ¦ &B807 ¦ &10  ¦ filename (padded with NULs)

&B12F ¦ &B817 ¦ 1    ¦ number of block being loaded, or next to

¦       ¦      ¦   be loaded

&B130 ¦ &B818 ¦ 1    ¦ last block flag (&FF=last block; &00=not)

&B131 ¦ &B819 ¦ 1    ¦ file type (&00=BASIC; &01=Protected BASIC;

¦       ¦      ¦   &02=Binary; &08=Screen; &16=ASCII)

&B132 ¦ &B81A ¦ 2    ¦ length of this block

&B134 ¦ &B81C ¦ 2    ¦ address to load this or the next block

¦       ¦      ¦   at,or the address of the byte after last

¦       ¦      ¦   one loaded

&B136 ¦ &B81E ¦ 1    ¦ first block flag (&FF=first block;

¦       ¦      ¦   &00=not)

&B137 ¦ &B81F ¦ 2    ¦ total length of file (all blocks)

&B139 ¦ &B821 ¦ 2    ¦ execution address for BIN files (&0000 if

¦       ¦      ¦   not saved as such)

&B13B ¦ &B823 ¦ &24  ¦ not allocated

------+-------+------+------------------------------------------

&B15F ¦ &B847 ¦ 1    ¦ file OUT flag (&00=closed; &02=IN file;

¦       ¦      ¦   &03=opened; &05=IN char)

&B160 ¦ &B848 ¦ 2    ¦ address to start the next block save from,

¦       ¦      ¦   or the address of the buffer if it is

¦       ¦      ¦   OPENOUT

&B162 ¦ &B84A ¦ 2    ¦ address of start of the last block saved,

¦       ¦      ¦   or the address of the buffer if it is

¦       ¦      ¦   OPENOUT

------+-------+------+------------------------------------------

&B164 ¦ &B84C ¦ &40  ¦ OUT Channel Header (details as IN Channel

¦       ¦      ¦   Header):

&B164 ¦ &B84C ¦ &10  ¦ filename

&B174 ¦ &B85C ¦ 1    ¦ number of the block being saved, or next

¦       ¦      ¦   to be saved

&B175 ¦ &B85D ¦ 1    ¦ last block flag (&FF=last block; &00=not)

&B176 ¦ &B85E ¦ 1    ¦ file type (as at &B131

&B177 ¦ &B85F ¦ 2    ¦ length saved so far

&B179 ¦ &B861 ¦ 2    ¦ address of start of area to save, or

¦       ¦      ¦   address of buffer if it is an OPENOUT

¦       ¦      ¦   instruction

&B17B ¦ &B863 ¦ 1    ¦ first block flag (&FF=first block;

¦       ¦      ¦   &00=not)

&B17C ¦ &B864 ¦ 2    ¦ total length of file to be saved

&B17E ¦ &B866 ¦ 2    ¦ execution address for BIN files (&0000 if

¦       ¦      ¦   parameter not supplied)

&B180 ¦ &B868 ¦ &24  ¦ not allocated

&B1A4 ¦ &B88C ¦ &40  ¦ used to construct IN Channel header:

&B1B5 ¦ &B89D ¦ 1    ¦

&B1B7 ¦ &B89F ¦ 2    ¦

&B1B8 ¦ &B8A3 ¦ 1    ¦

&B1BE ¦ &B8A6 ¦ 1    ¦

------+-------+------+------------------------------------------

&B1B9 ¦ &B51D ¦      ¦ base address for calculating relevant

¦       ¦      ¦   Sound Channel block

&B1BC ¦ &B520 ¦      ¦ base address for calculating relevant

¦       ¦      ¦   Sound Channel ?

&B1BE ¦ &B522 ¦      ¦ base address for calculating relevant

¦       ¦      ¦   Sound Channel ?

&B1D5 ¦ &B539 ¦      ¦ base address for calculating relevant

¦       ¦      ¦   Sound Channel ?

&B1E4 ¦ &B8CC ¦ 1    ¦

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B1E5 ¦ &B8CD ¦ 1    ¦ synchronisation byte

&B1E6 ¦ &B8CE ¦ 2    ¦ &55, &62

&B1E8 ¦ &B8D0 ¦ 1    ¦

&B1E9 ¦ &B8D1 ¦ 1    ¦ cassette precompensation (default &06;

¦       ¦      ¦   SPEED WRITE 1 &0C @4µs)

&B1EA ¦ &B8D2 ¦ 1    ¦ cassette 'Half a Zero' duration (default

¦       ¦      ¦   &53; SPEED WRITE 1 &29 @ 4µS)

&B1EB ¦ &B8D3 ¦ 2    ¦

¦ &B550 ¦ 1    ¦ used by sound routines

¦ &B551 ¦ 1    ¦ used by sound routines

&B1ED ¦       ¦ 1    ¦ used by sound routines

&B1EE ¦ &B552 ¦ 1    ¦ used by sound routines

&B1F0 ¦ &BB54 ¦ 1    ¦ used by sound routines

¦ &BB55 ¦ 7    ¦ used by sound and cassette routines

------+-------+------+------------------------------------------

&B1F8 ¦ &B55C ¦ &3F  ¦ Sound Channel A (1) data:

&B212 ¦ &B576 ¦ 1    ¦ number of sounds still queuing

&B213 ¦ &B577 ¦ 1    ¦ number of sounds originally queuing

&B217 ¦ &B57B ¦ 8    ¦ first or fifth sound in Channel 1 (A)

¦       ¦      ¦   queue:

&B217 ¦ &B57B ¦ 1    ¦ status: b0 to b2 = rendezvous with channel

¦       ¦      ¦   1, 2 or 4; b3 = Hold; b4 = Flush

&B218 ¦ &B57C ¦ 1    ¦ b0 to b3 = tone envelope number;

¦       ¦      ¦   b4 to b7 = volume envelope number (ie

¦       ¦      ¦   ENV nurnber*16)

&B219 ¦ &B57D ¦ 2    ¦ pitch

&B21B ¦ &B57F ¦ 1    ¦ noise

&B21C ¦ &B580 ¦ 1    ¦ volume

&B21D ¦ &B581 ¦ 2    ¦ duration (in 0.01 seconds)

&B21F ¦ &B583 ¦ 8    ¦ second sound in Channel 1 queue (as &B217)

&B227 ¦ &B58B ¦ 8    ¦ third sound in Channel 1 queue (as &B217)

&B22F ¦ &B593 ¦ 8    ¦ fourth sound in Channel 1 queue (as &B217)

------+-------+------+------------------------------------------

&B237 ¦ &B59B ¦ &3F  ¦ Sound Channel B (2) data - as described at

¦       ¦      ¦   &B1F8

&B256 ¦ &B5BA ¦ 8    ¦ first or fifth sound in Channel 2 queue

¦       ¦      ¦   (as &B217)

&B25E ¦ &B5C2 ¦ 8    ¦ second sound in Channel 2 queue (as &B217)

&B266 ¦ &B5CA ¦ 8    ¦ third sound in Channel 2 queue (as &B217)

&B26E ¦ &B5D2 ¦ 8    ¦ fourth sound in Channel 2 queue (as &B217)

------+-------+------+------------------------------------------

&B276 ¦ &B5DA ¦ &3F  ¦ Sound Channel C (4) data - as described at

¦       ¦      ¦   &B1F8

&B295 ¦ &B5F9 ¦ 8    ¦ first or fifth sound in Channel 4 queue

¦       ¦      ¦   (as   &B217)

&B29D ¦ &B601 ¦ 8    ¦ 2nd sound in Channel 4 queue (as &B217)

&B2A5 ¦ &B609 ¦ 8    ¦ 3rd sound in Channel 4 queue (as &B217)

&B2AD ¦ &B611 ¦ 8    ¦ 4th sound in Channel 4 queue (as &B217)

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B2A6 ¦ &B60A ¦      ¦ base address for calculating relevant ENV

¦       ¦      ¦   parameter block

&B2B6 ¦ &B61A ¦ 15*16¦ ENV parameter block area (each arranged as

¦       ¦      ¦   &ADA2):

&B2B6 ¦ &B61A ¦ &10  ¦ ENV 1

&B2C6 ¦ &B62A ¦ &10  ¦ ENV 2

&B2D6 ¦ &B63A ¦ &10  ¦ ENV 3

&B2E6 ¦ &B64A ¦ &10  ¦ ENV 4

&B2F6 ¦ &B65A ¦ &10  ¦ ENV 5

&B306 ¦ &B66A ¦ &10  ¦ ENV 6

&B316 ¦ &B67A ¦ &10  ¦ ENV 7

&B326 ¦ &B68A ¦ &10  ¦ ENV 8

&B336 ¦ &B69A ¦ &10  ¦ ENV 9

&B346 ¦ &B6AA ¦ &10  ¦ ENV 10

&B356 ¦ &B6BA ¦ &10  ¦ ENV 11

&B366 ¦ &B6CA ¦ &10  ¦ ENV 12

&B376 ¦ &B6DA ¦ &10  ¦ ENV 13

&B386 ¦ &B6EA ¦ &10  ¦ ENV 14

&B396 ¦ &B6FA ¦ &10  ¦ ENV 15

------+-------+------+------------------------------------------

&B396 ¦ &B6FA ¦      ¦ base address for calculating relevant ENT

¦       ¦      ¦   parameter block

&B3A6 ¦ &B70A ¦ 15*16¦ ENT parameter block area (each arranged as

¦       ¦      ¦   &ADA2):

&B3A6 ¦ &B70A ¦ &10  ¦ ENT 1

&B3B6 ¦ &B71A ¦ &10  ¦ ENT 2

&B3C6 ¦ &B72A ¦ &10  ¦ ENT 3

&B3D6 ¦ &B73A ¦ &10  ¦ ENT 4

&B3E6 ¦ &B74A ¦ &10  ¦ ENT 5

&B3F6 ¦ &B75A ¦ &10  ¦ ENT 6

&B406 ¦ &B76A ¦ &10  ¦ ENT 7

&B416 ¦ &B77A ¦ &10  ¦ ENT 8

&B426 ¦ &B78A ¦ &10  ¦ ENT 9

&B436 ¦ &B79A ¦ &10  ¦ ENT 10

&B446 ¦ &B7AA ¦ &10  ¦ ENT 11

&B456 ¦ &B7BA ¦ &10  ¦ ENT 12

&B466 ¦ &B7CA ¦ &10  ¦ ENT 13

&B476 ¦ &B7DA ¦ &10  ¦ ENT 14

&B486 ¦ &B7EA ¦ &10  ¦ ENT 15

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B496 ¦ &B34C ¦ &50  ¦ Normal Key Table:

¦       ¦      ¦ Cur U Cur R Cur D f9    f6    f3    Enter f.

¦       ¦      ¦ Cur L Copy  f7    f8    f5    f1    f2    f0

¦       ¦      ¦ Clr   [     Ret   ]     f4          \

¦       ¦      ¦ ^     -     @     p     ;     :     /     .

¦       ¦      ¦ 0     9     o     i     l     k     m     j

¦       ¦      ¦ 8     7     u     y     h     j     n     Space

¦       ¦      ¦ 6     5     r     t     g     f     b     v

¦       ¦      ¦ 4     3     e     w     s     d     c     x

¦       ¦      ¦ 1     2     Esc   q     Tab   a     Caps  z

¦       ¦      ¦ [VT]  [LF]  [BS]  [TAB] Fire2 Fire1       Del

------+-------+------+------------------------------------------

&B4E6 ¦ &B39C ¦ &50  ¦ Shifted Key Table:

¦       ¦      ¦ Cur U Cur R Cur D f9    f6    f3    Enter f.

¦       ¦      ¦ Cur L Copy  f7    f8    f5    f1    f2    f0

¦       ¦      ¦ Clr   {     Ret   }     f4          `

¦       ¦      ¦ £     =     |     P     +     *     ?     >

¦       ¦      ¦ _     )     O     I     L     K     M     <

¦       ¦      ¦ (     '     U     Y     H     J     N     Space

¦       ¦      ¦ &     %     R     T     G     F     B     V

¦       ¦      ¦ $     #     E     W     S     D     C     X

¦       ¦      ¦ !     "     Esc   Q     ->    A     Caps  Z

¦       ¦      ¦ [VT]  [LF]  [BS]  [TAB] Fire2 Fire1       Del

------+-------+------+------------------------------------------

&B536 ¦ &B3EC ¦ &50  ¦ Control Key Table:

¦       ¦      ¦ Cur U Cur R Cur D f9    f6    f3    Enter f.

¦       ¦      ¦ Cur L Copy  f7    f8    f5    f1    f2    f0

¦       ¦      ¦ Clr   (ESC) Ret   (GS)  f4          (FS)

¦       ¦      ¦ (RS)        (NUL) (DLE)

¦       ¦      ¦ (US)        (SI)  (HT)  (FF)  (VT)  (CR)

¦       ¦      ¦             (NAK) (EM)  (BS)  (LF)  (SO)

¦       ¦      ¦             (DC2) (DC4) (BEL) (ACK) (STX) (SYN)

¦       ¦      ¦             (ENQ) (ETB) (DC3) (EOT) (ETX) (CAN)

¦       ¦      ¦       ~     Esc   (DC1) Ins   (SOH) S-lck (SUB)

¦       ¦      ¦                                     Del

------+-------+------+------------------------------------------

&B586 ¦ &B43C ¦ 10   ¦ KB repeats table (each byte/bit applies to

¦       ¦      ¦   all three key tables): 1 byte is used

¦       ¦      ¦   per line of the tables; b0 to b7 give

¦       ¦      ¦   the columns (left to right), repeat if

¦       ¦      ¦   set

&B590 ¦ &B446 ¦ &98  ¦ DEF KEY's definition area (for Keys &80 to

¦       ¦      ¦   &9F in sequence): each definition has

¦       ¦      ¦   either a single byte of &00 if it is

¦       ¦      ¦   unused/unaltered, or:

¦       ¦      ¦  byte 1: length of definition

¦       ¦      ¦  bytes 2 to x: definition, either a single

¦       ¦      ¦   key or a string of keys

&B628 ¦ &B4DE ¦ 1    ¦ Byte after end of DEF KEY area

&B629 ¦ &B4DF ¦ 1    ¦

&B62A ¦ &B4E0 ¦ 1    ¦

&B62B ¦ &B4E1 ¦ 2    ¦ address of DEF KEY area

&B62D ¦ &B4E3 ¦ 2    ¦ address of byte after end of DEF KEY area

&B62F ¦ &B4E5 ¦ 1    ¦

&B630 ¦ &B4E6 ¦ 1    ¦

&B631 ¦ &B4E7 ¦ 1    ¦ Shift lock flag (&00=off; &FF=on)

&B632 ¦ &B4E8 ¦ 1    ¦ Caps lock flag (&00=off; &FF=on)

&B633 ¦ &B4E9 ¦ 1    ¦ KB repeat period (SPEED KEY - default &02

¦       ¦      ¦   @ 0.02 seconds)

&B634 ¦ &B4EA ¦ 1    ¦ KB delay period (SPEED KEY - default &1E

¦       ¦      ¦   @ 0.02 seconds)

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B635 ¦ &B4EB ¦ 2*10 ¦ Tables used for key scanning; bits 0 to 7

¦       ¦      ¦   give the table columns (from left to

¦       ¦      ¦   right):

&B635 ¦ &B4EB ¦ 1    ¦ Cur U Cur R Cur D f9    f6    f3    Enter f.

&B636 ¦ &B4EC ¦ 1    ¦ Cur L Copy  f7    f8    f5    f1    f2    f0

&B637 ¦ &B4ED ¦ 1    ¦ Clr   [     Ret   ]     f4    Shift \     Ctrl

&B638 ¦ &B4EE ¦ 1    ¦ ^     -     @     p     ;     :     /     .

&B639 ¦ &B4EF ¦ 1    ¦ 0     9     o     i     l     k     m     j

&B63A ¦ &B4F0 ¦ 1    ¦ 8     7     u     y     h     j     n     Space

&B63B ¦ &B4F1 ¦ 1    ¦ Down  Up    Left  Right Fire2 Fire1 (Joystick 1)

&B63B ¦ &B4F1 ¦ 1    ¦ 6     5     r     t     g     f     b     v

&B63C ¦ &B4F2 ¦ 1    ¦ 4     3     e     w     s     d     c     x

&B63D ¦ &B4F3 ¦ 1    ¦ 1     2     Esc   q     Tab   a     Caps  z

&B63E ¦ &B4F4 ¦ 1    ¦ Down  Up    Left  Right Fire2 Fire1 (Joystick 2)

&B63E ¦ &B4F4 ¦ 1    ¦                                           Del

¦       ¦      ¦

&B63F ¦ &B4F5 ¦ 1    ¦ complement of &B635

&B640 ¦ &B4F6 ¦ 1    ¦ complement of &B636

&B641 ¦ &B4F7 ¦ 1    ¦ complement of &B637

&B642 ¦ &B4F8 ¦ 1    ¦ complement of &B638

&B643 ¦ &B4F9 ¦ 1    ¦ complement of &B639

&B644 ¦ &B4FA ¦ 1    ¦ complement of &B63A

&B645 ¦ &B4FB ¦ 1    ¦ complement of &B63B

&B646 ¦ &B4FC ¦ 1    ¦ complement of &B63C

&B647 ¦ &B4FD ¦ 1    ¦ complement of &B63D

&B648 ¦ &B4FE ¦ 1    ¦ complement of &B63E

------+-------+------+------------------------------------------

&B64B ¦ &B501 ¦      ¦

&B653 ¦ &B509 ¦ 1    ¦

&B654 ¦ &B5OA ¦ 1    ¦

&B655 ¦ &B50B ¦ 1    ¦

&B656 ¦ &B50C ¦ 1    ¦

&B657 ¦ &B50D ¦ 7    ¦ event block for Keyboard handling,

¦       ¦      ¦   comprises:

&B657 ¦ &B50D ¦ 2    ¦ chain address

&B659 ¦ &B50F ¦ 1    ¦ count

&B65A ¦ &B510 ¦ 1    ¦ class: express event

&B65B ¦ &B511 ¦ 2    ¦ ROM routine address: &C492

&B65D ¦ &B513 ¦ 1    ¦ ROM select number: &FD

&B65E ¦ &B514 ¦ 20*2 ¦ store for last keys pressed and each entry

¦       ¦      ¦   is as follows:

¦       ¦      ¦   byte 1: +0 to +10=key tables' line

¦       ¦      ¦   number; if bit 5 is set then Shift is

¦       ¦      ¦   pressed; bit 7=Control is pressed

¦       ¦      ¦   byte 2: b0 to b7=key tables' column

¦       ¦      ¦   number - see &B496 etc

&B67F ¦ &B67F ¦ 2    ¦ vestige from the 464?

&B686 ¦ &B53C ¦ 1    ¦

&B687 ¦ &B53D ¦ 1    ¦ accumulated count of the number of keys

¦       ¦      ¦   pressed (MOD 20)

&B688 ¦ &B53E ¦ 1    ¦ number of keys left in key buffer

&B689 ¦ &B53F ¦ 1    ¦ accumulated count of the number of keys

¦       ¦      ¦   removed from the buffer (MOD 20)

&B68A ¦ &B540 ¦ 1    ¦

&B68B ¦ &B541 ¦ 2    ¦ address of the normal key table

&B68D ¦ &B543 ¦ 2    ¦ address of the shifted key table

&B68F ¦ &B545 ¦ 2    ¦ address of the control key table

&B691 ¦ &B547 ¦ 2    ¦ address of the KB repeats table

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B692 ¦       ¦ 1    ¦

&B693 ¦ &B328 ¦ 2    ¦ ORIGIN x

&B695 ¦ &B32A ¦ 2    ¦ ORIGIN y

&B697 ¦ &B32C ¦ 2    ¦ graphics text x position (pixel)

&B699 ¦ &B32E ¦ 2    ¦ graphics text y position(pixel)

&B69B ¦ &B330 ¦ 2    ¦ graphics window x of one edge (pixel)

&B69D ¦ &B332 ¦ 2    ¦ graphics window x of other edge (pixel)

&B69F ¦ &B334 ¦ 2    ¦ graphics window y of one side (pixel)

&B6A1 ¦ &B336 ¦ 2    ¦ graphics window y of other side (pixel)

&B6A3 ¦ &B338 ¦ 1    ¦ GRAPHICS PEN

&B6A4 ¦ &B339 ¦ 1    ¦ GRAPHICS PAPER

------+-------+------+------------------------------------------

&B6A5 ¦ &B33A ¦ 8/14 ¦ (This area is 14 bytes on the 464) Used by

¦       ¦      ¦   line drawing (and other) routines, as

¦       ¦      ¦   follows:

&B6A7 ¦ &B33A ¦ 2    ¦ x+1()

&B6A9 ¦ &B33C ¦ 2    ¦ y/2+1()

&B6AB ¦ &B33E ¦ 2    ¦ y/2-x()

&B6AD ¦ &B340 ¦ 2    ¦

¦ &B342 ¦ 2    ¦

&B6AF ¦ &B344 ¦ 1    ¦

&B6B0 ¦ &B345 ¦ 1    ¦

&B6B1 ¦ &B346 ¦ 1    ¦

&B6B2 ¦       ¦ 1    ¦ first point on drawn line flag (<>0=to be

¦       ¦      ¦   plotted; 0=don't plot)

&B6B3 ¦       ¦ 1    ¦ line MASK

&B6B4 ¦       ¦ 1    ¦

¦ &B207 ¦ 2    ¦

&B6B5 ¦ &B20C ¦ 1    ¦ current stream number

------+-------+------+------------------------------------------

&B6B6 ¦ &B20D ¦ 14/15¦ (These areas are 15 bytes on the 464)

¦       ¦      ¦   Stream (window) 0 parameter block.

¦       ¦      ¦   These areas are arranged as &B726

&B6C4 ¦ &B21C ¦ 14/15¦ stream (window) 1 parameter block

&B6D2 ¦ &B22B ¦ 14/15¦ stream (window) 2 parameter block

&B6E0 ¦ &B23A ¦ 14/15¦ stream (window) 3 parameter block

&B6EE ¦ &B249 ¦ 14/15¦ stream (window) 4 parameter block

&B6FC ¦ &B258 ¦ 14/15¦ stream (window) 5 parameter block

&B70A ¦ &B267 ¦ 14/15¦ stream (window) 6 parameter block

&B718 ¦ &B276 ¦ 14/15¦ stream (window) 7 parameter block

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B726 ¦ &B285 ¦ 14/15¦ Current Stream (Window) parameter block:

&B726 ¦ &B285 ¦ 1    ¦ cursor y position (line) with respect to

¦       ¦      ¦   the whole screen (starting from 0)

&B727 ¦ &B286 ¦ 1    ¦ cursor x position (column) with respect to

¦       ¦      ¦   the whole screen (starting from 0)

&B728 ¦ &B287 ¦      ¦

&B729 ¦ &B288 ¦ 1    ¦ window top line (y) with respect to the

¦       ¦      ¦   whole screen (starting from 0)

&B72A ¦ &B289 ¦ 1    ¦ window left column (x) with respect to the

¦       ¦      ¦   whole screen (starting from 0)

&B72B ¦ &B28A ¦ 1    ¦ window bottom line (y) with respect to the

¦       ¦      ¦   whole screen (starting from 0)

&B72C ¦ &B28B ¦ 1    ¦ window right colwnn (x) with respect to

¦       ¦      ¦   the whole screen (starting from 0)

&B72D ¦ &B28C ¦ 1    ¦ scroll count

&B72E ¦ &B28D ¦ 1    ¦ cursor flag (&01=disable; &02=off; &FD=on;

¦       ¦      ¦   &FE=enable)

¦ &B28E ¦ 1    ¦

&B72F ¦ &B28F ¦ 1    ¦ current PEN number (encoded, not its INK

¦       ¦      ¦   number)

&B730 ¦ &B290 ¦ 1    ¦ current PAPER number (encoded, not its INK

¦       ¦      ¦   number)

&B731 ¦ &B291 ¦ 2    ¦ address of text background routine:

¦       ¦      ¦   opaque=&1392; transparent=&13A0

&B733 ¦ &B293 ¦ 1    ¦ graphics character writing flag (0=off;

¦       ¦      ¦   <>0=on)

&B734 ¦ &B294 ¦ 1    ¦ ASCII number of the first character in

¦       ¦      ¦   User Defined Graphic (UDG) matrix table

&B735 ¦ &B295 ¦ 1    ¦ UDG matrix table flag (&00=non-existent;

¦       ¦      ¦   &FF=present)

&B736 ¦ &B296 ¦ 2    ¦ address of UDG matrix table

&B738 ¦ &B298 ¦ 2    ¦

&B758 ¦ &B2B8 ¦ 1    ¦

&B759 ¦ &B2B9 ¦ 1    ¦

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B763 ¦ &B2C3 ¦ 32*3 ¦ Control Code handling routine table - each

¦       ¦      ¦   code's entry comprises:

¦       ¦      ¦   byte 1: +0 to +9=number of parameters;

¦       ¦      ¦   +&80=re-run routine at a System Reset

¦       ¦      ¦   bytes 2 and 3: address of the control

¦       ¦      ¦     code's handling routine

&B763 ¦ &B2C3 ¦ 3    ¦ ASC 0:  &80,&1513: NUL

&B766 ¦ &B2C6 ¦ 3    ¦ ASC 1:  &81,&1335: Print control code

¦       ¦      ¦     chararacter [,char]

&B769 ¦ &B2C9 ¦ 3    ¦ ASC 2:  &80,&1297: Disable cursor

&B76C ¦ &B2CC ¦ 3    ¦ ASC 3:  &80,&1286: Enable cursor

&B76F ¦ &B2CF ¦ 3    ¦ ASC 4:  &81,&0AE9: Set mode [,mode]

&B772 ¦ &B2D2 ¦ 3    ¦ ASC 5:  &81,&1940: Print character using

¦       ¦      ¦     graphics mode [,char]

&B775 ¦ &B2D5 ¦ 3    ¦ ASC 6:  &00,&1459: Enable VDU

&B778 ¦ &B2D8 ¦ 3    ¦ ASC 7:  &80,&14E1: Beep

&B77B ¦ &B2DB ¦ 3    ¦ ASC 8:  &80,&1519: Back-space

&B77E ¦ &B2DE ¦ 3    ¦ ASC 9:  &80,&151E: Step-right

&B781 ¦ &B2E1 ¦ 3    ¦ ASC 10: &80,&1523: Linefeed

&B784 ¦ &B2E4 ¦ 3    ¦ ASC 11: &80,&1528: Previous line

&B787 ¦ &B2E7 ¦ 3    ¦ ASC 12: &80,&154F: Clear window and locate

¦       ¦      ¦     the cursor at position  1,1

&B78A ¦ &B2EA ¦ 3    ¦ ASC 13: &80,&153F: RETURN

&B78D ¦ &B2ED ¦ 3    ¦ ASC 14: &81,&12AB: Set paper [,pen]

&B790 ¦ &B2F0 ¦ 3    ¦ ASC 15: &81,&12A6: Set pen [,pen]

&B793 ¦ &B2F3 ¦ 3    ¦ ASC 16: &80,&155E: Delete the character at

¦       ¦      ¦     the cursor position

&B796 ¦ &B2F6 ¦ 3    ¦ ASC 17: &80,&1599: Clear the line up to

¦       ¦      ¦     the current cursor position

&B799 ¦ &B2F9 ¦ 3    ¦ ASC 18: &80,&158F: Clear from the cursor

¦       ¦      ¦     position to the end of the line

&B79C ¦ &B2FC ¦ 3    ¦ ASC 19: &80,&1578: Clear from start of the

¦       ¦      ¦     window to the cursor position

&B79F ¦ &B2FF ¦ 3    ¦ ASC 20: &80,&1565: Clear from the cursor

¦       ¦      ¦     position to the end of a window

&B7A2 ¦ &B302 ¦ 3    ¦ ASC 21: &80,&1452: Disable VDU

&B7A5 ¦ &B305 ¦ 3    ¦ ASC 22: &81,&14EC: Set text write mode

¦       ¦      ¦     [,mode]

&B7A8 ¦ &B308 ¦ 3    ¦ ASC 23: &81,&0C55: Set graphics draw mode

¦       ¦      ¦     [,mode]

&B7AB ¦ &B30B ¦ 3    ¦ ASC 24: &80,&12C6: Exchange pen and paper

&B7AE ¦ &B30E ¦ 3    ¦ ASC 25: &89,&150D: Define user defined

¦       ¦      ¦     character [,char,8 rows of char]

&B7B1 ¦ &B311 ¦ 3    ¦ ASC 26: &84,&1501: Define window

¦       ¦      ¦     [,left,right,top,bottom]

&B7B4 ¦ &B314 ¦ 3    ¦ ASC 27: &00,&14EB: ESC (=user)

&B7B7 ¦ &B317 ¦ 3    ¦ ASC 28: &83,&14F1: Set the pen inks

¦       ¦      ¦     [,pen,ink 1,ink 2]

&B7BA ¦ &B31A ¦ 3    ¦ ASC 29: &82,&14FA: Set border colours

¦       ¦      ¦     [,ink,ink2]

&B7BD ¦ &B31D ¦ 3    ¦ ASC 30: &80,&1539: Locate the text cursor

¦       ¦      ¦     at position 1,1

&B7C0 ¦ &B320 ¦ 3    ¦ ASC 31: &82,&1547: Locate the text cursor

¦       ¦      ¦     at [,column,line]

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B7C3 ¦ &B1C8 ¦ 1    ¦ MODE number

&B7C4 ¦ &B1C9 ¦ 2    ¦ screen offset

&B7C6 ¦ &B1CB ¦ 1    ¦ screen base HB (LB taken as &00)

&B7C7 ¦ &B1CC ¦ 3    ¦ graphics VDU write mode indirection -

¦       ¦      ¦   JP &0C74

¦ &B1CF ¦ 8    ¦ list of bytes having only one bit set,

¦       ¦      ¦   from b7 down to b0

&B7D2 ¦ &B1D7 ¦ 1    ¦ first flash period (SPEED INK - default

¦       ¦      ¦   &0A @ 0.02 seconds)

&B7D3 ¦ &B1D8 ¦ 1    ¦ second flash period (SPEED INK - default

¦       ¦      ¦   &0A @ 0.02 seconds)

------+-------+------+------------------------------------------

&B7D4 ¦ &B1D9 ¦ 1+16 ¦ Border and Pens' First Inks (as hardware

¦       ¦      ¦   numbers):

&B7D4 ¦ &B1D9 ¦ 1    ¦ hw &04 = sw 1    (blue)            border

&B7D5 ¦ &B1DA ¦ 1    ¦ hw &04 = sw 1    (blue)            pen 0

&B7D6 ¦ &B1DB ¦ 1    ¦ hw &0A = sw 24   (bright yellow)   pen 1

&B7D7 ¦ &B1DC ¦ 1    ¦ hw &13 = sw 20   (bright cyan)     pen 2

&B7D8 ¦ &B1DD ¦ 1    ¦ hw &0C = sw 6    (bright red)      pen 3

&B7D9 ¦ &B1DE ¦ 1    ¦ hw &0B = sw 26   (bright white)    pen 4

&B7DA ¦ &B1DF ¦ 1    ¦ hw &14 = sw 0    (black)           pen 5

&B7DB ¦ &B1E0 ¦ 1    ¦ hw &15 = sw 2    (bright blue)     pen 6

&B7DC ¦ &B1E1 ¦ 1    ¦ hw &0D = sw 8    (bright magenta)  pen 7

&B7DD ¦ &B1E2 ¦ 1    ¦ hw &06 = sw 10   (cyan)            pen 8

&B7DE ¦ &B1E3 ¦ 1    ¦ hw &1E = sw 12   (yellow)          pen 9

&B7DF ¦ &B1E4 ¦ 1    ¦ hw &1F = sw 14   (pale blue)       pen 10

&B7E0 ¦ &B1E5 ¦ 1    ¦ hw &07 = sw 16   (pink)            pen 11

&B7E1 ¦ &B1E6 ¦ 1    ¦ hw &12 = sw 18   (bright green)    pen 12

&B7E2 ¦ &B1E7 ¦ 1    ¦ hw &19 = sw 22   (pale green)      pen 13

&B7E3 ¦ &B1E8 ¦ 1    ¦ hw &04 = sw 1    (blue)            pen 14

&B7E4 ¦ &B1E9 ¦ 1    ¦ hw &17 = sw 11   (sky blue)        pen 15

------+-------+------+------------------------------------------

&B7E5 ¦ &B1EA ¦ 1+16 ¦ Border and Pens' Second Inks (as hardware

¦       ¦      ¦   numbers):

&B7E5 ¦ &B1EA ¦ 1    ¦ hw &04 = sw 1    (blue)            border

&B7E6 ¦ &B1EB ¦ 1    ¦ hw &04 = sw 1    (blue)            pen 0

&B7E7 ¦ &B1EC ¦ 1    ¦ hw &0A = sw 24   (bright yellow)   pen 1

&B7E8 ¦ &B1ED ¦ 1    ¦ hw &13 = sw 20   (bright cyan)     pen 2

&B7E9 ¦ &B1EE ¦ 1    ¦ hw &0C = sw 6    (bright red)      pen 3

&B7EA ¦ &B1FF ¦ 1    ¦ hw &0B = sw 26   (bright white)    pen 4

&B7EB ¦ &B1F0 ¦ 1    ¦ hw &14 = sw 0    (black)           pen 5

&B7EC ¦ &B1F1 ¦ 1    ¦ hw &15 = sw 2    (bright blue)     pen 6

&B7ED ¦ &B1F2 ¦ 1    ¦ hw &0D = sw 8    (bright magenta)  pen 7

&B7EE ¦ &B1F3 ¦ 1    ¦ hw &06 = sw 10   (cyan)            pen 8

&B7EF ¦ &B1F4 ¦ 1    ¦ hw &1E = sw 12   (yellow)          pen 9

&B7F0 ¦ &B1F5 ¦ 1    ¦ hw &1F = sw 14   (pale blue)       pen 10

&B7F1 ¦ &B1F6 ¦ 1    ¦ hw &07 = sw 16   (pink)            pen 11

&B7F2 ¦ &B1F7 ¦ 1    ¦ hw &12 = sw 18   (bright green)    pen 12

&B7F3 ¦ &B1F8 ¦ 1    ¦ hw &19 = sw 22   (pale green)      pen 13

&B7F4 ¦ &B1F9 ¦ 1    ¦ hw &04 = sw 1    (bright yellow)   pen 14

&B7F5 ¦ &B1FA ¦ 1    ¦ hw &17 = sw 11   (pink)            pen 15

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B7F6 ¦ &B1FB ¦ 1    ¦

&B7F7 ¦ &B1FC ¦ 1    ¦

&B7F8 ¦ &B1FD ¦ 1    ¦

&B7F9 ¦ &BlFE ¦ 2    ¦

&B7FB ¦ &B200 ¦ 2    ¦

&B7FD ¦       ¦      ¦

&B802 ¦       ¦ 1+1  ¦

&B804 ¦       ¦ 1    ¦ number of entries in the Printer

¦       ¦      ¦   Translation Table (normally 10)

&B805 ¦       ¦ 20*2 ¦ Printer Translation Table; each entry

¦       ¦      ¦   comprises:

¦       ¦      ¦   byte 1: screen code

¦       ¦      ¦   byte 2: pnnter code

&B805 ¦       ¦ 2    ¦ screen &A0 printer &5E (acute accent)

&B807 ¦       ¦ 2    ¦ screen &A1 printer &5C (\)

&B809 ¦       ¦ 2    ¦ screen &A2 printer &7B ({)

&B80B ¦       ¦ 2    ¦ screen &A3 printer &23 (#)

&B80D ¦       ¦ 2    ¦ screen &A6 printer &40 (@)

&B80F ¦       ¦ 2    ¦ screen &AB printer &7C (|)

&B811 ¦       ¦ 2    ¦ screen &AC printer &7D (})

&B813 ¦       ¦ 2    ¦ screen &AD printer &7E (~)

&B815 ¦       ¦ 2    ¦ screen &AE printer &5D (])

&B817 ¦       ¦ 2    ¦ screen &AF printer &SE ([)

&B819 ¦       ¦ 20   ¦ room for ten more translations

------+-------+------+------------------------------------------

&B82D ¦ &B100 ¦ 1    ¦

&B82E ¦ &B101 ¦ 1    ¦

&B82F ¦ &B102 ¦ 2    ¦

&B831 ¦ &B104 ¦ 1    ¦

&B832 ¦ &B105 ¦ 2    ¦ temporary store for stack pointer (SP)

¦       ¦      ¦   during interrupt handling

&B834 ¦ &B107 ¦ &70  ¦ temporary machine stack (from &B8B3

¦       ¦      ¦   downwards) during interrupt handling

&B8B4 ¦ &B187 ¦ 4    ¦ TIME (stored with the LB first - four

¦       ¦      ¦   bytes give >166 days; three bytes give

¦       ¦      ¦   >15 hours)

&B8B8 ¦ &B18B ¦ 1    ¦

&B8B9 ¦ &B18C ¦ 2    ¦

&B8BB ¦ &B18E ¦ 2    ¦

&B8BD ¦ &B190 ¦ 2    ¦ address of the first ticker block in chain

¦       ¦      ¦   (if any)

&B8BF ¦ &B192 ¦ 1    ¦ Keyboard scan flag (&00=scan not needed;

¦       ¦      ¦   &01=scan needed)

&B8C0 ¦ &B193 ¦ 2    ¦ address of the first event block in chain

¦       ¦      ¦   (if any)

&B8C2 ¦ &B195 ¦ 1    ¦

&B8C3 ¦ &B196 ¦ &10  ¦ buffer for last RSX or RSX command name

¦       ¦      ¦   (last character has bit 7 set)

&B8D3 ¦ &B1A6 ¦ 2    ¦ address of first ROM or RSX chaining block

¦       ¦      ¦   in chain

&B8D5 ¦       ¦ 1    ¦ RAM bank number

&B8D6 ¦ &B1A8 ¦ 1    ¦ Upper ROM status (eg select number)

&B8D7 ¦ &B1A9 ¦ 2    ¦ entry point of foreground ROM in use (eg

¦       ¦      ¦   &C006 for BASIC ROM)

&B8D9 ¦ &B1AB ¦ 1    ¦ foreground ROM select address (0 for the

¦       ¦      ¦   BASIC ROM)

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&B8DA ¦       ¦ 16*2 ¦ ROM entry IY value (ie the address table)

¦       ¦      ¦   - the 6128 has ROMs numbered from 0 to

¦       ¦      ¦   15:

¦ &B1AC ¦ 7*2  ¦ ROM entry IY value (ie the address table)

¦       ¦      ¦   -the 464 has ROMs 1 to 7 only:

&B8DA ¦       ¦ 2    ¦ ROM 0  IY (not for the 464)

&B8DC ¦ &B1AC ¦ 2    ¦ ROM 1  IY

&B8DE ¦ &B1AE ¦ 2    ¦ ROM 2  IY

&B8E0 ¦ &B1B0 ¦ 2    ¦ ROM 3  IY

&B8E2 ¦ &B1B2 ¦ 2    ¦ ROM 4  IY

&B8E4 ¦ &B1B4 ¦ 2    ¦ ROM 5  IY

&B8E6 ¦ &B1B6 ¦ 2    ¦ ROM 6  IY

&B8E8 ¦ &B1B8 ¦ 2    ¦ ROM 7  IY (usually &A700 for AMSDOS/CPM

¦       ¦      ¦   ROM)

&B8EA ¦       ¦ 2    ¦ ROM 8  IY (not 464)

&B8EC ¦       ¦ 2    ¦ ROM 9  IY (not 464)

&B8EE ¦       ¦ 2    ¦ ROM 10 IY (not 464)

&B8F0 ¦       ¦ 2    ¦ ROM 11 IY (not 464)

&B8F2 ¦       ¦ 2    ¦ ROM 12 IY (not 464)

&B8F4 ¦       ¦ 2    ¦ ROM 13 IY (not 464)

&B8F6 ¦       ¦ 2    ¦ ROM 14 IY (not 464)

&B8F8 ¦       ¦ 2    ¦ ROM 15 IY (not 464)

&B8FA ¦       ¦ 6    ¦ 6  bytes of &FF

¦ &B1BA ¦ 14   ¦ 14 bytes of &00

------+-------+------+------------------------------------------

&B900 ¦ &B900 ¦ 12*3 ¦ High Kernel Jumpblock (on the 464 this

¦       ¦      ¦   block is 11*3 bytes in size)

&B924 ¦ &B921 ¦ &1C0 ¦ routines used by the High Kernel

¦       ¦      ¦   Jumpblock (on the 464 this is &1C8 bytes

¦       ¦      ¦   in size)

&BAE4 ¦ &BAE9 ¦      ¦ bytes of &FF (&1C bytes on 6128, &17 bytes

¦       ¦      ¦   on 464)

&BB00 ¦ &BB00 ¦ 26*3 ¦ Key Manager Jumpblock

&BB4E ¦ &BB4E ¦ 36*3 ¦ Text VDU Jumpblock

&BBBA ¦ &BBBA ¦ 23*3 ¦ Graphics VDU Jumpblock

&BBFF ¦ &BBFF ¦ 34*3 ¦ Screen Pack Jumpblock

&BC65 ¦ &BC65 ¦ 22*3 ¦ Cassette (and Disc if fitted) Manager

¦       ¦      ¦   Jumpblock

&BCA7 ¦ &BCA7 ¦ 11*3 ¦ Sound Manager Jumpblock

&BCC8 ¦ &BCC8 ¦ 25*3 ¦ Kernel Jumpblock

&BD13 ¦ &BD13 ¦ 26*3 ¦ Machine Pack Jumpblock (on the 464 this

¦       ¦      ¦   block is 14*3 bytes in size)

&BD61 ¦ &BD3D ¦ 32*3 ¦ Maths Jumpblock (on the 464 this block is

¦       ¦      ¦   48*3 bytes in size)

&BDCD ¦ &BDCD ¦ 14*3 ¦ Firmware Indirections (on the 464 this

¦       ¦      ¦   block is 13*3 bytes in size

&BDF7 ¦ &BDF4 ¦      ¦ bytes of &00 (&09 bytes on 6128, &0C bytes

¦       ¦      ¦   on the 464) the lower limit of Machine

¦       ¦      ¦   Stack if no Disc Drive

&BE00 ¦ &BE00 ¦ &40  ¦ &40 bytes of &FF

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&BE40 ¦ &BE40 ¦ &4x  ¦ used by the AMSDOS ROM if a disc drive is

¦       ¦      ¦   fitted (otherwise &4x bytes of &FF)

&BE40 ¦ &BE40 ¦ 2    ¦ (address &A910)

&BE42 ¦ &BE42 ¦ 2    ¦ address of drive A XDPB

&BE44 ¦ &BE44 ¦ 9    ¦ Disc Set Up timing block:

&BE44 ¦ &BE44 ¦ 2    ¦ motor on period (default &0032; fastest

¦       ¦      ¦   &0023 @ 20mS)

&BE46 ¦ &BE46 ¦ 2    ¦ motor off period (default &00FA; fastest

¦       ¦      ¦   &00C8 @ 20mS)

&BE48 ¦ &BE48 ¦ 1    ¦ write current off period (default &AF

¦       ¦      ¦   @ 10µS)

&BE49 ¦ &BE49 ¦ 1    ¦ head settle time (default &0F @ 1mS)

&BE4A ¦ &BE4A ¦ 1    ¦ step rate period (default &0C; fastest &0A

¦       ¦      ¦   @ 1mS)

&BE4B ¦ &BE4B ¦ 1    ¦ head unload delay (default &01)

&BE4C ¦ &BE4C ¦ 1    ¦ b0=non DMA mode; b1 to b7=head load delay

¦       ¦      ¦   (default &03)

&BE4D ¦ &BE4D ¦ 2    ¦

&BE4F ¦ &BE4F ¦ 1    ¦ Drive Header Information Block:

&BE4F ¦ &BE4F ¦ 1    ¦ last track used

&BE50 ¦ &BE50 ¦ 1    ¦ head number (&00)

&BE51 ¦ &BE51 ¦ 1    ¦ last sector used

&BE52 ¦ &BE52 ¦ 1    ¦ log2(sector size)-7

&BE53 ¦ &BE53 ¦ 1    ¦

&BE54 ¦ &BE54 ¦ 1    ¦

&BE55 ¦ &BE55 ¦ 1    ¦

&BE56 ¦ &BE56 ¦ 1    ¦

&BE58 ¦ &BE58 ¦ 1    ¦

&BE59 ¦ &BE59 ¦ 1    ¦

&BE5D ¦ &BE5D ¦ 1    ¦

&BE5E ¦ &BE5E ¦ 1    ¦

&BE5F ¦ &BE5F ¦ 1    ¦ disc motor flag (&00=off;&01=on

¦       ¦      ¦   - strangely reversed)

&BE60 ¦ &BE60 ¦ 2    ¦ address of buffer for directory entries

¦       ¦      ¦   block (&A930)

&BE62 ¦ &BE62 ¦ 2    ¦ as &BE76 (ie&A9B0)

&BE64 ¦ &BE64 ¦ 2    ¦

&BE66 ¦ &BE66 ¦ 1    ¦ disc retries (default &10)

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&BE67 ¦ &BE67 ¦ &11  ¦ AMSDOS Ticker and Event Block:

&BE67 ¦ &BE67 ¦ 2    ¦ ticker chaining address

&BE69 ¦ &BE69 ¦ 2    ¦ tick count

&BE6B ¦ &BE6B ¦ 2    ¦ recharge count

&BE6D ¦ &BE6D ¦ 2    ¦ event chaining address

&BE6F ¦ &BE6F ¦ 1    ¦ count

&BE70 ¦ &BE70 ¦ 1    ¦ class (asynchronous event)

&BE71 ¦ &BE71 ¦ 2    ¦ ROM routine address (&C9D6)

&BE73 ¦ &BE73 ¦ 1    ¦ ROM select number (&07 ie the AMSDOS/CPM

¦       ¦      ¦   ROM)

&BE74 ¦ &BE74 ¦ 1    ¦ last sector number used

&BE75 ¦ &BE75 ¦ 1    ¦

&BE76 ¦ &BE76 ¦ 2    ¦ address of ½K buffer, or of header info

¦       ¦      ¦   block (for WRlTE SECTOR etc)

&BE78 ¦ &BE78 ¦ 1    ¦ disc error message flag (&00=on; &FF=off

¦       ¦      ¦   - reversed again)

&BE7D ¦ &BE7D ¦ 2    ¦ address of AMSDOS reserved area (&A700)

&BE7F ¦ &BE7F ¦ x    ¦ area used by AMSDOS to copy routines into

¦       ¦      ¦   RAM for running

&BE80 ¦ &BE80 ¦ &80  ¦ &80 bytes of &FF (limit of machine stack

¦       ¦      ¦   if disc drive fitted)

&BF00 ¦ &BF00 ¦ xy   ¦ &xy bytes of &00)

&BFxy ¦       ¦      ¦ machine stack (in theory this stack could

¦       ¦      ¦   extend down much further)

&BFFF ¦ &BFFF ¦      ¦ upper limit of machine stack

The area from &C000 to &FFFF  is  taken  up by the screen memory -

the layout of which is  illustrated  on  the next page.  Printed below

are diagrams which show how the CPC uses the bytes of screen memory in

the different MODEs.  For each byte:

in MODE 2 (where there are two colours only, each pixel needs only one

bit - either on or off)

bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0

p0   p1   p2   p3   p4   p5   p6   p7

(the pixels are arranged with p0 being the leftmost one, etc)

in MODE 1 (where four colours are available and so two bits are needed

for each pixel - 1 byte represents 4 pixels)

bit7  bit6  bit5  bit4  bit3  bit2  bit1  bit0

p0(1) p1(1) p2(1) p3(1) p0(0) p1(0) p2(0) p3(0)

(each pixel is twice as wide as in MODE 2)

in MODE 0 (where sixteen colours are possible and four bits are needed

for each pixel - 1 byte represents 2 pixels)

bit7  bit6  bit5  bit4  bit3  bit2  bit1  bit0

p0(0) p1(0) p0(2) p1(2) p0(1) p1(1) p0(3) p1(3)

(each pixel is four times as wide as in MODE 2)

NB: the numbers in brackets show  which  bit of the pixel's pen number

the screen byte bit refers  to.   For  example  in  MODE 1, the 4 most

significant bits of the byte hold bit  1  of the pixel's pen value and

the 41east signifcant bits hold bit 0 of the pen value.

6128  ¦ 464   ¦ Size ¦ Comments on the memory locations

------+-------+------+------------------------------------------

&C000   &C000   &4000  normal (upper) screen area.  The

alternative (lower) screen area is from

&4000 to &7FFF.  The addresses of the

starts of lines and rows in the normal

screen area after a MODE instruction

are:

LINE    R0W0   R0W1   R0W2   R0W3   R0W4   R0W5   R0W6   R0W7

1       C000   C800   D000   D800   E000   E800   F000   F800

2       C050   C850   D050   D850   E050   E850   F050   F850

3       C0A0   C8A0   D0A0   D8A0   E0A0   E8A0   F0A0   F8A0

4       C0F0   C8F0   D0F0   D8F0   E0F0   E8F0   F0F0   F8F0

5       C140   C940   D140   D940   E140   E940   F140   F940

6       C190   C990   D190   D990   E190   E990   F190   F990

7       C1E0   C9E0   D1E0   D9E0   E1E0   E9E0   F1E0   F9E0

8       C230   CA30   D230   DA30   E230   EA30   F230   FA30

9       C280   CA80   D280   DA80   E280   EA80   F280   FA80

10      C2D0   CAD0   D2D0   DAD0   E2D0   EAD0   F2D0   FAD0

11      C320   CB20   D320   DB20   E320   EB20   F320   FB20

12      C370   CB70   D370   DB70   E370   EB70   F370   FB70

13      C3C0   CBC0   D3C0   DBC0   E3C0   EBC0   F3C0   FBC0

14      C410   CC10   D410   DC10   E410   EC10   F410   FC10

15      C460   CC60   D460   DC60   E460   EC60   F460   FC60

16      C4B0   CCB0   D4B0   DCB0   E4B0   ECB0   F4B0   FCB0

17      C500   CD00   D500   DD00   E500   ED00   F500   FD00

18      C550   CD50   D550   DD50   E550   ED50   F550   FD50

19      C5A0   CDA0   D5A0   DDA0   E5A0   EDA0   F5A0   FDA0

20      C5F0   CDF0   D5F0   DDF0   E5F0   ED50   F550   FD50

21      C640   CE40   D640   DE40   E640   EE40   F640   FE40

22      C690   CE90   D690   DE90   E690   EE90   F690   FE90

23      C6E0   CEE0   D6E0   DEE0   E6E0   EEE0   F6E0   FEE0

24      C730   CF30   D730   DF30   E730   EF30   F730   FF30

25      C780   CF80   D780   DF80   E780   EF80   F780   FF80

spare start C7D0   CFD0   D7D0   DFD0   E7D0   EFD0   F7D0   FFD0

spare end  C7FF   CFFF   D7FF   DFFF   E7FF   EFFF   F7FF   FFFF

Once the whole screen  has  been  scrolled  in any direction, the

above table will  become  incorrect.   On  scrolling,  all  the above

addresses will have an offset (MOD &800) added, derived as follows:

+&02 per scroll to the left (=2, 1 or ½ character in MODE 2, MODE 1

or MODE 0 respectively)

-&02 per scroll to the right (=2, 1 or ½ character in MODE 2, MODE 1

or MODE 0 respectively)

+&50 per scroll up one line

-&50 per scroll down one line

If  scrolled  far  enough,  a  screen  row  may  sit  across  the

boundaries of the screen memory area, whose bottom end will then wrap

around to join up with the  top  (ie  byte  &FFFF will be followed by

byte &C000 assuming the  normal  screen  area).   If before scrolling

however, a window had been set up  smaller than the whole screen then

the table will remain accurate despite any scrolling.

The 'spare' areas of  screen  memory  are  filled  with  bytes of the

relevant PAPER value each time there  is  a  full screen CLS, and are

not really available for other uses.  After scrolling the spare areas

may be used as screen with other bytes becoming spare.

FIRMWARE SUMMARY

The Firmware Jumpblock is the  recommended method of communicating

with the routines in the lower  ROM  -  it  is  used by BASIC, and it

should also be used  by  other  programs.  The  reason  for using the

jumpblock is that  the  routines  in  the  lower  ROM  are located at

different positions on the  different  machines.   The entries in the

jumpblock, however, are all in the  same  place - the instructions in

the jumpblock redirect the computer to the correct place in the lower

ROM.  Thus, providing a program uses the jumpblock, it should work on

any CPC computer. By altering  the  firmware jumpblock it is possible

to make the computer run a different routine from normal.  This could

either be a different routine in the lower or upper ROM, or a routine

written by the user - this is  known as 'patching the jumpblock'.  It

is worth noting that because BASIC  uses the firmware jumpblock quite

heavily, it is possible to  alter  the  effect of BASIC commands. The

following example will change  the  effect  of  calling  SCR SET MODE

(&BCOE) - instead of changing  the  mode,  any calls to this location

will print the letter 'A'.  The first  thing to do is to assemble the

piece of code that will be used to print the letter - this is printed

below and starts at &4000.

ORG &4000

LD A,65      ; 65 IS ASCII FOR 'A'

CALL &BB5A   ; TXT OUTPUT

RET          ; RETURN FROM SUBROUTINE

The jumpblock entry for SCR  SET  MODE  is  now patched so that it

reroutes all calls to &BC0E away from the lower ROM and to our custom

routine at &4000.  This is done by changing the bytes at &BC0E, &BC0F

and &BC10 to &C3, &00, &40 respectively  (ie JP &4000).  Any calls to

&BC0E or MODE  commands  will  now  print  the  letter  A  instead of

changing mode.  The indirections jumpblock contains a small number of

routines which are called by the  rest  of the firmware.  By altering

this jumpblock, it is possible to alter the way in which the firmware

operates on a large scale - thus  it is not always necessary to patch

large numbers of entries  in  the  firmware  jumpblock. There are two

jumpblocks which are to do with the Kemel (ie the high and low Kernel

jumpblocks).  The high jumpblock allows  ROM states and interrupts to

be altered, and also  controls  the  introduction  of  RSXs.  The low

jumpblock contains general  routines  and  restart instructions which

are used by the computer for its own purposes.

THE FIRMWARE IN DETAIL

Low Kernel Jumpblock

000   &0000   RESET ENTRY (RST 0)

Action: Resets the computer as if it has just been switched on

Entry:  No entry conditions

Exit:   This routine is never returned from

Notes:  After initialisation  of  the  hardware  and  firmware,

control is handed over to ROM 0 (usually BASIC)

001   &0008   LOW JUMP (RST 1)

Action: Jumps to a routine in either the lower ROM or low RAM

Entry:  No entry conditions - all  the  registers are passed to

the destination routine unchanged

Exit:   The registers are as set  by  the  routine in the lower

ROM or RAM or are returned unaltered

Notes:  The RST 1 instruction  is  followed  by  a two byte low

address, which is defmed as follows:

if bit 15 is set, then the upper ROM is disabled

if bit 14 is set, then the lower ROM is disabled

bits 13 to 0 contain the address of the routine to

jump to

This command is used by the  majority of entries in the

main firmware jumpblock

002   &000B   KL LOW PCHL

Action: Jumps to a routine in either the lower ROM or low RAM

Entry:  HL contains the low  address  -  all  the registers are

passed to the destination routine unchanged

Exit:   The registers are as set  by  the  routine in the lower

ROM or RAM or are returned unaltered

Notes:  The two byte low  address  in  the  HL register pair is

defined as follows:

if bit 15 is set, then the upper ROM is disabled

if bit 14 is set, then the lower ROM is disabled

bits 13 to 0 contain the address of the routine to

jump to

003   &000E   PCBC INSTRUCTION

Action: Jumps to the specified address

Entry:  BC contains the address to jump  to - all the registers

are passed to the destination routine unaltered

Exit:   The registers are as set  by the destination routine or

are returned unchanged

004   &0010   SIDE CALL (RST 2)

Action: Calls a routine  in  ROM,  in  a  group  of  up to four

foreground ROMs

Entry:  No entry conditions - all  the  registers apart from IY

are passed to the destination routine unaltered

Exit:   IY is corrupt, and the  other  registers  are as set by

the destination routine or are returned unchanged

Notes:  The RST 2 instruction is  followed  by  a two byte side

address, which is defined as follows:

bits 14 and 15 give a number between 0 and 3, which

is added to the main foreground ROM select address

- this is then used as the ROM select address

bits 0 to 13 contain the address to which is added

&C000 - this gives the address of the routine to be

called

005   &0013   KL SIDE PCHL

Action: Calls a routine in another ROM

Entry:  HL contains the side address  - all the registers apart

from IY are passed to the destination routine unaltered

Exit:   IY is corrupt, and the  other  registers  are as set by

the destination routine or are returned unchanged

Notes:  The two byte side address is defined as follows:

bits 14 and 15 give a number between 0 and 3, which

is added to the main foreground ROM select address

- this is then used as the ROM select address

bits 0 to 13 contain the address to which is added

&C000 - this gives the address of the routine to be

called

006   &0016   PCDE INSTRUCTION

Action: Jumps to the specified address

Entry:  DE contains the address to jump  to - all the registers

are passed to the destination routine unaltered

Exit:   The registers are as set  by the destination routine or

are returned unchanged

007   &0018   FAR CALL (RST 3)

Action: Calls a routine anywhere in ROM or ROM

Entry:  No entry conditions - all  the  registers apart from IY

are passed to the destination routine unaltered

Exit:   IY is preserved, and the other  registers are as set by

the destination routine or are returned unchanged

Notes:  The RST 3 instruction is followed by a two byte in-line

address.  At this address,  there  is  a three byte far

address, which is defined as follows:

bytes 0 and 1 give the address of the routine to be

called

byte 2 is the ROM select byte which has values as

follows:

&00 to &FB-- select the given upper ROM, enable the

upper ROM and disable the lower ROM

&FC - no change to the ROM selection, enable the

upper and lower ROMs

&FD - no change to the ROM selection, enable the

upper ROM and disable the lower ROM

&FE - no change to the ROM selection, disable the

upper ROM and enable the lower ROM

&FF - no change to the ROM selection, disable the

upper and lower ROMs

When it is retumed  from,  the  ROM selection and state

are restored to their settings before the RST 3 command

008   &001B   KL FAR PCHL

Action: Calls a routine, given by  the  far  address in HL & C,

anywhere in RAM or ROM

Entry:  HL holds the address of the routine to be called, and C

holds the ROM select  byte  -  all  the registers apart

from IY are passed to the destination routine unaltered

Exit:   IY is preserved, and the other  registers are as set by

the destination routine or are returned unchanged

Notes:  See FAR CALL (RST 3) above  for more details on the ROM

select byte

009   &001E   PCHL INSTRUCTION

Action: Jumps to the specified address

Entry:  HL contains the address to jump  to - all the registers

are passed to the destination routine unaltered

Exit:   The registers are as set  by the destination routine or

are returned unchanged

010   &0020   RAM LAM

Action: Puts the contents of a  RAM  memory location into the A

register

Entry:  HL contains the address of the memory location

Exit:   A holds the contents  of  the  memory location, and all

other registers are preserved

Notes:  This routine always reads from  RAM,  even if the upper

or lower ROM is enabled

011   &0023   KL FAR CALL

Action: Calls a routine anywhere in RAM or ROM

Entry:  HL holds the address of the three byte far address that

is to be used -  all  the  registers  apart from IY are

passed to the destination routine unaltered

Exit:   IY is preserved, and the other  registers are as set by

the destination routine or are returned unchanged

Notes:  See FAR CALL above for  more  details on the three byte

far address

012   &0028   FIRM JUMP (RST 5)

Action: Jumps to a  routine  in  either  the  lower  ROM or the

central 32K of RAM

Entry:  No entry conditions - all  the  registers are passed to

the destination routine unchanged

Exit:   The registers are as set  by  the  routine in the lower

ROM or RAM or are returned unaltered

Notes:  The RST  5  instruction  is  followed  by  a  two  byte

address, which is the  address  to  jump to; before the

jump is made, the lower ROM is enabled, and is disabled

when the destination routine is returned from

013   &0030   USER RESTART (RST 6)

Action: This is an RST instruction that may be set aside by the

user for any purpose

Entry:  Defined by the user

Exit:   Defined by the user

Notes:  The bytes from &0030  to  &0037  are  available for the

user to put their own code in if they wish

014   &0038   INTERRUPT ENTRY (RST 7)

Action: Deals with normal interrupts

Entry:  No entry conditions

Exit:   All registers are preserved

Notes:  The RST 7 instruction must not be used by the user; any

external interrupts that are  generated  by hardware on

the expansion  port  will  be  dealt  with  by  the EXT

INTERRUPT routine (see  Low  Kernel  Jumpblock, routine

015)

015   &003B   EXT INTERRUPT

Action: This area  is  set  aside  for  dealing  with  external

interrupts that are generated by any extra hardware

Entry:  No entry conditions

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  If any external hardware  is  going to generate interr-

upts, then the user must  patch  the area from &003B to

&003F so that the computer  can  deal with the external

interrupt; when an external interrupt occurs, the lower

ROM is disabled and the  code  at  &003B is called; the

default external  interrupt  routine  at  &003B  simply

returns, and  this  will  cause  the  computer  to hang

because the interrupt will continue to exist

High Kernel Jumpblock

000   &B900   KL U ROM ENABLE

Action: Enables the current upper ROM

Entry:  No entry conditions

Exit:   A contains the previous state of the ROM, the flags are

corrupt, and all other registers are preserved

Notes:  After this routine has  been  called,  all reading from

addresses between &C000 and  &FFFF  refers to the upper

ROM, and not the top  16K  of  RAM which is usually the

screen memory; any  writing  to  these  addresses still

affects the  RAM  as,  by  its  nature,  ROM  cannot be

written to

001   &B903   KL U ROM DISABLE

Action: Disables the upper ROM

Entry:  No entry conditions

Exit:   A contains the previous state of the ROM, the flags are

corrupt, and all other registers are preserved

Notes:  After this routine has  been  called,  all reading from

addresses between &C000 and ~F~FF refers to the top 16K

of RAM which is usually the screen memory

002   &B906   KL L ROM ENABLE

Action: Enables the lower ROM

Entry:  No entry conditions

Exit:   A contains the previous state of the ROM, the flags are

corrupt, and all other registers are preserved

Notes:  After this routine has  been  called,  all reading from

addresses between &0000 and  &4000  refers to the lower

ROM, and not the  bottom  16K  of  RAM;  any writing to

these addresses still affects the  RAM  as a ROM cannot

be written to; the  lower  ROM is automatically enabled

when a firmware routine is called, and is then disabled

when the routine returns

003   &B909   KL L ROM DISABLE

Action: Disables the lower ROM

Entry:  No entry conditions

Exit:   A contains the previous state of the ROM, the flags are

corrupt, and all other registers are preserved

Notes:  After this routine has  been  called,  all reading from

addresses between &0000 and &4000  refers to the bottom

16K of RAM; the lower ROM is automatically enabled when

a firmware routine is called, and is then disabled when

the routine returns

004   &B90C   KL ROM RESTORE

Action: Restores the ROM to its previous state

Entry:  A contains the previous state of the ROM

Exit:   AF is corrupt, and all other registers are preserved

Notes:  The previous four routines all  return  values in the A

register which are suitable for use by KL ROM RESTORE

005   &B90F   KL ROM SELECT

Action: Selects an upper ROM and also enables it

Entry:  C contains the ROM select address of the required ROM

Exit:   C contains the ROM select  address of the previous ROM,

and B contains the state of the previous ROM

006   &B912   KL CURR SELECTION

Action: Gets the ROM select address of the current ROM

Entry:  No entry conditions

Exit:   A contains the ROM select  address  of the current ROM,

and all other registers are preserved

007   &B915   KL PROBE ROM

Action: Gets the class and version of a specified ROM

Entry:  C contains the ROM select address of the required ROM

Exit:   A contains the class of  the  ROM,  H holds the version

number, L holds me  mark  number,  B  and the flags are

corrupt, and all other registers are preserved

Notes:  The ROM class may be one of ine following:

&00 - a foregroumd ROM

&01 - a background ROM

&02 - an extension foreground ROM

&80 - the built in ROM (ie the BASIC ROM)

008   &B918   KL ROM DESELECT

Action: Selects the previous upper ROM and sets its state

Entry:  C contains me  ROM  select  address  of  the  ROM to be

reselected, and B contains  the  state  of the required

ROM

Exit:   C contains the ROM select address  of me current ROM, B

is corrupt, and all others are preserved

Notes:  This routine reverses the acoon  of  KL ROM SELECT, and

uses the values that it returns in B and C

009   &B91B   KL LDIR

Action: Switches off the  upper  and  lower  ROMs,  and moves a

block of memory

Entry:  As for a standard  LDIR  instruction  (ie  DE holds the

destination location, HL points to the first byte to be

moved, and BC  holds  the  length  of  the  block to be

moved)

Exit:   F, BC,  DE  amd  HL  are  set  as  for  a  normal  LDIR

instruction, and all other registers are preserved

010   &B91E   KL LDDR

Action: Switches off the  upper  and  lower  ROMs,  amd moves a

block of memory

Entry:  As for a standard  LDDR  instruction  (ie  DE holds the

first desination location,  HL  points  to  the highest

byte lit in memory to be moved, amd BC holds the number

of bytes to be moved)

Exit:   F, BC,  DE  amd  HL,  are  set  as  for  a  nommal LDDR

instruction, and all other registers are preserved

011   &B921   KL POLL SYNCHRONOUS

Action: Tests whether an event with  a higher priority than the

current event is waiting to be dealt with

Entry:  No entry conditions

Exit:   If there is  a  higher  priority  event,  then Carry is

false; if there is no higher priority event, then Carry

is true; in either  case,  A  and  the  other flags are

corrupt, and all other registers are preserved

014   &B92A   KL SCAN NEEDED

Action: Ensures that the  keyboard  is  scanned  when  the next

ticker interrupt occurs

Entry:  No entry conditions

Exit:   AF amd HL  are  corrupt,  amd  all  other registers are

preserved

Notes:  This routine is useful  for  scanning the keyboard when

interrupts are disabled and normal  key scanning is not

occuring

The Key Manager

000   &BB00   KM INITIALISE

Action: Initialises the Key Manager  and  sets up everything as

it is when the computer  is  first switched on; the key

buffer is emptied, Shift  and  Caps  lock are tumed off

amd all the expansion and  translation tables are reset

to normal; also see the routine KM RESET below

Entry:  No entry conditions

Exit:   AF, BC, DE and HL corrupt,  and all other registers are

preserved

001   &BB03   KM RESET

Action: Resets the Key Manager; the  key  buffer is emptied and

all current keys/characters are ignored

Entry:  No entry conditions

Exit:   AF, BC, DE and HL  are  corrupt and all other registers

are preserved

Notes:  See also KM INITIALISE above:  on  the 664 or 6128, the

key buffer can also  be  cleared  separately by calling

the KM FLUSH routine

002   &BB06   KM WAIT CHAR

Action: Waits for the next character from the keyboard buffer

Entry:  No entry conditions

Exit:   Carry is true, A holds  the  character value, the other

flags  are  corrupt,  and   all   other  registers  are

preserved

003   &BB09   KM READ CHAR

Action: Tests to see  if  a  character  is  available  from the

keyboard buffer, but  doesn't  wait  for  one to become

available

Entry:  No entry conditions

Exit:   If a character was available, then Carry is true, and A

contains the character; otherwise Carry is false, and A

is corrupt; in  both  cases,  the  other  registers are

preserved

004   &BB0C   KM CHAR RETURN

Action: Saves a character for the next  use  of KM WAIT CHAR or

KM READ CHAR

Entry:  A contains the ASCII code  of  the  character to be put

back

Exit:   All registers are preserved

005   &BB0F   KM SET EXPAND

Action: Assigns a string to a key code

Entry:  B holds the key code; C holds the length of the string;

HL contains the address of the string (must be in RAM)

Exit:   If it is OK,  then  Carry  is  true; otherwise Carry is

false; in either case, A,  BC,  DE  and HL are corrupt,

and all other registers rlre preserved

006   &BB12   KM GET EXPAND

Action: Reads a character from an expanded string of characters

Entry:  A holds an expansion token (ie  a key code) and L holds

the character position number (starts from 0)

Exit: If it is  OK,  then  Carry  is  true,  and  A  holds  the

character; otherwise Carry is false,  and A is corrupt;

in either case, DE and flags are corrupt, and the other

registers are preserved

007   &BB15   KM EXP BUFFER

Action: Sets  aside  a  buffer  area  for  character  expansion

strings

Entry:  DE holds the address  of  the  buffer  and HL holds the

length of the buffer

Exit:   If it is OK,  then  Carry  is  true; otherwise Carry is

false; in either case, A, BC, DE and HL are corrupt

Notes:  The buffer must be in the  central  32K of RAM and must

be at least 49 bytes long

008   &BB18   KM WAIT KEY

Action: Waits for a key to be  pressed  - this routine does not

expand any expansion tokens

Entry:  No entry conditions

Exit:   Carry is  true,  A  holds  the  character  or expansion

token, and all other registers are preserved

009   &BB1B   KM READ KEY

Action: Tests whether a key is available from the keyboard

Entry:  No entry conditions

Exit:   If a key  is  available,  then  Carry  is  true,  and A

contains the character; otherwise Carry is false, and A

is corrupt; in  either  case,  the  other registers are

preserved

Notes:  Any expansion tokens are not expanded

010   &BB1E   KM TEST KEY

Action: Tests if a  particular  key  (or  joystick direction or

button) is pressed

Entry:  A contains the key/joystick nurnber

Exit:   If the requested key  is  pressed,  then Zero is false;

otherwise Zero is true for  both,  Carry is false A and

HL are corrupt.  C holds  the  Sbift and Control status

and others are preserved

Notes:  After calling this, C will hold  the state of shift and

control - if bit 7 is set then Control was pressed, and

if bit 5 is set then Shift was pressed

011   &BB21   KM GET STATE

Action: Gets the state of the Shift and Caps locks

Entry:  No entry conditions

Exit:   If L holds &FF then  the  shift  lock  is  on, but if L

holds &00 then the Shift  lock  is  off; if H holds &FF

then the caps lock is on,  and  if H holds &00 then the

Caps lock is off; whatever  the  outcome, all the other

registers are preserved

012   &BB24   KM GET JOYSTICK

Action: Reads the present state of any joysticks attached

Entry:  No entry conditions

Exit:   H and A contains the state  of joystick 0, L holds that

state of joystick 1, and all others are preserved

Notes:  The joystick states  are  bit  significant  and  are as

follows: Bit 0 - Up Bit

1 - Down Bit

2 - Left Bit

3 - Right Bit

4 - Fire2 Bit

5 - Fire1 Bit

6 - Spare Bit

7 - Always zero

The bits are  set  when  the  corresponding  buttons or

directions are operated

013   &BB27   KM SET TRANSLATE

Action: Sets the token or character  that  is assigned to a key

when neither Shift nor Control are pressed

Entry:  A contains the key number and  B contains the new token

or character

Exit:   AF and HL  are  corrupt,  and  all  other registers are

preserved

Notes:  Special values for B are as follows:

&80  to  &9F  -  these   values  correspond  to  the

expansion tokens

&FD - this causes the caps lock to toggle on and off

&FE - this causes the  shift  lock  to toggle on and

off

&FF - causes this key to be ignored

014   &BB2A   KM GET TRANSLATE

Action: Finds out what token or character will be assigned to a

key when neither Shift nor Control are pressed

Entry:  A contains the key number

Exit:   A contains the token/character that is assigned, HL and

flags are corrupt, and all others are preserved

Notes:  See KM SET TRANSLATE  for  special  values  that can be

returned

015   &BB2D   KM SET SHIFT

Action: Sets the token or character that  will be assigned to a

key when Shift is pressed as well

Entry:  A contains the key number and  B contains the new token

or character

Exit:   AF and HL are corrupt, and all others are preserved

Notes:  See KM SET TRANSLATE for special values that can be set

016   &BB30   KM GET SHIFT

Action: Finds out what token/character  will  be  assigned to a

key when Shift is pressed as well

Entry:  A contains the key number

Exit:   A contains the token/character that is assigned, HL and

flags are corrupt, and all others are preserved

Notes:  See KM SET TRANSLATE  for  special  values  that can be

returned

017   &BB33   KM SET CONTROL

Action: Sets the token or character that  will be assigned to a

key when Control is pressed as well

Entry:  A contains  the  key  number  and  B  contains  the new

token/character

Exit:   AF and HL are corrupt, and all others are preserved

Notes:  See KM SET TRANSLATE for special values that can be set

018   &BB36   KM GET CONTROL

Action: Finds out what token or character will be assigned to a

key when Control is pressed as well

Entry:  A contains the key number

Exit:   A contains the token/character that is assigned, HL and

flags are corrupt and all others are preserved

Notes:  See KM SET TRANSLATE for special values that can be set

019   &BB39   KM SET REPEAT

Action: Sets whether a key may repeat or not

Entry:  A contains the key number B contains &00 if there is no

repeat and &FF is it is to repeat

Exit:   AF, BC and HL are corrupt, and all others are preserved

020   &BB3C   KM GET REPEAT

Action: Finds out whether a key is set to repeat or not

Entry:  A contains a key number

Exit:   If the key repeats, then Zero is false; if the key does

not repeat, then Zero is  true;  in  either case, A, HL

and flags are corrupt,  Carry  is  false, and all other

registers are preserved

021   &BB3F   KM SET DELAY

Action: Sets the time that elapses before the first repeat, and

also set the repeat speed

Entry:  H contains the  time  before  the  first  repeat, and L

holds the time between repeats (repeat speed)

Exit:   AF is corrupt, and all others are preserved

Notes:  The values for the times  are  given in 1/5Oth seconds,

and a value of 0 counts as 256

022   &BB42   KM GET DELAY

Action: Finds out the time that elapses before the first repeat

and also the repeat speed

Entry:  No entry conditions

Exit:   H contains the  time  before  the  first  repeat, and L

holds the time  between  repeats,  and  all  others are

preserved

023   &BB45   KM ARM BREAK

Action: Arms the Break mechanism

Entry:  DE holds the address of  the  Break handling routine, C

holds the ROM select address for this routine

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all  the other

registers are preserved

024   &BB48   KM DISARM BREAK

Action: Disables the Break mechanism

Entry:  No entry conditions

Exit:   AF and HL are corrupt, and  all the other registers are

preserved

025   &BB4B   KM BREAK EVENT

Action: Generates a Break interrupt if a Break routine has been

specified by KM ARM BREAK

Entry:  No entry conditions

Exit:   AF and HL  are  corrupt,  and  all  other registers are

preserved

The Text VDU

026   &BB4E   TXT INITIALISE

Action: Initialise the  text  VDU  to  its  settings  when  the

computer is switched  on,  includes  resetting  all the

text VDU indirections,  selecting  Stream  0, resetting

the text paper to pen  0  and  the  text  pen to pen 1,

moving the cursor to the top  left corner of the screen

and setting the writing mode to be opaque

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

027   &BB51   TXT RESET

Action: Resets the text VDU  indirections  and the control code

table

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all  the other

registers are preserved

028   &BB54   TXT VDU ENABLE

Action: Allows characters to be  printed  on  the screen in the

current stream

Entry:  No entry conditions

Exit:   AF is corrupt, and all other registers are preserved

029   &BB57   TXT VDU DISABLE

Action: Prevents characters from being  printed  to the current

stream

Entry:  No entry conditions

Exit:   AF  is  corrupt,  and  al1   the  other  registers  are

preserved

030   &BB5A   TXT OUTPUT

Action: Output a character or control code  (&00 to &1F) to the

screen

Entry:  A contains the character to output

Exit:   All registers are preserved

Notes:  Any control codes are obeyed  and nothing is printed if

the VDU is disabled;  characters  are printed using the

TXT OUT  ACTION  routine;  if  using  graphics printing

mode, then control codes are printed and not obeyed

031   &BB5D   TXT WR CHAR

Action: Print a character  at  the  current  cursor  position -

control codes are printed and not obeyed

Entry:  A contains the character to be printed

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

Notes:  This routine uses the TXT WRITE CHAR indirection to put

the character on the screen

032   &BB60   TXT RD CHAR

Action: Read a character from the  screen at the current cursor

position

Entry:  No entry conditions

Exit:   If it was successful then A contains the character that

was read from the screen  and  Carry is true; otherwise

Carry is false, and  A  holds  0;  in  either case, the

other  flags  are  corrupt,   and   all  registers  are

preserved

Notes:  This routine uses the TXT UNWRITE indirection

033   &BB63   TXT SET GRAPHIC

Action: Enables or disables graphics print character mode

Entry:  To switch graphics printing  mode  on,  A  must be non-

zero; to turn it off, A must contain zero

Exit:   AF corrupt, and all other registers are preserved

Notes:  When turned  on,  control  codes  are  printed  and not

obeyed; characters are printed by GRA WR CHAR

034   &BB66   TXT WIN ENABLE

Action: Sets the boundaries of the  current  text window - uses

physical coordinates

Entry:  H hoIds the column  number  of  one  edge,  D holds the

column number of  the  other  edge,  L  holds  the line

number of one edge, and E  holds the line number of the

other edge Exit: AF, BC, DE and HL are corrupt

Notes:  The window is not cleared  but  the  cursor is moved to

the top left corner of the window

035   &BB69   TXT GET WINDOW

Action: Returns the  size  of  the  current  window  -  returns

physical coordinates

Entry:  No entry conditions

Exit:   H holds the column number of the left edge, D holds the

column number of  the  right  edge,  L  holds  the line

number of the top edge, E  holds the line number of the

bottom edge, A is corrupt, Carry is false if the window

covers the entire screen,  and  the other registers are

always preserved

036   &BB6C   TXT CLEAR WINDOW

Action: Clears the window (of the current stream) and moves the

cursor to the top left corner of the window

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  alI others are

preserved

037   &BB6F   TXT SET COLUMN

Action: Sets the cursor's horizontal position

Entry:  A contains the logical column number to move the cursor

to

Exit:   AF and HL are corrupt, and  all the other registers are

preserved

Notes:  See also TXT SET CURSOR

038   &BB72   TXT SET ROW

Action: Sets the cursor's vertical position

Entry:  A contains the logical line  number  to move the cursor

to

Exit:   AF and HL are corrupt, and all others are preserved

Notes:  See also TXT SET CURSOR

039   &BB75   TXT SET CURSOR

Action: Sets the cursor's vertical and horizontal position

Entry:  H contains the logical column number and L contains the

logical line number

Exit:   AF and HL are corrupt, and all the others are preserved

Notes:  See also TXT SET COLUMN and TXT SET ROW

040   &BB78   TXT GET CURSOR

Action: Gets the cursor's current position

Entry:  No entry conditions

Exit:   H holds the logical column  number, L holds the logical

line number, and A contains  the  roll count, the flags

are corrupt, and all the other registers are preserved

Notes:  The roll count is increased when the screen is scrolled

down, and is decreased when it is scrolled up

041   &BB7B   TXT CUR ENABLE

Action: Allows the  text  cursor  to  be  displayed  (if  it is

allowed by TXT CUR ON) - intended for use by the user

Entry:  No entry conditions

Exit:   AF is corrupt, and all other registers are preserved

042   &BB7E   TXT CUR DISABLE

Action: Prevents the text cursor from being displayed -intended

for use by the user

Entry:  No entry conditions

Exit:   AF is corrupt, and all others are preserved

043   &BB81   TXT CUR ON

Action: Allows the text cursor to  be  displayed - intended for

use by the operating system

Entry:  No entry conditions

Exit:   All registers and flags are preserved

044   &BB84   TXT CUR OFF

Action: Prevents the text cursor from being displayed -intended

for use by the operating system

Entry:  No entry conditions

Exit:   All registers and flags are preserved

045   &BB87   TXT VALIDATE

Action: Checks whether a cursor position  is within the current

window

Entry:  H contains the logical  column  number  to check, and L

holds the logical line number

Exit:   H holds  the  logical  column  number  where  the  next

character will be  printed,  L  holds  the logical line

number; if printing  at  this  position  would make the

window scroll up, then Carry is  false and B holds &FF;

if printing at  this  position  would  make  the window

scroll down, then Carry is false and B contains &00; if

printing at the  specified  cursor  position  would not

scroll the window, then Carry is true and B is corrupt;

always, A and  the  other  flags  are  corrupt, and all

others are preserved

046   &BB8A   TXT PLACE CURSOR

Action: Puts a 'cursor  blob'  on  the  screen  at  the current

cursor position

Entry:  No entry conditions

Exit:   AF is corrupt, and all other registers are preserved

Notes:  It is possible to have more than one cursor in a window

(see also TXT DRAW  CURSOR);  do  not use this  routine

twice without using TXT REMOVE CURSOR between

047   &BB8D   TXT REMOVE CURSOR

Action: Removes  a  'cursor  blob'   from  the  current  cursor

position

Entry:  No entry conditions

Exit:   AF is corrupt, and all the others are preserved

Notes:  This should be used only  to  remove cursors created by

TXT PLACE CURSOR, but see also TXT UNDRAW CURSOR

048   &BB90   TXT SET PEN

Action: Sets the foreground PEN for the current stream

Entry:  A contams the PEN number to use

Exit:   AF and HL  are  corrupt,  and  all  other registers are

preserved

049   &BB93   TXT GET PEN

Action: Gets the foreground PEN for the current stream

Entry:  No entry conditions

Exit:   A contains the PEN number,  the  flags are corrupt, and

all other registers are preserved

050   &BB96   TXT SET PAPER

Action: Sets the background PAPER for the current stream

Entry:  A contains the PEN number to use

Exit:   AF and HL  are  corrupt,  and  all  other registers are

preserved

051   &BB99   TXT GET PAPER

Action: Gets the background PAPER for the current stream

Entry:  No entry conditions

Exit:   A contains the PEN number,  the  flags are corrupt, and

all other registers are preserved

052   &BB9C   TXT INVERSE

Action: Swaps the current PEN  and  PAPER  colours over for the

current stream

Entry:  No entry conditions

Exit:   AF and HL are corrupt, and all others are preserved

053   &BB9F   TXT SET BACK

Action: Sets the  character  write  mode  to  either  opaque or

transparent

Entry:  For transparent mode, A  must  be  non-zero; for opaque

mode, A has to hold zero

Exit:   AF and HL  are  corrupt,  and  all  other registers are

preserved

Notes:  Setting the character write mode  has no effects on the

graphics VDU

054   &BBA2   TXT GET BACK

Action: Gets the character write mode for the current stream

Entry:  No entry conditions

Exit:   If in transparent mode, A  is non-zero; in opaque mode,

A is zero; in either case DE, HL and flags are corrupt,

and the other registers are preserved

055   &BBA5   TXT GET MATRIX

Action: Gets the address of a character matrix

Entry:  A contains the character whose matrix is to be found

Exit:   If it is a user-defined matrix,  then Carry is true; if

it is in the lower ROM  then  Carry is false; in either

event, HL contains the  address  of  the  matrix, A and

other flags are corrupt, and others are preserved

Notes:  The character matrix is  stored  in  8 bytes; the first

byte is for the top row  of the character, and the last

byte refers to the bottom  row  of the character; bit 7

of a byte refers to the  leftmost  pixel of a line, and

bit 0 refers to the rightmost pixel in Mode 2.

056   &BBA8   TXT SET MATRIX

Action: Installs a matrix for a user-defined character

Entry:  A contains the character which  is being defined and HL

contains the address of the matrix to be used

Exit:   If the character is user-definable  then Carry is true;

otherwise Carry is false,  and  no  action is taken; in

both cases AF, BC, DE and HL are corrupt, and all other

registers are preserved

057   &BBAB   TXT SET M TABLE

Action: Sets the address of a user-defined matrix table

Entry:  DE is the first character  in  the  table and HL is the

table's address (in the central 32K of RAM)

Exit:   If there are no  existing  tables  then Carry is false,

and A and HL are both corrupt; otherwise Carry is true,

A is the first character and HL is the table's address;

in both cases BC, DE and the other flags are corrupt

058   &BBAE   TXT GET M TABLE

Action: Gets the address of a user-defined matrix table

Entry:  No entry conditions

Exit:   See TXT SET M  TABLE  above  for  details of the values

that can be returned

059   &BBB1   TXT GET CONTROLS

Action: Gets the address of the control code table

Entry:  No entry conditions

Exit:   HL contains the address  of  the  table, and all others

are preserved

Notes:  The table has  32  entries,  and  each  entry has three

bytes: byte 1 is the number of parameters needed by the

control code, bytes 2  and  3  are  the  address of the

routine, in the Lower ROM, to execute the control code

060   &BBB4   TXT STR SELECT

Action: Selects a new VDU text stream

Entry:  A contains the value of the stream to change to

Exit:   A contains the previously  selected  stream, HL and the

flags are corrupt, and all others are preserved

061   &BBB7   TXT SWAP STREAMS

Action: Swaps the states of two stream attribute tables

Entry:  B contains a stream  number,  and  C contains the other

stream number

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  The foreground pen  and  paper,  the  window  size, the

cursor position, the character  write  mode and graphic

character  mode  are  all  exchanged  between  the  two

streams

The Graphics VDU

062   &BBBA   GRA INITIALISE

Action: Initialises the graphics VDU to  its default set-up (ie

its set-up when the computer is switched on)

Entry:  No entry conditions

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  Sets the graphics indirections  to their defaults, sets

the graphic paper to text pen  0 and the graphic pen to

text pen 1,  reset  the  graphics  origin  and move the

graphics cursor to the bottom left of the screen, reset

the graphics window and write mode to their defaults

063   &BBBD   GRA RESET

Action: Resets the graphics VDU

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

Notes:  Resets the graphics indirections and the graphics write

mode to their defaults

064   &BBC0   GRA MOVE ABSOLUTE

Action: Moves  the  graphics  cursor   to  an  absolute  screen

position

Entry:  DE contains the user X-coordinate and HL holds the user

Y-coordinate

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

065   &BBC3   GRA MOVE RELATIVE

Action: Moves the graphics cursor  to  a  point relative to its

present screen position

Entry:  DE contains the X-distance to move  and HL holds the Y-

distance

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

066   &BBC6   GRA ASK CURSOR

Action: Gets the graphics cursor's current position

Entry:  No entry conditions

Exit:   DE holds the user X-coordirlate,  HL  holds the user Y-

coordinate, AF is corrupt, and all others nre preserved

067   &BBC9   GRA SET ORIGIN

Action: Sets the graphics user origin's screen position

Entry:  DE contains the standard X-coordinate  and HL holds the

standard Y-coordinate

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

068   &BBCC   GRA GET ORIGIN

Action: Gets the graphics user origin's screen position

Entry:  No entry conditions

Exit:   DE contains the standard X-coordinate  and HL holds the

standard Y-coordinate, and all others are preserved

069   &BBCF   GRA WIN WIDTH

Action: Sets the left and right edges of the graphics window

Ently:  DE contains the standard  X-coordinate  of one edge and

HL holds the standard X-coordinate of the other side

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all  the other

registers are preserved

Notes:  The default window  covers  the  entire  screen  and is

restored to its default when  the mode is changed; used

in conjunction with GRA WIN HEIGHT

070   &BBD2   GRA WIN HEIGHT

Action: Sets the top and bottom edges of the graphics window

Entry:  DE contains the standard  Y-coordinate  of one side and

HL holds the standard Y-coordinate of the other side

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

Notes:  See GRA WIN WIDTH for further details

071   &BBD5   GRA GET W WIDTH

Action: Gets the left and right edges of the graphics window

Entry:  No entry conditions

Exit:   DE contains the standard X-coordinate  of the left edge

and HL contains the standard  Y-coordinate of the right

edge, AF  is  corrupt,  and  all  other  registers  are

preserved

072   &BBD8   GRA GET W HEIGHT

Action: Gets the top and bottom edges of the graphics window

Entry:  No entry conditions

Exit:   DE contains the standard  Y-coordinate  of the top edge

and HL contains the standard Y-coordinate of the bottom

edge, AF  is  corrupt,  and  all  other  registers  are

preserved

073   &BBDB   GRA CLEAR WINDOW

Action: Clears the graphics window to the graphics paper colour

and moves the cursor back to the user origin

Entry:  No entry conditions

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

074   &BBDE   GRA SET PEN

Action: Sets the graphics PEN

Entry:  A contains the required text PEN number

Exit:   AF is corrupt, and all other registers are preserved

075   &BBE1   GRA GET PEN

Action: Gets the graphics PEN

Entry:  No entry conditions

Exit:   A contains the text PEN  number, the flags are corrupt,

and all other registers are preserved

076   &BBE4   GRA SET PAPER

Action: Sets the graphics PAPER

Entry:  A contains the required text PEN number

Exit:   AF corrupt, and all others are preserved

077   &BBE7   GRA GET PAPER

Action: Gets the graphics PAPER

Entry:  No entry conditions

Exit:   A contains the text PEN  number, the flags are corrupt,

and all others are preserved

078   &BBEA   GRA PLOT ABSOLUTE

Action: Plots a point at an absolute user coordinate, using the

GRA PLOT indirection

Entry:  DE contains the user X-coordinate and HL holds the user

Y-coordinate

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

079   &BBED   GRA PLOT RELATIVE

Action: Plots a point at  a  position  relative  to the current

graphics cursor, using the GRA PLOT indirection

Entry:  DE contains the relative  X-coordinate  and HL contains

the relative Y-coordinate

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

080   &BBF0   GRA TEST ABSOLUTE

Action: Moves to an  absolute  position,  and  tests  the point

there using the GRA TEST indirection

Entry:  DE contains the user X-coordinate and HL holds the user

Y-coordinate for the point you wish to test

Exit:   A contains the pen at  the  point,  and  BC, DE, HL and

flags are corrupt, and all others are preserved

081   &BBF3   GRA TEST RELATIVE

Action: Moves to a position  relative  to the current position,

and  tests  the  point   there   using   the  GRA  TEST

indirection

Entry:  DE contains the relative  X-coordinate  and HL contains

the relative Y-coordinate

Exit:   A contains the pen at  the  point,  and  BC, DE, HL and

flags are corrupt, and all others are preserved

082   &BBF6   GRA LlNE ABSOLUTE

Action: Draws a line from the  current  graphics position to an

absolute position, using GRA LINE

Entry:  DE contains the user X-coordinate and HL holds the user

Y-coordinate of the end point

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

Notes:  The line will be  plotted  in  the current graphics pen

colour (may be masked  to  produce  a  dotted line on a

6128)

083   &BBF9   GRA LINE RELATIVE

Action: Draws a line from  the  current  graphics position to a

relative screen position, using GRA LINE

Entry:  DE contains the relative  X-coordinate  and HL contains

the relative Y-coordinate

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  See GRA LINE ABSOLUTE above for details of how the line

is plotted

084   &BBFC   GRA WR CHAR

Action: Writes a  character  onto  the  screen  at  the current

graphics position

Entry:  A contains the character to be put onto the screen

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all  the other

registers are preserved

Notes:  As in BASIC, all characters including control codes are

printed; the character  is  printed  with  its top left

comer at the  current  graphics  position; the graphics

position is moved one character  width  to the right so

that it is ready for another character to be printed

The Screen Pack

085   &BBFF   SCR INITIALISE

Action: Initialises the Screen Pack to  the default values used

when the computer is first switched on

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

Notes:  All screen indirections are  restored  to their default

settings, as are inks and  flashing speeds; the mode is

switched to MODE 1 and  the  screen is cleared with PEN

0; the screen address is moved  to &C000 and the screen

offset is set to zero

086   &BC02   SCR RESET

Action: Resets the Screen Pack's  indirections, flashing speeds

and inks to their default values

Entry:  No entry conditions

Exit:   AF, BC, DE r1nd HL are corrupt, and all other registers

are preserved

087   &BC05   SCR SET OFFSET

Action: Sets the screen offset to  the  specified values - this

can cause the screen to scroll

Entry:  HL contains the required offset, which should be even

Exit:   AF and HL are corrupt, and alI others are preserved

Notes:  The screen offset is reset  to  0  whenever its mode is

set, or it is  cleared  by  SCR  CLEAR (but not BASIC's

CLS)

088   &BC08   SCR SET BASE

Action: Sets the location in memory of the screen - effectively

can only be &C000 or &4000

Entry:  A contains the  most  significant  byte  of  the screen

address required

Exit:   AF and HL  are  corrupt,  and  all  other registers are

preserved

Notes:  The screen memory can only be  set at 16K intervals (ie

&0000, &4000, &8000, &C000)  and  when  the computer is

first switched on the 16K  of  screen memory is located

at &C000)

089   &BC0B   SCR GET LOCATION

Action: Gets the location of  the  screen  memory  and also the

screen offset

Entry:  No entry conditions

Exit:   A  holds  the  most  significant  byte  of  the  screen

address, HL holds the  current  offset,  and all others

are preserved

090   &BC0E   SCR SET MODE

Action: Sets the screen mode

Entry:  A contains the mode number -  it has the same value and

characteristics as in BASIC

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

Notes:  The windows are set to  cover  the whole screen and the

graphics origin is set to the bottom left corner of the

screen; in addition, the current stream is set to zero,

and the screen offset is zeroed

091   &BC11   SCR GET MODE

Action: Gets the current screen mode

Ently:  No entry conditions

Exit:   If the mode is 0,  then  Carry  is true, Zero is false,

and A contains 0;  if  the  mode  is  1,  then Carry is

false, Zero is true, and A  contains  1; if the mode is

2, then Carry is false,  Zero  is false, and A contains

2; in all cases the other flags are corrupt and all the

other registers are preserved

092   &BC14   SCR CLEAR

Action: Clears the whole of the screen

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

093   &BC17   SCR CHAR LIMITS

Action: Gets the size  of  the  whole  screen  in  terms of the

numbers of characters that can be displayed

Entry:  No entry conditions

Exit:   B contains the number of  characters across the screen,

C contains the number of characters down the screen, AF

is corrupt, and all other registers are preserved

094   &BC1A   SCR CHAR POSITION

Action: Gets the memory address  of  the  top  left corner of a

specified character position

Entry:  H contains the character physical column and L contains

the character physical row

Exit:   HL contains the memory address of the top left comer of

the  character,  B  holds  the  width  in  bytes  of  a

character in the present mode,  AF  is corrupt, and all

other registers are preserved

095   &BC1D   SCR DOT POSITION

Action: Gets the memory  address  of  a  pixel  at  a specified

screen position

Entry:  DE contains the base X-coordinate  of the pixel, and HL

contains the base Y-coordinate

Exit:   HL contains the memory address of the pixel, C contains

the bit mask for this  pixel,  B contains the number of

pixels stored in a byte minus 1, AF and DE are corrupt,

and all others are preserved

096   &BC20   SCR NEXT BYTE

Action: Calculates the screen address of  the byte to the right

of the specified screen  address  (may  be  on the next

line)

Entry:  HL contains the screen address

Exit:   HL holds the screen address of the byte to the right of

the original screen address, AF  is corrupt, all others

are preserved

097   &BC23   SCR PREV BYTE

Action: Calculates the screen address of  the  byte to the left

of the  specified  screen  address  (this  address  may

actually be on the previous line)

Entry:  HL contains the screen address

Exit:   HL holds the screen address of  the byte to the left of

the original address,  AF  is  corrupt,  all others are

preserved

098   &BC26   SCR NEXT LINE

Action: Calculates the screen  address  of  the  byte below the

specified screen address Ently:  HL contains the screen

address

Exit:   HL contains the screen  address  of  the byte below the

original screen address,  AF  is  corrupt,  and all the

other registers are preserved

099   &BC29   SCR PREV LINE

Action: Calculates the screen  address  of  the  byte above the

specified screen address

Entry:  HL contains the screen address

Exit:   HL holds the  screen  address  of  the  byte  above the

original address, AF  is  corrupt,  and  all others are

preserved

100   &BC2C   SCR INK ENCODE

Action: Converts a PEN to provide a mask which, if applied to a

screen byte, will convert all of the pixels in the byte

to the appropriate PEN

Entry:  A contains a PEN number

Exit:   A contains the encoded value of  the PEN, the flags are

corrupt, and all other registers are preserved

Notes:  The mask returned is  different  in  each of the screen

modes

101   &BC2F   SCR INK DECODE

Action: Converts a PEN mask into  the  PEN  number (see SCR INK

ENCODE for the re~ erse process)

Entry:  A contains the encoded value of the PEN

Exit:   A contains the PEN number,  the  flags are corrupt, and

all others are preserved

102   &BC32   SCR SET INK

Action: Sets the colours of a PEN  - if the two values supplied

are different then the colours will alternate (flash)

Entry:  contains the PEN number,  B  contains the first colour,

and C holds the second colour

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

103   &BC35   SCR GET INK

Action: Gets the colours of a PEN

Entry:  A contains the PEN nurnber

Exit:   B contains the first colour, C holds the second colour,

and AF, DE  and  HL  are  corrupt,  and  all others are

preserved

104   &BC38   SCR SET BORDER

Action: Sets the colours of the border - again if two different

values are supplied, the border will flash

Entry:  B contains the first colour,  and C contains the second

colour

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

105   &BC3B   SCR GET BORDER

Action: Gets the colours of the border

Entry:  No entry conditions

Exit:   B contains the first colour, C holds the second colour,

and AF, DE  and  HL  are  corrupt,  and  all others are

preserved

106   &BC3E   SCR SET FLASHING

Action: Sets the  speed  with  which  the  border's  and  PENs'

colours flash

Entry:  H holds the time that the  first colour is displayed, L

holds the time the second colour is displayed for

Exit:   AF and HL  are  corrupt,  and  all  other registers are

preserved

Notes:  The length  of  time  that  each  colour  is  shown  is

measured in 1/5Oths of a  second,  and  a value of 0 is

taken to mean 256 * 1/50 seconds - the default value is

10 * 1/50 seconds

107   &BC41   SCR GET FLASHING

Action: Gets the periods with which  the  colours of the border

and PENs flash

Entry:  No entry conditions

Exit:   H holds the duration of  the  first colour, L holds the

duration of the second colour,  AF  is corrupt, and all

other registers are preserved  -  see  SCR SET FLASHING

for the units of time used

108   &BC44   SCR FILL BOX

Action: Fills an area of the  screen  with  an  ink - this only

works for 'character-sized' blocks of screen

Entry:  A contains the mask for the  ink  that is to be used, H

contains the left hand colurnn  of  the area to fill, D

contains the right hand column,  L  holds the top line,

and E holds the bottom line of the area (using physical

coordinates)

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

109   &BC17   SCR FLOOD BOX

Action: Fills an area of the  screen  with  an  ink - this only

works for 'byte-sized' blocks of screen

Entry:  C contains the  encoded  PEN  that  is  to  be used, HL

contains the screen address of the top left hand corner

of the area to fill, D  contains  the width of the area

to be filled in bytes, and E contains the height of the

area to be filled in screen lines

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  The whole of the  area  to  be  filled  must lie on the

screen otherwise unpredictable results may occur

110   &BC4A   SCR CHAR INVERT

Action: Inverts a character's colours; all  pixels in one PEN's

colour are printed in  another  PEN's  colour, and vice

versa

Entry:  B contains  one  encoded  PEN,  C  contains  the  other

encoded PEN, H contains the physical column number, and

L contains the physical  line  number  of the character

that is to be inverted

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  alI  the other

registers are preserved

111   &BC4D   SCR HW ROLL

Action: Scrolls the entire screen  up  or  down  by eight pixel

rows (ie one character line)

Entry:  B holds the  direction  that  the  screen  will roll, A

holds the encoded PAPER which  the new line will appear

in

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

Notes:  This alters the screen  offset;  to  roll  down, B must

hold zero, and to roll upwards B must be non-zero

112   &BCS0   SCR SW ROLL

Action: Scrolls part of the screen  up  or  down by eight pixel

lines - only for 'character-sized' blocks of the screen

Entry:  B holds the direction to  roll  the screen, A holds the

encoded PAPER which  the  new  line  will  appear in, H

holds the left column of  the  area  to scroll, D holds

the right colurnn, L holds  the  top  line, E holds the

bottom line

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  The area of the screen is  moved by copying it; to roll

down, B must hold zero, and  to  roll upwards B must be

non-zero; this routine uses physical roordinates

113   &BC53   SCR UNPACK

Action: Changes a character matrix from its eight byte standard

form into a set of  pixel  masks which are suitable for

the current mode - four *8  bytes are needed in mode 0,

two *8 bytes in mode l, and 8 bytes in mode 2

Entry:  HL contains the address of  the matrix, and DE contains

the address where the masks are to be stored

Exit:   AF. BC, DE and HL are  corrupt, and all other registers

are preserved

114   &BC56   SCR REPACK

Action: Changes a set of  pixel  masks  (for  the current mode)

into a standard eight byte character matrix

Entry:  A contains the  encoded  foreground  PEN  to be matched

against (ie the PEN that is to be regarded as being set

in the character), H holds  the  physical column of the

character to be `repacked',  L  holds the physical line

of the character, and  DE  contains  the address of the

area where the character matrix will be built

Exit:   AF, BC, DE amd HL are  corrupt,  and all the others are

preserved

115   &BC59   SCR ACCESS

Action: Sets the screen write mode for graphics

Entry:  A contains the write mode (0=Fill, 1=XOR, 2=AND, 3=OR)

Exit:   AF. BC, DE and HL are  corrupt, amd all other registers

are preserved

Notes:  The fill mode  means  that  the  ink  that plotting was

requested in is the ink that  appears on the screen; in

XOR mode, the specified ink  is  XORed with ink that is

at that point on the  screen already before plotting; a

simiIar situation occurs with the AND and OR modes

116   &BC5C   SCR PIXELS

Action: Puts a pixel or pixels on  the screen regardless of the

write mode specified by SCR ACCESS above

Entry:  B contains the mask  of  the  PEN  to  be drawn with, C

contains the  pixel  mask,  and  HL  holds  the  screen

address of the pixel

Exit:   AF is corrupt, amd all others are preserved

117   &BC5F   SCR HORIZONTAL

Action: Draws a honzontal line on  the screen using the current

graphics write mode

Entry:  A contains  the  encoded  PEN  to  be  drawn  with,  DE

contains the base  X-coordinate  of  the  start  of the

line, BC contains  the  end  base  X-coordinate, and HL

contains the base Y-coordinate

Exit:   AF, BC, DE and HL  are  conupt, and all other registers

are preserved

Notes:  The start X-coordinate must  be  less  than  the end X-

coordmate

118   &BC62   SCR VERTICAL

Action: Draws a vertical line on  the  screen using the current

graphics write mode

Entry:  A contains  the  encoded  PEN  to  be  drawn  with,  DE

contains the base X-coordinate  of  the  line, HL holds

the start base Y-coordinate,  and  BC  contains the end

base Y-coordinate - the  start  coordinate must be less

than the end coordinate

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all  the other

registers are preserved

The Cassette/AMSDOS Manager

NOTE: Some of  these  routines  are  only  applicable  to the cassette

manager; where a disc version  exists  it  is indicated by an asterisk

(*) next to  the  command  name.   These  disc  version jumpblocks are

automatically installed by the Operating System on switch on.

119   &BC65   CAS INITIALISE

Action: Initialises the cassette manager

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all  the other

registers are preserved

Notes:  Both read and write  streams  are closed; tape messages

are switched on; the default speed is reselected

120   &BC68   CAS SET SPEED

Action: Sets the speed  at  which  the  cassette  manager saves

programs

Entry:  HL holds  the  length  of  'half  a  zero'  bit,  and A

contains the amount of precompensation

Exit:   AF and HL are corrupt

Notes:  The value in HL is the length  of time that half a zero

bit is written as; a one  bit  is twice the length of a

zero bit; the default values (ie SPEED WRITE 0) are 333

microseconds (HL) and  25  microseconds  (A)  for SPEED

WRITE 1, the values are  given  as 107 microseconds and

50 microseconds respectiveIy

121   &BC6B   CAS NOISY

Action: Enables or disables  the  display  of cassette handling

messages

Entry:  To enable the messages then A  must be 0, otherwise the

messages are disabled

Exit:   AF is corrupt, and all other registers are preserved

122   &BC6E   CAS START MOTOR

Action: Switches on the tape motor

Entry:  No entry conditions

Exit:   If the motor operates properly  then  Carry is true; if

ESC was pressed then Carry is  false; in either case, A

contains the  motor's  previous  state,  tbe  flags are

corrupt, and all others are preserved

123   &BC71   CAS STOP MOTOR

Action: Switches off the tape motor

Entry:  No entry conditions

Exit:   If the motor turns off then  Carry  is true; if ESC was

pressed then Carry is false; in both cases, A holds tbe

motor's previous state,  the  other  flags are corrupt,

all others are preserved

124   &BC74   CAS RESTORE MOTOR

Action: Resets the tape motor to its previous state

Entry:  A contains the previous state of the motor (eg from CAS

START MOTOR or CAS STOP MOTOR)

Exit:   If the motor operates properly  then  Carry is true; if

ESC was pressed then Carry  is  false;  in all cases, A

and the other  flags  are  corrupt  and  all others are

preserved

125   &BC77   *CAS IN OPEN

Action: Opens an input buffer and reads  the first block of the

file

Entry:  B contains the length of  the filename, HL contains the

filename's address, and DE contains  the address of the

2K buffer to use for reading the file

Exit:   If the file  was  opened  successfully,  then  Carry is

true, Zero is false, HL  holds  the address of a buffer

contauling the file header  data,  DE holds the address

of the destination  for  the  file,  BC  holds the file

length, and A holds the  file  type; if the read stream

is already  open  then  Carry  and  Zero  are  false, A

contains an error nurnber  (664/6128  only)  and BC, DE

and HL are corrupt;  if  ESC  was  pressed by the user,

then Carry is false,  Zero  is  true,  A holds an error

number (664/6128 only) and BC,  DE  and HL are corrupt;

in all cases, IX and  the  other flags are corrupt, and

the others are preserved

Notes:A filename of zero length  means  'read  the neXt file on

the tape'; the stream remains  open  until it is closed

by either CAS IN CLOSE or CAS IN ABANDON

Disc:   Similar to tape except that  if  there  is no header on

the file, then a fake header is put into memory by this

routine

126   &BC7A   *CAS IN CLOSE

Action: Closes an input file

Entry:  No entry conditions

Exit:   If the file was closed successfully, then Carry is true

and A is corrupt; if the read stream was not open, then

Carry is false, and  A  holds  an  error code (664/6128

only); in both cases, BC,  DE,  HL  and the other flags

are all corrupt

Disc:   All the above applies, but  also  if the file failed to

close for any other reason,  then  Carry is false, Zero

is true and A contains  an  error  number; in all cases

the drive motor is turned off immediately

127   &BC7D   *CAS IN ABANDON

Action: Abandons an input file

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

Disc:   All the above applies for the disc routine

128   &BC80   *CAS IN CHAR

Action: Reads in a single byte from a file

Entry:  No entry conditions

Exit:   If a byte was read, then  Carry is true, Zero is false,

and A contains the byte read  from the file; if the end

of file was reached, then  Carry  and Zero are false, A

contains an error number (664/6128  only) or is corrupt

(for the 464); if ESC was pressed, then Carry is false,

Zero is true, and  A  holds  an  error number (664/6128

only) or is corrupt (for the 464); in all cases, IX and

the  other  flags  are  corrupt,  and  all  others  are

preserved

Disc:   All the above applies for the disc routine

129   &BC83   *CAS IN DIRECT

Action: Reads an entire file directly into memory

Entry:  HL contains the address where the file is to be  placed

in RAM

Exit:   If the operation was  successful,  then  Carry is true,

Zero is false, HL contains  the  entry address and A is

corrupt; if it was not  open,  then  Carry and Zero are

both false, HL is corrupt,  and  A  holds an error code

(664/6128) or is  corrupt  (464);  if  ESC was pressed,

Carry is false, Zero  is  true,  HL  is  corrupt, and A

holds an error code (664/6128  only); in all cases, BC,

DE and IX and  the  other  flags  are  corrupt, and the

others are preserved

Notes:  This routine cannot be used  once  CAS IN CHAR has been

used

Disc:   All the above applies to the disc routine

130   &BC86   *CAS RETURN

Action: Puts the last byte read  back  into the input buffer so

that it can be read again at a later time

Entry:  No entry conditions

Exit:   All registers are preserved

Notes:  The routine can only return  the  last byte read and at

least one byte must have been read

Disc:   All the above applies to the disc routine

131   &BC89   *CAS TEST EOF

Action: Tests whether the end of file has been encountered

Entry:  No entry conditions

Exit:   If the end of  file  has  been  reached, then Carry and

Zero are false, and A  is  corrupt;  if the end of file

has not been encountered, then  Carry  is true, Zero is

false, and A is corrupt; if  ESC was pressed then Carry

is false, Zero is true  and  A contains an error number

(664/6128 only); in all cases,  IX  and the other flags

are corrupt, and all others are preserved

Disc:   All the above applies to the disc routine

132   &BC8C   *CAS OUT OPEN

Action: Opens an output file

Entry:  B contains the length of  the filename, HL contains the

address of the filename,  and  DE  holds the address of

the 2K buffer to be used

Exit:   If the file was opened  correctly,  then Carry is true,

Zero is false,  HL  holds  the  address  of  the buffer

containing the file header data that will be written to

each block, and A is  corrupt;  if  the write stream is

already open, then Carry and Zero are false, A holds an

error nurnber (66~/6128) and HL  is corrupt; if ESC was

pressed then Carry is false,  Zero  is true, A holds an

error number  (664/6128)  and  HL  is  corrupt;  in all

cases, BC, DE, IX and the  other flags are corrupt, and

the others are preserved

Notes:  The buffer is used  to  store  the  contents  of a file

block before it is actually written to tape

Disc:   The same as for tape  except  that the filename must be

present in its usual AMSDOS format

133   &BC8F   *CAS OUT CLOSE

Action: Closes an output file

Entry:  No entry conditions

Exit:   If the file  was  closed  successfully,  then  Carry is

true, Zero is false,  and  A  is  corrupt; if the write

stream was not open, then Carry  and Zero are false and

A holds an  error  code  (664/6128  only);  if  ESC was

pressed then  Carry  is  false,  Zero  is  true,  and A

contains an error code  (664/6128  only); in all cases,

BC, DE, HL, IX and the other flags are all corrupt

Notes:  The last block of  a  file  is  written  only when this

routine is  called;  if  writing  the  file  is  to  be

abandoned, then CAS OUT ABANDON should be used instead

Disc:   All the above applies to the disc routine

134   &BC92   *CAS OUT ABANDON

Action: Abandons an output file

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

Notes:  When using this routine, the  current last block of the

file is not written to the tape

Disc:   Similar to the tape routine; if more than 16K of a file

has been written to the disc, then the first 16K of the

file will exist on the  disc  with  a file extension of

.$$$ because each 16K  section  of  the file requires a

separate directory entry

135   &BC95   *CAS OUT CHAR

Action: Writes a single byte to a file

Entry:  A contains the byte to  be  written  to the file output

buffer

Exit:   If a byte was  written  to  the  buffer,  then Carry is

true, Zero is false, and A  is corrupt; if the file was

not open, then Carry and Zero are false, and A contains

an error number (664/6128 only)  or  is corrupt (on the

464); if ESC was pressed, then  Carry is false, Zero is

true, and A contains an error number (664/6128 only) or

it is corrupt (on the  464);  in  all cases, IX and the

other flags are corrupt, and all others are preserved

Notes:  If the 2K buffer is full of  data then it is written to

the tape before  the  new  character  is  placed in the

buffer; it is important to call  CAS OUT CLOSE when all

the data has been sent  to  the  file  so that the last

block is written to the tape

Disc:   All the above applies to the disc routine

136   &BC98   *CAS OUT DIRECT

Action: Writes an entire file directly to tape

Entry:  HL contains the address  of  the  data  which  is to be

written to tape, DE contains  the  length of this data,

BC contains the e~ecution  address,  and A contains the

file type

Exit:   If the operation was  successful,  then  Carry is true,

Zero is false, and A  is  corrupt;  if the file was not

open, Carry and Zero are false, A holds an error number

(664/6128) or is  corrupt  (464);  if  ESC was pressed,

then Carry is false, Zero is true, and A holds an error

code (664/6128 only); in all  cases  BC, DE, HL, IX and

the  other  flags  are  corrupt,  and  the  others  are

preserved

Notes:  This routine cannot be used once  CAS OUT CHAR has been

used

Disc:   All the above applies to the disc routine

137   &BC9B   *CAS CATALOG

Action: Creates a catalogue of all the files on the tape

Entry:  DE contains the address of the  2K buffer to be used to

store the information

Exit:   If the operation was  successful,  then  Carry is true,

Zero is false, and A is  corrupt; if the read stream is

already being used, then Carry  and Zero are false, and

A holds an error code (664/6128  or is corrupt (for the

464); in all cases, BC, DE,  HL, IX and the other flags

are corrupt and all others are preserved

Notes:  This routine is only left  when  the ESC key is pressed

(cassette only) and is identical to BASIC's CAT command

Disc:   All tbe above applies,  except  that  a  sorted list of

files is displayed; system files are not listed by this

routine

138   &BC9E   CAS WRITE

Action: Writes data to the tape in one  long file (ie not in 2K

blocks)

Entry:  HL contains the address of  the  data  to be written to

tape, DE contains the length of the data to be written,

and A contains the sync character

Exit:   If the operation was successful, then Carry is true and

A is corrupt; if an error  occurred then Carry is false

and A contains an error code; in both cases, BC, DE, HL

and  lX  are  corrupt,  and  all  other  registers  are

preserved

Notes:  For header records the sync  character  is &2C, and for

data it is  &16;  this  routine  starts  and  stops the

cassette motor  and  also  tums  off  interrupts whilst

writing data

139   &BCA1   CAS READ

Action: Reads data from  the  tape  in  one  long  file  (ie as

originally written by CAS WRITE only)

Entry:  HL holds the address to  place  the  file, DE holds the

length of the  data,  and  A  holds  the  expected sync

character

Exit:   If the operation was successful, then Carry is true and

A is corrupt; if an error  occurred then Carry is false

and A contains an error code; in both cases, BC, DE, HL

and  IX  are  corrupt,  and  all  other  registers  are

preserved

Notes:  For header records the sync  character  is &2C, and for

data it is  &16;  this  routine  starts  and  stops the

cassette motor and turns  off interrupts whilst reading

data

140   &BCA4   CAS CHECK

Action: Compares the contents of memory  with a file record (ie

header or data) on tape

Entry:  HL contains  the  address  of  the  data  to  check, DE

contains the length of the  data  and  A holds the sync

character that was used  when  the  file was originally

written to the tape

Exit:   If the two are identical, then  Carry  is true and A is

corrupt; if an error occurred then Carry is false and A

holds an error code; in all  cases,  BC, DE, HL, IX and

other flags are corrupt,  and  all  other registers are

preserved

Notes:  For header records the sync  character  is &2C, and for

data it is  &16;  this  routine  starts  and  stops the

cassette motor and turns  off interrupts whilst reading

data; does not have to read  the whole of a record, but

must start at the beginning

The Sound Manager

141   &BCA7   SOUND RESET

Action: Resets the sound manager  by  clearing the sound queues

and abandoning any current sounds

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

142   &BCAA   SOUND QUEUE

Action: Adds a sound to the sound queue of a channel

Entry:  HL contains the  address  of  a  series  of bytes which

define the sound and are  stored  in the central 32K of

RAM

Exit:   If the sound was successfully  added to the queue, then

Carry is true and HL  is  corrupt;  if one of the sound

queues  was  full,  then  Carry  is  false  and  HL  is

preserved; in either case, A, BC,  DE, IX and the other

flags are corrupt, and all others are preserved

Notes:  The bytes required to define  the sound are as follows:

byte 0 - channel status byte

byte 1 - volume envelope to use

byte 2 - tone envelope to use

bytes 3&4 - tone period

byte 5 - noise period

byte 6 - start volume

bytes 7&8 - duration of  the  sound, or envelope repeat

count

143   &BCAD   SOUND CHECK

Action: Gets the status of a sound channel

Entry:  A contains the channel to test  -  for channel A, bit 0

set; for channel B, bit 1 set; for channel C, bit 2 set

Exit:   A contains the channel status, BC, DE, HL and flags are

corrupt, and all others are preserved

Notes:  The channel  status  returned  is  bit  significant, as

follows:

bits 0 to 2  - the  number  of free spaces in the sound

queue

bit 3 - trying to rendezvous with channel A

bit 4 - trying to rendezvous with channel B

bit 5 - trying to rendezvous with channel C

bit 6 - holding the channel

bit 7 - producing a sound

144   &BCB0   SOUND ARM EVENT

Action: Sets up an event which  will  be activated when a space

occurs in a sound queue

Entry:  A contains the channel  to  set  the  event up for (see

SOUND CHECK for the bit  values  this can take), and HL

holds the address of the event block

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  The event block must  be  initialised  by KL INIT EVENT

and is disarmed when the event itself is run

145   &BCB3   SOUND RELEASE

Action: Allows the playing of sounds  on specific channels that

had been stopped by SOUND HOLD

Entry:  A contains the sound channels to be released (see SOUND

CHECK for the bit values this can take)

Exit:   AF, BC, DE, HL and IX  are  corrupt, and all others are

preserved

146   &BCB6   SOUND HOLD

Action: Immediately stops all sound output (on all channels)

Entry:  No entry conditions

Exit:   If a sound was being  made,  then  Carry is true; if no

sound was being  made,  then  Carry  is  false;  in all

cases, A, BC, HL and  other  flags are corrupt, and all

others are preserved

Notes:  When the sounds  are  restarted,  they  will begin from

exactly the same place that they were stopped

147   &BCB9   SOUND CONTINUE

Action: Restarts all sound output (on all channels)

Entry:  No entry conditions

Exit:   AF, BC, DE  and  IX  are  corrupt,  and  all others are

preserved

148   &BCBC   SOUND AMPL ENVELOPE

Action: Sets up avolume envelope

Entry:  A holds an envelope number (from 1 to 15), HL holds the

address of a block of data for the envelope

Exit:   If it was set up properly,  Carry is true, HL holds the

data block address + 16, A  and  BC are corrupt; if the

envelope number is invalid, then Carry is false, and A,

B and HL are  preserved;  in  either  case,  DE and the

other flags are corrupt,  and  all  other registers are

preserved

Notes:  All the rules of  enevelopes  in  BASIC also apply; the

block of  the  data  for  the  envelope  is  set  up as

follows:

byte 0 - number of sections in the envelope

bytes 1 to 3 - first section of the envelope

bytes 4 to 6 - second section of the envelope

bytes 7 to 9 - third section of the envelope

bytes 10 to 12 - fourth section of the envelope

bytes 13 to 15 - fifth section of the envelope

Each section of the envelope has three bytes set out as

follows:

byte 0 - step count (with bit 7 set)

byte 1 - step size

byte 2 - pause time

or if it  is  a  hardware  envelope,  then each section

takes the following form:

byte 0 - envelope shape (with bit 7 not set)

bytes 1 and 2 - envelope period

See also SOUND TONE ENVELOPE below

149   &BCBF   SOUND TONE ENVELOPE

Action: Sets up a tone envelope

Entry:  A holds an envelope number (from 1 to 15), HL holds the

address of a block of data for the envelope

Exit:   If it was set up properly,  Carry is true, HL holds the

data block address + 16, A and BC are corrupt; í if the

envelope number is invalid, then Carry is false, and A,

B and HL are  preserved;  in  either  case,  DE and the

other flags are corrupt,  and  all  other registers are

preserved

Notes:  All the rules of  envelopes  in  BASIC  also apply; the

block of  the  data  for  the  envelope  is  set  up as

follows:

byte 0 - number of sections in the envelope

bytes 1 to 3 - first section of the envelope

bytes 4 to 6 - second section of the envelope

bytes 7 to 9 - third section of the envelope

bytes 10 to 12 - fourth section of the envelope

bytes 13 to 15 - fifth section of the envelope

Each section of the envelope has three bytes set out as

follows:

byte 0 - step count

byte 1 - step size

byte 2 - pause time

See also SOUND AMPL ENVELOPE above

150   &BCC2   SOUND A ADDRESS

Action: Gets the address of  the  data  block associated with a

volume envelope

Entry:  A contains an envelope number (from 1 to 15)

Exit:   If it was found, then Carry  is true, HL holds the data

block's address,  and  BC  holds  its  length;  if  the

envelope number is invalid, then  Carry is false, HL is

corrupt and BC is preserved;  in  both cases, A and the

other flags are corrupt, and all others are preserved

151   &BCC5   SOUND T ADDRESS

Action: Gets the address of  the  data  block associated with a

tone envelope

Entry:  A contains an envelope number (from 1 to 15)

Exit:   If it was found, then Carry  is true, HL holds the data

block's address,  and  BC  holds  its  length;  if  the

envelope number is invalid, then  Carry is false, HL is

corrupt and BC is preserved;  in  both cases, A and the

other flags are corrupt, and all others are preserved

The Kernel

152   &BCC8   KL CHOKE OFF

Action: Clears all  event  queues  and  timer  lists,  with the

exception of keyboard scanning and sound routines

Entry:  No entry conditions

Exit:   B contains the foreground ROM  select address (if any),

DE contains the  ROM  entry  address,  C  holds the ROM

select address for a RAM  foreground program, AF and HL

are corrupt, and all others are preserved

153   &BCCB   KL ROM WALK

Action: Finds and initialises all background ROMs

Entry:  DE holds  the  address  of  the  first  usable  byte of

memory, HL holds the address of the last usable byte

Exit:   DE holds the address of  the  new  first usable byte of

memory, HL holds the  address  of  the  new last usable

byte, AF and BC  are  corrupt,  and all other registers

are preserved

Notes:  This routine looks at the  ROM  select addresses from 0

to l5 (1 to 7 for the 464) and calls the initialisation

routine of any ROMs present; these routines may reserve

memory by adjusting DE and  HL before returning control

to KL ROM WALK, and the  ROM  is then added to the list

of command handling routines

154   &BCCE   KL INIT BACK

Action: Finds and initialises a specific background ROM

Entry:  C contains the ROM select address  of the ROM, DE holds

the address of  the  first  usable  byte  of memorv, HL

holds the address of the last usable byte of memory

Exit:   DE holds the address  of  the  new  first usaUe byte of

memory, HL holds the  address  of  the  new last usable

byte.  AF and B  are  corrupt,  and all other registers

are preserved

Notes:  The ROM select address must be in  the range of 0 to 15

(or 1 to 7 for the  464)  although address 7 is tor the

AMSDOS/CPM ROM if  present.   The  ROM's initialisation

routine is then called and  some memory may be reserved

for the ROM by adjusting the values of DE and HL before

returning control to KL INlT BACK

155   &BCD1   KL LOG EXT

Action: Logs on a new RSX to the firmware

Entry:  BC contains the address of  the RSX's command table, HL

contains the address of four  bytes exclusively for use

by the firmware

Exit:   DE is corrupt, and all other registers are preserved

156   &BCD4   KL FIND COMMAND

Action: Searches an RSX, background  ROM  or foreground ROM, to

find a command in its table

Entry:  HL contains the address  of  the  command  name (in RAM

only) which is being searched for

Exit:   If the narne was found in  a RSX or background ROM then

Carry is true, C contains  the  ROM select address, and

HL contains the address of  the routine; if the command

was not found,  then  Carry  is  false,  C  and  HL are

corrupt; in either case, A,  B  and DE are corrupt, and

all others are preserved

Notes:  The command names should be in  upper case and the last

character should have &80 added  to it; the sequence of

searching is RSXs, then ROMs  with lower numbers before

ROMs with higher numbers

157   &BCD7   KL NEW FRAME FLY

Action: Sets up a frame flyback event block which will be acted

on whenever a frame flyback occurs

Entry:  HL contains the  address  of  the  event  block  in the

central 32K of  RAM,  B  contains  the  event class.  C

contains the  ROM  select  address  (if  any),  and  DE

contains the address if the event routine

Exit:   AF, DE and HL are corrupt,  and all other registers are

preserved

158   &BCDA   KL ADD FRAME FLY

Action: Adds an existing but deleted  frame flyback event block

to the list of routines run when a frame flyback occurs

Entry:  HL contains the  address  of  the  event  block (in the

central 32K of RAM)

Exit:   AF, DE and HL are corrupt, and all others are preserved

159   &BCDD   KL DEL FRAME FLY

Action: Removes a frame flyback  event  block  from the list of

routines which are mn when a frame flyback occurs

Entry:  HL contains the address of the event block

Exit:   AF, DE and HL are corrupt, and all others are preserved

160   &BCE0   KL NEW FAST TICKER

Action: Sets up a fast  ticker  event  block  which will be run

whenever the l/300th second ticker interrupt occurs

Entry:  HL contains the  address  of  the  event  block (in the

central 32K of  RAM),  B  contains  the  event class, C

contains the  ROM  select  address  (if  any),  and  DE

contains the address of the event routine

Exit:   AF, DE and HL are corrupt,  and all other registers are

preserved

161   &BCE3   KL ADD FAST TICKER

Action: Adds an existing but deleted fast ticker event block to

the list of routines which are run when the l/300th sec

ticker interrupt occurs

Entry:  HL contains the address of the event block

Exit:   AF, DE and HL are corrupt,  and all the other registers

are preserved

162   &BCE6   KL DEL FAST TICKER

Action: Removes a fast  ticker  event  block  from  the list of

routines run  when  the  l/300th  sec  ticker interrupt

occurs

Entry:  HL contains the address of the event block

Exit:   AF, DE and HL are corrupt, and all others are preserved

163   &BCE9   KL ADD TICKER

Action: Sets up a ticker event block which will be run whenever

a 1/50th second ticker interrupt occurs

Entry:  HL contains the  address  of  the  event  block (in the

central 32K of RAM), DE  contains the initial value for

the counter, and BC  holds  the  value that the counter

will be given whenever it reaches zero

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all  the other

registers are preserved

Notes:  Every 1/50th of a second all the tick blocks are looked

at and  their  counter  is  decreased  by  1;  when the

counter reaches zero,  the  event  is  'kicked' and the

counter is loaded with the value  in BC; any tick block

with a counter of 0  is  ignored,  and therefore if the

value in BC is 0,  the  event  will be kicked only once

and ignored after that

164   &BCEC   KL DEL TICKER

Action: Removes a ticker event block  from the list of routines

that are run when a l/50th sec ticker interrupt occurs

Entry:  HL contains the address of the event block

Exit:   If the event block was  found,  then Carry is true, and

DE holds the value  remaining  of  the  counter; if the

event block was not found, then  Carry is false, and DE

is corrupt; in both cases,  A,  HL  and the other flags

are corrupt, and all other registers are preserved

165   &BCEF   KL INIT EVENT

Action: Initialises an event block

Entry:  HL contains the  address  of  the  event  block (in the

central 32K of RAM), B contains the class of event, and

C contains the ROM  select  address,  and  DE holds the

address of the event routine

Exit:   HL holds the  address  of  the  event  block+7, and all

other registers are preserved

Notes:  The event class is derived as follows:

bit 0 -indicates a near address

bits 1 to 4 - hold the synchronous event priority

bit 5 - always zero

bit 6 - if bit 6 is set, then it is an express event

bit 7 - if bit  7  is  set,  then it is an asynchronous

event.  Asynchronous events do  not have priorities; if

it is an  express  asynchronous  event,  then its event

routine is called from the  interrupt  path; if it is a

normal asynchronous event,  then  its  event routine is

called just before returning from  the interrupt; if it

is an express synchronous event,  then  it has a higher

priority than normal synchronous events, and it may not

be disabled through use  of  KL  EVENT  DISABLE; if the

near address bit is set, then the routine is located in

the central 32K  of  RAM  and  is  called  directly, so

saving time; no event may have a priority of zero

166   &BCF2   KL EVENT

Action: Kicks an event block

Ently:  HL contains the address of the event block

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

167   &BCF5   KL SYNC RESET

Action: Clears the synchronous event queue

Entry:  No entry conditions

Exit:   AF and HL  are  corrupt,  and  all  other registers are

preserved

Notes:  When using this routine, all events that are waiting to

be dealt with are simply discarded

168   &BCF8   KL DEL SYNCHRONOUS

Action: Removes a synchronous event from the event queue

Entry:  HL contains the address of the event block

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

169   &BCFB   KL NEXT SYNC

Action: Finds out if there is a synchronous event with a higher

priority

Entry:  No entry conditions

Exit:   If there is an  event  to  be  processed, then Carry is

true, HL contains the address of the event block, and A

contains the priority of  the  previous event; if there

is no event to be processed, then Carry is false, and A

and HL are corrupt; in either  case, DE is corrupt, and

all other registers are preserved

170   &BCFE   KL DO SYNC

Action: Runs a synchronous event routine

Entry:  HL contains the address of the event block

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  See KL DONE SYNC below

171   &BD01   KL DONE SYNC

Action: Finishes running a synchronous event routine

Entry:  A contains the priority of  the  previous event, and HL

contains the address of the event block

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  When an event that is waiting  to be processed has been

found by KL NEXT SYNC, the  event routine should be run

by KL DO SYNC; after this KL DONE SYNC should be called

so that the event  counter  can  be  decreased - if the

counter is greater than zero  then  the event is placed

back on the synchronous event queue

172   &BD04   KL EVENT DISABLE

Action: Disables norrnal synchronous events

Entry:  No entry conditions

Exit:   HL is corrupt, and all other registers are preserved

173   &BD07   KL EVENT ENABLE

Action: Enables normal synchronous events

Entry:  No entry conditions

Exit:   HL is corrupt, and all other registers are preserved

174   &BD0A   KL DISARM EVENT

Action: Disarrns a specific event and stops it from occurring

Entry:  HL contains the address of the event block

Exit:   AF is corrupt, and all other registers are preserved

Notes:  This routine should be used to disarm only asynchronous

events; see also KL DEL SYNCHRONOUS

175   &BD0D   KL TIME PLEASE

Action: Returns the time that  has  elapsed  since the computer

was switched on or reset (in 1/300ths of a second)

Entry:  No entry conditions

Exit:   DEHL contains the four byte  count of the time elapsed,

and all other registers are preserved

Notes:  D holds the most signifilcant byte of the time elapsed,

and L holds the least  significant; the four byte count

overflows after approximately l66 days have elapsed.

176   &BD10   KL TIME SET

Action: Sets the elapsed time (in lí300ths of a second)

Entry:  DEHL contains the four byte count of the time to set

Exit:   AF is corrupt, and all other registers are preserved

The Machine Pack

177   &BD13   MC BOOT PROGRAM

Action: Loads a program into RAM and then executes it

Entry:  HL contains the address of the routine which is used to

load the program

Exit:   Control is  handed  over  to  the  program  and  so the

routine is not returned from

Notes:  All events, sounds and  interrupts  are turned off, the

fiImware indirections  are  returned  to  their default

settings, and the stack  is  reset;  the routine to run

the program should be in  the  central block of memory,

and should obey the  following  exit conditions: if the

program was loaded  successfully,  then  Carry is true,

and HL  contains  the  prograrn  entry  point;  if  the

program failed to load, then Carry  is false, and HL is

corrupt; in either case,  A,  BC,  DE,  IX,  IY and the

other flags are all corrupt  Should the program fail to

load, control is  returned  to  the previous foreground

program

178   &BD16   MC START PROGRAM

Action: Runs a foreground program

Entry:  HL contains the  entry  point  for  the  program, and C

contains the ROM selection number

Exit:   Control is handed  over  to  the  prograrn  and  so the

routine is not returned from

179   &BD19   MC WAIT FLYBACK

Action: Waits until a frame flyback occurs

Entry:  No entry conditions

Exit:   All registers are preserved

Notes:  When the frame flyback occurs  the  screen is not being

written to and so the  screen c~n be manipulated during

this period without any  flickering  or ghosting on the

screen

180   &BD1C   MC SET MODE

Action: Sets the screen mode

Entry:  A contains the required mode

Exit:   AF is corrupt, and all other registers are preserved

Notes:  Although this routine changes  the  screen mode it does

not inform the routines which  write to the screen that

the mode has  been  changed;  therefore  these routines

will write to the screen  as  if  the mode had not been

changed; however as  the  hardware  is now interpreting

these signals differently, unusual effects may occur

181   &BD1F   MC SCREEN OFFSET

Action: Sets the screen offset

Entry:  A contains the screen base,  and HL contains the screen

offset

Exit:   AF is corrupt, and all other registers are preserved

Notes:  As with MC SET MODE,  this routine changes the hardware

setting without telling the routines  that write to the

screen; therefore these  routines  may cause unpredict-

able effects if called; the default screen base is &C0

182   &BD22   MC CLEAR INKS

Action: Sets all the PENs  and  the  border  to  one colour, so

making it seem as if the screen has been cleared

Entry:  DE contains the address of the ink vector

Exit:   AF is corrupt, and all other registers are preserved

Notes:  The ink vector takes the following form:

byte 0 - holds the colour for the border

byte 1 - holds  the  colour  for  all  of  the PENs The

values for the colours are all given as hardware values

183   &BD25   MC SET INKS

Action: Sets the colours of all the PENs and the border

Entry:  DE contains the address of the ink vector

Exit:   AF is corrupt, and all other registers are preserved

Notes:  The ink vector takes the following form:

byte 0 - holds the colour for the border

byte 1 - holds the colour for PEN 0...

... byte 16 - holds the colour for PEN 15

The values for the  colours  are  all given as hardware

values; the routine sets all sixteen PEN's

184   &BD28   MC RESET PRINTER

Action: Sets the MC WAIT  PRINTER  indirection  to its original

routine

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

185   &BD2B   MC PRINT CHAR

Action: Sends a character to the  printer  and detects if it is

busy for too long (more than 0.4 seconds)

Entry:  A contains the character to  be  printed - only charac-

ters upto ASCII 127 can be printed

Exit:   If the character was sent properly, then Carry is true;

if the printer was busy, then Carry is false; in either

case, A and the other flags  are corrupt, and all other

registers are preserved

Notes:  This routine uses the MC WAIT PRINTER indirection

186   &BD2E   MC BUSY PRINTER

Action: Tests to see if the printer is busy

Entry:  No entry conditions

Exit:   If the printer is  busy,  then  Carry  is  true; if the

printer is not  busy,  then  Carry  is  false;  in both

cases, the  other  flags  are  corrupt,  and  all other

registers are preserved

187   &BD31   MC SEND PRINTER

Action: Sends a character to  the  printer,  which  must not be

busy

Entry:  A contains tlle character to  be printed - only charac-

ters up to ASCII 127 can be printed

Exit:   Carry is true, A and  the  other flags are corrupt, and

all other registers are preserved

188   &BD34   MC SOUND REGISTER

Action: Sends data to a sound chip register

Entry:  A contains the  register  nurnber,  and  C contains the

data to be sent

Exit:   AF and BC  are  corrupt,  and  all  other registers are

preserved

189   &BD37   JUMP RESTORE

Action: Restores the jumpblock to its default state

Entry:  No entry conditions

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  This routine does  not  affect  the  indirections jump-

block, but restores all entries in the main jumpblock

664 and 6128 only

190   &BD3A   KM SET LOCKS

Action: Turns the shift and caps locks on and off

Entry:  H contains the  caps  lock  state,  and  L contains the

shift lock state

Exit:   AF is corrupt, and all others are preserved

Notes:  In this routine, &00  means  turned  off, and &FF means

turned on

191   &BD3D   KM FLUSH

Action: Empties the key buffer

Entry:  No entry conditions

Exit:   AF is corrupt, and all other registers are preserved

Notes:  This routine also discards any current expansion string

192   &BD40   TXT ASK STATE

Action: Gets the VDU and cursor state

Entry:  No entry conditions

Exit:   A contains the  VDU  and  cursor  state,  the flags are

corrupt, and all others are preserved

Notes:  The value in  the  A  register  is  bit significant, as

follows:

if bit 0 is set, then the cursor is disabled, otherwise

it is enabled

if bit  1  is  set,  then  the  cursor  is  turned off,

otherwise it is on

if bit 7 is set, then  the VDU is enabled, otherwise it

is disabled

193   &BD43   GRA DEFAULT

Action: Sets the graphics VDU to its default mode

Entry:  No entry conditions

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  Sets the background to opaque,  the first point of line

is plotted, lines aren't dotted,  and the write mode is

force

194   &BD46   GRA SET BACK

Action: Sets the graphics background  mode  to either opaque or

transparent

Entry:  A holds zero if opaque mode  is wanted, or holds &FF to

select transparent mode

Exit:   All registers are preserved

195   &BD49   GRA SET FIRST

Action: Sets whether the first point  of  a  line is plotted or

not

Entry:  A holds zero if the first  point  is not to be plotted,

or holds &FF if it is to be plotted

Exit:   All registers are preserved

196   &BD4C   GRA SET LINE MASK

Action: Sets how the points in a  line are plotted - ie defines

whether a line is dotted or not

Entry:  A contains the line mask that will be used when drawing

lines

Exit:   All registers are preserved

Notes:  The first point in the line corresponds to bit 7 of the

line mask and after bit 0 the mask repeats; if a bit is

set then that point will be plotted; the mask is always

applied from left to right, or from bottom to top

197   &BD4F   GRA FROM USER

Action: Converts user coordinates into base coordinates

Entry:  DE contains the user X  coordinate, and HL contains the

user Y coordinate

Exit:   DE holds the base X coordinate, and HL holds the base Y

coordinate, AF is corrupt, and all others are preserved

198   &BD52   GRA FILL

Action: Fills an area of the  screen  starting from the current

graphics position and extending until it reaches either

the edge of the window or a pixel set to the PEN

Entry:  A holds a PEN to fill with, HL holds the address of the

buffer, and DE holds the length of the buffer

Exit:   If the area was filled properly, then Carry is true; if

the area was not filled, then Carry is false; in either

case, A, BC, DE, HL  and  the  other flags are corrupt,

and all others are preserved

Notes:  The buffer is  used  to  store  complex  areas to fill,

which are remembered and  filled  when  the basic shape

has been done;  each  entry  in  the  buffer uses seven

bytes and so the more complex  the shape the larger the

buffer; if it runs out of  space to store these complex

areas, it will fill what  it  can  and then return with

Carry false

199   &BD55   SCR SET POSITION

Action: Sets the screen  base  and  offset  without telling the

hardware

Entry:  A contains the screen base,  and HL contains the screen

offset

Exit:   A contains the masked screen  base, and HL contains the

masked screen offset, the  flags  are  corrupt, and all

other registers are preserved

200   &BD58   MC PRINT TRANSLATION

Action: Sets how ASCII  characters  will  be  translated before

being sent to the printer

Entry:  HL contains the address of the table

Exit:   If the table is too long,  then Carry is false (ie more

than 20 entries); if  the  table  is correctly set out,

then Carry is true; in either  case,  A, BC, DE, HL and

the  other  flags  are  corrupt,  and  all  others  are

preserved

Notes:  The first byte in the  table  is the number of entries;

each entry requires two bytes, as follows:

byte 0 - the character to be translated

byte 1 -  the  character  that  is  to  be  sent to the

printer If the character to  be  sent to the printer is

&FF, then the character is ignored and nothing is sent

201   &BD5B   KL BANK SWITCH (6128 only)

Action: Sets which RAM banks are being accessed by the Z80

Entry:  A contains the organisation that is to be used

Exit:   A contains the  previous  organisation,  the  flags are

corrupt, and all other registers are preserved

The Firmware Indirections

000   &BDCD   TXT DRAW CURSOR

Action: Places the cursor  on  the  screen,  if  the  cursor is

enabled

Entry:  No entry conditions

Exit:   AF is corrupt, and all other registers are preserved

Notes:  The cursor is  an  inverse  blob  which  appears at the

current text position

001   &BDD0   TXT UNDRAW CURSOR

Action: Removes the cursor from  the  screen,  if the cursor is

enabled

Entry:  No entry conditions

Exit:   AF  is  corrupt,  and  all   the  other  registers  are

preserved

002   &BDD3   TXT WRITE CHAR

Action: Writes a character onto the screen

Entry:  A holds  the  character  to  be  wntten,  H  holds  the

physical column number, and  L  holds the physical line

number

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

003   &BDD6   TXT UNWRITE

Action: Reads a character from the screen

Entry:  H contains the physical  column  number, and L contains

the physical line number to read from

Exit:   If a character was  found,  then  Carry  is true, and A

contains the character; if no character was found, then

Carry is false, and  A  contains  zero; in either case,

BC, DE, HL and  the  other  nags  are  corrupt, and all

other registers are preserved Notes: This routine works

by comparing the image on the screen with the character

matrices; therefore if the character matrices have been

altered the routine may not find a readable a character

004   &BDD9   TXT OUT ACTION

Action: Writes a character to  the  screen  or  obeys a control

code (&00 to &1F)

Entry:  A contains the character or code

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  Control codes may take  a  maximum  of nine parameters;

when a control code  is  found,  the required number of

parameters is read into  the  control  code buffer, and

then the control code  is  acted  upon; if the graphics

character wnte mode  is  enabled,  then  characters and

codes are printed using  the  graphics  VDU; when using

the graphics VDU  control  codes  are  printed  and not

obeyed

005   &BDDC   GRA PLOT

Action: Plots a point in the current graphics PEN

Entry:  DE contains the user X  coordinate, and HL contains the

user Y coordinate of the point

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  This routine uses the  SCR  WRITE  indirection to write

the point to the screen

006   &BDDF   GRA TEST

Action: Tests a point and finds out what PEN it is set to

Entry:  DE contains the user X  coordinate, and HL contains the

user Y coordinate of the point

Exit:   A contains the PEN that the point is written in, BC, DE

and HL are corrupt, and all others are preserved

Notes:  This routine uses the  SCR  READ  indirection to test a

point on the screen

007   &BDE2   GRA LINE

Action: Draws a line  in  the  current  graphics  PEN, from the

current graphics position to the specified point

Entry:  DE contains the user X  coordinate, and HL contains the

user Y coordinate for the endpoint

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all others are

preserved

Notes:  This routine uses the  SCR  WRITE  indirection to write

the points of the line on the screen

008   &BDE5   SCR READ

Action: Reads a pixel from the screen  and returns its decode a

PEN

Entry:  HL contains the  screen  address  of  the  pixel, and C

contains the mask for the pixel

Exit:   A contains the decoded PEN of  the pixel, the flags are

corrupt, and all others are preserved

Notes:  The mask should be for a single pixel, and is dependent

on the screen mode

009   &BDE8   SCR WRITE

Action: Writes one or more pixels to the screen

Entry:  HL contains the screen address of the pixel, C contains

the mask, and B contains the encoded PEN

Exit:   AF is corrupt, and all other registers are preserved

Notes:  The mask should determine  which  pixels  in the screen

byte are to be plotted

010   &BDEB   SCR MODE CLEAR

Action: Fills the entire screen  memory  with &00, which clears

the screen to PEN 0

Entry:  No entry conditions

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all  the other

registers are preserved

011   &BDEE   KM TEST BREAK

Action: Tests if  the  ESC  key  has  been  pressed,  and  acts

accordingly

Entry:  C contains  the  Shift  and  Control  key  states,  and

interrupts must be disabled

Exit:   AF and HL  are  corrupt,  and  all  other registers are

preserved

Notes:  If bit 7 of C is set,  then the Control key is pressed;

if bit 5 of C is set, then the Shift key is pressed; if

ESC, Shift and Control are  pressed  at the sarne time,

then it initiates a system  reset; otherwise it reports

a break event

012   &BDF1   MC WAIT PRINTER

Action: Sends a character to the printer if it is not busy

Entry:  A contains the character to be sent to the printer

Exit:   If the character was  printed  successfully, then Carry

is true; if the  printer  was  busy  for too long (more

than 0.4 seconds), then Carry is false; in either case,

A and BC  are  corrupt,  and  all  other  registers are

preserved

013   &BDF4   KM SCAN KEYS

Action: Scans the  keyboard  every  1/50th  of  a  second,  and

updates the status of all keys

Entry:  All interrupts must be disabled

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

The Maths Firmware

000   &BDC1   MOVE REAL (&BD3D for the 464)

Action: Copies the five bytes that are  pointed to by DE to the

location held in HL

Entry:  DE points to the source  real  value,  and HL points to

the destination

Exit:   HL points to the real  value  in the destination, Carry

is true if the move  went  properly,  F is corrupt, and

all other registers are preserved

Notes:  For the 464 only, A holds the exponent byte of the real

value when the routine is exited

001   &BD64   INTEGER TO REAL (&BD40 for the 464)

Action: Converts an integer value into a real value

Entry:  HL holds the integer  value,  DE  points  to the desti-

nation for the real value, bit 7 of A holds the sign of

the integer value - it is taken to be negative if bit 7

is set

Exit:   HL points to the real value  in the destination, AF and

DE are corrupt, and all others are preserved

002   &BD67   BINARY TO REAL (&BD43 for the 464)

Action: Converts a four byte binary value  into a real value at

the same location

Entry:  HL points to the binary  value,  bit  7  of A holds the

sign of the binary value - negative if it is set

Exit:   HL points to the real  value  in  lieu of the four byte

binary  value,  AF  is  corrupt,  and  all  others  are

preserved

Notes:  A four byte binary value  is  an unsigned integer up to

&FFFFFFFF and is stored with the least significant byte

first, and with the most significant byte last

003   &BD6A   REAL TO INTEGER (&BD46 for the 464)

Action: Converts a real  value,  rounding  it  into an unsigned

integer value held in HL

Entry:  HL points to the real value

Exit:   HL holds  the  integer  value,  Carry  is  true  if the

conversion worked successfully, the Sign flag holds the

sign of the integer (negative if it is set).  A, IX and

the other flags are  corrupt,  and  all other registers

are preserved

Notes:  This rounds the decimal part  down  if  it is less than

0.5, but rounds up if it  is  greater than, or equal to

0.5

004   &BD6D   REAL TO BINARY (&BD49 for the 464)

Action: Converts a real value,  rounding  it  into  a four byte

binary value at the same location

Entry:  HL points to the real value

Exit:   HL points to  the  binary  value  in  lieu  of the real

value, bit 7 of B holds  the  sign for the binary value

(it is negative if bit  7  is  set),  AF,  B and IX are

corrupt, and all other registers are preserved

Notes:  See REAL TO INTEGER for  details  of how the values are

rounded up or down

005   &BD70   REAL FIX (&BD4C for the 464)

Action: Performs an equivalent  of  BASIC's  FIX  function on a

real value, leaving the  result  as  a four byte binary

value at the same location

Entry:  HL points to the real value

Exit:   HL points to  the  binary  value  in  lieu  of the real

value, bit 7 of B has the  sign of the binary value (it

is negative if bit 7 is set), AF, B and IX are corrupt,

and all others are preserved

Notes:  FIX removes any  decimal  part  of  the value, rounding

down whether  positive  or  negative  -  see  the BASIC

handbook for more details on the FIX command

006   &BD73   REAL INT (&BD4F for the 464)

Action: Performs an equivalent  of  BASIC's  INT  function on a

real value, leaving the  result  as  a four byte binary

value at the same location

Entry:  HL points to the real value

Exit:   HL points to  the  binary  value  in  lieu  of the real

value, bit 7 of B has the  sign of the binary value (it

is negative if bit 7 is set), AF, B and IX are corrupt,

and all others are preserved

Notes:  INT removes any  decimal  part  of  the value, rounding

down if the nurnber is positive,  but rounding up if it

is negative

007   &BD76   INTERNAL SUBROUTINE - not useful (&BD52 for the 464)

008   &BD79   REAL *10^A (&BD55 for the 464)

Action: Multiplies a real value by 10 to the power of the value

in the A  register,  leaving  the  result  at  the same

location

Entry:  HL points to the real value,  and  A holds the power of

10

Exit:   HL points to the  result,  AF,  BC,  DE,  IX and IY are

corrupt

009   &BD7C   REAL ADDITION (&BD58 for the 464)

Action: Adds two real values, and leaves  the result in lieu of

the first real number

Entry:  HL points to the first real value, and DE points to the

second real value

Exit:   HL points to the  result,  AF,  BC,  DE,  IX and IY are

corrupt

010   &BD82   REAL REVERSE SUBTRACTION (&BD5E for the 464)

Action: Subtracts the first  real  value  from  the second real

value, and leaves  the  result  in  lieu  of  the first

number

Entry:  HL points to the first real value, and DE points to the

second real value

Exit:   HL points to the  result  in  place  of  the first real

value, AF, BC, DE, IX and IY are corrupt

011   &BD85   REAL MULTIPLICATION (&BD61 for the 464)

Action: Multiplies two real  values  together,  and  leaves the

result in lieu of the first number

Entry:  HL points to the first real value, and DE points to the

second real value

Exit:   HL points to the  result  in  place  of  the first real

value, AF, BC, DE, IX and IY are corrupt

012   &BD88   REAL DIVISION (&BD64 for the 464)

Action: Divides the first real value  by the second real value,

and leaves the result in lieu of the first number

Entry:  HL points to the first real value, and DE points to the

second real value

Exit:   HL points to the  result  in  place  of  the first real

value, AF, BC, DE, IX and IY are corrupt

013   &BD8E   REAL COMPARISON (&BD6A for the 464)

Action: Compares two real values

Entry:  HL points to the first real value, and DE points to the

second real value

Exit:   A holds the result of  the  comparison process, IX, IY,

and the other flags  are  corrupt,  and  all others are

preserved

Notes:  After this routine  has  been  called,  the  value in A

depends on the result of the comparison as follows:

if the first real  number  is  greater  than the second

real number, then A holds &01

if the first real number is the same as the second real

number, then A holds &00  if  the second real number is

greater than the first real number, then A holds &FF

014   &BD91   REAL UNARY MINUS (&BD6D for the 464)

Action: Reverses the sign of a real value

Entry:  HL points to the real value

Exit:   HL points to the new value of the real number (which is

stored in place of  the  original  number),  bit 7 of A

holds the sign of the result  (it  is negative if bit 7

is set), AF and IX are corrupt, and all other registers

are preserved

015   &BD94   REAL SIGNUM/SGN (&BD70 for the 464)

Action: Tests a real value, and compares it with zero

Entry:  HL points to the real value

Exit:   A holds the result of  this  comparison process, IX and

the  other  ílags  are  corrupt,  and  all  others  are

preserved

Notes:  After this routine  has  been  called,  the  value in A

depends on the result of the comparison as follows:

if the real number is greater than 0, then A holds &01,

Carry is false, and Zero is false

if the real number is the same  as 0, then A holds &00,

Carry is false, and Zero is true

if the real number is smaller than 0, then A holds &FF,

Carry is true, and Zero is false

016   &BD97   SET ANGLE MODE (&BD73 for the 464)

Action: Sets the angular  calculation  mode  to  either degrees

(DEG) or radians (RAD)

Entry:  A holds the mode setting  -  0  for  RAD, and any other

value for DEG

Exit:   All registers are preserved

017   &BD9A   REAL PI (&BD76 for the 464)

Action: Places the real value of pi at a given memory location

Entry:  HL holds the address at which the  value of pi is to be

placed

Exit:   AF and DE  are  corrupt,  and  all  other registers are

preserved

018   &BD9D   REAL SQR (&BD79 for the 464)

Action: Calculates the square root of a real value, leaving the

result in lieu of the real value

Entry:  HL points to the real value

Exit:   HL points to the result of the calculation, AF, BC, DE,

IX and IY are corrupt

019   &BDA0   REAL POWER (&BD7C for the 464)

Action: Raises the first real value to  the power of the second

real value, leaving the  result  in  lieu  of the íirst

real value

Entry:  HL points to the first real value, and DE points to the

second real value

Exit:   HL points to the result of the calculation, AF, BC, DE,

IX and IY are corrupt

020   &BDA3   REAL LOG (&BD7F for the 464)

Action: Returns the naperian logarithm  (to  base  e) of a real

value, leaving the result in lieu of the real value

Entry:  HL points to the real value

Exit:   HL points to the  logarithrn  that has been calculated,

AF, BC, DE, LY and IY are corrupt

021   &BDA6   REAL LOG 10 (&BD82 for the 464)

Action: Returns the logarithm (to  base  10)  of  a real value,

leaving the result in lieu of the real value

Entry:  HL points to the real value

Exit:   HL points to the  logarithrn  that has been calculated,

AF, BC, DE, IX and IY are corrupt

022   &BDA9   REAL EXP (&BD85 for the 464)

Action: Returns the antilogarithm  (base  e)  of  a real value,

leaving the result in lieu of the real value

Entry:  HL points to the real value

Exit:   HL points  to  the  antilogarithm  that  has  been cal-

culated, AF, BC, DE, IX and IY are corrupt

Notes:  See the BASIC handbook for details of EXP

023   &BDAC   REAL SINE (&BD88 for the 464)

Action: Returns the sine of a real value, leaving the result in

lieu of the real value

Entry:  HL points to the real value (ie all angle)

Exit:   HL points to the sine  value  that has been calculated,

AF, BC, DE, IX and IY are corrupt

024   &BDAF   REAL COSINE (&BD8B for the 464)

Action: Returns the cosine  of  a  real  value,  leaving  a the

result in lieu of the real value

Entry:  HL points to the real value (ie an angle)

Exit:   HL points to the cosine value that has been calculated,

AF, BC, DE, IX and IY are corrupt

025   &BDB2   REAL TANGENT (&BD8E for the 464)

Action: Returns the tangent of a real value, leaving the result

in lieu of the real value

Entry:  HL points to the real value (ie an angle)

Exit:   HL points to  the  tangent  value  that  has  been cal-

culated, AF, BC, DE, IX and IY are corrupt

026   &BDB5   REAL ARCTANGENT (&BD91 for the 464)

Action: Returns the arctangent  of  a  real  value, leaving the

result in lieu of the real value

Entry:  HL points to the real value (ie an angle)

Exit:   HL  points  to  the  arctangent  value  that  has  been

calculated, AF, BC, DE, IX and IY are corrupt

All of the above routines to calculate sine, cosine, tangent

and

arctangent are slightly inaccuarate

027   &BDB8   INTERNAL SUBROUTINE - not useful (&BD94 for the 464)

028   &BDBB   INTERNAL SUBROUTINE - not useful (&BD97 for the 464)

029   &BDBE   INTERNAL SUBROUTINE - not useful (&BD9A for the 464)

Maths Routines for the 464 only

&BD5B   REAL SUBTRACTION

Action: Subtracts the second  real  value  from  the first real

value, and leaves  the  result  in  lieu  of  the first

number

Entry:  HL points to the first real value, and DE points to the

second real value

Exit:   HL points to the  result  in  place  of  the first real

value, AF, BC, DE, IX and IY are corrupt

&BD67   REAL EXPONENT ADDITION

Action: Adds the value of the  A  register to the exponent byte

of a real number

Entry:  HL points to the real value,  and  A holds the value to

he added

Exit:   HL points to the  result  in  place  of  the first real

value, AF  and  IX  are  corrupt,  and  all  others are

preserved

&BD9D   INTERNAL SUBROUTINE - not useful

&BDA0   INTERNAL SUBROUTINE - not useful

&BDA3   INTERNAL SUBROUTINE - not useful

&BDA6   INTERNAL SUBROUTINE - not useful

&BDA9   INTERNAL SUBROUTINE - not useful

&BDAC   INTEGER ADDITION

Action: Adds two signed integer values

Entry:  HL holds the  first  integer  value,  and  DE holds the

second integer value

Exit:   HL holds the result  of  the  addition,  A holds &FF if

there is an overflow  but  is  preserved otherwise, the

flags Z  are  corrupt,  and  all  other  registers  are

preserved

&BDAF   INTEGER SUBTRACTION

Action: Subtracts the  second  signed  integer  value  from the

first signed integer value

Entry:  HL holds the  first  integer  value,  and  DE holds the

second integer value

Exit:   HL holds the result of the  subtraction, A holds &FF if

there is an overflow  but  is  preserved othenvise, the

flags are corrupt,  and  all  the  other  registers are

preserved

&BDB2   INTEGER REVERSE SUBTRACTION

Action: Subtracts the  first  signed  integer  value  from  the

second signed integer value

Entry:  HL holds the  first  integer  value,  and  DE holds the

second integer value

Exit:   HL holds the result of  the  subtraction, AF and DE are

corrupt, and all others are preserved

&BDB5   INTEGER MULTIPLICATION

Action: Multiplies two  signed  integer  values  together,  and

leaves the result in lieu of the first number

Entry:  HL holds the  first  integer  value,  and  DE holds the

second integer value

Exit:   HL holds the result of  the multiplication, A holds &FF

if there is an overflow but is corrupted otherwise, the

flags, BC and DE are  corrupt,  and the other registers

are preserved

Notes:  Multiplication of signed integers  does not produce the

sarne result as with unsigned integers

&BDB8   INTEGER DIVISION

Action: Divides the first signed  integer  value  by the second

signed integer value

Entry:  HL holds the  first  integer  value,  and  DE holds the

second integer value

Exit:   HL holds the  result  of  the  division,  DE  holds the

remainder, AF and BC  are  corrupt,  and all others are

preserved

Notes:  Division of signed integers  does  not produce the same

result as with unsigned integers

&BDBB   INTEGER DIVISION 2

Action: Divides the first signed  integer  value  by the second

signed integer value

Entry:  HL holds the  first  integer  value,  and  DE holds the

second integer value

Exit:   DE holds the  result  of  the  division,  HL  holds the

remainder, AF and BC  are  corrupt,  and all others are

preserved

Notes:  Division of signed integers  does  not produce the same

result as with unsigned integers

&BDBE   INTERNAL SUBROUTINE - not useful

&BDC1   INTERNAL SUBROUTINE - not useful

&BDC4   INTEGER COMPARISON

Action: Compares two signed integer values

Entry:  HL holds the  first  integer  value,  and  DE holds the

second integer value

Exit:   A holds the result of the comparison process, the flags

are corrupt, and all others are preserved

Notes:  After this routine  has  been  called,  the  value in A

depends on the result of the comparison as follows:

if the first real  number  is  greater  than the second

real number, then A holds &01

if the first real  number  is  the  sarne as the second

real number, then A holds &00

if the second real  number  is  greater  than the first

real number, then A holds &FF

With signed integers,  the  range  of  values runs from

&8000 (-32768) via zero  to  &7FFF  (+32767) and so any

value which is  greater  than  &8000  is  considered as

being less than a value of &7FFF or less

&BDC7   INTEGER UNARY MINUS

Action: Reverses the sign of  an  integer value (by subtracting

it from &10000)

Entry:  HL holds the integer value

Exit:   HL holds the new  value  of  the  integer number, AF is

corrupt, cmd all other registers are preserved

&BDCA   INTEGER SIGNUM/SGN

Action: Tests a signed integer value

Entry:  HL holds the integer value

Exit:   A holds the  result  of  this  comparison  process, the

flags are corrupt, and all others are preserved

Notes:  After this routine  has  been  called,  the  value in A

depends on the result of the comparison as follows:

if the integer number  is  greater  than  0 and is less

than &8000, then A holds &01

if the integer number is  the  same  as 0, then A holds

&00

if the integer number  is  greater  than &7FFF and less

than or equal to &FFFF, then A holds &FF

See INTEGER COMPARISON for more details on the way that

signed integers are laid out

Maths Subroutines for the 664 and 6128 only

&BD5E   TEXT INPUT

Action: Allows  upto  255  characters  to  be  input  from  the

keyboard into a buffer

Entry:  HL points to the start of  the buffer - a NUL character

must be placed after any characters already present, or

at the start of the buffer if there is no text

Exit:   A has the last key pressed,  HL  points to the start of

the buffer, the flags are  corrupt,  and all others are

preserved

Notes:  This routine prints any existing contents of the buffer

(upto the NUL character) and then echoes any keys used;

it allows full line  editing  with  the cursor keys and

DEL, etc; it is exited only by use of ENTER or ESC

&BD7F   REAL RND

Action: Creates a new RND real  value  at a location pointed to

by HL

Entry:  HL points to the destination for the result

Exit:   HL points to the RND value, AF, BC, DE and IX registers

are corrupt; and all others are preserved

&BD8B   REAL RND(0)

Action: Returns the last RND value  created,  and  puts it in a

location pointed to by HL

Entry:  HL points  to  the  place  where  the  value  is  to be

returned to

Exit:   HL points to  the  value  created,  AF,  DE  and IX are

corrupt, and all other registers are preserved

Notes:  See the BASIC handbook for more details on RND(0)

AMSDOS and BIOS Firmware

A     &C033   BIOS SET MESSAGE

Action: Enables or disables disc error messages

Entry:  To enable messages, A holds &00; to disable messages, A

holds &FF

Exit:   A holds  the  previous  state,  HL  and  the  flags are

corrupt, and all others are preserved

Notes:  Enabling  and  disabling  the   messages  can  also  be

achieved by poking &BE78 with &00 or &FF

B     &C036   BIOS SETUP DISC

Action: Sets the parameters which effect the disc speed

Entry:  HL holds the address of  the  nine  bytes which make up

the parameter block

Exit:   AF, BC, DE and HL are  corrupt, and all other registers

are preserved

Notes:  The parameter block is arranged as follows:

bytes 0&1 -  the  motor  on  time  in  20ms  units; the

default is &0032; the fastest is &0023

bytes 2&3 -  the  motor  off  time  in  20ms units; the

default is &00FA; the fastest is &00C8

byte 4 - the write off  time in l0µs units; the default

is &AF; should not be changed

byte 5 - the head settle time in 1ms units; the default

is &0F; should not be changed

byte 6 - the step rate  time  in 1ms units; the default

is &0C; the fastest is &0A

byte 7 - the  head  unload  delay;  the default is &01;

should not be changed

byte 8  -  a  byte  of  &03  and  this  should  be left

unaltered

C     &C039   BIOS SELECT FORMAT

Action: Sets a format for a disc

Entry:  A holds the type of format that is to be selected

Exit:   AF, BC, DE  and  HL  are  corrupt,  and  all  the other

registers are preserved

Notes:  To select one of the normal disc formats, the following

values should be put into the A register:

Data format - &C1

System format - &41 - Used by CP/M

IBM format - &01 - compatible with CP/M-86

This routine sets  the  extended  disc  parameter block

(XDPB) at &A890 to &A8A8  -  to  set other formats, the

XDPB must be altered directly

D     &C03C   BIOS READ SECTOR

Action: Reads a sector from a disc into memory

Entry:  HL holds the address in memory where the sector will be

read to, E holds the drive number (&00 for drive A, and

&01 for drive B), D holds the track number, and C holds

the sector number

Exit:   If the sector was read properly,  then Carry is true, A

holds 0, and HL is preserved;  if the read failed, then

Carry is false, A  holds  an  error  number,  and HL is

corrupt; in either case,  the  other flags are corrupt,

and all other registers are preserved

E     &C03F   BIOS WRITE SECTOR

Action: Writes a sector from memory onto disc

Entry:  HL holds the address of memory which will be written to

the disc, E holds the  drive  number  (&00 for drive A,

and &01 for drive B), D  holds  the track number, and C

holds the sector number

Exit:   If the sector was written properly, then Carry is true,

A holds 0, and HL  is  preserved;  if the write failed,

then Carry is false, A holds an error number, and HL is

corrupt; in either case,  the  other flags are corrupt,

and all other registers are preserved

F     &C042   BIOS FORMAT TRACK

Action: Formats a complete  track,  inserts  sectors, and fills

the track with bytes of &E5

Entry:  HL contains  the  address  of  the  header  information

buffer which holds  the  header  information  blocks, E

contains the drive number (&00 for drive A, and &01 for

drive B), and D holds the track number

Exit:   if the formatting process was successful, then Carry is

true, A holds 0, and HL is preserved; if the formatting

process failed, then Carry is  false,  A holds an error

number, and HL is  corrupt;  in  either case, the other

flags are corrupt,  and  all  the  other  registers are

preserved

Notes:  The header information block is laid out as follows:

byte 0 - holds the track number

byte 1 - holds the head number (set to zero)

byte 2 - holds the sector number

byte 3 - holds  log2(sector  size) -7  (usually  either

&02=512 bytes, or &03=1024  bytes).  Header information

blocks must be set up  contiguously for every sector on

the track, and in the same sequence that they are to be

laid down (eg &C1, &C6,  &C2,  &C7, &C3, &C8, &C4, &C9,

&C5)

G     &C045   BIOS MOVE TRACK

Action: Moves the disc drive head to the specified track

Entry:  E holds the drive number (&00  for drive A, and &01 for

drive B), and D holds the track number

Exit:   If the head was moved successfully, then Carry is true,

A holds 0, and  HL  is  preserved;  if the move failed,

then Carry is false, A holds an error number, and HL is

corrupt; in both cases,  the  other  flags are corrupt,

and all other registers are preserved

Notes:  There is normally no need to  call this routine as READ

SECTOR, WRITE  SECTOR  and  FORMAT  TRACK automatically

move the head to the correct position

H     &C048   BIOS GET STATUS

Action: Returns the status of the specified drive

Entry:  A holds the drive number (&00  for drive A, and &01 for

drive B)

Exit:   If Carry is true, then A  holds the status byte, and HL

is preserved; if Carry is false, then A is corrupt, and

HL holds the  address  of  the  byte  before the status

byte; in either case,  the  other  flags are preserved,

and all other registers are preserved

Notes:  The  status  byte  indicates   the  drive's  status  as

follows:

if bit 6 is set, then  either  the write protect is set

or the disc is missing

if bit 5 is set, then  the  drive is ready and the disc

is fitted (whether the disc is formatted or not)

if bit 4 is set, then the head is at track 0

I     &C04B   BIOS SET RETRY COUNT

Action: Sets the number of  times  the  operation is retried in

the event of disc error

Entry:  A holds the number of retries required

Exit:   A holds the  previous  number  of  retries,  HL and the

flags are corrupt, and all others are preserved

Notes:  The default setting is &10,  and the minimum setting is

&01; the number  of  retries  can  also  be  altered by

poking &BE66 with the required value

J     &C56C   GET SECTOR DATA

Action: Gets the data of a sector on the current track

Entry:  E holds the drive number

Exit:   If a formatted disc is present, then Carry is true, and

HL is preserved; if an  unforrnatted disc is present or

the disc is missing, then Carry  is false, and HL holds

the address of  the  byte  before  the  status byte; in

either case, A and the other flags are corrupt, and all

other registers are preserved

Notes:  The track number is held  at  &BE4F, the head number is

held at &BE50, the sector number  is held at &BE51, and

the log2(sector size)-7 is  held  at  &BE52; disc para-

meters do not need to be set to the format of the disc;

this routine is best used  with the disc error messages

turned off

Instruction Set for the Z80 Microprocessor

The following list contains all  the  normal machine code instruc-

tions for the microprocessor, plus a number of undocumented ones.  The

latter comprise those which operate on  the  high  or low bytes of the

Index registets (IX and lY) which  are  notated  here as HIX, LIX, HIY

and LIY - some assemblers may use  the  form  IXH,  etc - and a set of

rotation instructions complementary to SRL, which are designated SLL.

The Opcodes and T states

Within the instructions, a number of abbreviations are used:

d ...... displacement (a value from -128 (&80) to +127 (&7F))

n ...... a single byte value (from 0 (&00) to 255 (&FF))

hilo ... a double byte  value  (from  -32769  (&8000)  via  0 to 65535

(&FFFF))

addr ... an address value (from 0  (&0000)  to  65535 (&FFFF)) (in the

sequence of opcode bytes, 'addr'  and  'hilo' are entered low

byte first)

The next two colurnns  detail  the  number  of bytes applicable to

each instruction, and the number of  T states (clock pulses) that each

requires - some have two figures which are distinguised as follows:

f ...... means 'the number of T states  required when the condition is

false'

t ...... means 'the number of T  states  needed  when the condition is

true'

= ...... means 'the number of T states  needed when either BC=0 and/or

A matches the contents of HL'

# ...... means 'the number of T  states  required  when both the above

conditions are false'

z ...... means 'the number of T states needed when B=0'

nz ..... means 'the number of T states requrred when B<>0'

The Flag Register

The last columns give  the  effect  on  the  flag  bits which each

instruction causes:

? ...... means the setting of the bit is unpredicatable

- ...... means the setting of the bit is unchanged

0 ...... means that the flag bit is reset to zero

1 ...... means that the flag bit is set to one

In addition, the Sign flag (bit 7) is also set:

7 ...... if bit 7 of the A register is set

15 ..... if bit 15  of  the  HL  register  pair  (ie  bit  7  of the H

register) is set

=7 ..... if bit 7 of the  A  register  would  be set by subtraction in

lieu of CP

The Zero flag (bi6 6) is also set:

z ...... if the A register or the HL register pair equals zero

= ...... if the A register matches the compared register or value

=A ..... if the A register matches the contents of the address pointed

to by HL

<>B .... if the B register holds zero

<>b .... if the bit tested is zero

The Parity/Overflow flag (bit 2) is also set:

p ...... if the register concemed contains an even number of set bits

v ...... if an overflow has occured in Two's Complement arithrnetic

BC ..... if BC is not zero

A80 .... if the  A  register  was  &80  before  this  instruction  was

perforrned

i ...... to the contents of  the  rnicroprocessor's internal interrupt

register

The Carry flag (bit 0) is also set:

c ...... if an addition resulted  in  a  carry  out  of  bit  7 (for a

register) or bit 15 (for a register pair)

b ...... if a  subtraction  required  a  borrow  from  bit  7  (for  a

register) or bit 15 (for a register pair)

< ...... if the A register is less than  the value or register that is

being compared

r0 ..... by the bit rotated in from bit 0 of the register concerned

r7 ..... by the bit rotated in from bit 7 of the register concerned

x ...... if the Carry was reset (ie  zero) before this instruction was

performed

A0 ..... if tbe  A  register  was  &00  before  this  instruction  was

performed

Z80 OPCODES

Instruction  ¦ Opcode     ¦ B ¦ Ticks ¦ S  ¦ Z ¦ P   ¦ C

-------------+------------+---+-------+----+---+-----+--

ADC A,(HL)   ¦ 8E         ¦ 1 ¦ 7     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,(IX+d) ¦ DD 8E d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ v   ¦ c

ADC A,(IY+d) ¦ FD 8E d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ v   ¦ c

ADC A,A      ¦ 8F         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,B      ¦ 88         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,C      ¦ 89         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,D      ¦ 8A         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,E      ¦ 8B         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,H      ¦ 8C         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,HIX    ¦ DD 8C      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,HIY    ¦ FD 8C      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,L      ¦ 8D         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,LIX    ¦ DD 8D      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,LIY    ¦ FD 8D      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

ADC A,n      ¦ CE n       ¦ 2 ¦ 7     ¦ 7  ¦ z ¦ v   ¦ c

ADC HL,BC    ¦ ED 4A      ¦ 2 ¦ 15    ¦ 15 ¦ z ¦ v   ¦ c

ADC HL,DE    ¦ ED 5A      ¦ 2 ¦ 15    ¦ 15 ¦ z ¦ v   ¦ c

ADC HL,HL    ¦ ED 6A      ¦ 2 ¦ 15    ¦ 15 ¦ z ¦ v   ¦ c

ADC HL,SP    ¦ ED 7A      ¦ 2 ¦ 15    ¦ 15 ¦ z ¦ v   ¦ c

ADD A,(HL)   ¦ 86         ¦ 1 ¦ 7     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,(IX+d) ¦ DD 86 d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ v   ¦ c

ADD A,(IY+d) ¦ FD 86 d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ v   ¦ c

ADD A,A      ¦ 87         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,B      ¦ 80         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,C      ¦ 81         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,D      ¦ 82         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,E      ¦ 83         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,H      ¦ 84         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,HIX    ¦ DD 86 84   ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,HIY    ¦ FD 86 84   ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,L      ¦ 85         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,LIX    ¦ DD 86 85   ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,LIY    ¦ FD 86 85   ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

ADD A,n      ¦ C6 n       ¦ 2 ¦ 7     ¦ 7  ¦ z ¦ v   ¦ c

ADD HL,BC    ¦ 09         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ c

ADD HL,DE    ¦ 19         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ c

ADD HL,HL    ¦ 29         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ c

ADD HL,SP    ¦ 39         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ c

ADD IX,BC    ¦ DD 09      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ c

ADD IX,DE    ¦ DD 19      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ c

ADD IX,HL    ¦ DD 29      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ c

ADD IY,BC    ¦ FD 09      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ c

ADD IY,DE    ¦ FD 19      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ c

ADD IY,HL    ¦ FD 29      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ c

ADD IY,SP    ¦ FD 39      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ c

ADD LY,SP    ¦ DD 39      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ c

AND (HL)     ¦ A6         ¦ 1 ¦ 7     ¦ 7  ¦ z ¦ p   ¦ c

AND (IX+d)   ¦ DD A6 d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ p   ¦ c

AND (IY+d)   ¦ FD A6 d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ p   ¦ c

AND A        ¦ A7         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ c

AND B        ¦ A0         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ c

AND C        ¦ A1         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ c

AND D        ¦ A2         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ c

AND E        ¦ A3         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ c

AND H        ¦ A4         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ c

AND HIX      ¦ DD A4      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

AND HIY      ¦ FD A4      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

AND L        ¦ A5         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ c

AND LIX      ¦ DD A5      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

AND LIY      ¦ FD A5      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ c

AND n        ¦ E6 n       ¦ 2 ¦ 7     ¦ 7  ¦ z ¦ p   ¦ c

BIT 0,(HL)   ¦ CB 46      ¦ 2 ¦ 12    ¦ ?  ¦ <>b ?   ¦ -

BIT 0,(IX+d) ¦ CB DD 46 d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 0,(IY+d) ¦ CB FD 46 d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 0,A      ¦ CB 47      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 0,B      ¦ CB 40      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 0,C      ¦ CB 41      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 0,D      ¦ CB 42      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 0,E      ¦ CB 43      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 0,H      ¦ CB 44      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 0,L      ¦ CB 45      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 1,(HL)   ¦ CB 4E      ¦ 2 ¦ 12    ¦ ?  ¦ <>b ?   ¦ -

BIT 1,(IX+d) ¦ CB DD 4E d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 1,(IY+d) ¦ CB FD 4E d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 1,A      ¦ CB 1F      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 1,B      ¦ CB 48      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 1,C      ¦ CB 49      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 1,D      ¦ CB 4A      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 1,E      ¦ CB 4B      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 1,H      ¦ CB 4C      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 1,L      ¦ CB 4D      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 2,(HL)   ¦ CB 56      ¦ 2 ¦ 12    ¦ ?  ¦ <>b ?   ¦ -

BIT 2,(IY+d) ¦ CB FD 56 d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 2,(LY+d) ¦ CB DD 56 d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 2,A      ¦ CB 57      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 2,B      ¦ CB 50      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 2,C      ¦ CB 51      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 2,D      ¦ CB 52      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 2,E      ¦ CB 53      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 2,H      ¦ CB 54      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 2,L      ¦ CB 55      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 3,(HL)   ¦ CB 5E      ¦ 2 ¦ 12    ¦ ?  ¦ <>b ?   ¦ -

BIT 3,(IX+d) ¦ CB DD 5E d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 3,(IY+d) ¦ CB FD 5E d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 3,A      ¦ CB 5F      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 3,B      ¦ CB 58      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 3,C      ¦ CB 59      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 3,D      ¦ CB 5A      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 3,E      ¦ CB 5B      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 3,H      ¦ CB 5C      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 3,L      ¦ CB 5D      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 4,(HL)   ¦ CB 66      ¦ 2 ¦ 12    ¦ ?  ¦ <>b ?   ¦ -

BIT 4,(IY+d) ¦ CB FD 66 d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 4,(LY+d) ¦ CB DD 66 d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 4,A      ¦ CB 67      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 4,B      ¦ CB 60      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 4,C      ¦ CB 61      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 4,D      ¦ CB 62      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 4,E      ¦ CB 63      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 4,H      ¦ CB 64      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 4,L      ¦ CB 65      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 5,(HL)   ¦ CB 6E      ¦ 2 ¦ 12    ¦ ?  ¦ <>b ?   ¦ -

BIT 5,(IX+d) ¦ CB DD 6E d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 5,(IY+d) ¦ CB FD 6E d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 5,A      ¦ CB 6F      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 5,B      ¦ CB 68      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 5,C      ¦ CB 69      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 5,D      ¦ CB 6A      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 5,E      ¦ CB 6B      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 5,H      ¦ CB 6C      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 5,L      ¦ CB 6D      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 6,(HL)   ¦ CB 76      ¦ 2 ¦ 12    ¦ ?  ¦ <>b ?   ¦ -

BIT 6,(IX+d) ¦ CB DD 76 d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 6,(IY+d) ¦ CB FD 76 d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 6,A      ¦ CB 77      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 6,B      ¦ CB 70      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 6,C      ¦ CB 71      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 6,D      ¦ CB 72      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 6,E      ¦ CB 73      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 6,H      ¦ CB 74      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 6,L      ¦ CB 75      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 7,(HL)   ¦ CB 7E      ¦ 2 ¦ 12    ¦ ?  ¦ <>b ?   ¦ -

BIT 7,(IX+d) ¦ CB DD 7E d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 7,(IY+d) ¦ CB FD 7E d ¦ 4 ¦ 20    ¦ ?  ¦ <>b ?   ¦ -

BIT 7,A      ¦ CB 7F      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 7,B      ¦ CB 78      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 7,C      ¦ CB 79      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 7,D      ¦ CB 7A      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 7,E      ¦ CB 7B      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 7,H      ¦ CB 7C      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

BIT 7,L      ¦ CB 7D      ¦ 2 ¦ 8     ¦ ?  ¦ <>b ?   ¦ -

CALL addr    ¦ CD dr ad   ¦ 3 ¦ 17    ¦ -  ¦ - ¦ -   ¦ -

CALL c,addr  ¦ DC dr ad   ¦ 3 ¦ t17f10¦ -  ¦ - ¦ -   ¦ -

CALL m,addr  ¦ FC dr ad   ¦ 3 ¦ t17f10¦ -  ¦ - ¦ -   ¦ -

CALL nc,addr ¦ D4 dr ad   ¦ 3 ¦ t17f10¦ -  ¦ - ¦ -   ¦ -

CALL nz,addr ¦ C4 dr ad   ¦ 3 ¦ t17f10¦ -  ¦ - ¦ -   ¦ -

CALL p,addr  ¦ F4 dr ad   ¦ 3 ¦ t17f10¦ -  ¦ - ¦ -   ¦ -

CALL p<>,addr¦ E4 dr ad   ¦ 3 ¦ t17f10¦ -  ¦ - ¦ -   ¦ -

CALL pe,addr ¦ EC dr ad   ¦ 3 ¦ t17f10¦ -  ¦ - ¦ -   ¦ -

CALL z,addr  ¦ CC dr ad   ¦ 3 ¦ t17f10¦ -  ¦ - ¦ -   ¦ -

CCF          ¦ 3F         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ x

CP (HL)      ¦ BE         ¦ 1 ¦ 7     ¦ =7 ¦ = ¦ v   ¦ <

CP (IX+d)    ¦ DD BE d    ¦ 3 ¦ 19    ¦ =7 ¦ = ¦ v   ¦ <

CP (IY+d)    ¦ FD BE d    ¦ 3 ¦ 19    ¦ =7 ¦ = ¦ v   ¦ <

CP A         ¦ BF         ¦ 1 ¦ 4     ¦ =7 ¦ = ¦ v   ¦ <

CP B         ¦ B8         ¦ 1 ¦ 4     ¦ =7 ¦ = ¦ v   ¦ <

CP C         ¦ B9         ¦ 1 ¦ 4     ¦ =7 ¦ = ¦ v   ¦ <

CP D         ¦ BA         ¦ 1 ¦ 4     ¦ =7 ¦ = ¦ v   ¦ <

CP E         ¦ BB         ¦ 1 ¦ 4     ¦ =7 ¦ = ¦ v   ¦ <

CP H         ¦ BC         ¦ 1 ¦ 4     ¦ =7 ¦ = ¦ v   ¦ <

CP HIX       ¦ DD BC      ¦ 2 ¦ 8     ¦ =7 ¦ = ¦ v   ¦ <

CP HIY       ¦ FD BC      ¦ 2 ¦ 8     ¦ =7 ¦ = ¦ v   ¦ <

CP L         ¦ BD         ¦ 1 ¦ 4     ¦ =7 ¦ = ¦ v   ¦ c

CP LIX       ¦ DD BD      ¦ 2 ¦ 8     ¦ =7 ¦ = ¦ v   ¦ <

CP LIY       ¦ FD BD      ¦ 2 ¦ 8     ¦ =7 ¦ = ¦ v   ¦ <

CP n         ¦ FE n       ¦ 2 ¦ 7     ¦ =7 ¦ = ¦ v   ¦ <

CPD          ¦ ED A9      ¦ 2 ¦ 16    ¦ ?  ¦ =A¦ BC  ¦ -

CPDR         ¦ ED B9      ¦ 2 ¦ =16#21¦ ?  ¦ =A¦ BC  ¦ -

CPI          ¦ ED A1      ¦ 2 ¦ 16    ¦ ?  ¦ =A¦ BC  ¦ -

CPIR         ¦ ED B2      ¦ 2 ¦ =16#21¦ ?  ¦ =A¦ BC  ¦ -

CPL          ¦ 2F         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

DAA          ¦ 27         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ c

DEC (HL)     ¦ 35         ¦ 1 ¦ 11    ¦ 7  ¦ z ¦ v   ¦ -

DEC (IX+d)   ¦ DD 35 d    ¦ 3 ¦ 23    ¦ 7  ¦ z ¦ v   ¦ -

DEC (IY+d)   ¦ FD 35 d    ¦ 3 ¦ 23    ¦ 7  ¦ z ¦ v   ¦ -

DEC A        ¦ 3D         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

DEC B        ¦ 05         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

DEC BC       ¦ 0B         ¦ 1 ¦ 6     ¦ -  ¦ - ¦ -   ¦ -

DEC C        ¦ 0D         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

DEC D        ¦ 15         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

DEC DE       ¦ 1B         ¦ I ¦ 6     ¦ -  ¦ - ¦ -   ¦ -

DEC E        ¦ 1D         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

DEC H        ¦ 25         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

DEC HIX      ¦ DD 25      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ -

DEC HIY      ¦ FD 25      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ -

DEC HL       ¦ 2B         ¦ 1 ¦ 6     ¦ -  ¦ - ¦ -   ¦ -

DEC IX       ¦ DD 2B      ¦ 2 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

DEC IY       ¦ FD 2B      ¦ 2 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

DEC L        ¦ 2D         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

DEC LIX      ¦ DD 2D      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ -

DEC LIY      ¦ FD 2D      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ -

DEC SP       ¦ 3B         ¦ 1 ¦ 6     ¦ -  ¦ - ¦ -   ¦ -

DI           ¦ F3         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

DJNZ d       ¦ 10 d       ¦ 2 ¦ t13f8 ¦ -  ¦ - ¦ -   ¦ -

EI           ¦ FB         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

EX (SP),HL   ¦ E3         ¦ 1 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

EX (SP),IX   ¦ DD E3      ¦ 2 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

EX (SP),IY   ¦ FD E3      ¦ 2 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

EX AF,AF'    ¦ 08         ¦ 1 ¦ 4     ¦ s' ¦ z'¦ p'  ¦ c'

EX DE,HL     ¦ EB         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

EXX          ¦ D9         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

HALT         ¦ 76         ¦ 1 ¦ min 4 ¦ -  ¦ - ¦ -   ¦ -

IM 0         ¦ ED 46      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

IM 1         ¦ ED 56      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

IM 2         ¦ ED 5E      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

IN A,(C)     ¦ ED 78      ¦ 2 ¦ 12    ¦ 7  ¦ z ¦ p   ¦ 0

IN A,(n)     ¦ DB n       ¦ 2 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

IN B,(C)     ¦ ED 40      ¦ 2 ¦ 12    ¦ 7  ¦ z ¦ p   ¦ 0

IN C,(C)     ¦ ED 48      ¦ 2 ¦ 12    ¦ 7  ¦ z ¦ p   ¦ 0

IN D,(C)     ¦ ED 50      ¦ 2 ¦ 12    ¦ 7  ¦ z ¦ p   ¦ 0

IN E,(C)     ¦ ED 58      ¦ 2 ¦ 12    ¦ 7  ¦ z ¦ p   ¦ 0

IN H,(C)     ¦ ED 60      ¦ 2 ¦ 12    ¦ 7  ¦ z ¦ p   ¦ 0

IN L,(C)     ¦ ED 68      ¦ 2 ¦ 12    ¦ 7  ¦ z ¦ p   ¦ 0

INC (HL)     ¦ 34         ¦ 1 ¦ 11    ¦ 7  ¦ z ¦ v   ¦ -

INC (IX+d)   ¦ DD 34 d    ¦ 3 ¦ 23    ¦ 7  ¦ z ¦ v   ¦ -

INC (IY+d)   ¦ FD 34 d    ¦ 3 ¦ 23    ¦ 7  ¦ z ¦ v   ¦ -

INC A        ¦ 3C         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

INC B        ¦ 04         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

INC BC       ¦ 03         ¦ 1 ¦ 6     ¦ -  ¦ - ¦ -   ¦ -

INC C        ¦ 0C         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

INC D        ¦ 14         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

INC DE       ¦ 13         ¦ 1 ¦ 6     ¦ -  ¦ - ¦ -   ¦ -

INC E        ¦ 1C         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

INC H        ¦ 24         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

INC HIX      ¦ DD 24      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ -

INC HIY      ¦ FD 24      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ -

INC HL       ¦ 23         ¦ 1 ¦ 6     ¦ -  ¦ - ¦ -   ¦ -

INC IX       ¦ DD 23      ¦ 2 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

INC IY       ¦ FD 23      ¦ 2 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

INC L        ¦ 2C         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ -

INC LIX      ¦ DD 2C      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ -

INC LIY      ¦ FD 2C      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ -

INC SP       ¦ 33         ¦ 1 ¦ 6     ¦ -  ¦ - ¦ -   ¦ -

IND          ¦ ED AA      ¦ 2 ¦ 16    ¦ ?  ¦ <>B ?   ¦ -

INDR         ¦ ED BA      ¦ 2 ¦ z16nz21 ?  ¦ 1 ¦ ?   ¦ -

INI          ¦ ED A2      ¦ 2 ¦ 16    ¦ ?  ¦ <>B ?   ¦ -

INIR         ¦ ED B2      ¦ 2 ¦ z16nz21 ?  ¦ 1 ¦ ?   ¦ -

JP (HL)      ¦ E9         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

JP (IX)      ¦ DD E9      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

JP (IY)      ¦ FD E9      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

JP addr      ¦ C3 dr ad   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

JP c,addr    ¦ DA dr ad   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

JP m,addr    ¦ FA dr ad   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

JP nc,addr   ¦ D2 dr ad   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

JP nz,addr   ¦ C2 dr ad   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

JP p,addr    ¦ F2 dr ad   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

JP p<>,addr  ¦ E2 dr ad   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

JP pe,addr   ¦ EA dr ad   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

JP z,addr    ¦ CA dr ad   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

JR c,d       ¦ 38 d       ¦ 2 ¦ t12f7 ¦ -  ¦ - ¦ -   ¦ -

JR d         ¦ 18 d       ¦ 2 ¦ 12    ¦ -  ¦ - ¦ -   ¦ -

JR nc,d      ¦ 30 d       ¦ 2 ¦ t12f7 ¦ -  ¦ - ¦ -   ¦ -

JR nz,d      ¦ 20 d       ¦ 2 ¦ t12f7 ¦ -  ¦ - ¦ -   ¦ -

JR z,d       ¦ 28 d       ¦ 2 ¦ t12f7 ¦ -  ¦ - ¦ -   ¦ -

LD (addr),A  ¦ 32 dr ad   ¦ 3 ¦ 13    ¦ -  ¦ - ¦ -   ¦ -

LD (addr),BC ¦ ED 43 dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD (addr),DE ¦ ED 53 dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD (addr),HL ¦ 22 dr ad   ¦ 3 ¦ 16    ¦ -  ¦ - ¦ -   ¦ -

LD (addr),HL ¦ ED 63 dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD (addr),IX ¦ DD 22 dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD (addr),IY ¦ FD 22 dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD (addr),SP ¦ ED 73 dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD (BC),A    ¦ 02         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD (DE),A    ¦ 12         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD (HL),A    ¦ 77         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD (HL),A    ¦ 77         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD (HL),B    ¦ 70         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD (HL),C    ¦ 71         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD (HL),D    ¦ 72         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD (HL),E    ¦ 73         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD (HL),H    ¦ 74         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD (HL),L    ¦ 75         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD (HL),n    ¦ 36 n       ¦ 2 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

LD (IX+d),A  ¦ DD 77 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IX+d),B  ¦ DD 70 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IX+d),C  ¦ DD 71 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IX+d),D  ¦ DD 72 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IX+d),E  ¦ DD 73 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IX+d),H  ¦ DD 71 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IX+d),L  ¦ DD 75 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IX+d),n  ¦ DD 36 d n  ¦ 4 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IY+d),A  ¦ FD 77 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IY+d),B  ¦ FD 70 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IY+d),C  ¦ FD 71 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IY+d),D  ¦ FD 72 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IY+d),E  ¦ FD 73 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IY+d),H  ¦ FD 74 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IY+d),L  ¦ FD 75 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD (IY+d),n  ¦ FD 36 d n  ¦ 4 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD A,(addr)  ¦ 3A dr ad   ¦ 3 ¦ 13    ¦ -  ¦ - ¦ -   ¦ -

LD A,(BC)    ¦ 0A         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD A,(DE)    ¦ 1A         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD A,(HL)    ¦ 7E         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD A,(HL)    ¦ 7E         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD A,(IX+d)  ¦ DD 7E d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD A,(IY+d)  ¦ FD 7E d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD A,A       ¦ 7F         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD A,B       ¦ 78         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD A,C       ¦ 79         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD A,D       ¦ 7A         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD A,E       ¦ 7B         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD A,H       ¦ 7C         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD A,HIX     ¦ DD 7C      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD A,HIY     ¦ FD 7C      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD A,I       ¦ ED 57      ¦ 2 ¦ 9     ¦ 7  ¦ z ¦ i   ¦ 0

LD A,L       ¦ 7D         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD A,LIX     ¦ DD 7D      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD A,LIY     ¦ FD 7D      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD A,n       ¦ 3E n       ¦ 2 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD A,R       ¦ ED 5F      ¦ 2 ¦ 9     ¦ 7  ¦ z ¦ i   ¦ 0

LD B,(HL)    ¦ 46         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD B,(IX+d)  ¦ DD 46 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD B,(IY+d)  ¦ FD 46 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD B,A       ¦ 47         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD B,B       ¦ 40         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD B,C       ¦ 41         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD B,D       ¦ 42         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD B,E       ¦ 43         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD B,H       ¦ 44         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD B,HIX     ¦ DD 44      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD B,HIY     ¦ FD 44      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD B,L       ¦ 45         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD B,LIX     ¦ DD 45      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD B,LIY     ¦ FD 45      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD B,n       ¦ 06 n       ¦ 2 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD BC,(addr) ¦ ED 4B dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD BC,hilo   ¦ 01 lo hi   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

LD C,(HL)    ¦ 4E         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD C,(IX+d)  ¦ DD 4E d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD C,(IY+d)  ¦ DD 4E d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD C,A       ¦ 4F         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD C,B       ¦ 48         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD C,C       ¦ 49         ¦ 1 ¦ 1     ¦ -  ¦ - ¦ -   ¦ -

LD C,D       ¦ 4A         ¦ 1 ¦ 1     ¦ -  ¦ - ¦ -   ¦ -

LD C,E       ¦ 4B         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD C,H       ¦ 4C         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD C,HIX     ¦ DD 4C      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD C,HIY     ¦ FD 4C      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD C,L       ¦ 4D         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD C,LIX     ¦ DD 4D      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD C,LIY     ¦ FD 4D      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD C,n       ¦ 0E n       ¦ 2 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD D,(HL)    ¦ 56         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD D,(IX+d)  ¦ DD 56 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD D,(IY+d)  ¦ FD 56 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD D,A       ¦ 57         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD D,B       ¦ 50         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD D,C       ¦ 51         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD D,D       ¦ 52         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD D,E       ¦ 53         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD D,H       ¦ 54         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD D,HIX     ¦ DD 54      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD D,HIY     ¦ FD 54      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD D,L       ¦ 55         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD D,LIX     ¦ DD 55      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD D,LIY     ¦ FD 55      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD D,n       ¦ 16 n       ¦ 2 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD DE,(addr) ¦ ED 5B dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD DE,hilo   ¦ 11 lo hi   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

LD E,(HL)    ¦ 5E         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD E,(IX+d)  ¦ DD 5E d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD E,(IY+d)  ¦ FD 5E d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD E,A       ¦ 5F         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD E,B       ¦ 58         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD E,C       ¦ 59         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD E,D       ¦ 5A         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD E,E       ¦ 5B         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD E,H       ¦ 5C         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD E,HIX     ¦ DD 5C      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD E,HIY     ¦ FD 5C      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD E,L       ¦ 5D         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD E,LIX     ¦ DD 5D      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD E,LIY     ¦ FD 5D      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD E,n       ¦ 1E n       ¦ 2 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD H,(HL)    ¦ 66         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD H,(IX+d)  ¦ DD 66 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD H,(IY+d)  ¦ FD 66 d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD H,A       ¦ 67         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD H,B       ¦ 60         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD H,C       ¦ 61         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD H,D       ¦ 62         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD H,E       ¦ 63         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD H,H       ¦ 64         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD H,L       ¦ 65         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD H,n       ¦ 26 n       ¦ 2 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD HIX,A     ¦ DD 67      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIX,B     ¦ DD 60      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIX,C     ¦ DD 61      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIX,D     ¦ DD 62      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIX,E     ¦ DD 63      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIX,HIX   ¦ DD 64      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIX,LIX   ¦ DD 65      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIX,n     ¦ DD 26 n    ¦ 3 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

LD HIY,A     ¦ FD 67      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIY,B     ¦ FD 60      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIY,C     ¦ FD 61      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIY,D     ¦ FD 62      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIY,E     ¦ FD 63      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIY,HIY   ¦ FD 64      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIY,LIY   ¦ FD 65      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD HIY,n     ¦ FD 26 n    ¦ 3 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

LD HL,(addr) ¦ 2A dr ad   ¦ 3 ¦ 16    ¦ -  ¦ - ¦ -   ¦ -

LD HL,(addr) ¦ ED 6B dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD HL,hilo   ¦ 21 lo hi   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

LD I,A       ¦ ED 47      ¦ 2 ¦ 9     ¦ -  ¦ - ¦ -   ¦ -

LD IX,(addr) ¦ DD 2A dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD IX,hilo   ¦ DD 2A lo hi¦ 4 ¦ 14    ¦ -  ¦ - ¦ -   ¦ -

LD IY,(addr) ¦ FD 2A dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD IY,hilo   ¦ FD 21 lo hi¦ 4 ¦ 14    ¦ -  ¦ - ¦ -   ¦ -

LD L,(HL)    ¦ 6E         ¦ 1 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD L,(IX+d)  ¦ DD 6E d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD L,(IY+d)  ¦ FD 6E d    ¦ 3 ¦ 19    ¦ -  ¦ - ¦ -   ¦ -

LD L,A       ¦ 6F         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD L,B       ¦ 68         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD L,C       ¦ 69         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD L,D       ¦ 6A         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD L,E       ¦ 6B         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD L,H       ¦ 6C         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD L,L       ¦ 6D         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

LD L,n       ¦ 2E n       ¦ 2 ¦ 7     ¦ -  ¦ - ¦ -   ¦ -

LD LIX,A     ¦ DD 6F      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIX,B     ¦ DD 68      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIX,C     ¦ DD 69      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIX,D     ¦ DD 6A      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIX,E     ¦ DD 6B      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIX,HIX   ¦ DD 6C      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIX,LIX   ¦ DD 6D      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIX,n     ¦ DD 2E n    ¦ 3 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

LD LIY,A     ¦ FD 6F      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIY,B     ¦ FD 68      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIY,C     ¦ FD 69      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIY,D     ¦ FD 6A      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIY,E     ¦ FD 6B      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIY,HIY   ¦ FD 6C      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIY,LIY   ¦ FD 6D      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

LD LIY,n     ¦ FD 2E n    ¦ 3 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

LD R,A       ¦ ED 4F      ¦ 2 ¦ 9     ¦ -  ¦ - ¦ -   ¦ -

LD SP,(addr) ¦ ED 7B dr ad¦ 4 ¦ 20    ¦ -  ¦ - ¦ -   ¦ -

LD SP,hilo   ¦ 31 lo hi   ¦ 3 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

LD SP,HL     ¦ F9         ¦ 1 ¦ 6     ¦ -  ¦ - ¦ -   ¦ -

LD SP,IX     ¦ DD F9      ¦ 2 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

LD SP,IY     ¦ FD F9      ¦ 2 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

LDD          ¦ ED A8      ¦ 2 ¦ 16    ¦ -  ¦ - ¦ BC  ¦ -

LDDR         ¦ ED B8      ¦ 2 ¦ z16nz21 -  ¦ - ¦ 0   ¦ -

LDI          ¦ ED A0      ¦ 2 ¦ 16    ¦ -  ¦ - ¦ BC  ¦ -

LDIR         ¦ ED B0      ¦ 2 ¦ zI6nz21 -  ¦ - ¦ 0   ¦ -

NEG          ¦ ED 44      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ A80 ¦ A0

NOP          ¦ 00         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ -

OR (HL)      ¦ B6         ¦ 1 ¦ 7     ¦ 7  ¦ z ¦ P   ¦ 0

OR (IX+d)    ¦ DD B6 d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ p   ¦ 0

OR (IY+d)    ¦ FD B6 d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ p   ¦ 0

OR A         ¦ B7         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

OR B         ¦ B0         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

OR C         ¦ B1         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

OR D         ¦ B2         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

OR E         ¦ B3         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

OR H         ¦ B4         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

OR HIX       ¦ DD B4      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ 0

OR HIY       ¦ FD B4      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ 0

OR L         ¦ B5         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

OR LIX       ¦ DD B5      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ 0

OR LIY       ¦ FD B5      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ 0

OR n         ¦ F6 n       ¦ 2 ¦ 7     ¦ 7  ¦ z ¦ p   ¦ 0

OTDR         ¦ ED BB      ¦ 2 ¦ z16nz21 ?  ¦ 1 ¦ ?   ¦ -

OTIR         ¦ ED B3      ¦ 2 ¦ zI6nz21 ?  ¦ I ¦ ?   ¦ -

OUT (C),A    ¦ ED 79      ¦ 2 ¦ 12    ¦ -  ¦ - ¦ -   ¦ -

OUT (C),B    ¦ ED 41      ¦ 2 ¦ 12    ¦ -  ¦ - ¦ -   ¦ -

OUT (C),C    ¦ ED 49      ¦ 2 ¦ 12    ¦ -  ¦ - ¦ -   ¦ -

OUT (C),D    ¦ ED 51      ¦ 2 ¦ 12    ¦ -  ¦ - ¦ -   ¦ -

OUT (C),E    ¦ ED 59      ¦ 2 ¦ 12    ¦ -  ¦ - ¦ -   ¦ -

OUT (C),H    ¦ ED 61      ¦ 2 ¦ 12    ¦ -  ¦ - ¦ -   ¦ -

OUT (C),L    ¦ ED 69      ¦ 2 ¦ 12    ¦ -  ¦ - ¦ -   ¦ -

OUT (n),A    ¦ D3 n       ¦ 2 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

OUTD         ¦ ED AB      ¦ 2 ¦ 16    ¦ ?  ¦ <>B ?   ¦ -

OUTI         ¦ ED A3      ¦ 2 ¦ 16    ¦ ?  ¦ <>B ?   ¦ -

POP AF       ¦ F1         ¦ 1 ¦ 10    ¦ [POP flags]  ¦

POP BC       ¦ C1         ¦ 1 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

POP DE       ¦ D1         ¦ 1 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

POP HL       ¦ E1         ¦ 1 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

POP IX       ¦ DD E1      ¦ 2 ¦ 14    ¦ -  ¦ - ¦ -   ¦ -

POP IY       ¦ FD E1      ¦ 2 ¦ 14    ¦ -  ¦ - ¦ -   ¦ -

PUSH AF      ¦ F5         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

PUSH BC      ¦ C5         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

PUSH DE      ¦ D5         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

PUSH HL      ¦ E5         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

PUSH IX      ¦ DD E5      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

PUSH IY      ¦ FD E5      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

RES 0,(HL)   ¦ CB 86      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

RES 0,(IX+d) ¦ DD CB d 86 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 0,(IY+d) ¦ FD CB d 86 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 0,A      ¦ CB 87      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 0,B      ¦ CB 80      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 0,C      ¦ CB 81      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 0,D      ¦ CB 82      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 0,E      ¦ CB 83      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 0,H      ¦ CB 81      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 0,L      ¦ CB 85      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 1,(HL)   ¦ CB 8E      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

RES 1,(IX+d) ¦ DD CB d 8E ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 1,(IY+d) ¦ FD CB d 8E ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 1,A      ¦ CB 8F      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 1,B      ¦ CB 88      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 1,C      ¦ CB 89      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 1,D      ¦ CB 8A      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 1,E      ¦ CB 8B      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 1,H      ¦ CB 8C      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 1,L      ¦ CB 8D      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 2,(HL)   ¦ CB 96      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

RES 2,(IX+d) ¦ DD CB d 96 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 2,(IY+d) ¦ FD CB d 96 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 2,A      ¦ CB 97      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 2,B      ¦ CB 90      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 2,C      ¦ CB 91      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 2,D      ¦ CB 92      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 2,E      ¦ CB 93      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 2,H      ¦ CB 94      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 2,L      ¦ CB 95      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 3,(HL)   ¦ CB 9E      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

RES 3,(IX+d) ¦ DD CB d 9E ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 3,(IY+d) ¦ FD CB d 9E ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 3,A      ¦ CB 9F      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 3,B      ¦ CB 98      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 3,C      ¦ CB 99      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 3,D      ¦ CB 9A      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 3,E      ¦ CB 9B      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 3,H      ¦ CB 9C      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 3,L      ¦ CB 9D      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 4,(HL)   ¦ CB A6      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

RES 4,(IX+d) ¦ DD CB d A6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 4,(IY+d) ¦ FD CB d A6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 4,A      ¦ CB A7      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 4,B      ¦ CB A0      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 4,C      ¦ CB A1      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 4,D      ¦ CB A2      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 4,E      ¦ CB A3      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 4,H      ¦ CB A4      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 4,L      ¦ CB A5      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 5,(HL)   ¦ CB AE      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

RES 5,(IX+d) ¦ DD CB d AE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 5,(IY+d) ¦ FD CB d AE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 5,A      ¦ CB AF      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 5,B      ¦ CB A8      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 5,C      ¦ CB A9      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 5,D      ¦ CB AA      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 5,E      ¦ CB AB      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 5,H      ¦ CB AC      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 5,L      ¦ CB AD      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 6,(HL)   ¦ CB B6      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

RES 6,(IX+d) ¦ DD CB d B6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 6,(IY+d) ¦ FD CB d B6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 6,A      ¦ CB B7      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 6,B      ¦ CB B0      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 6,C      ¦ CB B1      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 6,D      ¦ CB B2      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 6,E      ¦ CB B3      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 6,H      ¦ CB B4      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 6,L      ¦ CB B5      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 7,(HL)   ¦ CB BE      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

RES 7,(IX+d) ¦ DD CB d BE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 7,(IY+d) ¦ FD CB d BE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

RES 7,A      ¦ CB BF      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 7,B      ¦ CB B8      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 7,C      ¦ CB B9      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 7,D      ¦ CB BA      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 7,E      ¦ CB BB      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 7,H      ¦ CB BC      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RES 7,L      ¦ CB BD      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

RET          ¦ C9         ¦ 1 ¦ 10    ¦ -  ¦ - ¦ -   ¦ -

RET C        ¦ D8         ¦ 1 ¦ t11f8 ¦ -  ¦ - ¦ -   ¦ -

RET M        ¦ F8         ¦ 1 ¦ t11f8 ¦ -  ¦ - ¦ -   ¦ -

RET NC       ¦ D0         ¦ 1 ¦ t11f8 ¦ -  ¦ - ¦ -   ¦ -

RET NZ       ¦ C0         ¦ 1 ¦ t11f8 ¦ -  ¦ - ¦ -   ¦ -

RET P        ¦ F0         ¦ 1 ¦ t11f8 ¦ -  ¦ - ¦ -   ¦ -

RET PE       ¦ E8         ¦ 1 ¦ t11f8 ¦ -  ¦ - ¦ -   ¦ -

RET PO       ¦ E0         ¦ 1 ¦ t11f8 ¦ -  ¦ - ¦ -   ¦ -

RET Z        ¦ C8         ¦ 1 ¦ t11f8 ¦ -  ¦ - ¦ -   ¦ -

RETI         ¦ ED 4D      ¦ 2 ¦ 14    ¦ -  ¦ - ¦ -   ¦ -

RETN         ¦ ED 45      ¦ 2 ¦ 14    ¦ -  ¦ - ¦ -   ¦ -

RL (HL)      ¦ CB 16      ¦ 2 ¦ 15    ¦ 7  ¦ z ¦ p   ¦ r7

RL (IX+d)    ¦ DD CB d 16 ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r7

RL (IY+d)    ¦ FD CB d 16 ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r7

RL A         ¦ CB 17      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RL B         ¦ CB 10      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RL C         ¦ CB 11      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RL D         ¦ CB 12      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RL E         ¦ CB 13      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RL H         ¦ CB 14      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RL L         ¦ CB 15      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RLA          ¦ 17         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ r7

RLC (HL)     ¦ CB 06      ¦ 2 ¦ 15    ¦ 7  ¦ z ¦ p   ¦ r7

RLC (IX+d)   ¦ DD CB d 06 ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r7

RLC (IY+d)   ¦ FD CB d 06 ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r7

RLC A        ¦ CB 07      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RLC B        ¦ CB 00      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RLC C        ¦ CB 01      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RLC D        ¦ CB 02      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RLC E        ¦ CB 03      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RLC H        ¦ CB 04      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RLC L        ¦ CB 05      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

RLCA         ¦ 07         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ r7

RLD          ¦ ED 6F      ¦ 2 ¦ 18    ¦ 7  ¦ z ¦ p   ¦ -

RR (HL)      ¦ CB 1E      ¦ 2 ¦ 15    ¦ 7  ¦ z ¦ p   ¦ r0

RR (IX+d)    ¦ DD CB d 1E ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r0

RR (IY+d)    ¦ FD CB d 1E ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r0

RR A         ¦ CB 1F      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RR B         ¦ CB 18      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RR C         ¦ CB 19      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RR D         ¦ CB 1A      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RR E         ¦ CB 1B      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RR H         ¦ CB 1C      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RR L         ¦ CB 1D      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RRA          ¦ 1F         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ r0

RRC (HL)     ¦ CB 0E      ¦ 2 ¦ 15    ¦ 7  ¦ z ¦ p   ¦ r0

RRC (IX+d)   ¦ DD CB d 0E ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r0

RRC (IY+d)   ¦ FD CB d 0E ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r0

RRC A        ¦ CB 0F      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RRC B        ¦ CB 08      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RRC C        ¦ CB 09      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RRC D        ¦ CB 0A      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RRC E        ¦ CB 0B      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RRC H        ¦ CB 0C      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RRC L        ¦ CB 0D      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

RRCA         ¦ 0F         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ r0

RRD          ¦ ED 67      ¦ 2 ¦ 18    ¦ 7  ¦ z ¦ p   ¦ -

RST 0        ¦ C7         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

RST 1,addr   ¦ CF dr ad   ¦ 3 ¦ (11)  ¦ -  ¦ - ¦ -   ¦ -

RST 2,addr   ¦ D7 dr ad   ¦ 3 ¦ (11)  ¦ -  ¦ - ¦ -   ¦ -

RST 3,addr   ¦ DF dr ad   ¦ 3 ¦ (11)  ¦ -  ¦ - ¦ -   ¦ -

RST 4        ¦ E7         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

RST 5,addr   ¦ EF dr ad   ¦ 3 ¦ (11)  ¦ -  ¦ - ¦ -   ¦ -

RST 6        ¦ F7         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

RST 7        ¦ FF         ¦ 1 ¦ 11    ¦ -  ¦ - ¦ -   ¦ -

SBC A,(HL)   ¦ 9E         ¦ 1 ¦ 7     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,(IX+d) ¦ DD 9E d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ v   ¦ b

SBC A,(IY+d) ¦ FD 9E d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ v   ¦ b

SBC A,A      ¦ 9F         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,B      ¦ 98         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,C      ¦ 99         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,D      ¦ 9A         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,E      ¦ 9B         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,H      ¦ 9C         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,HIX    ¦ DD 9C      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,HIY    ¦ FD 9C      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,L      ¦ 9D         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,LIX    ¦ DD 9D      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,LIY    ¦ FD 9D      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ b

SBC A,n      ¦ DE n       ¦ 2 ¦ 7     ¦ 7  ¦ z ¦ v   ¦ b

SBC HL,BC    ¦ ED 42      ¦ 2 ¦ 15    ¦ 15 ¦ z ¦ v   ¦ b

SBC HL,DE    ¦ ED 52      ¦ 2 ¦ 15    ¦ 15 ¦ z ¦ v   ¦ b

SBC HL,HL    ¦ ED 62      ¦ 2 ¦ 15    ¦ 15 ¦ z ¦ v   ¦ b

SBC HL,SP    ¦ ED 72      ¦ 2 ¦ 15    ¦ 15 ¦ z ¦ v   ¦ b

SCF          ¦ 37         ¦ 1 ¦ 4     ¦ -  ¦ - ¦ -   ¦ 1

SET 0,(HL)   ¦ CB C6      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

SET 0,(IX+d) ¦ DD CB d C6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 0,(IY+d) ¦ FD CB d C6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 0,A      ¦ CB C7      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 0,B      ¦ CB C0      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 0,C      ¦ CB C1      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 0,D      ¦ CB C2      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 0,E      ¦ CB C3      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 0,H      ¦ CB C4      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 0,L      ¦ CB C5      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 1,(HL)   ¦ CB CE      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

SET 1,(IX+d) ¦ DD CB d CE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 1,(IY+d) ¦ FD CB d CE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 1,A      ¦ CB CF      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 1,B      ¦ CB C8      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 1,C      ¦ CB C9      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 1,D      ¦ CB CA      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 1,E      ¦ CB CB      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 1,H      ¦ CB CC      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 1,L      ¦ CB CD      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 2,(HL)   ¦ CB D6      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

SET 2,(IX+d) ¦ DD CB d D6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 2,(IY+d) ¦ FD CB d D6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 2,A      ¦ CB D7      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 2,B      ¦ CB D0      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 2,C      ¦ CB D1      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 2,D      ¦ CB D2      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 2,E      ¦ CB D3      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 2,H      ¦ CB D4      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 2,L      ¦ CB D5      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 3,(HL)   ¦ CB DE      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

SET 3,(IX+d) ¦ DD CB d DE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 3,(IY+d) ¦ FD CB d DE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 3,A      ¦ CB DF      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 3,B      ¦ CB D8      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 3,C      ¦ CB D9      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 3,D      ¦ CB DA      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 3,E      ¦ CB DB      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 3,H      ¦ CB DC      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 3,L      ¦ CB DD      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 4,(HL)   ¦ CB E6      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

SET 4,(IX+d) ¦ DD CB d E6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 4,(IY+d) ¦ FD CB d E6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 4,A      ¦ CB E7      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 4,B      ¦ CB E0      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 4,C      ¦ CB E1      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 4,D      ¦ CB E2      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 4,E      ¦ CB E3      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 4,H      ¦ CB E4      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 4,L      ¦ CB E5      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 5,(HL)   ¦ CB EE      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

SET 5,(IX+d) ¦ DD CB d EE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 5,(IY+d) ¦ FD CB d EE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 5,A      ¦ CB EF      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 5,B      ¦ CB E8      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 5,C      ¦ CB E9      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 5,D      ¦ CB EA      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 5,E      ¦ CB EB      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 5,H      ¦ CB EC      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 5,L      ¦ CB ED      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 6,(HL)   ¦ CB F6      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

SET 6,(IX+d) ¦ DD CB d F6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 6,(IY+d) ¦ FD CB d F6 ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 6,A      ¦ CB F7      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 6,B      ¦ CB F0      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 6,C      ¦ CB F1      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 6,D      ¦ CB F2      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 6,E      ¦ CB F3      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 6,H      ¦ CB F4      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 6,L      ¦ CB F5      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 7,(HL)   ¦ CB FE      ¦ 2 ¦ 15    ¦ -  ¦ - ¦ -   ¦ -

SET 7,(IX+d) ¦ DD CB d FE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 7,(IY+d) ¦ FD CB d FE ¦ 4 ¦ 23    ¦ -  ¦ - ¦ -   ¦ -

SET 7,A      ¦ CB FF      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 7,B      ¦ CB F8      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 7,C      ¦ CB F9      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 7,D      ¦ CB FA      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 7,E      ¦ CB FB      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 7,H      ¦ CB FC      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SET 7,L      ¦ CB FD      ¦ 2 ¦ 8     ¦ -  ¦ - ¦ -   ¦ -

SLA (HL)     ¦ CB 26      ¦ 2 ¦ 15    ¦ 7  ¦ z ¦ p   ¦ r7

SLA (IX+d)   ¦ DD CB d 26 ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r7

SLA (IY+d)   ¦ FD CB d 26 ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r7

SLA A        ¦ CB 27      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLA B        ¦ CB 20      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLA C        ¦ CB 21      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLA D        ¦ CB 22      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLA E        ¦ CB 23      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLA H        ¦ CB 24      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLA L        ¦ CB 25      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLL (HL)     ¦ CB 36      ¦ 2 ¦ 15    ¦ 7  ¦ z ¦ p   ¦ r7

SLL (IX+d)   ¦ DD CB d 36 ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r7

SLL (IY+d)   ¦ FD CB d 36 ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r7

SLL A        ¦ CB 37      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLL B        ¦ CB 30      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLL C        ¦ CB 31      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLL D        ¦ CB 32      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLL E        ¦ CB 33      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLL H        ¦ CB 34      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SLL L        ¦ CB 35      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r7

SRA (HL)     ¦ CB 2E      ¦ 2 ¦ 15    ¦ 7  ¦ z ¦ p   ¦ r0

SRA (IX+d)   ¦ DD CB d 2E ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r0

SRA (IY+d)   ¦ FD CB d 2E ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r0

SRA A        ¦ CB 2F      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRA B        ¦ CB 28      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRA C        ¦ CB 29      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRA D        ¦ CB 2A      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRA E        ¦ CB 2B      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRA H        ¦ CB 2C      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRA L        ¦ CB 2D      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRL (HL)     ¦ CB 3E      ¦ 2 ¦ 15    ¦ 7  ¦ z ¦ p   ¦ r0

SRL (IX+d)   ¦ DD CB d 3E ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r0

SRL (IY+d)   ¦ FD CB d 3E ¦ 4 ¦ 23    ¦ 7  ¦ z ¦ p   ¦ r0

SRL A        ¦ CB 3F      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRL B        ¦ CB 38      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRL C        ¦ CB 39      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRL D        ¦ CB 3A      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRL E        ¦ CB 3B      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRL H        ¦ CB 3C      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SRL L        ¦ CB 3D      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ r0

SUB (HL)     ¦ 96         ¦ 1 ¦ 7     ¦ 7  ¦ z ¦ v   ¦ b

SUB (IX+d)   ¦ DD 96 d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ v   ¦ b

SUB (IY+d)   ¦ FD 96 d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ v   ¦ b

SUB A        ¦ 97         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SUB B        ¦ 90         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SUB C        ¦ 91         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SUB D        ¦ 92         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SUB E        ¦ 93         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SUB H        ¦ 94         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SUB HIX      ¦ DD AC      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ b

SUB HIY      ¦ FD AC      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ b

SUB L        ¦ 95         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ v   ¦ b

SUB LIX      ¦ DD AD      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ b

SUB LIY      ¦ FD AD      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ v   ¦ b

SUB n        ¦ D6 n       ¦ 2 ¦ 7     ¦ 7  ¦ z ¦ v   ¦ b

XOR (HL)     ¦ AE *       ¦ 1 ¦ 7     ¦ 7  ¦ z ¦ p   ¦ 0

XOR (IX+d)   ¦ DD AC d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ p   ¦ 0

XOR (IY+d)   ¦ FD AC d    ¦ 3 ¦ 19    ¦ 7  ¦ z ¦ p   ¦ 0

XOR A        ¦ AF         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

XOR B        ¦ A8         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

XOR C        ¦ A9         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

XOR D        ¦ AA         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

XOR E        ¦ AB         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

XOR H        ¦ AC         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

XOR HIX      ¦ DD AC      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ 0

XOR HIY      ¦ FD AD      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ 0

XOR L        ¦ AD         ¦ 1 ¦ 4     ¦ 7  ¦ z ¦ p   ¦ 0

XOR LIX      ¦ DD AC      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ 0

XOR LIY      ¦ FD AD      ¦ 2 ¦ 8     ¦ 7  ¦ z ¦ p   ¦ 0

XOR n        ¦ EE n       ¦ 2 ¦ 7     ¦ 7  ¦ z ¦ p   ¦ 0

* - the opcode given for XOR (HL) in the paper version of the guide is

AC, the same as that given for XOR H.

The code for XOR (HL) should be AE, according to Rodnay Zaks' book

"PROGRAMMING THE Z80", (pub Sydex, ISBN 0-89588-069-5)

The flag register is bit significant, and is defined as follows:

bit 7 - Sign

6 - Zero

5 - unused

4 - Half Carry (cannot test)

3 - unused

2 - Parity/Overflow

1 - Add/Subtract (cannot test)

0 - Carry

Snippets

To unerase files - Poke &A701 with &E5,  CAT the disc, and try to load

the file you want.  Save it back to  USER 0 if it loads correctly.  If

it fails to load, it is probably that it has already been written over

by another file.

To disable ESC - Poke &BDEE with  &C9.   To restore ESC, poke the same

address with &C3

The CRTC Registers

To change the value of these  registers, the register number should be

output on address &BCxx and then the data output on &BDxx

Reg ¦ Function               ¦ Default value

----+------------------------+--------------

R0  ¦ Horizontal Total       ¦ 63

R1  ¦ Horizontal Displayed   ¦ 40

R2  ¦ Horizontal Sync Pos.   ¦ 46

R3  ¦ Sync Width             ¦ 112

R4  ¦ Vertical Total         ¦ 38

R5  ¦ Vertical Total Adjust  ¦ 0

R6  ¦ Vertical Displayed     ¦ 25

R7  ¦ Vertical Sync Position ¦ 30

R8  ¦ Interlace and Skew     ¦ 0

R9  ¦ Maximum Raster Addr    ¦ 7

R10 ¦ Cursor Start Raster    ¦ 0

R11 ¦ Cursor End Raster      ¦ 0

R12 ¦ Start Address (H)      ¦ 48

R13 ¦ Start Address (L)      ¦ 0

R14 ¦ Cursor Register (H)    ¦ 192

R15 ¦ Cursor Register (L)    ¦ 07