SINGLE BYTE OPCODES

|  | +00/08 | +01/09 | +02/0A | +03/0B | +04/0C | +05/0D | +06/0E | +07/0F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00 | nop | ld bc,word | ld (bc),a | inc bc | inc b | dec b | ld b,byte | rlca |
| 0x08 | ex af,af | add hl,bc | ld a,(bc) | dec bc | inc c | dec c | ld c,byte | rrca |
| 0x10 | djnz | ld de,word | ld (de),a | inc de | inc d | dec d | ld d,byte | rla |
| 0x18 | jr | add hl,de | ld a,(de) | dec de | inc e | dec e | ld e,byte | rra |
| 0x20 | jr nz | ld hl,word | ld (word),hl | inc hl | inc h | dec h | ld h,byte | daa |
| 0x28 | jr z | add hl,hl | ld hl,(word) | dec hl | inc l | dec l | ld l,byte | cpl |
| 0x30 | jr nc | ld sp,word | ld (word),a | inc sp | inc (hl) | dec (hl) | ld (hl),byte | scf |
| 0x38 | jr c | add hl,sp | ld a,(word) | dec sp | inc a | dec a | ld a,byte | ccf |
| 0x40 | ld b,b | ld b,c | ld b,d | ld b,e | ld b,h | ld b,l | ld b,(hl) | ld b,a |
| 0x48 | ld c,b | ld c,c | ld c,d | ld c,e | ld c,h | ld c,l | ld c,(hl) | ld c,a |
| 0x50 | ld d,b | ld d,c | ld d,d | ld d,e | ld d,h | ld d,l | ld d,(hl) | ld d,a |
| 0x58 | ld e,b | ld e,c | ld e,d | ld e,e | ld e,h | ld e,l | ld e,(hl) | ld e,a |
| 0x60 | ld h,b | ld h,c | ld h,d | ld h,e | ld h,h | ld h,l | ld h,(hl) | ld h,a |
| 0x68 | ld l,b | ld l,c | ld l,d | ld l,e | ld l,h | ld l,l | ld l,(hl) | ld l,a |
| 0x70 | ld (hl),b | ld (hl),c | ld (hl),d | ld (hl),e | ld (hl),h | ld (hl),l | halt | ld (hl),a |
| 0x78 | ld a,b | ld a,c | ld a,d | ld a,e | ld a,h | ld a,l | ld a,(hl) | ld a,a |
| 0x80 | add a,b | add a,c | add a,d | add a,e | add a,h | add a,l | add a,(hl) | add a,a |
| 0x88 | adc a,b | adc a,c | adc a,d | adc a,e | adc a,h | adc a,l | adc a,(hl) | adc a,a |
| 0x90 | sub b | sub c | sub d | sub e | sub h | sub l | sub (hl) | sub a |
| 0x98 | sbc a,b | sbc a,c | sbc a,d | sbc a,e | sbc a,h | sbc a,l | sbc a,(hl) | sbc a,a |
| 0xA0 | and b | and c | and d | and e | and h | and l | and (hl) | and a |
| 0xA8 | xor b | xor c | xor d | xor e | xor h | xor l | xor (hl) | xor a |
| 0xB0 | or b | or c | or d | or e | or h | or l | or (hl) | or a |
| 0xB8 | cp b | cp c | cp d | cp e | cp h | cp l | cp (hl) | cp a |
| 0xC0 | ret nz | pop bc | jp nz | jp | call nz | push bc | add a,byte | rst 00 |
| 0xC8 | ret z | ret | jp z | shift cb | call z | call | adc a,byte | rst 08 |
| 0xD0 | ret nc | pop de | jp nc | out byte,a | call nc | push de | sub byte | rst 10 |
| 0xD8 | ret c | exx | jp c | in a,byte | call c | shift dd | sbc a,byte | rst 18 |
| 0xE0 | ret po | pop hl | jp po | ex (sp),hl | call po | push hl | and byte | rst 20 |
| 0xE8 | ret pe | jp hl | jp pe | ex de,hl | call pe | shift ed | xor byte | rst 28 |
| 0xF0 | ret p | pop af | jp p | di | call p | push af | or byte | rst 30 |
| 0xF8 | ret m | ld sp,hl | jp m | ei | call m | shift fd | cp byte | rst 38 |

CB OPCODES

|  | +00/08 | +01/09 | +02/0A | +03/0B | +04/0C | +05/0D | +06/0E | +07/0F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00 | rlc b | rlc c | rlc d | rlc e | rlc h | rlc l | rlc (hl) | rlc a |
| 0x08 | rrc b | rrc c | rrc d | rrc e | rrc h | rrc l | rrc (hl) | rrc a |
| 0x10 | rl b | rl c | rl d | rl e | rl h | rl l | rl (hl) | rl a |
| 0x18 | rr b | rr c | rr d | rr e | rr h | rr l | rr (hl) | rr a |
| 0x20 | sla b | sla c | sla d | sla e | sla h | sla l | sla (hl) | sla a |
| 0x28 | sra b | sra c | sra d | sra e | sra h | sra l | sra (hl) | sra a |
| 0x30 | sll b | sll c | sll d | sll e | sll h | sll l | sll (hl) | sll a |
| 0x38 | srl b | srl c | srl d | srl e | srl h | srl l | srl (hl) | srl a |
| 0x40 | bit 0,b | bit 0,c | bit 0,d | bit 0,e | bit 0,h | bit 0,l | bit 0,(hl) | bit 0,a |
| 0x48 | bit 1,b | bit 1,c | bit 1,d | bit 1,e | bit 1,h | bit 1,l | bit 1,(hl) | bit 1,a |
| 0x50 | bit 2,b | bit 2,c | bit 2,d | bit 2,e | bit 2,h | bit 2,l | bit 2,(hl) | bit 2,a |
| 0x58 | bit 3,b | bit 3,c | bit 3,d | bit 3,e | bit 3,h | bit 3,l | bit 3,(hl) | bit 3,a |
| 0x60 | bit 4,b | bit 4,c | bit 4,d | bit 4,e | bit 4,h | bit 4,l | bit 4,(hl) | bit 4,a |
| 0x68 | bit 5,b | bit 5,c | bit 5,d | bit 5,e | bit 5,h | bit 5,l | bit 5,(hl) | bit 5,a |
| 0x70 | bit 6,b | bit 6,c | bit 6,d | bit 6,e | bit 6,h | bit 6,l | bit 6,(hl) | bit 6,a |
| 0x78 | bit 7,b | bit 7,c | bit 7,d | bit 7,e | bit 7,h | bit 7,l | bit 7,(hl) | bit 7,a |
| 0x80 | res 0,b | res 0,c | res 0,d | res 0,e | res 0,h | res 0,l | res 0,(hl) | res 0,a |
| 0x88 | res 1,b | res 1,c | res 1,d | res 1,e | res 1,h | res 1,l | res 1,(hl) | res 1,a |
| 0x90 | res 2,b | res 2,c | res 2,d | res 2,e | res 2,h | res 2,l | res 2,(hl) | res 2,a |
| 0x98 | res 3,b | res 3,c | res 3,d | res 3,e | res 3,h | res 3,l | res 3,(hl) | res 3,a |
| 0xA0 | res 4,b | res 4,c | res 4,d | res 4,e | res 4,h | res 4,l | res 4,(hl) | res 4,a |
| 0xA8 | res 5,b | res 5,c | res 5,d | res 5,e | res 5,h | res 5,l | res 5,(hl) | res 5,a |
| 0xB0 | res 6,b | res 6,c | res 6,d | res 6,e | res 6,h | res 6,l | res 6,(hl) | res 6,a |
| 0xB8 | res 7,b | res 7,c | res 7,d | res 7,e | res 7,h | res 7,l | res 7,(hl) | res 7,a |
| 0xC0 | set 0,b | set 0,c | set 0,d | set 0,e | set 0,h | set 0,l | set 0,(hl) | set 0,a |
| 0xC8 | set 1,b | set 1,c | set 1,d | set 1,e | set 1,h | set 1,l | set 1,(hl) | set 1,a |
| 0xD0 | set 2,b | set 2,c | set 2,d | set 2,e | set 2,h | set 2,l | set 2,(hl) | set 2,a |
| 0xD8 | set 3,b | set 3,c | set 3,d | set 3,e | set 3,h | set 3,l | set 3,(hl) | set 3,a |
| 0xE0 | set 4,b | set 4,c | set 4,d | set 4,e | set 4,h | set 4,l | set 4,(hl) | set 4,a |
| 0xE8 | set 5,b | set 5,c | set 5,d | set 5,e | set 5,h | set 5,l | set 5,(hl) | set 5,a |
| 0xF0 | set 6,b | set 6,c | set 6,d | set 6,e | set 6,h | set 6,l | set 6,(hl) | set 6,a |
| 0xF8 | set 7,b | set 7,c | set 7,d | set 7,e | set 7,h | set 7,l | set 7,(hl) | set 7,a |

DD OPCODES

|  | +00/08 | +01/09 | +02/0A | +03/0B | +04/0C | +05/0D | +06/0E | +07/0F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0x08 | Note 1 | add ix,bc | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0x10 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0x18 | Note 1 | add ix,de | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0x20 | Note 1 | ld ix,word | ld (word),ix | inc ix | inc ixh | dec ixh | ld ixh,byte | Note 1 |
| 0x28 | Note 1 | add ix,ix | ld ix,(word) | dec ix | inc ixl | dec ixl | ld ixl,byte | Note 1 |
| 0x30 | Note 1 | Note 1 | Note 1 | Note 1 | inc (ix+d) | dec (ix+d) | ld (ix+d),byte | Note 1 |
| 0x38 | Note 1 | add ix,sp | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0x40 | Note 1 | Note 1 | Note 1 | Note 1 | ld b,ixh | ld b,ixl | ld b,(ix+d) | Note 1 |
| 0x48 | Note 1 | Note 1 | Note 1 | Note 1 | ld c,ixh | ld c,ixl | ld c,(ix+d) | Note 1 |
| 0x50 | Note 1 | Note 1 | Note 1 | Note 1 | ld d,ixh | ld d,ixl | ld d,(ix+d) | Note 1 |
| 0x58 | Note 1 | Note 1 | Note 1 | Note 1 | ld e,ixh | ld e,ixl | ld e,(ix+d) | Note 1 |
| 0x60 | ld ixh,b | ld ixh,c | ld ixh,d | ld ixh,e | ld ixh,ixh | ld ixh,ixl | ld h,(ix+d) | ld ixh,a |
| 0x68 | ld ixl,b | ld ixl,c | ld ixl,d | ld ixl,e | ld ixl,ixh | ld ixl,ixl | ld l,(ix+d) | ld ixl,a |
| 0x70 | ld (ix+d),b | ld (ix+d),c | ld (ix+d),d | ld (ix+d),e | ld (ix+d),h | ld (ix+d),l | Note 1 | ld (ix+d),a |
| 0x78 | Note 1 | Note 1 | Note 1 | Note 1 | ld a,ixh | ld a,ixl | ld a,(ix+d) | Note 1 |
| 0x80 | Note 1 | Note 1 | Note 1 | Note 1 | add a,ixh | add a,ixl | add a,(ix+d) | Note 1 |
| 0x88 | Note 1 | Note 1 | Note 1 | Note 1 | adc a,ixh | adc a,ixl | adc a,(ix+d) | Note 1 |
| 0x90 | Note 1 | Note 1 | Note 1 | Note 1 | sub ixh | sub ixl | sub (ix+d) | Note 1 |
| 0x98 | Note 1 | Note 1 | Note 1 | Note 1 | sbc a,ixh | sbc a,ixl | sbc a,(ix+d) | Note 1 |
| 0xA0 | Note 1 | Note 1 | Note 1 | Note 1 | and ixh | and ixl | and (ix+d) | Note 1 |
| 0xA8 | Note 1 | Note 1 | Note 1 | Note 1 | xor ixh | xor ixl | xor (ix+d) | Note 1 |
| 0xB0 | Note 1 | Note 1 | Note 1 | Note 1 | or ixh | or ixl | or (ix+d) | Note 1 |
| 0xB8 | Note 1 | Note 1 | Note 1 | Note 1 | cp ixh | cp ixl | cp (ix+d) | Note 1 |
| 0xC0 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xC8 | Note 1 | Note 1 | Note 1 | shift dd,cb | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xD0 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xD8 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xE0 | Note 1 | pop ix | Note 1 | ex (sp),ix | Note 1 | push ix | Note 1 | Note 1 |
| 0xE8 | Note 1 | jp ix | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xF0 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xF8 | Note 1 | ld sp,ix | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |

| Note 1 | - | Decrement PC and retry with this as first opcode |
| --- | --- | --- |

DD CB OPCODES

|  |  | +00/08 |  | +01/09 |  | +02/0A |  | +03/0B |  | +04/0C |  | +05/0D |  | +06/0E |  | +07/0F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00 |  | rlc (ix+d) |  | rlc (ix+d) |  | rlc (ix+d) |  | rlc (ix+d) |  | rlc (ix+d) |  | rlc (ix+d) |  | rlc (ix+d) |  | rlc (ix+d) |
| 0x08 |  | rrc (ix+d) |  | rrc (ix+d) |  | rrc (ix+d) |  | rrc (ix+d) |  | rrc (ix+d) |  | rrc (ix+d) |  | rrc (ix+d) |  | rrc (ix+d) |
| 0x10 |  | rl (ix+d) |  | rl (ix+d) |  | rl (ix+d) |  | rl (ix+d) |  | rl (ix+d) |  | rl (ix+d) |  | rl (ix+d) |  | rl (ix+d) |
| 0x18 |  | rr (ix+d) |  | rr (ix+d) |  | rr (ix+d) |  | rr (ix+d) |  | rr (ix+d) |  | rr (ix+d) |  | rr (ix+d) |  | rr (ix+d) |
| 0x20 |  | sla (ix+d) |  | sla (ix+d) |  | sla (ix+d) |  | sla (ix+d) |  | sla (ix+d) |  | sla (ix+d) |  | sla (ix+d) |  | sla (ix+d) |
| 0x28 |  | sra (ix+d) |  | sra (ix+d) |  | sra (ix+d) |  | sra (ix+d) |  | sra (ix+d) |  | sra (ix+d) |  | sra (ix+d) |  | sra (ix+d) |
| 0x30 |  | sll (ix+d) |  | sll (ix+d) |  | sll (ix+d) |  | sll (ix+d) |  | sll (ix+d) |  | sll (ix+d) |  | sll (ix+d) |  | sll (ix+d) |
| 0x38 |  | srl (ix+d) |  | srl (ix+d) |  | srl (ix+d) |  | srl (ix+d) |  | srl (ix+d) |  | srl (ix+d) |  | srl (ix+d) |  | srl (ix+d) |
| 0x40 |  | bit 0,(ix+d) |  | bit 0,(ix+d) |  | bit 0,(ix+d) |  | bit 0,(ix+d) |  | bit 0,(ix+d) |  | bit 0,(ix+d) |  | bit 0,(ix+d) |  | bit 0,(ix+d) |
| 0x48 |  | bit 1,(ix+d) |  | bit 1,(ix+d) |  | bit 1,(ix+d) |  | bit 1,(ix+d) |  | bit 1,(ix+d) |  | bit 1,(ix+d) |  | bit 1,(ix+d) |  | bit 1,(ix+d) |
| 0x50 |  | bit 2,(ix+d) |  | bit 2,(ix+d) |  | bit 2,(ix+d) |  | bit 2,(ix+d) |  | bit 2,(ix+d) |  | bit 2,(ix+d) |  | bit 2,(ix+d) |  | bit 2,(ix+d) |
| 0x58 |  | bit 3,(ix+d) |  | bit 3,(ix+d) |  | bit 3,(ix+d) |  | bit 3,(ix+d) |  | bit 3,(ix+d) |  | bit 3,(ix+d) |  | bit 3,(ix+d) |  | bit 3,(ix+d) |
| 0x60 |  | bit 4,(ix+d) |  | bit 4,(ix+d) |  | bit 4,(ix+d) |  | bit 4,(ix+d) |  | bit 4,(ix+d) |  | bit 4,(ix+d) |  | bit 4,(ix+d) |  | bit 4,(ix+d) |
| 0x68 |  | bit 5,(ix+d) |  | bit 5,(ix+d) |  | bit 5,(ix+d) |  | bit 5,(ix+d) |  | bit 5,(ix+d) |  | bit 5,(ix+d) |  | bit 5,(ix+d) |  | bit 5,(ix+d) |
| 0x70 |  | bit 6,(ix+d) |  | bit 6,(ix+d) |  | bit 6,(ix+d) |  | bit 6,(ix+d) |  | bit 6,(ix+d) |  | bit 6,(ix+d) |  | bit 6,(ix+d) |  | bit 6,(ix+d) |
| 0x78 |  | bit 7,(ix+d) |  | bit 7,(ix+d) |  | bit 7,(ix+d) |  | bit 7,(ix+d) |  | bit 7,(ix+d) |  | bit 7,(ix+d) |  | bit 7,(ix+d) |  | bit 7,(ix+d) |
| 0x80 |  | res 0,(ix+d) |  | res 0,(ix+d) |  | res 0,(ix+d) |  | res 0,(ix+d) |  | res 0,(ix+d) |  | res 0,(ix+d) |  | res 0,(ix+d) |  | res 0,(ix+d) |
| 0x88 |  | res 1,(ix+d) |  | res 1,(ix+d) |  | res 1,(ix+d) |  | res 1,(ix+d) |  | res 1,(ix+d) |  | res 1,(ix+d) |  | res 1,(ix+d) |  | res 1,(ix+d) |
| 0x90 |  | res 2,(ix+d) |  | res 2,(ix+d) |  | res 2,(ix+d) |  | res 2,(ix+d) |  | res 2,(ix+d) |  | res 2,(ix+d) |  | res 2,(ix+d) |  | res 2,(ix+d) |
| 0x98 |  | res 3,(ix+d) |  | res 3,(ix+d) |  | res 3,(ix+d) |  | res 3,(ix+d) |  | res 3,(ix+d) |  | res 3,(ix+d) |  | res 3,(ix+d) |  | res 3,(ix+d) |
| 0xA0 |  | res 4,(ix+d) |  | res 4,(ix+d) |  | res 4,(ix+d) |  | res 4,(ix+d) |  | res 4,(ix+d) |  | res 4,(ix+d) |  | res 4,(ix+d) |  | res 4,(ix+d) |
| 0xA8 |  | res 5,(ix+d) |  | res 5,(ix+d) |  | res 5,(ix+d) |  | res 5,(ix+d) |  | res 5,(ix+d) |  | res 5,(ix+d) |  | res 5,(ix+d) |  | res 5,(ix+d) |
| 0xB0 |  | res 6,(ix+d) |  | res 6,(ix+d) |  | res 6,(ix+d) |  | res 6,(ix+d) |  | res 6,(ix+d) |  | res 6,(ix+d) |  | res 6,(ix+d) |  | res 6,(ix+d) |
| 0xB8 |  | res 7,(ix+d) |  | res 7,(ix+d) |  | res 7,(ix+d) |  | res 7,(ix+d) |  | res 7,(ix+d) |  | res 7,(ix+d) |  | res 7,(ix+d) |  | res 7,(ix+d) |
| 0xC0 |  | set 0,(ix+d) |  | set 0,(ix+d) |  | set 0,(ix+d) |  | set 0,(ix+d) |  | set 0,(ix+d) |  | set 0,(ix+d) |  | set 0,(ix+d) |  | set 0,(ix+d) |
| 0xC8 |  | set 1,(ix+d) |  | set 1,(ix+d) |  | set 1,(ix+d) |  | set 1,(ix+d) |  | set 1,(ix+d) |  | set 1,(ix+d) |  | set 1,(ix+d) |  | set 1,(ix+d) |
| 0xD0 |  | set 2,(ix+d) |  | set 2,(ix+d) |  | set 2,(ix+d) |  | set 2,(ix+d) |  | set 2,(ix+d) |  | set 2,(ix+d) |  | set 2,(ix+d) |  | set 2,(ix+d) |
| 0xD8 |  | set 3,(ix+d) |  | set 3,(ix+d) |  | set 3,(ix+d) |  | set 3,(ix+d) |  | set 3,(ix+d) |  | set 3,(ix+d) |  | set 3,(ix+d) |  | set 3,(ix+d) |
| 0xE0 |  | set 4,(ix+d) |  | set 4,(ix+d) |  | set 4,(ix+d) |  | set 4,(ix+d) |  | set 4,(ix+d) |  | set 4,(ix+d) |  | set 4,(ix+d) |  | set 4,(ix+d) |
| 0xE8 |  | set 5,(ix+d) |  | set 5,(ix+d) |  | set 5,(ix+d) |  | set 5,(ix+d) |  | set 5,(ix+d) |  | set 5,(ix+d) |  | set 5,(ix+d) |  | set 5,(ix+d) |
| 0xF0 |  | set 6,(ix+d) |  | set 6,(ix+d) |  | set 6,(ix+d) |  | set 6,(ix+d) |  | set 6,(ix+d) |  | set 6,(ix+d) |  | set 6,(ix+d) |  | set 6,(ix+d) |
| 0xF8 |  | set 7,(ix+d) |  | set 7,(ix+d) |  | set 7,(ix+d) |  | set 7,(ix+d) |  | set 7,(ix+d) |  | set 7,(ix+d) |  | set 7,(ix+d) |  | set 7,(ix+d) |
|  | Also copies result to B |  | Also copies result to C |  | Also copies result to D |  | Also copies result to E |  | Also copies result to H |  | Also copies result to L |  |  |  | Also copies result to A |  |

ED OPCODES

|  | +00/08 | +01/09 | +02/0A | +03/0B | +04/0C | +05/0D | +06/0E | +07/0F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x08 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x10 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x18 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x20 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x28 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x30 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x38 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x40 | in b,c | out c,b | sbc hl,bc | ld (word),bc | neg | retn | im 0 | ld i,a |
| 0x48 | in c,c | out c,c | adc hl,bc | ld bc,(word) | neg | reti | im 0 | ld r,a |
| 0x50 | in d,c | out c,d | sbc hl,de | ld (word),de | neg | retn | im 1 | ld a,i |
| 0x58 | in e,c | out c,e | adc hl,de | ld de,(word) | neg | reti | im 2 | ld a,r |
| 0x60 | in h,c | out c,h | sbc hl,hl | ld (word),hl | neg | retn | im 0 | rrd |
| 0x68 | in l,c | out c,l | adc hl,hl | ld hl,(word) | neg | reti | im 0 | rld |
| 0x70 | in 0,c | out c,0 | sbc hl,sp | ld (word),sp | neg | retn | im 1 | nop |
| 0x78 | in a,c | out c,a | adc hl,sp | ld sp,(word) | neg | reti | im 2 | nop |
| 0x80 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x88 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x90 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0x98 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0xA0 | ldi | cpi | ini | outi | nop | nop | nop | nop |
| 0xA8 | ldd | cpd | ind | outd | nop | nop | nop | nop |
| 0xB0 | ldir | cpir | inir | otir | nop | nop | nop | nop |
| 0xB8 | lddr | cpdr | indr | otdr | nop | nop | nop | nop |
| 0xC0 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0xC8 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0xD0 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0xD8 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0xE0 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0xE8 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0xF0 | nop | nop | nop | nop | nop | nop | nop | nop |
| 0xF8 | nop | nop | nop | nop | nop | nop | nop | nop |

FD OPCODES

|  | +00/08 | +01/09 | +02/0A | +03/0B | +04/0C | +05/0D | +06/0E | +07/0F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0x08 | Note 1 | add iy,bc | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0x10 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0x18 | Note 1 | add iy,de | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0x20 | Note 1 | ld iy,word | ld (word),iy | inc iy | inc iyh | dec iyh | ld iyh,byte | Note 1 |
| 0x28 | Note 1 | add iy,iy | ld iy,(word) | dec iy | inc iyl | dec iyl | ld iyl,byte | Note 1 |
| 0x30 | Note 1 | Note 1 | Note 1 | Note 1 | inc (iy+d) | dec (iy+d) | ld (iy+d),byte | Note 1 |
| 0x38 | Note 1 | add iy,sp | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0x40 | Note 1 | Note 1 | Note 1 | Note 1 | ld b,iyh | ld b,iyl | ld b,(iy+d) | Note 1 |
| 0x48 | Note 1 | Note 1 | Note 1 | Note 1 | ld c,iyh | ld c,iyl | ld c,(iy+d) | Note 1 |
| 0x50 | Note 1 | Note 1 | Note 1 | Note 1 | ld d,iyh | ld d,iyl | ld d,(iy+d) | Note 1 |
| 0x58 | Note 1 | Note 1 | Note 1 | Note 1 | ld e,iyh | ld e,iyl | ld e,(iy+d) | Note 1 |
| 0x60 | ld iyh,b | ld iyh,c | ld iyh,d | ld iyh,e | ld iyh,iyh | ld iyh,iyl | ld h,(iy+d) | ld iyh,a |
| 0x68 | ld iyl,b | ld iyl,c | ld iyl,d | ld iyl,e | ld iyl,iyh | ld iyl,iyl | ld l,(iy+d) | ld iyl,a |
| 0x70 | ld (iy+d),b | ld (iy+d),c | ld (iy+d),d | ld (iy+d),e | ld (iy+d),h | ld (iy+d),l | Note 1 | ld (iy+d),a |
| 0x78 | Note 1 | Note 1 | Note 1 | Note 1 | ld a,iyh | ld a,iyl | ld a,(iy+d) | Note 1 |
| 0x80 | Note 1 | Note 1 | Note 1 | Note 1 | add a,iyh | add a,iyl | add a,(iy+d) | Note 1 |
| 0x88 | Note 1 | Note 1 | Note 1 | Note 1 | adc a,iyh | adc a,iyl | adc a,(iy+d) | Note 1 |
| 0x90 | Note 1 | Note 1 | Note 1 | Note 1 | sub iyh | sub iyl | sub (iy+d) | Note 1 |
| 0x98 | Note 1 | Note 1 | Note 1 | Note 1 | sbc a,iyh | sbc a,iyl | sbc a,(iy+d) | Note 1 |
| 0xA0 | Note 1 | Note 1 | Note 1 | Note 1 | and iyh | and iyl | and (iy+d) | Note 1 |
| 0xA8 | Note 1 | Note 1 | Note 1 | Note 1 | xor iyh | xor iyl | xor (iy+d) | Note 1 |
| 0xB0 | Note 1 | Note 1 | Note 1 | Note 1 | or iyh | or iyl | or (iy+d) | Note 1 |
| 0xB8 | Note 1 | Note 1 | Note 1 | Note 1 | cp iyh | cp iyl | cp (iy+d) | Note 1 |
| 0xC0 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xC8 | Note 1 | Note 1 | Note 1 | shift fd,cb | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xD0 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xD8 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xE0 | Note 1 | pop iy | Note 1 | ex (sp),iy | Note 1 | push iy | Note 1 | Note 1 |
| 0xE8 | Note 1 | jp iy | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xF0 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| 0xF8 | Note 1 | ld sp,iy | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |

| Note 1 | - | Decrement PC and retry with this as first opcode |
| --- | --- | --- |

FD CB OPCODES

|  |  | +00/08 |  | +01/09 |  | +02/0A |  | +03/0B |  | +04/0C |  | +05/0D |  | +06/0E |  | +07/0F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00 |  | rlc (iy+d) |  | rlc (iy+d) |  | rlc (iy+d) |  | rlc (iy+d) |  | rlc (iy+d) |  | rlc (iy+d) |  | rlc (iy+d) |  | rlc (iy+d) |
| 0x08 |  | rrc (iy+d) |  | rrc (iy+d) |  | rrc (iy+d) |  | rrc (iy+d) |  | rrc (iy+d) |  | rrc (iy+d) |  | rrc (iy+d) |  | rrc (iy+d) |
| 0x10 |  | rl (iy+d) |  | rl (iy+d) |  | rl (iy+d) |  | rl (iy+d) |  | rl (iy+d) |  | rl (iy+d) |  | rl (iy+d) |  | rl (iy+d) |
| 0x18 |  | rr (iy+d) |  | rr (iy+d) |  | rr (iy+d) |  | rr (iy+d) |  | rr (iy+d) |  | rr (iy+d) |  | rr (iy+d) |  | rr (iy+d) |
| 0x20 |  | sla (iy+d) |  | sla (iy+d) |  | sla (iy+d) |  | sla (iy+d) |  | sla (iy+d) |  | sla (iy+d) |  | sla (iy+d) |  | sla (iy+d) |
| 0x28 |  | sra (iy+d) |  | sra (iy+d) |  | sra (iy+d) |  | sra (iy+d) |  | sra (iy+d) |  | sra (iy+d) |  | sra (iy+d) |  | sra (iy+d) |
| 0x30 |  | sll (iy+d) |  | sll (iy+d) |  | sll (iy+d) |  | sll (iy+d) |  | sll (iy+d) |  | sll (iy+d) |  | sll (iy+d) |  | sll (iy+d) |
| 0x38 |  | srl (iy+d) |  | srl (iy+d) |  | srl (iy+d) |  | srl (iy+d) |  | srl (iy+d) |  | srl (iy+d) |  | srl (iy+d) |  | srl (iy+d) |
| 0x40 |  | bit 0,(iy+d) |  | bit 0,(iy+d) |  | bit 0,(iy+d) |  | bit 0,(iy+d) |  | bit 0,(iy+d) |  | bit 0,(iy+d) |  | bit 0,(iy+d) |  | bit 0,(iy+d) |
| 0x48 |  | bit 1,(iy+d) |  | bit 1,(iy+d) |  | bit 1,(iy+d) |  | bit 1,(iy+d) |  | bit 1,(iy+d) |  | bit 1,(iy+d) |  | bit 1,(iy+d) |  | bit 1,(iy+d) |
| 0x50 |  | bit 2,(iy+d) |  | bit 2,(iy+d) |  | bit 2,(iy+d) |  | bit 2,(iy+d) |  | bit 2,(iy+d) |  | bit 2,(iy+d) |  | bit 2,(iy+d) |  | bit 2,(iy+d) |
| 0x58 |  | bit 3,(iy+d) |  | bit 3,(iy+d) |  | bit 3,(iy+d) |  | bit 3,(iy+d) |  | bit 3,(iy+d) |  | bit 3,(iy+d) |  | bit 3,(iy+d) |  | bit 3,(iy+d) |
| 0x60 |  | bit 4,(iy+d) |  | bit 4,(iy+d) |  | bit 4,(iy+d) |  | bit 4,(iy+d) |  | bit 4,(iy+d) |  | bit 4,(iy+d) |  | bit 4,(iy+d) |  | bit 4,(iy+d) |
| 0x68 |  | bit 5,(iy+d) |  | bit 5,(iy+d) |  | bit 5,(iy+d) |  | bit 5,(iy+d) |  | bit 5,(iy+d) |  | bit 5,(iy+d) |  | bit 5,(iy+d) |  | bit 5,(iy+d) |
| 0x70 |  | bit 6,(iy+d) |  | bit 6,(iy+d) |  | bit 6,(iy+d) |  | bit 6,(iy+d) |  | bit 6,(iy+d) |  | bit 6,(iy+d) |  | bit 6,(iy+d) |  | bit 6,(iy+d) |
| 0x78 |  | bit 7,(iy+d) |  | bit 7,(iy+d) |  | bit 7,(iy+d) |  | bit 7,(iy+d) |  | bit 7,(iy+d) |  | bit 7,(iy+d) |  | bit 7,(iy+d) |  | bit 7,(iy+d) |
| 0x80 |  | res 0,(iy+d) |  | res 0,(iy+d) |  | res 0,(iy+d) |  | res 0,(iy+d) |  | res 0,(iy+d) |  | res 0,(iy+d) |  | res 0,(iy+d) |  | res 0,(iy+d) |
| 0x88 |  | res 1,(iy+d) |  | res 1,(iy+d) |  | res 1,(iy+d) |  | res 1,(iy+d) |  | res 1,(iy+d) |  | res 1,(iy+d) |  | res 1,(iy+d) |  | res 1,(iy+d) |
| 0x90 |  | res 2,(iy+d) |  | res 2,(iy+d) |  | res 2,(iy+d) |  | res 2,(iy+d) |  | res 2,(iy+d) |  | res 2,(iy+d) |  | res 2,(iy+d) |  | res 2,(iy+d) |
| 0x98 |  | res 3,(iy+d) |  | res 3,(iy+d) |  | res 3,(iy+d) |  | res 3,(iy+d) |  | res 3,(iy+d) |  | res 3,(iy+d) |  | res 3,(iy+d) |  | res 3,(iy+d) |
| 0xA0 |  | res 4,(iy+d) |  | res 4,(iy+d) |  | res 4,(iy+d) |  | res 4,(iy+d) |  | res 4,(iy+d) |  | res 4,(iy+d) |  | res 4,(iy+d) |  | res 4,(iy+d) |
| 0xA8 |  | res 5,(iy+d) |  | res 5,(iy+d) |  | res 5,(iy+d) |  | res 5,(iy+d) |  | res 5,(iy+d) |  | res 5,(iy+d) |  | res 5,(iy+d) |  | res 5,(iy+d) |
| 0xB0 |  | res 6,(iy+d) |  | res 6,(iy+d) |  | res 6,(iy+d) |  | res 6,(iy+d) |  | res 6,(iy+d) |  | res 6,(iy+d) |  | res 6,(iy+d) |  | res 6,(iy+d) |
| 0xB8 |  | res 7,(iy+d) |  | res 7,(iy+d) |  | res 7,(iy+d) |  | res 7,(iy+d) |  | res 7,(iy+d) |  | res 7,(iy+d) |  | res 7,(iy+d) |  | res 7,(iy+d) |
| 0xC0 |  | set 0,(iy+d) |  | set 0,(iy+d) |  | set 0,(iy+d) |  | set 0,(iy+d) |  | set 0,(iy+d) |  | set 0,(iy+d) |  | set 0,(iy+d) |  | set 0,(iy+d) |
| 0xC8 |  | set 1,(iy+d) |  | set 1,(iy+d) |  | set 1,(iy+d) |  | set 1,(iy+d) |  | set 1,(iy+d) |  | set 1,(iy+d) |  | set 1,(iy+d) |  | set 1,(iy+d) |
| 0xD0 |  | set 2,(iy+d) |  | set 2,(iy+d) |  | set 2,(iy+d) |  | set 2,(iy+d) |  | set 2,(iy+d) |  | set 2,(iy+d) |  | set 2,(iy+d) |  | set 2,(iy+d) |
| 0xD8 |  | set 3,(iy+d) |  | set 3,(iy+d) |  | set 3,(iy+d) |  | set 3,(iy+d) |  | set 3,(iy+d) |  | set 3,(iy+d) |  | set 3,(iy+d) |  | set 3,(iy+d) |
| 0xE0 |  | set 4,(iy+d) |  | set 4,(iy+d) |  | set 4,(iy+d) |  | set 4,(iy+d) |  | set 4,(iy+d) |  | set 4,(iy+d) |  | set 4,(iy+d) |  | set 4,(iy+d) |
| 0xE8 |  | set 5,(iy+d) |  | set 5,(iy+d) |  | set 5,(iy+d) |  | set 5,(iy+d) |  | set 5,(iy+d) |  | set 5,(iy+d) |  | set 5,(iy+d) |  | set 5,(iy+d) |
| 0xF0 |  | set 6,(iy+d) |  | set 6,(iy+d) |  | set 6,(iy+d) |  | set 6,(iy+d) |  | set 6,(iy+d) |  | set 6,(iy+d) |  | set 6,(iy+d) |  | set 6,(iy+d) |
| 0xF8 |  | set 7,(iy+d) |  | set 7,(iy+d) |  | set 7,(iy+d) |  | set 7,(iy+d) |  | set 7,(iy+d) |  | set 7,(iy+d) |  | set 7,(iy+d) |  | set 7,(iy+d) |
|  | Also copies result to B |  | Also copies result to C |  | Also copies result to D |  | Also copies result to E |  | Also copies result to H |  | Also copies result to L |  |  |  | Also copies result to A |  |