TECHNICAL INFO

Using The Interface I

If you want to use the microdrive, you'll need cartridge files.  The

emulator can create an empty cartridge file for you.  You have to format

it before you can use it.  Type

FORMAT "m";1;"name"

to format the cartridge currently in Microdrive 1 giving it the name

'name'.  Next, type CAT 1 to get a catalogue of the files on it (none of

course) and the number of kilobytes free.  You can save a file by typing

for instance

SAVE *"m";1;"screen"SCREEN$

Instead of SCREEN$ you can use all other expressions that are permitted

also when saving to tape, like LINE nnnn or CODE x,y etcetera.  To load

a file back from cartridge, you type (you guessed it)

LOAD *"m";1;"screen"SCREEN$

If the file doesn't exist or is of the wrong type you'll get the

appropriate error message.  To erase a file, type for instance

ERASE "m";1;"screen"

Note that no * is needed (or even permitted), and that only the name

should be given.  There's another way to create a file on a cartridge,

and that is by using a command like OPEN #3;"m";1;"name", and printing

to that stream.  You can use MOVE to move data from stream to stream,

but I'll not go into that --- it's not very much used anyway.

Instead of to the microdrive, you can also 'save to the RS232 link'. For

instance, type SAVE *"b"SCREEN$ (note: there's no name!) to save a

screen.  On the emulator you can send the output to the RS232 channel to

a printer (then SAVE *"b" is useless), to a file (can be useful) or to

the COM port (very useful if you connect a real Spectrum to the PC's COM

port!).  You can load the data back by typing LOAD *"b"SCREEN$ and

making sure the RS232 channel is fed with the right input (from a COM

port or a file).  See also paragraph 2.6.

If you want to use the RS232 channel for printing, open stream 3 for

output to that channel by typing

OPEN #3,"b"

or

OPEN #3,"t"

The first will simply copy everything you send to stream 3 (using for

instance LPRINT or LLIST) to the RS232 channel; the second converts CR's

into CR/LF's, breaks off lines at 80 characters and translates keywords

into character sequences.  "t" is useful for LLISTings, but not for

anything else.

Useful extra commands: CLS #, to clear the screen and reset the

attributes to their reset defaults, and CLEAR # to do a CLS # and close

all currently open streams (discarding all data that may still be

buffered!)

The Interface I uses its own system variables.  At the first error

message you make (or RASP, or flashing question mark) and at the first

Interface I statement you execute, it inserts them automatically.  Some

programs will not run when the Interface I has inserted its system

variables.  So if you load a game from tape, reset the Spectrum first

and don't make an error typing LOAD "".  With a bit of exercise you

should be able to do this.

The Spectrum

The Spectrum is at the hardware level a very simple machine.  There's

the 16K ROM which occupies the lowest part of the address space, and 48K

of RAM which fills up the rest.  An ULA which reads the lowest 6912

bytes of RAM to display the screen, and contains the logic for just one

I/O port completes the machine, from a software point of view at least.

Every even I/O address will address the ULA, but to avoid problems with

other I/O devices only port FE should be used.  If this port is written

to, bits have the following meaning:

Bit   7   6   5   4   3   2   1   0

+-------------------------------+

|   |   |   | E | M |   Border  |

+-------------------------------+

The lowest three bits specify the border colour; a zero in bit 3

activates the MIC output, and a one in bit 4 activates the EAR output

(which sounds the internal speaker).  The real Spectrum also activates

the MIC when the ear is written to; the emulator doesn't.  This is no

problem; MIC is only used for saving, and when saving the Spectrum never

sounds the internal speaker.  The upper three bits are unused.

If port FE is read from, the highest eight address lines are important

too.  A zero on one of these lines selects a particular half-row of five

keys:

IN:    Reads keys (bit 0 to bit 4 inclusive)

#FEFE  SHIFT, Z, X, C, V            #EFFE  0, 9, 8, 7, 6

#FDFE  A, S, D, F, G                #DFFE  P, O, I, U, Y

#FBFE  Q, W, E, R, T                #BFFE  ENTER, L, K, J, H

#F7FE  1, 2, 3, 4, 5                #7FFE  SPACE, SYM SHFT, M, N,

A zero in one of the five lowest bits means that the corresponding key

is being pressed.  If more than one address line is made low, the result

is the logical AND of all single inputs, so a zero in a bit means that

at least one of the appropriate keys is pressed.  For example, only if

each of the five lowest bits of the result from reading from port 00FE

(for instance by XOR A/IN A,(FE)) is one, no key is pressed.

A final remark about the keyboard.  It is connected in a matrix-like

fashion, with 8 rows of 5 columns, as is obvious from the above remarks.

Any two keys pressed simultaneously can be uniquely decoded by reading

from the IN ports, however, if more than two keys are pressed decoding

may not be uniquely possible.  For instance, if you press Caps shift, B

and V, the Spectrum will think also the Space key is pressed, and react

by giving the 'Break into Program' report.  This matrix behaviour is

also emulated - without it, Zynaps for instance won't pause when you

press 5,6,7,8 and 0 simultaneously.

Bit 5 (value 64) of IN-port FE is the ear input bit.  When the line is

silent, its value is zero, except in the early Model 2 of the Spectrum,

where it was one.  When there is a signal, this bit toggles.  The

Spectrum loading software is not sensitive to the polarity of this bit

(which it definitely should not be, not only because of this model

difference, but also because you cannot be sure the tape recorder

doesn't change the polarity of the signal recorded!) Some old programs

rely on the fact that bit 5 is always one (for instance Spinads); for

these programs the emulator can mimic a Model 2 Spectrum.

Bits 6 and 7 are always one.

The ULA with the lower 16K of RAM, and the processor with the upper 32K

RAM and 16K ROM are working independently of each other.  The data and

address buses of the Z80 and the ULA are connected by small resistors;

normally, these do effectively decouple the buses.  However, if the Z80

wants to read of write the lower 16K, the ULA halts the processor if it

is busy reading, and after it's finished it lets the processor access

lower memory through the resistors.  A very fast, cheap and neat design

indeed!

If you run a program in the lower 16K of RAM, or read or write in that

memory, the processor is halted sometimes.  This part of memory is

therefore somewhat slower than the upper 32K block.  This is also the

reason that you cannot write a sound- or save-routine in lower memory;

the timing won't be exact, and the music will sound harsh.  Also, INning

from port FE will halt the processor, because the ULA has to supply the

result.  Therefore, INning from port FE is a tiny bit slower on average

than INning from other ports; whilst normally an IN A,(nn) instruction

would take 11 T states, it takes 12.15 T states on average if nn=FE. See

below for more exact information.

If the processor reads from a non-existing IN port, for instance FF, the

ULA won't stop, but nothing will put anything on the data bus.

Therefore, you'll read a mixture of FF's (idle bus), and screen and ATTR

data bytes (the latter being very scarce, by the way).  This will only

happen when the ULA is reading the screen memory, about 60% of the

1/50th second time slice in which a frame is generated.  The other 40%

the ULA is building the border or generating a vertical retrace.  This

behaviour is actually used in some program, for instance by Arkanoid,

and the emulator also emulates this behaviour.

Spectrum 128K

In comparison to the Spectrum 48K, and on the hardware side, the

Spectrum 128 offers more RAM (128K, you guessed it), more ROM (32K

instead of 16K), a soundchip, and a serial printer port.  Nothing

really spectacular.

Another difference with the 48K Spectrum is in the timing of the video

signals.  You can see this if you save something: the bars in the

border move differently.  The really important difference of the 128K

with respect to the video is that the 128K ULA is more relaxed in

giving the Z80 access to (screen) memory.  This allows programs to make

hi-resolution colour effects not only in the border, but also on the

screen itself.  Many 128K programs use this effect.  Note however that,

although the 128K ULA is more relaxed towards memory access, it does

still halt the Z80 occasionally.  Partly for this reason it is

impossible to have hi-res colour effect over the entire screen; there is

only time to change approximately half of it.  (This does not seem to

be true; the Shock Megademo manages to move 1-pixel thick lines up on

the screen one pixel per frame on the whole screen.  Very well done

indeed!  This is the only program I know to feature hi-res colour effect

over the entire width of the screen.  It might be, however, that this

program only changes the entire ATTR line every two scan line times.)

The basic video timings, with the Z80 out of the way, are as follows.

Each video line takes 228 T states, 4 T states more than on the 48K

Spectrum.  It starts with 128 T states of screen pixels (or border).

Then there's border, horizontal retrace and border again, of 100 T

states.  A complete '50 Hz' frame consists of 311 video lines (of which

a few are vertical retraces), that is, 1 less than for 48K models.  A

complete frame is 311 x 228 = 70908 T states long.

I don't know whether the 128K model uses a different crystal.  If not,

one frame on the Spectrum 128K is 1.5% longer than a 48K frame.

A quote from the +2 manual, page 279; Cliff Lawson writes: "For the

contended RAM [pages 4-7] (which shares time between the video

circuitry and the processor), during 128 out of every 228 CPU T states

(1 TV line), and during 192 out of every 311 TV lines (1 frame) the CPU

is allowed only 1 access to contended RAM in every 8 T states.  The CPU

is controlled by introducing wait states".  My guess is this holds true

for the 128K too.

Directly after an interrupt is generated by the ULA (so slightly before

the Z80 acts upon it), 63 video lines are written to the CRT.  A first

few may be verical retraces; this is difficult to find out without an

oscilloscope, but luckily we don't need this information.  Then 192

screen lines are written, and then 56 border lines and (possibly)

vertical retrace lines.  The first screen byte is written to the screen

14364 T states after the interrupt was generated.

Then the other things.  Memory is arranged in banks of 16K.  The bank

at 0000-3FFF contains either the original 48K ROM or the new 128K ROM.

The latter is active at reset.  The bank at 4000-7FFF always contains

RAM page 5.  The bank at 8000-BFFF is always page 2.  The bank at

C000-FFFF contains any page from 0-7, including page 2 and 5.  If page

2 or 5 is enabled in the high bank, every byte written in this bank is

mirrored in the other bank at 4000 or at 8000, and vice versa.  The

screen information is read, by the ULA, from the first 6912 bytes of

either page 5 or page 7.

All this is controlled by writing to port 7FFD (or, in fact, by writing

to any address with bit 15 and 1 zero; don't use this fact, though; it

does not work on the +3, where there's another port 1FFD, and neither

will this work in full generality on the emulator.)

Port #7FFD:

Bit    7   6   5   4   3   2   1   0

+-------------------------------+

WRITE|   |   | P | R | S |  page no  |

+-------------------------------+

Bits 0-2 determine which page is to appear in bank C000-FFFF.  If S=0,

the ULA reads the screen from page 5, otherwise it reads page 7.  If

R=0, the 128K Rom is selected in bank 0000-3FFF; otherwise the 48K Rom.

If P=1, port #7FFD will be disabled and keep its value until the

computer is reset.  This bit is set if you select 48K Spectrum in the

128K startup menu, so that no 48K program is able to (accidentally)

swap itself into oblivion.

The sound chip of the Spectrum 128 is described in the next section.

The AY-3-8912 sound chip

The following section was put together using information collected from

comp.sys.sinclair.  Thanks, and large parts of the information below,

are due to Alastair Booker, who put a detailed description on the net

in April '95, and to Ian Collier who recently carried out some thorough

investigations on the AY chip.

This chip is used in for instance the Sinclair ZX Spectrum 128/+2/+3,

Amstrad CPC 464/664/6128, Mattel Intellivision, Atari ST, Sega Master

System and the MSX.

The AY has 16 internal registers.  A register is selected by OUTing the

register number in bits 0-3 to port #FFFD (only A15, A14 and A1 are

decoded).  Then write to a register by OUTing to #BFFD, read it by

INning from #FFFD.  When reading from a register, unused bits are

always 0.  Reading always yields the value last written to the

register, except for R14 and R15 when bit 6 or 7 of R7 are reset (R14 /

R15 used for input).  On the AY-3-8912, when R7 bit 7 is reset, R15

always reads 255.  Writing to R14 or R15 when they are selected for

input does load the output register.

Here are the names of the AY registers:

Register        Name                                Bits used:

R0              Fine tone control (FTC) channel A      0-7

R1              Coarse tone control (CTC) channel A    0-3

R2              FTC channel B                          0-7

R3              CTC channel B                          0-3

R4              FTC channel C                          0-7

R5              CTC channel C                          0-3

R6              Noise generator pitch control          0-4

R7              Mixer and I/O control                  0-7

R8              Amplitude channel A                    0-4

R9              Amplitude channel B                    0-4

R10             Amplitude channel C                    0-4

R11             Envelope fine period control           0-7

R12             Envelope coarse period control         0-7

R13             Envelope control                       0-3

R14             RS232 i/o                              0-7

R15             I/O port 2                             0-7

The AY chip consists of three tone generators, one noise generator, an

envelope generator, three mixers, and three volume generators.

Tone generator A is controlled by R0 and R1.  It contains a 12 bit up

counter which is reset to 0 whenever it is larger than or equal to the

value of R1R0 (most significant bits are in R1), and is counted up at a

frequency of 221660 Hz (which is the driving frequency of the chip

divided by 8). Loading R0 or R1 takes effect directly.  If the internal

counter is below the new value, it simply continues; if it is above, it

immediately resets to 0.  Every time the internal counter is reset, the

tone generator changes it output from 0 to 1 or vice versa, so the

frequency of the tone generated is 110830/R1R0 Hz (one period consists

of two transitions). If R1R0 contains 0, the counter behaves as if R1R0

contained 1.

The noise generator contains a 5 bit up counter, which is reset to 0

whenever it is >= R6.  It is counted up at a frequency of 110830 Hz

(driving frequency divided by 16) [1].  Every time it reaches zero, it

randomly chooses 0 or 1 as its new output [2].  When R6 is zero, the

noise generated is the same as when R6 is 1.  Changes to R6 take effect

only when the internal counter reaches 0.

Bit      7    6    5    4    3    2    1    0

+---------------------------------------+

R7    | D2 | D1 | Nc | Nb | Na | Tc | Tb | Ta |

+---------------------------------------+

If D1 is 1, R14 acts as output register (RS232 output: bit 2 is CTS,

bit 3 is data output); when it is 0 R14 acts as input register (bit 6

is DTR [3]).  D2 is ignored as the AY chip has only one I/O register;

the bit and its corresponding register are present however.  Reading

R15 in input mode always yields 255.  Changes made to R7 take effect

immediately.

The noise and tone output of a channel is combined in the mixer in the

following way:

Output_A = (Tone_A OR Ta) AND (Noise OR Na)

Here Tone_A is the binary output of tone generator A, and Noise is the

binary output of the noise generator.  Note that setting both Ta and Na

to 1 produces a constant 1 as output.  Also note that setting both Ta

and Na to 0 produces bursts of noise and half-periods of constant

output 0.

Each binary tone channel output is fed to a separate volume generator.

Each volume generator is controlled by its amplitude register (R8 for

channel A) and the 4-bit output of the envelope controller.

Bit      7    6    5    4    3    2    1    0

+---------------------------------------+

R8    |    |    |    | Ev | V3 | V2 | V1 | V0 |

+---------------------------------------+

If Ev=0, the current volume is given by V3V2V1V0.  If Ev=1, the current

volume is given by the output of the envelope generator.  The volume

controller produces an output voltage proportional to its channel's

binary output value times the current volume.  These analogue outputs

are then added together to give the final output (this is done outside

the chip actually).  Note that even when a channel is disabled (say Ta

= Na = 1), changing the volume level changes the final output, as the

binary tone channel output is constant and equal to 1, not 0.  Changes

to the amplitude registers take effect immediately.

The envelope generator is controlled by R11, R12 and R13.

Bit     7  6  5  4      3         2         1        0

+--------------------------------------------------+

R13   |  |  |  |  | Continue | Attack | Alternate | Hold |

+--------------------------------------------------+

The envelope generator contains a 16-bit up counter, operated at a

frequency of 110830 Hz.  The lowest frequency attainable by the

envelope clock is therefore 1.7 Hz.  If R12R11 contains 0, the clock

runs at 110830 Hz; otherwise it runs at 110830/R12R11 Hz.  The envelope

generator is reset by writing to R13 (but not by writing to R11 or

R12); otherwise it works just as the other counters.

Each time the envelope generator up counter is reset, it produces a

envelope clock tick.  A 'period' is the time taken by 16 clock ticks.

The output of the envelope generator during the first period is as

follows.  If Attack = 1, the output starts off at 0 and at each clock

tick is increased by 1 until it reaches 15.  If Attack = 0, the output

starts off at 15 and is falls to 0.

The output in the subsequent periods is 0 if Continue = 0.  Otherwise,

first the (internal) 'Attack' bit is toggled if Alternate is set,

otherwise it doesn't change.  How if Hold = 1, the output is a constant

15 in this and subsequent periods if Attack = 1, otherwise it is a

constant 0.  If Hold = 0, the envelope generator behaves just as in the

first period (except that Attack has possibly changed).  To sum it up:

0,1,2,3  \__________     single decay then off

4,5,6,7  /|_________     single attack then off

8        \|\|\|\|\|\     repeated decay

9        \__________     single decay then off

10       \/\/\/\/\/\     repeated decay-attack

_________

11       \|              single decay then hold

12       /|/|/|/|/|/     repeated attack

__________

13       /               single attack then hold

14       /\/\/\/\/\/     repeated attack-decay

15       /|_________     single attack then off

If the envelope generator is used to generate a tone, its frequency is

either 110830/(16*R12R11) Hz (R13 = 8 or 12) or 110830/(32*R12R11) Hz

(R13 = 10 or 14).

[1] This has been checked by Pierre Guerrier using an oscilloscope.

[2] The algorithm used for the pseudo random output is not known.  It

seems not to be too good, as sounds vaguely similar to tape loading

sounds are audible if you set R6 to 31.

[3] I don't know which bit is data input; it is probably not bit 3.

The Interface I

The Interface I is quite complicated.  It uses three different I/O

ports, and contains logic to page and unpage an 8K ROM if new commands

are used.  I won't be very detailed here; you could refer to the source

code of the emulator if you want to know some details, or read the

'Spectrum Shadow ROM Disassembly' by Gianlura Carri, published by

Melbourne House - but don't expect the same level of detail as of Ian

Logan and Frank O'Hara in their Rom disassembly book.

The ROM is paged if the processor executes the instruction at ROM

address 0008 or 1708 hexadecimal, the error and close# routines.  It is

inactivated when the Z80 executes the RET at address 0700.

I/O Port E7 is used to send or receive data to and from the microdrive.

Accessing this port will halt the Z80 until the Interface I has

collected 8 bits from the microdrive head; therefore, it the microdrive

motor isn't running, or there is no formatted cartridge in the

microdrive, the Spectrum hangs.  This is the famous 'IN 0 crash'.

Port EF is used for several things:

Bit    7   6    5    4    3    2    1     0

+---------------------------------------+

READ|   |   |    |busy| dtr |gap| sync|write|

|   |   |    |    |     |   |     |prot.|

+---------------------------------------+

WRITE|   |   |wait| cts|erase|r/w|comms|comms|

|   |   |    |    |     |   | clk | data|

+---------------------------------------+

Bits DTR and CTS are used by the RS232 interface.  The WAIT bit is used

by the Network to synchronise, GAP, SYNC, WR_PROT, ERASE, R/_W, COMMS

CLK and COMMS DATA are used by the microdrive system.  If the microdrive

is not being used, the COMMS DATA output selects the function of bit 0

of out-port F7:

Bit      7    6   5   4   3   2   1       0

+------------------------------------------+

READ|txdata|   |   |   |   |   |   |    net    |

|      |   |   |   |   |   |   |   input   |

+------------------------------------------+

WRITE|      |   |   |   |   |   |   |net output/|

|      |   |   |   |   |   |   |   rxdata  |

+------------------------------------------+

TXDATA and RXDATA are the input and output of the RS232 port.  COMMS

DATA determines whether bit 0 of F7 is output for the RS232 or the

network.

The ZX Printer

As always, how very Sinclair, this is a simple but ingenious piece of

equipment.  The ZX Printer is controlled through one I/O port, namely

#FB.  It is decoded using A2 only.  Port #FB:

Bit      7    6    5    4    3    2    1    0

+---------------------------------------+

READ  |styl|  0 |    |    |    |    |    | enc|

+---------------------------------------+

+---------------------------------------+

WRITE |Data|    |    |    |    | Mot|Slow|    |

+---------------------------------------+

To print, first read port #FB and check that bit 6 is 0 to make sure

that the printer is attached.  The printer motor is started by writing

a byte 0 to #FB (Mot=0).  Then check bit 7 until it goes high; this

means that the printer head is in the starting position.  A line is

printed as follows.  Wait for bit 0 (encoder) to go high, and write a

bit of data (1=black dot), and do this 256 times.  The Spectrum ROM

makes the 'Slow' bit 1 in the last two lines; this ensures that the

printer head stops at the start of the new line, instead of halfway

into it.  Stop the printer by writing a byte 4.  See #0EF4 in the ROM

for the relevant routine.

The emulator returns #FF on a port #FB IN if there is no printer

attached to the LPT port selected for ZX Printer output.  If the

printer is off-line, busy, or out of paper, the emulator will usually

return #FF too, so that the Spectrum program will not even consider to

print, unless you set option -xj.  In that case the emulator returns 1

so that the program will try to print, and will wait for the printer to

go on line.  The reason for including the -xj switch is that usually

the status lines of the printer are not too reliable, so that a

Spectrum program may halt on a COPY statement even if there's no

printer attached (e.g. with SuperSpy, which makes screendumps on the ZX

Printer without asking).  In the inner printing loop, the ROM does not

check for BREAKs, so that the encoder bit is always high to prevent

locking the emulated Spectrum.  If the printer is happy to accept data,

the emulator returns #81.

File formats

.TAP FILES:

The .TAP files contain blocks of tape-saved data.  All blocks start with

two bytes specifying how many bytes will follow (not counting the two

length bytes).  Then raw tape data follows, including the flag and

checksum bytes.  The checksum is the bitwise XOR of all bytes including

the flag byte.  For example, when you execute the line SAVE "ROM" CODE

0,2 this will result:

|------ Spectrum-generated data -------|       |---------|

13 00 00 03 52 4f 4d 7x20 02 00 00 00 00 80 f1 04 00 ff f3 af a3

^^^^^       first block is 19 bytes (17 bytes+flag+checksum)

^^    flag byte (A reg, 00 for headers, ff for datablocks)

^^ first byte of header, indicating a code block

filename    ^^^^^^^^^^^^^

header info               ^^^^^^^^^^^^^^^^^

checksum of header                          ^^

length of second block                         ^^^^^

flag byte                                             ^^

first two bytes of rom                                  ^^^^^

checksum (checkbittoggle would be better)                     ^^

The emulator will always start reading bytes at the beginning of a

block.  If less bytes are loaded than are available, the other bytes are

skipped, and the last byte loaded is used as checksum.  If more bytes

are asked for than exist in the block, the loading routine will

terminate with the usual tape-loading-error flags set, leaving the error

handling to the calling Z80 program.

Note that it is possible to join .TAP files by simply stringing them

together, for example COPY /B FILE1.TAP + FILE2.TAP ALL.TAP

For completeness, I'll include the structure of a tape header.  A header

always consists of 17 bytes:

Byte    Length  Description

0       1       Type (0,1,2 or 3)

1       10      Filename (padded with blanks)

11      2       Length of data block

13      2       Parameter 1

15      2       Parameter 2

The type is 0,1,2 or 3 for a Program, Number array, Character array or

Code file.  A screen$ file is regarded as a Code file with start address

16384 and length 6912 decimal.  If the file is a Program file, parameter

1 holds the autostart line number (or a number >=32768 if no LINE

parameter was given) and parameter 2 holds the start of the variable

area relative to the start of the program.  If it's a Code file,

parameter 1 holds the start of the code block when saved, and parameter

2 holds 32768.  For data files finally, the byte at position 14 decimal

holds the variable name.

.MDR FILES:

The emulator uses a cartridge file format identical to the 'Microdrive

File' format of Carlo Delhez' Spectrum emulator Spectator for the QL.

The following information is adapted from Carlo's documentation.  It can

also be found in the 'Spectrum Microdrive Book', by Ian Logan (co-writer

of the excellent 'Complete Spectrum ROM Disassembly').

A cartridge file contains 254 'sectors' of 543 bytes each, and a final

byte flag which is non-zero is the cartridge is write protected, so the

total length is 137923 bytes.  On the cartridge tape, after a GAP of

some time the Interface I writes 10 zeros and 2 FF bytes (the preamble),

and then a fifteen byte header-block-with-checksum.  After another GAP,

it writes a preamble again, with a 15-byte record-

descriptor-with-checksum (which has a structure very much like the

header block), immediately followed by the data block of 512 bytes, and

a final checksum of those 512 bytes.  The preamble is used by the

Interface I hardware to synchronise, and is not explicitly used by the

software.  The preamble is not saved to the microdrive file:

offset length name    contents

0      1   HDFLAG   Value 1, to indicate header block

1      1   HDNUMB   sector number (values 254 down to 1)

2      2            not used

4     10   HDNAME   microdrive cartridge name (blank padded)

14      1   HDCHK    header checksum (of first 14 bytes)

15      1   RECFLG   - bit 0: always 0 to indicate record block

- bit 1: set for the EOF block

- bit 2: reset for a PRINT file

- bits 3-7: not used (value 0)

16      1   RECNUM   data block sequence number (value starts at 0)

17      2   RECLEN   data block length (<=512, LSB first)

19     10   RECNAM   filename (blank padded)

29      1   DESCHK   record descriptor checksum (of previous 14 bytes)

30    512            data block

542      1   DCHK     data block checksum (of all 512 bytes of data

block, even when not all bytes are used)

---------

254 times

(Actually, this information is 'transparent' to the emulator.  All it

does is store 2 times 254 blocks in the .MDR file as it is OUTed,

alternatingly of length 15 and 528 bytes.  The emulator does check

checksums, see below; the other fields are dealt with by the emulated

Interface I software.)

A used record block is either an EOF block (bit 1 of RECFLG is 1) or

contains 512 bytes of data (RECLEN=512, i.e.  bit 1 of MSB is 1).  An

empty record block has a zero in bit 1 of RECFLG and also RECLEN=0.  An

unusable block (as determined by the FORMAT command) is an EOF block

with RECLEN=0.

The three checksums are calculated by adding all the bytes together

modulo 255; this will never produce a checksum of 255.  Possibly, this

is the value that is read by the Interface I if there's no or bad data

on the tape.

In normal operation, all first-fifteen-byte blocks of each header or

record block will have the right checksum.  If the checksum is not

right, the block will be treated as a GAP.  For instance, if you type

OUT 239,0 on a normal Spectrum with interface I, the microdrive motor

starts running and the cartridge will be erased completely in 7 seconds.

CAT 1 will respond with 'microdrive not ready'.  Try it on the

emulator...

.SCR FILES:

.SCR files are memory dumps of the first 6912 bytes of the Spectrum

memory.  A coordinate (x,y), x between 0 and 255 and y between 0 and

192, (0,0) being the upper left corner of the screen, corresponds to the

pixel address

16384+INT (x/8)+1792*INT (y/64)-2016*INT (y/8)+256*y

I admit this is not quite the clearest way to explain the organization

of Spectrum's video memory, but with a bit or (hard) thinking you can

extract from above formula all information you need...  The lowest three

bits of x determine which bit of this address corresponds to the pixel

(x,y).  This bit-map constitutes the larger part of the screen memory,

256*192/8=6144 bytes.  The final 768 bytes are attribute bytes.  The

address of the attribute byte corresponding to pixel (x,y) is

22528+INT (x/8)+32*INT (y/8)

The lowest three bits of the attribute byte control the foreground color

(the color of the pixel if the corresponding bit in the bitmap is set),

bits 3-5 control the background color, bit 6 is the bright bit and bit 7

is the flash bit - if it is set, every 16/50th of a second the ULA

effectively flips the foreground and background colours.

.Z80 and .SLT files:

The old .Z80 snapshot format (for version 1.45 and below) looks like

this:

Offset  Length  Description

0       1       A register

1       1       F register

2       2       BC register pair (LSB, i.e. C, first)

4       2       HL register pair

6       2       Program counter

8       2       Stack pointer

10      1       Interrupt register

11      1       Refresh register (Bit 7 is not significant!)

12      1       Bit 0  : Bit 7 of the R-register

Bit 1-3: Border colour

Bit 4  : 1=Basic SamRom switched in

Bit 5  : 1=Block of data is compressed

Bit 6-7: No meaning

13      2       DE register pair

15      2       BC' register pair

17      2       DE' register pair

19      2       HL' register pair

21      1       A' register

22      1       F' register

23      2       IY register (Again LSB first)

25      2       IX register

27      1       Interrupt flipflop, 0=DI, otherwise EI

28      1       IFF2 (not particularly important...)

29      1       Bit 0-1: Interrupt mode (0, 1 or 2)

Bit 2  : 1=Issue 2 emulation

Bit 3  : 1=Double interrupt frequency

Bit 4-5: 1=High video synchronisation

3=Low video synchronisation

0,2=Normal

Bit 6-7: 0=Cursor/Protek/AGF joystick

1=Kempston joystick

2=Sinclair 2 Left joystick (or user

defined, for version 3 .Z80 files)

3=Sinclair 2 Right joystick

Because of compatibility, if byte 12 is 255, it has to be regarded as

being 1.  Following this header block of 30 bytes the 48K bytes of

Spectrum memory are stored, in a compressed format (if bit 5 of byte 12

is set).

The compression method is this.  Repetitions of at least five equal

bytes are replaced by the four-byte code ED ED xx yy, which stands for

"byte yy repeated xx times".  Only sequences of length at least 5 are

coded. The exception is sequences consisting of ED's; if they are

encountered, even two ED's are encoded into ED ED 02 ED.  Finally,

every byte directly following a single ED is not taken into a block,

for example ED 6*00 is not encoded into ED ED ED 06 00 but into ED 00

ED ED 05 00.  The block is terminated by an end marker, 00 ED ED 00.

That's the format of .Z80 files as used by versions up to 1.45.

Starting from version 2.0, a different format is used, since from then

on also 128K snapshots had to be supported.  This new format is used

for all snapshots (48K or 128K).

Version 2.01 and 3.0x .Z80 files start with the same 30 byte header

that old .Z80 files used.  Bit 4 and 5 of the flag byte (offset 12)

have no meaning anymore, and the program counter (offset 6 and 7) are

zero to signal a version 2.01 (or later) snapshot file.

Starting from version 3.05, Z80 ignores the setting of byte 29, bit 3

(Double Interrupt frequency) when loading snapshots, and resets the bit

when saving, because of the altered behaviour of the -d switch.

After the first 30 bytes, the additional header follows:

Offset  Length  Description

* 30      2       Length of additional header block (see below)

* 32      2       Program counter

* 34      1       Hardware mode (see below)

* 35      1       If in SamRam mode, bitwise state of 74ls259.

For example, bit 6=1 after an OUT 31,13 (=2*6+1)

If in 128 mode, contains last OUT to 7ffd

* 36      1       Contains 0FF if Interface I rom paged

* 37      1       Bit 0: 1 if R register emulation on

Bit 1: 1 if LDIR emulation on

* 38      1       Last OUT to fffd (soundchip register number)

* 39      16      Contents of the sound chip registers

55      2       Low T state counter

57      1       Hi T state counter

58      1       Flag byte used by Spectator (QL spec. emulator)

Ignored by Z80 when loading, zero when saving

59      1       0FF if MGT Rom paged

60      1       0FF if Multiface Rom paged. Should always be 0.

61      1       0FF if 0-8191 is ROM, 0 if RAM

62      1       0FF if 8192-16383 is ROM, 0 if RAM

63      10      5x keyboard mappings for user defined joystick

(Default values: 0x0103,0x0203,0x0403,0x0803,0x1003)

73      10      5x ascii word: keys corresponding to mappings above

(Default values: 0x0031,0x0032,0x0033,0x0034,0x0035)

83      1       MGT type: 0=Disciple+Epson,1=Discipls+HP,16=Plus D

84      1       Disciple inhibit button status: 0=out, 0ff=in

85      1       Disciple inhibit flag: 0=rom pageable, 0ff=not

The value of the word at position 30 is 23 for version 2.01 files, and

54 for version 3.0x files.  The starred fields are the ones that

constitute the version 2.01 header, and their interpretation has

remained unchanged except for offset 34 (Hardware mode):

Value:          Meaning in v2.01        Meaning in v3.0x

0               48k                     48k

1               48k + If.1              48k + If.1

2               SamRam                  SamRam

3               128k                    48k + M.G.T.

4               128k + If.1             128k

5               -                       128k + If.1

6               -                       128k + M.G.T.

Fields 30-34, 36-37, 55-82 are valid in all modes.  Field 35 is valid

in mode 2,4,5,6.  Fields 38-54 are valid in modes 4-6.  Fields 83-85

are valid in modes 3 and 6 (all mode codes are as in v3.0x files).

"Invalid" fields are "don't care" for Z80 when loading, and "undefined"

when saving.

The hi T state counter counts up modulo 4.  Just after the ULA

generates its once-in-every-20-ms interrupt, it is 3, and is increased

by one every 5 emulated milliseconds to take the values 0,1,2 and 3

again respectively.  In these 1/200s intervals, the low T state counter

counts down from 17471 to 0 inclusive (or 17726 to 0 in 128K modes),

which make a total of 69888 T states (70908 T states) per frame.

The 5 ascii words (high byte always 0) at 73-82 are the keys

corresponding to the joystick directions left, right, down, up, fire

respectively.  Shift, Symbol Shift, Enter and Space are denoted by

[,],/,\ respectively.  The ascii values are used only to display the

joystick keys; the information in the 5 keyboard mapping words

determine which key is actually pressed (and should correspond to the

ascii values).  The low byte is in the range 0-7 and determines the

keyboard row.  The high byte is a mask byte and determines the column.

Enter for example is stored as 0x0106 (row 6 and column 1) and 'g' as

0x1001 (row 1 and column 4).  The default values correspond to the Left

Sinclair Interface 2 joystick.

Byte 60 must be zero, because the contents of the Multiface RAM is not

saved in the snapshot file.  If the Multiface was paged when the

snapshot was saved, the emulated program will most probably crash when

loaded back.

Bytes 61 and 62 are a function of the other flags, such as byte 34, 59,

60 and 83.

Hereafter a number of memory blocks follow, each containing the

compressed data of a 16K block.  The compression is according to the

old scheme, except for the end-marker, which is now absent.  The

structure of a memory block is:

Offset  Length  Description

0       2       Length of data (without this 3-byte header)

2       1       Page number of block

3       [0]     Compressed data

Starting with version 3.05, a length field of 65535 (-1) means that the

block is not compressed and exactly 16384 bytes long, and the length

field will never hold values larger than 16383.  (Previous versions of

Z80 could produce 'compressed' blocks of more than 16384 bytes, but

would refuse to load those.)

The pages are numbered, depending on the hardware mode, in the

following way:

Page    In '48 mode     In '128 mode    In SamRam mode

0      48K rom         rom (basic)     48K rom

1      Interface I, Disciple or Plus D rom, according to setting

2      -               rom (reset)     samram rom (basic)

3      -               page 0          samram rom (monitor,..)

4      8000-bfff       page 1          Normal 8000-bfff

5      c000-ffff       page 2          Normal c000-ffff

6      -               page 3          Shadow 8000-bfff

7      -               page 4          Shadow c000-ffff

8      4000-7fff       page 5          4000-7fff

9      -               page 6          -

10      -               page 7          -

11      Multiface rom   Multiface rom   -

In 48K mode, pages 4,5 and 8 are saved.  In SamRam mode, pages 4 to 8

are saved.  In '128 mode, all pages from 3 to 10 are saved.  This

version saves the pages in numerical order.  There is no end marker.

This concludes the specification of .Z80 files.  .SLT files (which

stands for 'super level loader trap files') are like .Z80 files except

that after the ordinary data another section follows, containing things

like level data (previously stored in .DAT files, and giving .SLT files

their name) or loading screens.  The ".Z80"-file preceding the SLT must

conform the format of v2.01 or v3.0x files (long header).

Though it is agreed that .Z80 files will not contain SLT data, and

files with SLT data will always have extension .SLT, Z80 does in fact

not distinguish between the extensions.

The SLT format was cooked up by Damien Burke, James McKay and yours

truly.  It starts as an ordinary >= v2.01 .Z80 file.  Directly

following this comes a

Offset  Length  Description

0       6       Separator (0,0,0,'S','L','T')

Then a table follows, each entry describing a piece of data.  The

format of a single table entry is:

Offset  Length  Description

0       2       Data type: 1=level data, 3=loading screen

2       2       Id word: Level number (between 0 and 255 inclusive)

for type 1, border colour (between 0 and 7) for

type 3.

4       4       Length of data block in bytes (Note: long word)

Data types other than type 1 and 3 are not supported by Z80 v3.04, and

are ignored.  The table ends with an all-zero end marker:

Offset  Length  Description

0       8       End marker (all zeroes)

Finally, the data blocks follow.  The blocks are stored in the order in

which they appear in the table,  i.e. the offset of a particular data

block is the sum of the lengths of the blocks that precede it.  The

internal format of these blocks depend on the data type:

Type:   Format:

0       (no data)

1       Compressed data (ED ED xx yy scheme, see above) expanding

to anything up to 48K

3       Compressed data expanding to exactly 6912 bytes of data

See the end of section 5.9 for an explanation how to load these blocks

using the ED FB opcode, and error handling.

All words in the format have the least significant byte first.

KEYMAP

KEYBOARD MATRIX

|  | b0 | b1 | b2 | b3 | b4 |
| --- | --- | --- | --- | --- | --- |
| b0 | Left shift | Z | X | C | V |
| b1 | A | S | D | F | G |
| b2 | Q | W | E | R | T |
| b3 | 1 | 2 | 3 | 4 | 5 |
| b4 | 0 | 9 | 8 | 7 | 6 |
| b5 | P | O | I | U | Y |
| b6 | ENTER | L | K | J | H |
| b7 | SPACE | Right shift | M | N | B |

VARS

| Length | Address (dec) | Address (hex) | Name | Contents |
| --- | --- | --- | --- | --- |
| N8 | 23552 | 5C00 | KSTATE | Used in reading the keyboard. |
| Nl | 23560 | 5C08 | LAST K | Stores newly pressed key. |
| 1 | 23561 | 5C09 | REPDEL | Time (in 50ths of a second in 60ths of a second in N. America) that a key must be held down before it repeats. This starts off at 35, but you can POKE in other values. |
| 1 | 23562 | 5C0A | REPPER | Delay (in 50ths of a second in 60ths of a second in N. America) between successive repeats of a key held down: initially 5. |
| N2 | 23563 | 5C0B | DEFADD | Address of arguments of user defined function if one is being evaluated; otherwise 0. |
| Nl | 23565 | 5C0D | K DATA | Stores 2nd byte of colour controls entered from keyboard. |
| N2 | 23566 | 5C0E | TVDATA | Stores bytes of coiour, AT and TAB controls going to television. |
| X38 | 23568 | 5C10 | STRMS | Addresses of channels attached to streams. |
| 2 | 23606 | 5C36 | CHARS | 256 less than address of character set (which starts with space and carries on to the copyright symbol). Normally in ROM, but you can set up your own in RAM and make CHARS point to it. |
| 1 | 23608 | 5C38 | RASP | Length of warning buzz. |
| 1 | 23609 | 5C39 | PIP | Length of keyboard click. |
| 1 | 23610 | 5C3A | ERR NR | 1 less than the report code. Starts off at 255 (for 1) so PEEK 23610 gives 255. |
| X1 | 23611 | 5C3B | FLAGS | Various flags to control the BASIC system. |
| X1 | 23612 | 5C3C | TV FLAG | Flags associated with the television. |
| X2 | 23613 | 5C3D | ERR SP | Address of item on machine stack to be used as error return. |
| N2 | 23615 | 5C3F | LIST SP | Address of return address from automatic listing. |
| N1 | 23617 | 5C41 | MODE | Specifies K, L, C. E or G cursor. |
| 2 | 23618 | 5C42 | NEWPPC | Line to be jumped to. |
|  | 23620 | 5C44 | NSPPC | Statement number in line to be jumped to. Poking first NEWPPC and then NSPPC forces a jump to a specified statement in a line. |
| 2 | 23621 | 5C45 | PPC | Line number of statement currently being executed. |
| 1 | 23623 | 5C47 | SUBPPC | Number within line of statement being executed. |
| 1 | 23624 | 5C48 | BORDCR | Border colour * 8; also contains the attributes normally used for the lower half of the screen. |
| 2 | 23625 | 5C49 | E PPC | Number of current line (with program cursor). |
| X2 | 23627 | 5C4B | VARS | Address of variables. |
| N2 | 23629 | 5C4D | DEST | Address of variable in assignment. |
| X2 | 23631 | 5C4F | CHANS | Address of channel data. |
| X2 | 23633 | 5C51 | CURCHL | Address of information currently being used for input and output. |
| X2 | 23635 | 5C53 | PROG | Address of BASIC program. |
| X2 | 23637 | 5C55 | NXTLIN | Address of next line in program. |
| X2 | 23639 | 5C57 | DATADD | Address of terminator of last DATA item. |
| X2 | 23641 | 5C59 | E LINE | Address of command being typed in. |
| 2 | 23643 | 5C5B | K CUR | Address of cursor. |
| X2 | 23645 | 5C5D | CH ADD | Address of the next character to be interpreted: the character after the argument of PEEK, or the NEWLINE at the end of a POKE statement. |
| 2 | 23647 | 5C5F | X PTR | Address of the character after the ? marker. |
| X2 | 23649 | 5C61 | WORKSP | Address of temporary work space. |
| X2 | 23651 | 5C63 | STKBOT | Address of bottom of calculator stack. |
| X2 | 23653 | 5C65 | STKEND | Address of start of spare space. |
| N1 | 23655 | 5C67 | BREG | Calculator's b register. |
| N2 | 23656 | 5C68 | MEM | Address of area used for calculator's memory. (Usually MEMBOT, but not always.) |
|  | 23658 | 5C6A | FLAGS2 | More flags. |
| X1 | 23659 | 5C6B | DF SZ | The number of lines (including one blank line) in the lower part of the screen. |
| 2 | 23660 | 5C6C | S TOP | The number of the top program line in automatic listings. |
| 2 | 23662 | 5C6E | OLDPPC | Line number to which CONTINUE jumps. |
|  | 23664 | 5C70 | OSPCC | Number within line of statement to which CONTINUE jumps. |
| N1 | 23665 | 5C71 | FLAGX | Various flags. |
| N2 | 23666 | 5C72 | STRLEN | Length of string type destination in assignment. |
| N2 | 23668 | 5C74 | T ADDR | Address of next item in syntax table (very unlikely to be useful). |
| 2 | 23670 | 5C76 | SEED | The seed for RND. This is the variable that is set by RANDOMIZE. |
| 3 | 23672 | 5C78 | FRAMES | 3 byte (least significant first), frame counter. Incremented every 20ms. See Chapter 18. |
| 2 | 23675 | 5C7B | UDG | Address of 1st user defined graphic You can change this for instance to save space by having fewer user defined graphics. |
| 1 | 23677 | 5C7D | COORDS | x-coordinate of last point plotted. |
| 1 | 23678 | 5C7E |  | y-coordinate of last point plotted. |
| 1 | 23679 | 5C7F | P POSN | 33 column number of printer position |
| 1 | 23680 | 5C80 | PR CC | Less significant byte of address of next position for LPRINT to print at (in printer buffer). |
| 1 | 23681 | 5C81 |  | Not used. |
| 2 | 23682 | 5C82 | ECHO E | 33 column number and 24 line number (in lower half) of end of input buffer. |
| 2 | 23684 | 5C84 | DF CC | Address in display file of PRINT position. |
| 2 | 23686 | 5C86 | DFCCL | Like DF CC for lower part of screen. |
| X1 | 23688 | 5C88 | S POSN | 33 column number for PRINT position |
| X1 | 23689 | 5C89 |  | 24 line number for PRINT position. |
| X2 | 23690 | 5C8A | SPOSNL | Like S POSN for lower part |
|  | 23692 | 5C8C | SCR CT | Counts scrolls: it is always 1 more than the number of scrolls that will be done before stopping with scroll? If you keep poking this with a number bigger than 1 (say 255), the screen will scroll on and on without asking you. |
| 1 | 23693 | 5C8D | ATTR P | Permanent current colours, etc (as set up by colour statements). |
| 1 | 23694 | 5C8E | MASK P | Used for transparent colours, etc. Any bit that is 1 shows that the corresponding attribute bit is taken not from ATTR P, but from what is already on the screen. |
| N1 | 23695 | 5C8F | ATTR T | Temporary current colours, etc (as set up by colour items). |
| N1 | 23696 | 5C90 | MASK T | Like MASK P, but temporary. |
|  | 23697 | 5C91 | P FLAG | More flags. |
| N30 | 23698 | 5C92 | MEMBOT | Calculator's memory area; used to store numbers that cannot conveniently be put on the calculator stack. |
| 2 | 23728 | 5CB0 |  | Not used. |
| 2 | 23730 | 5CB2 | RAMTOP | Address of last byte of BASIC system area. |
| 2 | 23732 | 5CB4 | P RAMT | Address of last byte of physical RAM. |