TECHNICAL INFO

Memory Map

| Start Address | End Address | Usage |
| --- | --- | --- |
| 0x0000 | 0x1fff | ROM area |
| 0x2000 | 0x3fff | Shadow of ROM area |
| 0x4000 | 0xffff | 48k of RAM; at startup, the ZX81 at most 16k of RAM to BASIC; the remaining 32k is kept free for data storage; more can be allocated to BASIC by moving(system variable at 4004h) upwards. |
| 0x4000 | 0x407c | system variables |
| 0x407d | (0x400c)-1 | BASIC program area |
| (0x400c) | (0x4010)-1 | display file (text mode!) |
| (0x4010) | (0x4014)-1 | BASIC variables area |
| (0x4014) | (0x401c)-1 | work space |
| (0x401c) | (0x4004)-1 | free memory available for BASIC; this area cleared by NEW; the machine code stack grows down from the top address of this area |
| (0x4004) | 0xffff | free memory that cannot be used by BASIC and that is also not influenced by NEW, useful for resident utilities or RAMdisk; provided (0x4004) is above 0x7fff, this area is also not altered by RST 0. |

Note : (addr) = PEEK (addr) + 256 * PEEK (addr+1)

| * | Register I points to the ZX81 character set. Default value is I=$1E which means that the character set starts at address $1E00 (last half K of ROM). If you want to change the characterset, first POKE the new set somewhere in memory (at an address which is a multiple of 512 bytes) and then alter register I accordingly (so: only even values for I are allowed). This will instantaneously change the display. The reversed sequence of this procedure will not work the way you would expect! I advise not to change the character set present in ROM. |
| --- | --- |
| * | According to docs, IX is used to hold the NMI. (IX is only set in the ROM via a pop(ix) at 0x292!!) |
| * | IM 1 seems to be the set interrupt mode |

INPUT PORTS

| Address | Usage |
| --- | --- |
| 0xfe | Keyboard (high byte defines which half-row to read). Also turns off HSYNC generation if NMI is disabled. |

OUTPUT PORTS

| Address | Usage |
| --- | --- |
| 0xfd | Turns off NMI generator |
| 0xfe | Turns on NMI generator |
| 0xff | Turns on HSYNC generation |
| 0xfb | Printer |

TS1000 .IMG SNAPSHOT FORMAT

The 16410-byte snapshot is broken down as follows:

- 16384 bytes representing the RAM contents from 0x4000 to 0x7fff

- 26 bytes representing the registers in the following (LSB) order:

AF, BC, DE, HL, IX, IY, SP, PC, AF', BC', DE', HL', I, R.

DISPLAY

ZX81 VIDEO DISPLAY SYSTEM

A tutorial by wilf rigter

last revision 7 Sept 1996

____________________________________________________________________________

INDEX

1.  INTRODUCTION

2.  ZX81 DISPLAY BASICS

3.  SLOW MODE VIDEO

4.  FAST MODE VIDEO

5.  ZX81 VIDEO HARDWARE

6.  ZX81 CHARACTER VIDEO HARDWARE

7.  PSEUDO HIRES VIDEO HARDWARE

8.  TRUE HIRES VIDEO HARDWARE

9.  ZX81 CHARACTER DISPLAY TIMING

10. PSEUDO HIRES DISPALY TIMING

11. TRUE HIRES DISPLAY TIMING

12. ZX81 SLOW MODE VIDEO ROUTINES

13. ZX81 FAST MODE VIDEO ROUTINES

14. TRUE HIRES VIDEO ROUTINES

15. PSEUDO HIRES VIDEO ROUTINES

____________________________________________________________________________

1.INTRODUCTION

**************

When circumstances combine innovative technical ideas with an economical design

and market opportunity some interesting things begin to happen. In 1980,

Sinclair was not yet a household word and was perhaps better known for his

digital watch and calculator than his ZX80 personal computer.

But Sinclair decided the time had come for an affordable and easy to use mass

produced version of the ZX80 with floating point math and a non-flicker

display.

The ZX81 was born and as they say "the rest is history".

A key to the economical design of the ZX81 was the video system. Not only was

it cheap to manufacture, but the ZX81 video circuit turned out to be versatile

with capabilities well beyond the designer's original goals.

2. ZX81 DISPLAY BASICS

**********************

The standard ZX81 video screen displays 24 rows of 32 characters. Every

character has height of 8 scan lines and a width of 8 pixels. The characters

to be displayed are located in a block of memory called DFILE. The set of 128

displayable characters includes 64 normal (white on black) uppercase only

letters, numbers, symbols and graphics characters and their inverse (black on

white). The ZX81 character codes CHR$ 0-63, CHR$118 and CHR$ 128-191, are

non-standard (not ASCII). A set of token codes is also used for keywords,

functions and commands but these are always expanded to the displayable

characters before printing to DFILE. The DFILE is formatted starting with the

Sinclair equivalent of a Carriage Return (CHR$ 118) followed by up to 32 CHR$

codes, this repeated 24 times and ending with a CHR$ 118. CHR$ 118 is the

opcode for the Z80 HALT instruction for reasons which will be explained later.

All other character codes are illegal and if loaded into DFILE will generallly

cause a system crash. The collapsed DFILE is used in the 1K and 2K basic ZX81

to minimize screen memory requirements. When empty a collapsed DFILE consists

of just 25 CHR$ 118 codes. Each line is expanded when characters are printed

to that line. When equipped with 4K or more of memory, DFILE is initialized to

the fully expanded format with 24 lines of 32 CHR$ 00 (space) characters and

25 CHR$ 118 line termination characters.

The character codes are not displayed directly but rather are used as address

pointers to a ROM video pattern table. The ROM pattern bytes are addressed by

a combination of the character code in DFILE and the ZX81 hardware and

is loaded into the video shiftregister. Bit 7 of the character code is used by

the video hardware to invert the pixels as they are shifted out of the

shift register. The display on the screen is generated by the serial bit stream

of pixels a video shift register which turns the TV CRT electron beam on and

off as it scans the phosphor coating on the inside face of the picture tube.

A fully expanded DFILE with 24 lines of 32 characters per row and 8 pattern

bytes per character displays 6144 pattern bytes or 49152 pixels per screen.

3. SLOW MODE VIDEO

******************

In the SLOW mode, the CPU is multitasking between video and program execution.

About 80% of the CPU time allocated to video and keyboard service routines and

only about 20 % of CPU time is available to execute the application program.

In fact, the CPU time is divided in four distinct task blocks per TV frame

as shown in TABLE 1. Tasks are switched using a Non Maskable Interupt (NMI)

generator to call a NMI service routine which controls task switching from

the asynchronous application program to the realtime video routines.

________________________________________________

|                                                |

|  1.  VSYNC, frame count and keyboard - NMI off |

|  2.  Blank lines/application code    - NMI on  |

|  3.  VIDEO DISPLAY routine           - NMI off |

|  4.  Blank lines/application code    - NMI on  |

|________________________________________________|

TABLE 1 SLOW MODE CPU TASK TABLE

Each task can be described in more detail as follows:

1. During the vertical sync interval, when no video is actually displayed, the

CPU executes a fixed length VSYNC routine which increments a FRAME counter,

reads 8 rows of keyboard data together with the 50/60Hz mode bit. Any I/O read

operation with A0 low (ie FE) addresses the ULA keyboard port. It also causes

the ULA to start the vertical sync pulse by clamping the video output to the 0V

sync level and simultaneously applies a reset to the ULA 3 bit line counter

(LCNTR). After the all the keyboard data is processed (400us later), the CPU

executes an OUT FF,A (any OUT will do) which restores the ULA video output to

the normal "white level with horizontal sync pulses" and releases the LCNTR

reset. At the end of the VSYNC routine, the number of blank lines to the start

of the live display are determined from the system variable MARGIN (50/60HZ).

Then the NMI generator is turned on and the CPU registers are switched back to

the application task.

2. While the CPU executes the application code, the CPU is interrupted every

64 us by the NMI generator at the same time the ULA generates a horizontal sync

pulse. The NMI routine increments a blank line counter in A' and returns if

there is more time left for application code excecution. When the blank line

counter is incremented to zero the NMI routine turns off the NMI generator

and switches to the VIDEO DISPLAY routine through a pointer in the IX register.

3. The video display routine sets up the display file pointer, the row and line

counters, enables INT and JP(HL) to the start of DFILE + 32K. Each

character in the DFILE is interpreted as a NOP instruction except the N/L

character which terminates the line. At the end of each line, the INT service

routine updates the row and line counters and returns to execute the

remaining lines. After 192 lines, the video display routine ends by turning on

the NMI generator and CPU switches back to execute application code.

4. As before, during the top blank lines, the NMI routine counts the number of

blank lines remaining. At the end of the bottom blank lines, the sequence

repeats when the NMI service routine switches back to the VSYNC routine.

4. FAST MODE VIDEO

******************

In the ZX80 compatable FAST mode, the CPU executes either the video routine or

any other program but not both which causes the familiar flicker of the display

when switching between these tasks. When the application program is running, it

is executed using 100% of the available CPU time. Only if the application

program is STOPped (in the command mode) or waiting for a keyboard INPUT, or in

PAUSE mode is the video is displayed. The video hardware is activated in the

same way as the SLOW mode but NMI is always off. In addition, the blank lines

at the top and bottom of the screen are also generated in software making the

ZX81 ROM fully compatable with the ZX80 hardware.

5. ZX81 VIDEO HARDWARE

**********************

The ZX81 video hardware consists of the Z80 CPU, ROM, RAM and the

larger part of the ZX81 Sinclair Logic Chip (usually called the ULA)

as shown in FIG 1 with all relevant connections including the

isolation resistors R. For simplicity only the 2K RAM is shown.

The ULA contains a 6.5 MHz crystal oscillator and a frequency divider which

generates horizontal sync pulses at the video output and NMI pulses on

the NMI output. The HSYNC and the NMI outputs can be controlled with the

following I/O operations.

1. OUT FD,A - turns off the NMI generator

2. OUT FE,A - turns on  the NMI generator

3. IN A,FE  - turns off the HSYNC generator (only if NMI is off)

4. OUT FF,A - turns on  the HSYNC generator

The ULA video output switches between 3 voltage levels. It is normally at the

+5V white level for blank lines. Characters patterns are displayed as black

pixels when the level is +2.5V . The narrow horizontal sync pulses and

wide vertical sync pulses are  0V level as shown in FIG 1 waveform. These

logic levels are reduced with a resistor divider to 1V, 0.5V and 0V (UK/US)

at the input of the TV RF modulator.

white _  _____    _____    _________//__________//____             ____//_____

black _   |   |__|     |__|   |             |         |<--400us-->|          |

sync  _   |<-------64us------>|<---64us---->|         |_____//____|          |

display line         blank        blank   vert sync     blank

FIG 1 - VIDEO LEVELS

The HSYNC pulses are 5 usec wide with 64 usec between HSYNC pulses.

The VSYNC is 400 usec wide with 16.6 msec or 20 msec between VSYNC pulses.

VSYNC is used to synchronize the TV vertical oscillator and start the raster

scan at the top of the screen. This occurs when IN A,FE (used for scanning the

keyboard) clamps the video output to the SYNC level. 400us later OUT FF,A

releases SYNC to enable the 64 us HSYNC pulses. The HSYNC pulses continue to

be generated independ of the CPU until the next VSYNC.

The CPU executes the application code during the blank lines at the top and

bottom of the screen while the NMI generator interrupts the CPU every 64 us

and increments a blank line counter to determine if it is time for the VIDEO

DISPLAY of VSYNC routines.

6. ZX81 CHARACTER VIDEO HARDWARE

*******************************

The Sinclair ZX81 character display generator consists of the Z80, ROM, RAM

and the larger part of the ZX81 Sinclair Logic Chip (usually called the ULA)

as shown in FIG 2 with all relevant connections including the isolation

resistors R. For simplicity only the 2K RAM is shown.

ULA               ROM        Z80        2K RAM

________________       _____       _____       _____

VIDEO<-| VSHFTREG <-DATA|-----|DATA |-----|DATA |--R--|DATA |

|  LINECTR ->A0-2|-----|A0-2 |--R--|A0-2 |-----|A0-2 |

| CHRLATCH ->A3-8|-----|A3-8 |--R--|A3-8 |-----|A3-8 |

|                |     |A9-12|-----|A9-13|-----|A9-11|

|           ROMCS|-----|CE   |     |  INT|-----|A6   |

|                |     |_____|     |     |     |     |

|           RAMCS|-----------------|-----|-----|CE/OE|

|             A14|-----------------|A14  |     |_____|

|             A15|-----------------|A15  |

|              WR|-----------------|WR   |

|              RD|-----------------|RD   |

|              M1|-----------------|M1   |

|            MREQ|-----------------|MREQ |

|            IORQ|-----------------|IORQ |

|             NMI|-----------------|NMI  |

|            HALT|-----------------|HALT |

|________________|                 |_____|

FIG 2 ZX81 CHARACTER VIDEO DISPLAY CIRCUIT

7. PSEUDO HIRES VIDEO HARDWARE

******************************

The pseudo hires graphics video display generator consists of the Z80 CPU,

ROM, RAM and a large part of the ZX81 Sinclair Logic Chip (usually called

the ULA) as shown in FIG 2 with all relevant connections including the

isolation resistors R. For simplicity only the 2K RAM is shown.

ULA               ROM        Z80        2K RAM

________________       _____       _____       _____

VIDEO<-| VSHFTREG <-DATA|-----|DATA |-----|DATA |--R--|DATA |

| *LINECTR ->A0-2|-----|A0-2 |--R--|A0-2 |-----|A0-2 |

| CHRLATCH ->A3-8|-----|A3-8 |--R--|A3-8 |-----|A3-8 |

|                |     |A9-12|-----|A9-13|-----|A9-11|

|           ROMCS|-----|CE   |     | *INT|-----|A6   |

|                |     |_____|     |     |     |     |

|           RAMCS|-----------------|-----|-----|CE/OE|

|             A14|-----------------|A14  |     |_____|

|             A15|-----------------|A15  |

|              WR|-----------------|WR   |

|              RD|-----------------|RD   |

|              M1|-----------------|M1   |

|            MREQ|-----------------|MREQ |

|            IORQ|-----------------|IORQ |

|             NMI|-----------------|NMI  |

|            HALT|-----------------|HALT |

|________________|                 |_____|

FIG 3 PSEUDO HIRES GRAPHICS DISPLAY CIRCUIT

The only difference between pseudo hiress and Sinclair character

hardware is the *ULA LCNTR and the use of the  *INT input.

Most pseudo hires core routines do not use INT and the ULA LCNTR is

reset to zero every horizontal line. The exception is XTRICATOR which

uses INT and makes dual use of the I register in INT mode 2 as a part of

the RST vector address when interupted at the end of each horizontal line

and at refresh time as a ROM pattern table pointer.

8. TRUE HIRES VIDEO HARDWARE

****************************

The portion of the ZX81 hardware required for true hires graphics

display consists of the Z80 CPU, the RAM, the video shift register and the

sync circuit of the ULA as shown in FIG 4 with all relevant connections.

Again the 2K SRAM is shown for simplicity but applies to larger SRAM

designs as well. If a 16K RAMPACK is used, this must be slightly

modified, as will be shown later, to enable the data outputs during RFSH

time as required for this hires display method.

ULA               ROM         Z80       2K RAM

________________       _____       _____       _____

VIDEO<-| VSHFTREG <-DATA|-----|DATA |-----|DATA |--R--|DATA |

|                |     |A0-12|-----|A0-15|-----|A0-10|

|           ROMCS|-----|CE   |     |     |     |     |

|                |     |_____|     |     |     |     |

|           RAMCE|-----------------|-----|-----|CE/OE|

|             A14|-----------------|A14  |     |_____|

|             A15|-----------------|A15  |

|              WR|-----------------|WR   |

|              RD|-----------------|RD   |

|              M1|-----------------|M1   |

|            MREQ|-----------------|MREQ |

|            IORQ|-----------------|IORQ |

|             NMI|-----------------|NMI  |

|            HALT|-----------------|HALT |

|________________|                 |_____|

FIG 5 TRUE HIRES GRAPHICS VIDEO DISPLAY CIRCUIT

With the exception of WRX1K which creates a miniature hires screen on a

1K ZX81 all hires programs need a 6K hires graphics file (HFILE).

Suitable RAM for true hires graphics can be implimented by modifying

a standard 16K RAMPACK with a couple of diodes and a resistor.

The RAMPACK is modified to enable the data output at RFSH time by cutting

the RD and RFSH lines at the edge connector and installing 2 only 1N34A

Germanium diodes and a 4.7K pullup resistor. Modify at your own risk!

+5V

|

[4.7K] 					(Figure 5)

1N34A   |

connector   RD _______|/|____|_____________ RD OF RAMPACK

|\|    |

|

connector RFSH _______|/|____|     +5V_____ RFSH of RAMPACK

|\|

1N34A

9. ZX81 CHARACTER DISPLAY TIMING

********************************

All the Sinclair ZX81 character display hardware shown in FIG 2 is

required to generate a standard screen of 24 lines of 32 characters.

The character display starts when the last blank line at the top of the screen

has occurred and and the video routine jumps to the DFILE echo above 32K.

The hardware in the ZX81 ULA takes control when any opcode is executed above

32K (A15 high and M1 low) with data bit 6 equal to zero. The video data

is loaded in these simplified steps:

1.The ULA loads the character code into a address register in the ULA

2.The ULA forces the data lines low.

3. The CPU interprets the byte as a NOP.

4. The ULA generates part of the ROM pattern table address and the

Z80 CPU generates the pattern table pointer with the I register.

5. The pattern byte is loaded into the ULA shift register.

One could say that the Dfile is literally executed with NOPs substituted for

each character code. Each NOP executes in 4 CPU clock cycles at 3.25 MHz or

8 pixels at 6.5MHz from the ULA video shift register.

<--------CHARACTER 1-----------><--------CHARACTER 2----------->

T STATE <--T1--><--T2--><--T3--><--T4--><--T1--><--T2--><--T3--><--T4-->

(ref)  ___    1___2   3___     ___5    ___     ___     ___     ___

CPU CLOCK |   |___|   |___|   |___|   |___|   |___|   |___|   |___|   |___|

_ _______________ _______________ _______________ _______________

A0-A15 _X_____PC________X___I+CHR+ULA___X______PC_______X___I+CHR+ULA___X

________  NOP  __________       ________  NOP  __________

DATA >---|__CHR___|_____|_ROM DATA_|-----|__CHR___|_____|_ROM DATA_|--

FIG 6 ZX81 CHARACTER DISPLAY TIMING

The detailed sequence of operations for each character byte is shown

in FIG 6 and described as follows:

1. Each character code (CHR$) byte in DFILE is addressed by the CPU PC, on the

rising edge T2 data is loaded from DFILE into the ULA : bits 0-5 into a 6

bit

ULA address latch while bit 7 is loaded into 1 bit ULA video invert latch

2. On the falling edge of T2, the ULA forces all CPU data lines to zero.

3. On the rising edge of T3 the low data lines are interpreted by the CPU

as a NOP instruction.

4. During T3/4, the CPU executes the Refresh cycle and ROM address lines are

generated with I register on A9-A15, the ULA 6 bit character code register

on A3-A8, and the ULA modulo 8 line counter on line A0-A2.

5. On the falling edge of T4, pattern data from the ROM is loaded into

ULA video shift register and 8 video pixels are shifted out at 6.5MHz

6. If character code bit 7 latch in ULA equals 1, video pixels are inverted.

7. The CPU increments the program counter and fetches the next character code.

8. This repeats until a HALT (Sinclair) is fetched.

9. HALT opcode bit 6 = 1 and is therefore executed (no NOP)

10.The ULA generates a HSYNC pulse independend of the CPU timing and the

ULA LCNTR is incremented

11.The halted CPU continues to execute NOPs, incrementing register R and

samples the INT input on the rising edge of each T4.

12.When A6, which is hardwired to INT, goes low during refresh time,

(bit 6 of the R reg = 0), the Z80 executes the INT routine (below 32K)

13.CPU returns from INT and resumes "excution" of DFILE CHR$ codes.

14.The process repeats 192 times and then INT routine returns to the main

video routine, turns on the NMI generator and switches back to the

application code.

10. PSEUDO HIRES DISPLAY TIMING

*******************************

All the ZX81 character display hardware shown in FIG 2 with some exceptions

is required to generate a standard screen of 192 lines of 32 pseudo hires

patterns. The display starts when the last blank line at the top of the

screen has occurred and and the video routine jumps to the 6K DFILE echo

above 32K. The hardware in the ZX81 ULA takes control when any opcode is

executed above 32K (A15 high and M1 low) with data bit 6 equal to zero.

The video data is loaded in five steps:

1. The ULA loads the character code into an address register

2. The ULA forces the data lines low.

3. The CPU interprets the byte as a NOP.

4. The ULA generates part of the ROM pattern table address. The CPU generates

the pattern table MSB address with the I register.

5. The quasi hires pattern byte is loaded into the ULA shift register.

Each NOP executes in 4 CPU clock cycles at 3.25 MHz or 8 pixels at 6.5MHz

from the ULA video shift register.

<--------CHARACTER 1-----------><--------CHARACTER 2----------->

T STATE <--T1--><--T2--><--T3--><--T4--><--T1--><--T2--><--T3--><--T4-->

(ref)  ___    1___2   3___     ___5    ___     ___     ___     ___

CPU CLOCK |   |___|   |___|   |___|   |___|   |___|   |___|   |___|   |___|

_ _______________ _______________ _______________ _______________

A0-A15 _X_____PC________X___I+CHR+ULA___X______PC_______X___I+CHR+ULA___X

________  NOP  __________       ________  NOP  __________

DATA >---|__CHR___|_____|_ROM DATA_|-----|__CHR___|_____|_ROM DATA_|--

FIG 7 SINCLAIR CHARACTER DISPLAY TIMING

The detailed sequence of operations for each character byte is as follows:

1. Each character code (CHR$) byte in DFILE is addressed by the CPU PC, on the

rising edge T2 data is loaded from DFILE into the ULA : bits 0-5 into a 6

bit ULA address latch while bit 7 is loaded into 1 bit ULA video invert latch

2. On the falling edge of T2, the ULA forces all CPU data lines to zero.

3. On the rising edge of T3 the low data lines are interpreted by the CPU

as a NOP instruction.

4. During T3/4, the CPU executes the Refresh cycle and ROM address lines are

generated with I register on A9-A15, the ULA 6 bit character code register

on A3-A8, and zero on line A0-A2.

5. On the falling edge of T4, pattern data from the ROM is loaded into

ULA video shift register and 8 video pixels are shifted out at 6.5MHz

6. If character code bit 7 latch in ULA equals 1, video pixels are inverted.

7. The CPU increments the program counter and fetches the next character code.

8. This repeats until a RET fetched which returns to the hires routine.

9. RET opcode bit 6 = 1 and is therefore executed (no NOP)

10.The ULA generates a HSYNC pulse independend of the CPU timing and the

ULA LCNTR is incremented  but the video software resets the LCNTR to zero.

11.CPU returns from the hires routine and resumes "excution" of DFILE CHR$

codes.

12.The process repeats 192 times and then hires routine ends by turning on the

NMI generator and by switching back to the application code.

11. TRUE HIRES DISPLAY TIMING

*****************************

Although I will use the WRX hires core routine as an example, the true hires

software developed independently by others is very similar. The true hires

display starts when the last blank line at the top of the screen has occurred,

and NMI jumps via IX to the HR video routine. HR sets up the I and R

register pointers to the hires display file (HFILE), then the HR routine jumps

to the LBUF routine echo above 32K and loads register R and points to the first

NOP opcode. The ULA loads the video shift register when any opcode is executed

with A15 high and M1 low and data bit 6 equal to zero.

The hires data is loaded in 3 steps:

1. The CPU executes each of the 4T state NOP instructions.

2. During T3/4 (refresh), the I and R registers appear on the A0-15 lines.

3. The hires byte addressed by I and R is loaded into the ULA shift register.

LBUF consists 32 NOP opcodes, each executing in 4 CPU clock cycles.

During the second part of the opcode excution, the I and R register address the

hires byte in HFILE which the ULA loads into the video shift register.

The ULA generated ROM address for the pattern table is not used since ROMCS is

not enabled as register I (A8-15) is set up to point to the RAM based HFILE.

<---------HIRESBYTE1-----------><---------HIRESBYTE2----------->

T STATE <--T1--><--T2--><--T3--><--T4--><--T1--><--T2--><--T3--><--T4-->

___     ___     ___     ___     ___     ___     ___     ___

CPU CLOCK |   |___|   |___|   |___|   |___|   |___|   |___|   |___|   |___|

_ _______________ _______________ _______________ _______________

A0-A15 _X______PC_______X______I_+_R____X______PC_______X______I_+_R____X

____________    _________       ____________    _________

DATA >---|_____NOP____|--|__HIRES__|-----|_____NOP____|--|__HIRES__|--

FIG 8 TRUE HIRES DISPLAY TIMING

The detailed sequence of operations for each HIRES byte is as follows:

1. The first opcode of the LBUF routine, LD R,A is executed.

2. The following 32 NOPs in LBUF are sequentially executed.

3. On the rising edge of T3 of each instruction fetch, the CPU executes the

NOP.

4. During T3/4, the address is generated with the R register on A0-7 and the I

register on A8-A15.

5. On the falling edge of T4, a hires data byte from HFILE is loaded into

ULA video shift register and 8 video pixels are shifted out at 6.5MHz

6. The CPU increments the program counter and the R register and fetches the

next NOP and the next hires byte .

7. This process repeats 32 times.

8. The last opcode of LBUF, JP (IX), is executed to return to the HR routine .

9. The ULA generates a horizontal syncpulse.

10.The HLINE routine increment the I/R register pair by 32 and jumps back to

the 32 NOP LBUF routine echo above 32K.

11.The process repeats 192 times.

12.The HIRES video routine may call the Sinclair character routine for the

bottom line and restores the registers etc to return to the application

code.

Like the other hires routines WRX intercepts the Sinclair video by loading a

new  video routine vector in the IX register.

12. ZX81 SLOW MODE VIDEO ROUTINES

*********************************

As shown in TABLE 1 the CPU is multitasking between the video routines

and the application program in 4 blocks of time.

1. VSYNC INTERVAL ROUTINES

0229 DISPLAY-1 DECREMENT FRAME COUNTER

023E DISPLAY-2 CHECK KEYBOARD (START OF VSYNC)

0277 OUT FF,A  ENDS THE VSYNC PULSE

0292 DISPLAY-3 SAVE THE VIDEO POINTER IN IX (0281), RETURN TO APPLICATION

2. APPLICATION PROGRAM

0066 NMI       COUNTS BLANK LINES, RETURN TO APPLICATION OR VIA JP (IX) TO 0281

3. DFILE DISPLAY ROUTINES

0281 VIDEO-1   SETUP DISPLAY PARAMETERS, CALL 2B5 (RETURN VIA INT)

02B5 DISPLAY-5 SETUP DISPLAY PARAMETERS, ENABLE INTERRUPT, JP (DFILE)

XXXX (DFILE)   EXECUTES HALTs AND FORCED NOPs IN DFILE

0038 INT       DECREMENTS ROW/LINES COUNTERS RETURN TO DFILE OR TO 028B

0292 DISPLAY-3 SAVE VIDEO POINTER (028F)

4. APPLICATION PROGRAM

0066 NMI       COUNTS BLANK LINES, RETURN TO APPLICATION OR VIA JP (IX) TO 028F

028F VIDEO-2 JP 229  BACK TO FRAME COUNTER in BLOCK 1

The ZX81 video routines follow in fully anotated listings showing more details

than IAN LOGAN ZX81 DISASSEMBLY. Needless to say, I used Dr. LOGAN's book

extensively during my research into the ZX81 video code.

NOTE: Only code which is relevant to video is shown here.

0038               ;INT SERVICE ROUTINE

DEC C         ;decrement the scan line counter in register C

JP NZ 0045    ;go SCAN-LINE : repeats 8 times for each DFILE character row

POP HL        ;point to the start of next DFILE row

DEC B         ;decrement the ROW counter in register B

RET Z         ;return to 028B

SET 3,C       ;load scan line counter in register C with 08 scan lines

0041               ;WAIT-INT

LD R,A        ;load value DD into register R

EI            ;enable INT

JP (HL)       ;execute the NOPs in DFILE

0045               ;SCAN-LINE

POP DE        ;discard the return address

RET Z         ;delay (never returns)

JR 0041       ;got WAIT-INT

;----------------------------------------------------------

0066               ;NMI SERVICE ROUTINE

;Interupts application program every 64 usec (HSYNC)

EX AF,AF'     ;retrieve blank line counter in AF'

INC A         ;next blank line

JP M 006D     ;RETURN via 006D if AF' = FF (to NMI-EXIT)

JR Z 006F     ;JR NMI-CONT if last line

006D EX AF,AF'     ;save blank line counter

RET           ;return to application or NMI-EXIT

006F               ;NMI-CONT

EX AF,AF'     ;retrieve main register AF

PUSH AF       ;now save the application program registers

PUSH BC

PUSH DE

PUSH HL

LD HL,(DFILE) ;needed only if IX=0281 and

SET 7,H       ;if DFILE is executed

HALT          ;1T state synchronization: this HALT is used with special

;hardware connected to the CPU WAIT and HALT lines and is

;released and synchronized on the falling of the NMI pulse

007A               ;NMI-EXIT

OUT FD,A      ;turn off NMI generator

JP (IX)       ;to VIDEO-1 or VIDEO-2

;---------------------------------------------------------

0229               ;DISPLAY-1

LD HL,(FRAMES);get the system variable FRAMES

DEC HL        ;decrement each frame

.....

LD (FRAMES),HL;save the system variable FRAMES

023E               ;DISPLAY-2

CALL 02BB     ;read the keyboard and load MARGIN with blank lines

.....         ;also starts the VSYNC pulse

0277 OUT FF,A      ;stops the VSYNC pulse

LD HL,(DFILE) ;(FAST VIDEO only) - point HL to first HALT for blank lines

SET 7,H       ;(FAST VIDEO only) - DFILE echo above 32K

027E CALL 0292

0281               ;VIDEO-1

;this vector is saved in register IX at 0292

LD A,R        ;delay

LD BC,1901    ;set up INT parameters for first HALT at (DFILE)

LD A,F5       ;set up R register for first HALT at (DFILE)

CALL 2B5      ;continue setup for DFILE display and return via INT

028B               ;return here from last INT

DEC HL        ;(FAST VIDEO only) - point HL to last HALT for blank lines

CALL 292      ;save VIDEO vector in IX, calculate blank lines, POP regs

028F JP 0229       ;VIDEO-2

;this vector is saved in register IX at 0292

0292               ;DISPLAY-3

POP IX        ;IX=0281 or 028F to vestor to VIDEO-1 or VIDEO-2

LD C,(IY+56)  ;load number of blank lines from MARGIN (1F in 60 Hz option)

BIT 7,(IY+59) ;test FAST/SLOW bit

JR Z,2A9      ;(FAST VIDEO)  branches to generate blank lines

LD A,C        ;C=(MARGIN)=1F for 60 Hz

NEG           ;

INC A         ;

EX AF,AF'     ;during NMI @ 0066 - AF' is incremented and tested for zero

OUT (FE),A    ;turn on NMI generator

POP HL        ;self explanatory

POP DE

POP BC

POP AF

RET           ;return to application program interupted every HSYNC by NMI

;-----------------------------------------------------------

02B5               ;DISPLAY-5

LD R,A        ;R increments with each opcode on A0 to A7 during RFSH

;until A6 goes low which generates the INT signal.

LD A,DD       ;Set the left margin of all other lines, load to R at 0041

EI            ;Now that R is set up enable INT

JP (HL)       ;"executes" the DFILE starting with HALT and waits for the

;first INT to come to the rescue.

;-----------------------------------------------------------

13. ZX81 FAST MODE VIDEO ROUTINES

*********************************

In order to speed up application program execution time, the FAST mode

uses 100% of the CPU time for executing the application program but there

are times when the application program is idle. When the program is STOPped

or PAUSEd or waiting for keyboard INPUT, the keyboard is checked and if

no key is down the video is generated independent of NMI pulses.

In fact, the ZX81 FAST mode video routine was designed to be compatable with

the ZX80 hardware so the ROM could be used as a retrofit ROM upgrade.

Since the ZX80 generates the blank lines in software, the ZX81 ROM does the

same when in the FAST mode.

The loop of VIDEO routines for FAST video starts with the

FRAME/KBD/VSYNC routine at 229:

0229 DECREMENT FRAME COUNTER

023D EXIT FAST VIDEO IF FRAMES=0 (END OF PAUSE)

023E CHECK KEYBOARD

0260 EXIT FAST VIDEO IF NEW KEY PRESSED

0292 SAVE THE VIDEO POINTER IN IX (0281)

029B JR Z 02A9 TO BLANK LINE ROUTINE

02A9 GENERATE BLANK LINES

02B3 JP (IX) TO  0281

0281 GENERATE THE DFILE DISPLAY

0292 SAVE VIDEO POINTER (028F)

029B JR Z 02A9 TO BLANK LINE ROUTINE

02A9 GENERATE BLANK LINES

02B3 JP (IX) TO 028F)

028F JP 229  BACK TO FRAME COUNTER

Since most of the routines were described in the SLOW MODE VIDEO

chapter, only  the differences are described here. Compare the way the

SLOW mode enters this loop from end of blank line application program

execution by saving the main registers of the program and restoring them

at the end of 0292. By contrast, the FAST mode does not save any registers

and branches out of the 0292 restore main register routine to literally

generate the blank lines. This is done at 029B after testing the FAST flag

and jumping to a less known routine called DISPLAY-4

02A9               ;DISPLAY-4

LD A,FC       ;first R delay to INT

LD B,01       ;one row

CALL 02B5     ;display blank lines

DEC HL        ;point back to HALT

EX (SP),HL    ;delay 19T

EX (SP),HL    ;delay 19T

JP (IX)       ;IX = 0281 or 028F

The routine at 02A9 is called twice each frame to generate the top and bottom

blank lines with HL pointing to either the first HALT at the start of DFILE

or the last HALT at end of DFILE. Reg C holds the number of blank lines and

reg B is set up for 1 row. After VSYNC the 31 top blank lines are generated by

calling the diplay routine at 02B5 and excecuting the first HALT at the START

of DFILE 31 times. After returning from the display routine HL points to the

last HALT+1 and DEC HL is required point HL back to the last HALT of DFILE.

After saving the return address in IX, the routine at 029A is reentered with

HL pointing to the last HALT and generates the bottom 31 blank lines by

excecuting the HALT at the END of DFILE.

14. TRUE HIRES VIDEO SOFTWARE

*****************************

The true hires core routines are distinguished by the use of the I and R

register pair as address pointers for the display file. The only other

requirement is to execute 32 NOP instructions (or equal) per horizontal

line and to update the I and R registers during HSYNC time. More blank

lines can be used above and below the display for faster application

execution. The listings are compatable source code for the ZXAS assembler

both on the ZX81 and under XTender, the ZX81 emulator form CARLO DELHEZ.

Check current version of XTender for hires compatability.

These ASCII listings can be used to prepare a formatted 2 REM .l file with

the ZXAS.COM program from Jack Raats.

WRX16 - 1984 wilf rigter

This is the hires core used in programs by FRED NACHBAUR and GREG HARDER.

It creates a 256x192 high resolution display in a 6144 byte array starting

at (HFILE), which can be poked directly from BASIC programs.

START is used to start the hires display and STOP restores the SINCLAIR

video. It has a characteristic signature with the I register value greater

than 2000 hex.

PART 1 calls LBUF 192 times, displaying 256x192 pixels, calculates blank

lines, saves pointer to PART 2 in IX and returns to application code.

PART 2 calls VSYNC etc, calculates blank lines, saves pointer to PART 2

in IX and returns to application code execution.

;ORIGIN = 16516 (hex 4084)

LBUF  ;Displays one line of 256 pixels

;------------------------------

;like DFILE, it is called above 32K to activate the ULA video

;hardware. The hires bytes may be inverted for special effects

;by setting bit 7 of the NOP codes . The hires data is loaded

;into the ULA video shift register during the refresh cycles of

;the 32 NOP opcodes when the I and R registers sequentially

;address 32 bytes of hires data in the 6144 byte HFILE

LBUF

LD R,A     ;Now load R register

00 00 00 00;32 bytes of 8 pixels

00 00 00 00

00 00 00 00

00 00 00 00

00 00 00 00

00 00 00 00

00 00 00 00

00 00 00 00

JP (IX)    ;Return to HR

HR    ;HIRES DISPLAY ROUTINE PART 1

;----------------------------

LD B,04    ;load delay

HR0   DJNZ HR0   ;delay 56T states to synchronize with HSYNC pulses.

LD HL,(HFILE);RAMTOP points to the first HFILE byte

LD B,C0    ;48 horizontal lines

LD IX,HR1  ;save the return vector in IX (for JP (IX) at end of LBUF)

JR HR2     ;skip HR1 first time through the loop

HR1   LD DE,20   ;this value for 32 bytes or 256 pixels per line is

ADD HL,DE  ;added to HL to point to the start of the next HLINE

DEC B      ;repeat 48 times

JP Z HR3   ;if this is the last line JP to HR3

HR2   LD A,H     ;the address in HL is then transferred to

LD I,A     ;I register

LD A,L     ;and during LBUF to the R register

JP C084    ;jump to LBUF @ 4084 + 8000 to start the ULA

HR3   LD IX,HR4  ;save the video vector so that NMI returns to HR4

JR HR5     ;now get blank lines and return to application code

HR4   ;HIRES DISPLAY ROUTINE PART 2

;----------------------------

CALL 220   ;first PUSH registers then jump to VSYNC, etc

LD IX,WRX16;save the video vector so that NMI returns to HR

HR5   LD A,(4028);33 or 19 blank lines in system variable MARGIN

JP 29E     ;save blank lines, start NMI, POP registers and RETURN

;--------------------- end of listing ---------------------

The hires video is started and stopped by changing the vector address

in register IX which is used by NMI to JP (IX) to the video routine.

The following routines are synchronized with the display so that the

changeover in video mode occurs without display breakup.

STOP  ;STOP hires video and return to SINCLAIR video

;---------------------------------------------

LD HL,0281 ;pointer to SINCLAIR video routine

LD A,1E    ;SINCLAIR ROM pattern table MSB base address (1E00)

LD I,A     ;pointer to I register

JR SYNC

START ;Start the hires video

;---------------------

LD HL,HR   ;pointer to the hires video routine

SYNC  ;used by START and STOP to smoothly change video mode

;----------------------------------------------------

PUSH HL

LD HL,4034 ;FRAMES counter

LD A,HL    ;get old FRAMES

SYNC1 CP A,(HL)  ;compare to new FRAMES

JR Z SYNC1 ;exit after a change is detected

POP IX     ;SINCLAIR video routine

;-------------- END OF LISTING ---------------

GUUS-FLATER by ENNO BORGESTEEDE (1984)

This hires core uses a ingenious way to intercept the video vector.

Instead of changing the value of the IX register, GUUS-FLATER intercepts

at the beginning of the DFILE execution by changing the first 4 bytes

including the HALT to DI and JP 409F which is the start of the hires

routine. At the end of the hires screen the program simply returns to

ROM routine at xxxx. It has a characteristic DFILE starting with the

DI and JP 409F and the HFILE starts at (4004)

(400C)  DI           ;these bytes are loaded into DFILE

JP 409F      ;to vector to the hires routine

409F    LD B,08      ;delay

40A1    DJNZ 40A1    ;delay

LD A,R       ;delay

LD B,C0      ;192 lines

LD DE,20     ;32 bytes per line

LD HL,(4004) ;hires file (HFILE) starts at RAMTOP

40AD    LD A,H       ;MSB address of HFILE

LD I,A       ;load MSB of HFILE pointer

LD A,L       ;LSB address of HFILE

40B3    JP C0B6      ;JUMP above 32K

40B6    LD R,A       ;load LSB of HFILE pointer

40B8    "COPYRIGHT 1984 ENNO BORGESTEEDE " ; same as 32 NOPs

40D8    JP 40DB      ;JUMP below 32K

40DB    ADD HL,DE    ;next hires line

40DC    DJNZ 40AD    ;next line repeats 192 times

40DE    LD A,1E      ;restore ROM pattern table pointer

40E0    LD I,A       ;load pointer

40E2    RET          ;join the SINCLAIR video in progress

HRG7 - in progress

WRX16K - 1996 wilf rigter  <rigter@cafe.net>

The original WRX was written in 1984 but recent renewed interest has yielded

newer more efficient versions. WRX16K 1996 is the most compact version of

the WRX yet and will display a true bit mapped 256x192 hires screen. It was

designed to work with the modified 16K RAMPACK or 16K SRAM and you must

first lower RAMTOP with POKE 16389,96 then NEW before loading.

The hires mode can be started and stopped with the same routines shown in

the WRX16 listing above. The simple START is used for starting the hires

mode by changing video vector address in the IX register.

Hires is stopped with the inline code segment called "BREAK" which returns

synchronously to the Sinclair video mode when the space key is pressed.

The HFILE is a 6K linear array starting at (4004) but is easily relocated.

Note that HFILE must start on a 32 byte boundary (2000,2020, etc).

ORG               ;16516 (hex 4084)

START LD IX,HR    ;simple start of the hres mode

RET

LBUF LD R,A      ;load HFILE address LSB

0 0 0 0     ;32 NOPs = 256 pixels

0 0 0 0

0 0 0 0

0 0 0 0

0 0 0 0

0 0 0 0

0 0 0 0

0 0 0 0

RET NZ      ;always returns

HR

LD B,7      ;delay

HR0   DJNZ HR0    ;delay

DEC B       ;reset Z flag

LD HL,(4004);HFILE starts at RAMTOP or HSCRN (note below)

LD DE,20    ;32 bytes per line

LD B,C0     ;192 lines per hires screen

HR1   LD A,H      ;get HFILE address MSB

LD I,A      ;load MSB into I register

LD A,L      ;get HFILE address LSB

CALL C089   ;CALL LBUF + 8000

ADD HL,DE   ;next line

DEC B       ;dec line counter

JP NZ HR1   ;last line

HR2

CALL 292    ;return to application program

CALL 220    ;extra register PUSH and VSYNC

BREAK             ;this code segment is optional

CALL F46    ;check break key

LD A,1E     ;restore pattern table pointer

LD I,A

JR NC STOP  ;skip the HR vector load if BREAK

LD IX,HR    ;load the HR vector

STOP  JP 2A4      ;return to application program

HSCRN 2000        ;this is used with SRAM at 8K - 16K

Note:

HFILE can be relocated to use SRAM between 8 to 16K by changing

LD HL,(4004) to LD HL,(HSCRN).

------------------ end of listing ------------------

WRX1K -1996 wilf rigter

This little true hires program is special because it runs on a 1K/2K ZX81.

It creates a miniature 64x48 high resolution display in a 384 byte array

starting at (RAMTOP), which can be poked directly from BASIC programs.

When START is called, it collapses the SINCLAIR DFILE and expands

the hires file above RAMTOP in order to efficiently utilize the 1K memory.

When STOP is called, it recovers the space above RAMTOP to make more RAM

avialable for DFILE.

;ORIGIN = 16516 (hex 4084)

LBUF  ;Dummy display file

;------------------

;like DFILE, it is called above 32K to activate the ULA video

;hardware but only bit 7 character code data is used. The hires

;data is loaded into the ULA video shift register during the refresh

;cycles of the 8 NOP opcodes when the I and R registers

;sequentially address 8 bytes of hires data in the 384 byte HFILE

;NOTE: in this special case of short (8 byte) video lines the delay

;opcodes (E3 and 40) have bit 6 high to suppress the video display

;at the start and end of each horizontal line

E3 E3 E3 E3;Delay 76T states

LD R,A     ;Now load R register

LBYTE 00 00 00 00;8 bytes of 8 pixels

00 00 00 00;

40 40 40 40;Delay 20T states

40         ;Delay 4T

JP (IX)    ;Return to HR

START ;Makes room above RAMTOP and starts the hires routine

;----------------------------------------------------

LD IX,HR    ;This is the start of the new video routine

LD BC,180   ;384 bytes are required for a 64X48 display

CALL EC5    ;is there enough room to lower ramtop?

LD HL,(4004);get the old RAMTOP (1K/2K)

CCF         ;calculate the new RAMTOP value by

SBC HL,BC   ;Subtracting the HFILE length

STACK ;used by START and STOP to change RAMTOP without NEW

OUT FD,A    ;Turn off the NMI generator during STACK move

LD (4004),HL;Save RAMTOP value in the sytem variable

DEC HL      ;point to the first byte below RAMTOP

LD (HL),3E  ;and mark it with 3E

DEC HL

LD SP,HL    ;Load the STACK POINTER

DEC HL

DEC HL

LD (4002),HL;Load the ERROR STACK POINTER

OUT FE,A    ;Turn on NMI

*      JP 676      ;resume BASIC program execution at NEXT LINE

HR    ;PART 1 calls LBUF 48 times, displaying 64x48 pixels, calculates blank

;lines, saves pointer to PART 2 in IX,and returns to application code

;PART 2 calls VSYNC etc, checks the BREAK key, calculates blank lines,

;saves pointer to PART 1 in IX and returns to application code.

LD B,04    ;load delay

HR0   DJNZ HR0   ;delay 56T states to synchronize with HSYNC pulses.

LD HL,(4004);RAMTOP points to the first HFILE byte

LD B,30    ;48 horizontal lines

LD IX,HR1  ;save the return vector in IX (for JP (IX) at end of LBUF)

JR HR2     ;skip HR1 first time through the loop

HR1   LD DE,08   ;this value for 8 bytes or 64 pixels per line is

ADD HL,DE  ;added to HL to point to the start of the next HLINE

DEC B      ;repeat 48 times

JP Z HR3   ;if this is the last line JP to HR3

HR2   LD A,H     ;the address in HL is then transferred to

LD I,A     ;I register

LD A,L     ;and during LBUF to the R register

JP C084    ;jump to LBUF (4084 + 8000) to start the video

HR3   LD IX,HR4  ;save the video vector so that NMI returns to PART 2

JR HR5     ;now get blank lines and return to application code

HR4   ;PART 2

CALL 220   ;first PUSH registers then jump to VSYNC, etc

CALL F46   ;test the BREAK key

JR NC STOP ;and exit HR if key is down

LD IX,HR   ;save the video vector so that NMI returns to PART 1

HR5   LD A,(4028);33 or 19 blank lines in system variable MARGIN

ADD A,47   ;71 more blank lines for fast application code execution

JP 29E     ;save blank lines, start NMI, POP registers and RETURN

STOP             ;EXIT hires video to restore RAMTOP and SINCLAIR video

LD A,1E    ;SINCLAIR ROM pattern table MSB base address (1E00)

LD I,A     ;pointer to I register

LD HL,(4004);load the current RAMTOP

LD DE,180  ;HFILE length

ADD HL,DE  ;is added to the current RAMTOP

LD IX,0281 ;SINCLAIR video routine

JR STACK   ;exit HR via STACK to change RAMTOP

;------------------END OF LISTING----------------

15. PSEUDO HIRES CORE ROUTINES

******************************

Pseudo hires software uses CHR$ code + register I to address 1 of 64 pattern

bytes in the ROM. The CHR$ codes are limited 0 to 63 and their inverse

(128 to 191) and the value of I is choses to point to a block of pattern

bytes in ROM which has the greatest randomness and least duplication of

values. This method is called "pseudo" hires because fewer than 50% of

the 256 patterns required for true hires are available for display. This

results in incomplete or missing pixel patterns but for many application

(games etc) this is not a problem. The advantage of this method is the fact

that it runs on systems the standard 16K RAMPACK and is emulated by Xtender.

It has a characteristic 6336 byte DFILE consisting of 192 RET opcodes

spaced at 33 byte intervals.

ROCK CRUSH by Steve McDonald

In progress

3DHIRES author unknown

In this example core routine the expanded hires "DFILE" starts at 6700 hex

START   LD A,04   ; select an "interesting" pattern table

LD I,A    ; load into ROM pattern table pointer

LD IX,42C4; load pseudo hires vector

RET

42C4    LD HL,E6DF; HL is used as the hires "DFILE" pointer

LD DE,0021; 32 CHR$ + RET = 33 bytes per line

DI        ; INT not used

LD C,FE   ; IO address to reset the ULA row counter

LD B,16   ; delay

42CF    DJNZ 42CF ; delay

LD B,C0   ; 192 lines of 33 bytes

42D3    IN A,(C)  ; apply reset to row counter

OUT FF,A  ; release reset from row counter

ADD HL,DE ; next "DFILE" line (E700,E721, etc)

CALL 42EC ; "execute" the "DFILE" via JP (HL) at 42EC

DEC B     ; decrement line counter

JP NZ 42D3; last line? repeat 192 times.

CALL 0292 ; restore main registers, return to application

CALL 0220 ; extra register PUSH then VSYNC

LD IX,42C4; hires routine vector

JP 02A4   ; restore main registers, return to application

JP (HL)   ; jump to the hires "DFILE" echo above 32K

XTRICATOR by Software Farm

This unusual pseudo hires core routine intercepts the video by

setting INT mode 2 in which the interrupting device supplies part

of the INT vector address. Since the idle data bus is FF and

the I register is set to 40, the INT vector is 4000 when the A6

line interupts at the end of the horizontal line.

4083    LD HL,40A5 ;return vector

4086    PUSH HL    ;save vector

4087    LD HL,E500 ;hires file

408A    PUSH HL    ;save vector

408B    LD B,07    ;delay

408D    DJNZ 408D  ;delay

408F    LD A,1E    ;Sinclair ROM patterns

4091    LD I,A     ;load pattern table pointer

4093    LD DE,C201 ;D = 194 lines

4096    DEC E      ;set Z FLAG

4097    JP Z 409A  ;delay

409A    POP HL     ;HL = E500

409B    DEC D      ;decrement the line counter

409C    RET Z      ;after last line only : return to 40A5

409D    SET 0,E    ;E = 01

409F    JP 40A2    ;delay

40A2    LD A,00    ;delay

40A4    JP (HL)    ;jump to 6500 above 32K

40A5    LD A,04    ;"special" ROM pattern

40A7    LD I,A     ;load pattern table pointer

40A9    RET        ;return to xxxx

40AA    LD A,40    ;INT mode 2 MSB address

40AC    LD I,A     ;load the INT mode 2 vector

40AE    IM2        ;start INT mode 2

40B0    RET

40B1    LD A,1E    ;Sinclair ROM patterns

40B3    LD I,A     ;load pattern table pointer

40B5    IM1        ;restore INT mode 1

40B7    RET

40B8    LD HL,6500 ;load a call instruction

40BB    LD B,C1    ;the start of HFILE (E500)

408D    LD (HL),CD ;CALL 4096

408F    INC HL

40C0    LD (HL),96

40C2    INC HL

40C3    LD (HL),40

40C5    INC HL

40C6    LD A,20     ;generate a 6144 spaces starting

40C8    LD (HL),00  ;at 6503

40CA    INC HL

40CB    DEC A

40CC    JR NZ 40C8

40CE    DJNZ 40C6

40D0    RET

KEYBOARD MATRIX

|  | b0 | b1 | b2 | b3 | b4 |
| --- | --- | --- | --- | --- | --- |
| b0 | Shift | Z | X | C | V |
| b1 | A | S | D | F | G |
| b2 | Q | W | E | R | T |
| b3 | 1 | 2 | 3 | 4 | 5 |
| b4 | 0 | 9 | 8 | 7 | 6 |
| b5 | P | O | I | U | Y |
| b6 | ENTER | L | K | J | H |
| b7 | SPACE | Period | M | N | B |

NB: Don’t know whether high of low indicates keypress.  Also relevence of top 3 bits unknown.

KEYBOARD POLLING ROUTINE

ld     hl,$ffff        ; 0002BB 21 FF FF

ld     bc,$fefe        ; 0002BE 01 FE FE

in     a,(c)           ; 0002C1 ED 78

or     $01             ; 0002C3 F6 01

or     $e0             ; 0002C5 F6 E0

ld     d,a             ; 0002C7 57

cpl                    ; 0002C8 2F

cp     $01             ; 0002C9 FE 01

sbc    a,a             ; 0002CB 9F

or     b               ; 0002CC B0

and    l               ; 0002CD A5

ld     l,a             ; 0002CE 6F

ld     a,h             ; 0002CF 7C

and    d               ; 0002D0 A2

ld     h,a             ; 0002D1 67

rlc    b               ; 0002D2 CB 00

in     a,(c)           ; 0002D4 ED 78

jr     c,$02c5         ; 0002D6 38 ED

rra                    ; 0002D8 1F

rl     h               ; 0002D9 CB 14

rla                    ; 0002DB 17

rla                    ; 0002DC 17

rla                    ; 0002DD 17

sbc    a,a             ; 0002DE 9F

and    $18             ; 0002DF E6 18

add    a,$1f           ; 0002E1 C6 1F

ld     ($4028),a       ; 0002E3 32 28 40

ret                    ; 0002E6 C9

KNOWN SYSTEM VARIABLES

| Address |  | Description |
| --- | --- | --- |
| 0x400c | DFILE | Pointer to display file. |
| 0x4010 | VARS | Points to variable storage (straight after DFILE) |
| 0x4025 | LASTK | Used in key reading routines. |
| 0x4028 | MARGIN | Number of blank lines above/below page. |
| 0x403b | FAST(?) | Bit 7 - FAST mode on/off. |