6502 Microprocessor

Most of the following information has been taking out of the "Commodore 64 Programmers Reference Manual" simply because it was available in electronic form and there appears to be no difference between this documentation and the 6502 documentation, they are both from the 6500 family after all. I've made changes and additions where appropriate.

In theory you should be able to use any code you can find for emulating the 6510 (the C64 processor).

THE REGISTERS INSIDE THE 6502 MICROPROCESSOR

Almost all calculations are done in the microprocessor. Registers are special pieces of memory in the processor which are used to carry out, and store information about calculations. The 6502 has the following registers:

THE ACCUMULATOR

This is THE most important register in the microprocessor. Various machine language instructions allow you to copy the contents of a memory location into the accumulator, copy the contents of the accumulator into a memory location, modify the contents of the accumulator or some other register directly, without affecting any memory. And the accumulator is the only register that has instructions for performing math.

THE X INDEX REGISTER

This is a very important register. There are instructions for nearly all of the transformations you can make to the accumulator. But there are other instructions for things that only the X register can do. Various machine language instructions allow you to copy the contents of a memory location into the X register, copy the contents of the X register into a memory location, and modify the contents of the X, or some other register directly.

THE Y INDEX REGISTER

This is a very important register. There are instructions for nearly all of the transformations you can make to the accumulator, and the X register. But there are other instructions for things that only the Y register can do. Various machine language instructions allow you to copy the contents of a memory location into the Y register, copy the contents of the Y register into a memory location, and modify the contents of the Y, or some other register directly.

THE STATUS REGISTER

This register consists of eight "flags" (a flag = something that indicates whether something has, or has not occurred). Bits of this register are altered depending on the result of arithmetic and logical operations. These bits are described below:

Bit No.       7   6   5   4   3   2   1   0

S   V       B   D   I   Z   C

Bit0 - C - Carry flag: this holds the carry out of the most significant bit in any arithmetic operation. In subtraction operations however, this flag is cleared - set to 0 - if a borrow is required, set to 1 - if no borrow is required. The carry flag is also used in shift and rotate   logical operations.

Bit1 - Z - Zero flag: this is set to 1 when any arithmetic or logical operation produces a zero result, and is set to 0 if the result is non-zero.

Bit 2 - I: this is an interrupt enable/disable flag. If it is set, interrupts are disabled. If it is cleared, interrupts are enabled.

Bit 3 - D: this is the decimal mode status flag. When set, and an Add with Carry or Subtract with Carry instruction is executed, the source values are treated as valid BCD (Binary Coded Decimal, eg. 0x00-0x99 = 0-99) numbers. The result generated is also a BCD number.

Bit 4 - B: this is set when a software interrupt (BRK instruction) is executed.

Bit 5: not used. Supposed to be logical 1 at all times.

Bit 6 - V - Overflow flag: when an arithmetic operation produces a result too large to be represented in a byte, V is set.

Bit 7 - S - Sign flag: this is set if the result of an operation is negative, cleared if positive.

The most commonly used flags are C, Z, V, S.

THE PROGRAM COUNTER

This contains the address of the current machine language instruction being executed. Since the operating system is always "RUN"ning in the Commodore VIC-20 (or, for that matter, any computer), the program counter is always changing. It could only be stopped by halting the microprocessor in some way.

THE STACK POINTER

This register contains the location of the first empty place on the stack. The stack is used for temporary storage by machine language programs, and by the computer.

ADDRESSING MODES

Instructions need operands to work on. There are various ways of indicating where the processor is to get these operands. The different methods used to do this are called addressing modes. The 6502 offers 11 modes, as described below.

| 1) | Immediate In this mode the operand's value is given in the instruction itself. In assembly language this is indicated by "#" before the operand. eg. LDA #$0A - means "load the accumulator with the hex value 0A" In machine code different modes are indicated by different codes. So LDA would be translated into different codes depending on the addressing mode. In this mode, it is: $A9 $0A |
| --- | --- |
| 2&3) | Absolute and Zero-page Absolute In these modes the operands address is given. eg. LDA $31F6 - (assembler) $AD $31F6 - (machine code) If the address is on zero page - i.e. any address where the high byte is 00 - only 1 byte is needed for the address. The processor automatically fills the 00 high byte. eg. LDA $F4 $A5 $F4 Note the different instruction codes for the different modes. Note also that for 2 byte addresses, the low byte is store first, eg. LDA $31F6 is stored as three bytes in memory, $AD $F6 $31. Zero-page absolute is usually just called zero-page. |
| 4) | Implied No operand addresses are required for this mode. They are implied by the instruction. eg. TAX - (transfer accumulator contents to X-register) $AA |
| 5) | Accumulator In this mode the instruction operates on data in the accumulator, so no operands are needed. eg. LSR - logical bit shift right $4A |
| 6&7) | Indexed and Zero-page Indexed In these modes the address given is added to the value in either the X or Y index register to give the actual address of the operand. eg. LDA $31F6, Y $D9 $31F6 LDA $31F6, X $DD $31F6 Note that the different operation codes determine the index register used. In the zero-page version, you should note that the X and Y registers are not interchangeable. Most instructions which can be used with zero-page indexing do so with X only. eg. LDA $20, X $B5 $20 |
| 8) | Indirect This mode applies only to the JMP instruction - JuMP to new location. It is indicated by parenthesis around the operand. The operand is the address of the bytes whose value is the new location. eg. JMP ($215F) Assume the following - byte value $215F $76 $2160 $30 This instruction takes the value of bytes $215F, $2160 and uses that as the address to jump to - i.e. $3076 (remember that addresses are stored with low byte first). |
| 9) | Pre-indexed indirect In this mode a zero-page address is added to the contents of the X-register to give the address of the bytes holding the address of the operand. The indirection is indicated by parenthesis in assembly language. eg. LDA ($3E, X) $A1 $3E Assume the following - byte value X-reg. $05 $0043 $15 $0044 $24 $2415 $6E Then the instruction is executed by: (i) adding $3E and $05 = $0043 (ii) getting address contained in bytes $0043, $0044 = $2415 (iii) loading contents of $2415 - i.e. $6E - into accumulator Note a) When adding the 1-byte address and the X-register, wrap around addition is used - i.e. the sum is always a zero-page address. eg. FF + 2 = 0001 not 0101 as you might expect. DON'T FORGET THIS WHEN EMULATING THIS MODE. b) Only the X register is used in this mode. |
| 10) | Post-indexed indirect In this mode the contents of a zero-page address (and the following byte) give the indirect addressm which is added to the contents of the Y-register to yield the actual address of the operand. Again, inassembly language, the instruction is indicated by parenthesis. eg. LDA ($4C), Y Note that the parenthesis are only around the 2nd byte of the instruction since it is the part that does the indirection. Assume the following - byte value $004C $00 $004D $21 Y-reg. $05 $2105 $6D Then the instruction above executes by: (i) getting the address in bytes $4C, $4D = $2100 (ii) adding the contents of the Y-register = $2105 (111) loading the contents of the byte $2105 - i.e. $6D into the accumulator. Note: only the Y-register is used in this mode. |
| 11) | Relative This mode is used with Branch-on-Condition instructions. It is probably the mode you will use most often. A 1 byte value is added to the program counter, and the program continues execution from that address. The 1 byte number is treated as a signed number - i.e. if bit 7 is 1, the number given by bits 0-6 is negative; if bit 7 is 0, the number is positive. This enables a branch displacement of up to 127 bytes in either direction. eg bit no. 7 6 5 4 3 2 1 0 signed value unsigned value value 1 0 1 0 0 1 1 1 -39 $A7 value 0 0 1 0 0 1 1 1 +39 $27 Instruction example: BEQ $A7 $F0 $A7 This instruction will check the zero status bit. If it is set, 39 decimal will be subtracted from the program counter and execution continues from that address. If the zero status bit is not set, execution continues from the following instruction. Notes: a) The program counter points to the start of the instruction after the branch instruction before the branch displacement is added. Remember to take this into account when calculating displacements. b) Branch-on-condition instructions work by checking the relevant status bits in the status register. Make sure that they have been set or unset as you want them. This is often done using a CMP instruction. c) If you find you need to branch further than 127 bytes, use the opposite branch-on-condition and a JMP. |

MCS6502 MICROPROCESSOR INSTRUCTION SET ALPHABETIC SEQUENCE

| ADC Add Memory to Accumulator with Carry AND "AND" Memory with Accumulator ASL Shift Left One Bit (Memory or Accumulator) BCC Branch on Carry Clear BCS Branch on Carry Set BEQ Branch on Result Zero BIT Test Bits in Memory with Accumulator BMI Branch on Result Minus BNE Branch on Result not Zero BPL Branch on Result Plus BRK Force Break BVC Branch on Overflow Clear BVS Branch on Overflow Set CLC Clear Carry Flag CLD Clear Decimal Mode CLI Clear interrupt Disable Bit CLV Clear Overflow Flag CMP Compare Memory and Accumulator CPX Compare Memory and Index X CPY Compare Memory and Index Y DEC Decrement Memory by One DEX Decrement Index X by One DEY Decrement Index Y by One EOR "ExclusiveOr" Memory with Accumulator INC Increment Memory by One INX Increment Index X by One INY Increment Index Y by One JMP Jump to New Location | JSR Jump to New Location Saving Return Address LDA Load Accumulator with Memory LDX Load Index X with Memory LDY Load Index Y with Memory LSR Shift Right One Bit (Memory or Accumulator) NOP No Operation ORA "OR" Memory with Accumulator PHA Push Accumulator on Stack PHP Push Processor Status on Stack PLA Pull Accumulator from Stack PLP Pull Processor Status from Stack ROL Rotate One Bit Left (Memory or Accumulator) ROR Rotate One Bit Right (Memory or Accumulator) RTI Return from Interrupt RTS Return from Subroutine SBC Subtract Memory from Accumulator with Borrow SEC Set Carry Flag SED Set Decimal Mode SEI Set Interrupt Disable Status STA Store Accumulator in Memory STX Store Index X in Memory STY Store Index Y in Memory TAX Transfer Accumulator to Index X TAY Transfer Accumulator to Index Y TSX Transfer Stack Pointer to Index X TXA Transfer Index X to Accumulator TXS Transfer Index X to Stack Pointer TYA Transfer Index Y to Accumulator |
| --- | --- |

The following notation applies to this summary:

A       Accumulator                  EOR     Logical Exclusive Or

X, Y    Index Registers              fromS   Transfer from Stack

M       Memory                       toS     Transfer to Stack

P       Processor Status Register    ->      Transfer to

S       Stack Pointer                <-      Transfer from

/       Change                       V       Logical OR

_       No Change                    PC      Program Counter

+       Add                          PCH     Program Counter High

/\      Logical AND                  PCL     Program Counter Low

-       Subtract                     OPER    OPERAND

#       IMMEDIATE ADDRESSING MODE

Note:

At the top of each table is located in parentheses a reference number (Ref: XX) which directs the user to that Section in the MCS6500 Microcomputer Family Programming Manual in which the instruction is defined and discussed.

ADC               Add memory to accumulator with carry                ADC

Operation:  A + M + C -> A, C                         N Z C I D V

/ / / _ _ /

(Ref: 2.2.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   ADC #Oper           |    69   |    2    |    2     |

|  Zero Page     |   ADC Oper            |    65   |    2    |    3     |

|  Zero Page,X   |   ADC Oper,X          |    75   |    2    |    4     |

|  Absolute      |   ADC Oper            |    60   |    3    |    4     |

|  Absolute,X    |   ADC Oper,X          |    70   |    3    |    4*    |

|  Absolute,Y    |   ADC Oper,Y          |    79   |    3    |    4*    |

|  (Indirect,X)  |   ADC (Oper,X)        |    61   |    2    |    6     |

|  (Indirect),Y  |   ADC (Oper),Y        |    71   |    2    |    5*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if page boundary is crossed.

AND                  "AND" memory with accumulator                    AND

Operation:  A /\ M -> A                               N Z C I D V

/ / _ _ _ _

(Ref: 2.2.3.0)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   AND #Oper           |    29   |    2    |    2     |

|  Zero Page     |   AND Oper            |    25   |    2    |    3     |

|  Zero Page,X   |   AND Oper,X          |    35   |    2    |    4     |

|  Absolute      |   AND Oper            |    2D   |    3    |    4     |

|  Absolute,X    |   AND Oper,X          |    3D   |    3    |    4*    |

|  Absolute,Y    |   AND Oper,Y          |    39   |    3    |    4*    |

|  (Indirect,X)  |   AND (Oper,X)        |    21   |    2    |    6     |

|  (Indirect,Y)  |   AND (Oper),Y        |    31   |    2    |    5     |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if page boundary is crossed.

ASL          ASL Shift Left One Bit (Memory or Accumulator)           ASL

+-+-+-+-+-+-+-+-+

Operation:  C <- |7|6|5|4|3|2|1|0| <- 0

+-+-+-+-+-+-+-+-+                    N Z C I D V

/ / / _ _ _

(Ref: 10.2)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Accumulator   |   ASL A               |    0A   |    1    |    2     |

|  Zero Page     |   ASL Oper            |    06   |    2    |    5     |

|  Zero Page,X   |   ASL Oper,X          |    16   |    2    |    6     |

|  Absolute      |   ASL Oper            |    0E   |    3    |    6     |

|  Absolute, X   |   ASL Oper,X          |    1E   |    3    |    7     |

+----------------+-----------------------+---------+---------+----------+

BCC                     BCC Branch on Carry Clear                     BCC

N Z C I D V

Operation:  Branch on C = 0                           _ _ _ _ _ _

(Ref: 4.1.1.3)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Relative      |   BCC Oper            |    90   |    2    |    2*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if branch occurs to same page.

* Add 2 if branch occurs to different page.

BCS                      BCS Branch on carry set                      BCS

Operation:  Branch on C = 1                           N Z C I D V

_ _ _ _ _ _

(Ref: 4.1.1.4)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Relative      |   BCS Oper            |    B0   |    2    |    2*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if branch occurs to same  page.

* Add 2 if branch occurs to next  page.

BEQ                    BEQ Branch on result zero                      BEQ

N Z C I D V

Operation:  Branch on Z = 1                           _ _ _ _ _ _

(Ref: 4.1.1.5)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Relative      |   BEQ Oper            |    F0   |    2    |    2*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if branch occurs to same  page.

* Add 2 if branch occurs to next  page.

BIT             BIT Test bits in memory with accumulator              BIT

Operation:  A /\ M, M7 -> N, M6 -> V

Bit 6 and 7 are transferred to the status register.   N Z C I D V

If the result of A /\ M is zero then Z = 1, otherwise M7/ _ _ _ M6

Z = 0

(Ref: 4.2.1.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Zero Page     |   BIT Oper            |    24   |    2    |    3     |

|  Absolute      |   BIT Oper            |    2C   |    3    |    4     |

+----------------+-----------------------+---------+---------+----------+

BMI                    BMI Branch on result minus                     BMI

Operation:  Branch on N = 1                           N Z C I D V

_ _ _ _ _ _

(Ref: 4.1.1.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Relative      |   BMI Oper            |    30   |    2    |    2*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if branch occurs to same page.

* Add 1 if branch occurs to different page.

BNE                   BNE Branch on result not zero                   BNE

Operation:  Branch on Z = 0                           N Z C I D V

_ _ _ _ _ _

(Ref: 4.1.1.6)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Relative      |   BMI Oper            |    D0   |    2    |    2*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if branch occurs to same page.

* Add 2 if branch occurs to different page.

BPL                     BPL Branch on result plus                     BPL

Operation:  Branch on N = 0                           N Z C I D V

_ _ _ _ _ _

(Ref: 4.1.1.2)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Relative      |   BPL Oper            |    10   |    2    |    2*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if branch occurs to same page.

* Add 2 if branch occurs to different page.

BRK                          BRK Force Break                          BRK

Operation:  Forced Interrupt PC + 2 toS P toS         N Z C I D V

_ _ _ 1 _ _

(Ref: 9.11)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   BRK                 |    00   |    1    |    7     |

+----------------+-----------------------+---------+---------+----------+

1. A BRK command cannot be masked by setting I.

BVC                   BVC Branch on overflow clear                    BVC

Operation:  Branch on V = 0                           N Z C I D V

_ _ _ _ _ _

(Ref: 4.1.1.8)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Relative      |   BVC Oper            |    50   |    2    |    2*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if branch occurs to same page.

* Add 2 if branch occurs to different page.

BVS                    BVS Branch on overflow set                     BVS

Operation:  Branch on V = 1                           N Z C I D V

_ _ _ _ _ _

(Ref: 4.1.1.7)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Relative      |   BVS Oper            |    70   |    2    |    2*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if branch occurs to same page.

* Add 2 if branch occurs to different page.

CLC                       CLC Clear carry flag                        CLC

Operation:  0 -> C                                    N Z C I D V

_ _ 0 _ _ _

(Ref: 3.0.2)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   CLC                 |    18   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

CLD                      CLD Clear decimal mode                       CLD

Operation:  0 -> D                                    N A C I D V

_ _ _ _ 0 _

(Ref: 3.3.2)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   CLD                 |    D8   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

CLI                  CLI Clear interrupt disable bit                  CLI

Operation: 0 -> I                                     N Z C I D V

_ _ _ 0 _ _

(Ref: 3.2.2)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   CLI                 |    58   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

CLV                      CLV Clear overflow flag                      CLV

Operation: 0 -> V                                     N Z C I D V

_ _ _ _ _ 0

(Ref: 3.6.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   CLV                 |    B8   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

CMP                CMP Compare memory and accumulator                 CMP

Operation:  A - M                                     N Z C I D V

/ / / _ _ _

(Ref: 4.2.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   CMP #Oper           |    C9   |    2    |    2     |

|  Zero Page     |   CMP Oper            |    C5   |    2    |    3     |

|  Zero Page,X   |   CMP Oper,X          |    D5   |    2    |    4     |

|  Absolute      |   CMP Oper            |    CD   |    3    |    4     |

|  Absolute,X    |   CMP Oper,X          |    DD   |    3    |    4*    |

|  Absolute,Y    |   CMP Oper,Y          |    D9   |    3    |    4*    |

|  (Indirect,X)  |   CMP (Oper,X)        |    C1   |    2    |    6     |

|  (Indirect),Y  |   CMP (Oper),Y        |    D1   |    2    |    5*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if page boundary is crossed.

CPX                  CPX Compare Memory and Index X                   CPX

N Z C I D V

Operation:  X - M                                     / / / _ _ _

(Ref: 7.8)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   CPX *Oper           |    E0   |    2    |    2     |

|  Zero Page     |   CPX Oper            |    E4   |    2    |    3     |

|  Absolute      |   CPX Oper            |    EC   |    3    |    4     |

+----------------+-----------------------+---------+---------+----------+

CPY                  CPY Compare memory and index Y                   CPY

N Z C I D V

Operation:  Y - M                                     / / / _ _ _

(Ref: 7.9)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   CPY *Oper           |    C0   |    2    |    2     |

|  Zero Page     |   CPY Oper            |    C4   |    2    |    3     |

|  Absolute      |   CPY Oper            |    CC   |    3    |    4     |

+----------------+-----------------------+---------+---------+----------+

DEC                   DEC Decrement memory by one                     DEC

Operation:  M - 1 -> M                                N Z C I D V

/ / _ _ _ _

(Ref: 10.7)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Zero Page     |   DEC Oper            |    C6   |    2    |    5     |

|  Zero Page,X   |   DEC Oper,X          |    D6   |    2    |    6     |

|  Absolute      |   DEC Oper            |    CE   |    3    |    6     |

|  Absolute,X    |   DEC Oper,X          |    DE   |    3    |    7     |

+----------------+-----------------------+---------+---------+----------+

DEX                   DEX Decrement index X by one                    DEX

Operation:  X - 1 -> X                                N Z C I D V

/ / _ _ _ _

(Ref: 7.6)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   DEX                 |    CA   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

DEY                   DEY Decrement index Y by one                    DEY

Operation:  X - 1 -> Y                                N Z C I D V

/ / _ _ _ _

(Ref: 7.7)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   DEY                 |    88   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

EOR            EOR "Exclusive-Or" memory with accumulator             EOR

Operation:  A EOR M -> A                              N Z C I D V

/ / _ _ _ _

(Ref: 2.2.3.2)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   EOR #Oper           |    49   |    2    |    2     |

|  Zero Page     |   EOR Oper            |    45   |    2    |    3     |

|  Zero Page,X   |   EOR Oper,X          |    55   |    2    |    4     |

|  Absolute      |   EOR Oper            |    40   |    3    |    4     |

|  Absolute,X    |   EOR Oper,X          |    50   |    3    |    4*    |

|  Absolute,Y    |   EOR Oper,Y          |    59   |    3    |    4*    |

|  (Indirect,X)  |   EOR (Oper,X)        |    41   |    2    |    6     |

|  (Indirect),Y  |   EOR (Oper),Y        |    51   |    2    |    5*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if page boundary is crossed.

INC                    INC Increment memory by one                    INC

N Z C I D V

Operation:  M + 1 -> M                                / / _ _ _ _

(Ref: 10.6)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Zero Page     |   INC Oper            |    E6   |    2    |    5     |

|  Zero Page,X   |   INC Oper,X          |    F6   |    2    |    6     |

|  Absolute      |   INC Oper            |    EE   |    3    |    6     |

|  Absolute,X    |   INC Oper,X          |    FE   |    3    |    7     |

+----------------+-----------------------+---------+---------+----------+

INX                    INX Increment Index X by one                   INX

N Z C I D V

Operation:  X + 1 -> X                                / / _ _ _ _

(Ref: 7.4)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   INX                 |    E8   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

INY                    INY Increment Index Y by one                   INY

Operation:  X + 1 -> X                                N Z C I D V

/ / _ _ _ _

(Ref: 7.5)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   INY                 |    C8   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

JMP                     JMP Jump to new location                      JMP

Operation:  (PC + 1) -> PCL                           N Z C I D V

(PC + 2) -> PCH   (Ref: 4.0.2)            _ _ _ _ _ _

(Ref: 9.8.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Absolute      |   JMP Oper            |    4C   |    3    |    3     |

|  Indirect      |   JMP (Oper)          |    6C   |    3    |    5     |

+----------------+-----------------------+---------+---------+----------+

JSR          JSR Jump to new location saving return address           JSR

Operation:  PC + 2 toS, (PC + 1) -> PCL               N Z C I D V

(PC + 2) -> PCH               _ _ _ _ _ _

(Ref: 8.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Absolute      |   JSR Oper            |    20   |    3    |    6     |

+----------------+-----------------------+---------+---------+----------+

LDA                  LDA Load accumulator with memory                 LDA

Operation:  M -> A                                    N Z C I D V

/ / _ _ _ _

(Ref: 2.1.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   LDA #Oper           |    A9   |    2    |    2     |

|  Zero Page     |   LDA Oper            |    A5   |    2    |    3     |

|  Zero Page,X   |   LDA Oper,X          |    B5   |    2    |    4     |

|  Absolute      |   LDA Oper            |    AD   |    3    |    4     |

|  Absolute,X    |   LDA Oper,X          |    BD   |    3    |    4*    |

|  Absolute,Y    |   LDA Oper,Y          |    B9   |    3    |    4*    |

|  (Indirect,X)  |   LDA (Oper,X)        |    A1   |    2    |    6     |

|  (Indirect),Y  |   LDA (Oper),Y        |    B1   |    2    |    5*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 if page boundary is crossed.

LDX                   LDX Load index X with memory                    LDX

Operation:  M -> X                                    N Z C I D V

/ / _ _ _ _

(Ref: 7.0)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   LDX #Oper           |    A2   |    2    |    2     |

|  Zero Page     |   LDX Oper            |    A6   |    2    |    3     |

|  Zero Page,Y   |   LDX Oper,Y          |    B6   |    2    |    4     |

|  Absolute      |   LDX Oper            |    AE   |    3    |    4     |

|  Absolute,Y    |   LDX Oper,Y          |    BE   |    3    |    4*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 when page boundary is crossed.

LDY                   LDY Load index Y with memory                    LDY

N Z C I D V

Operation:  M -> Y                                    / / _ _ _ _

(Ref: 7.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   LDY #Oper           |    A0   |    2    |    2     |

|  Zero Page     |   LDY Oper            |    A4   |    2    |    3     |

|  Zero Page,X   |   LDY Oper,X          |    B4   |    2    |    4     |

|  Absolute      |   LDY Oper            |    AC   |    3    |    4     |

|  Absolute,X    |   LDY Oper,X          |    BC   |    3    |    4*    |

+----------------+-----------------------+---------+---------+----------+

* Add 1 when page boundary is crossed.

LSR          LSR Shift right one bit (memory or accumulator)          LSR

+-+-+-+-+-+-+-+-+

Operation:  0 -> |7|6|5|4|3|2|1|0| -> C               N Z C I D V

+-+-+-+-+-+-+-+-+                    0 / / _ _ _

(Ref: 10.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Accumulator   |   LSR A               |    4A   |    1    |    2     |

|  Zero Page     |   LSR Oper            |    46   |    2    |    5     |

|  Zero Page,X   |   LSR Oper,X          |    56   |    2    |    6     |

|  Absolute      |   LSR Oper            |    4E   |    3    |    6     |

|  Absolute,X    |   LSR Oper,X          |    5E   |    3    |    7     |

+----------------+-----------------------+---------+---------+----------+

NOP                         NOP No operation                          NOP

N Z C I D V

Operation:  No Operation (2 cycles)                   _ _ _ _ _ _

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   NOP                 |    EA   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

ORA                 ORA "OR" memory with accumulator                  ORA

Operation: A V M -> A                                 N Z C I D V

/ / _ _ _ _

(Ref: 2.2.3.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   ORA #Oper           |    09   |    2    |    2     |

|  Zero Page     |   ORA Oper            |    05   |    2    |    3     |

|  Zero Page,X   |   ORA Oper,X          |    15   |    2    |    4     |

|  Absolute      |   ORA Oper            |    0D   |    3    |    4     |

|  Absolute,X    |   ORA Oper,X          |    10   |    3    |    4*    |

|  Absolute,Y    |   ORA Oper,Y          |    19   |    3    |    4*    |

|  (Indirect,X)  |   ORA (Oper,X)        |    01   |    2    |    6     |

|  (Indirect),Y  |   ORA (Oper),Y        |    11   |    2    |    5     |

+----------------+-----------------------+---------+---------+----------+

* Add 1 on page crossing

PHA                   PHA Push accumulator on stack                   PHA

Operation:  A toS                                     N Z C I D V

_ _ _ _ _ _

(Ref: 8.5)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   PHA                 |    48   |    1    |    3     |

+----------------+-----------------------+---------+---------+----------+

PHP                 PHP Push processor status on stack                PHP

Operation:  P toS                                     N Z C I D V

_ _ _ _ _ _

(Ref: 8.11)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   PHP                 |    08   |    1    |    3     |

+----------------+-----------------------+---------+---------+----------+

PLA                 PLA Pull accumulator from stack                   PLA

Operation:  A fromS                                   N Z C I D V

_ _ _ _ _ _

(Ref: 8.6)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   PLA                 |    68   |    1    |    4     |

+----------------+-----------------------+---------+---------+----------+

PLP               PLP Pull processor status from stack                PLA

Operation:  P fromS                                   N Z C I D V

From Stack

(Ref: 8.12)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   PLP                 |    28   |    1    |    4     |

+----------------+-----------------------+---------+---------+----------+

ROL          ROL Rotate one bit left (memory or accumulator)          ROL

+------------------------------+

|         M or A               |

|   +-+-+-+-+-+-+-+-+    +-+   |

Operation:   +-< |7|6|5|4|3|2|1|0| <- |C| <-+         N Z C I D V

+-+-+-+-+-+-+-+-+    +-+             / / / _ _ _

(Ref: 10.3)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Accumulator   |   ROL A               |    2A   |    1    |    2     |

|  Zero Page     |   ROL Oper            |    26   |    2    |    5     |

|  Zero Page,X   |   ROL Oper,X          |    36   |    2    |    6     |

|  Absolute      |   ROL Oper            |    2E   |    3    |    6     |

|  Absolute,X    |   ROL Oper,X          |    3E   |    3    |    7     |

+----------------+-----------------------+---------+---------+----------+

ROR          ROR Rotate one bit right (memory or accumulator)         ROR

+------------------------------+

|                              |

|   +-+    +-+-+-+-+-+-+-+-+   |

Operation:   +-> |C| -> |7|6|5|4|3|2|1|0| >-+         N Z C I D V

+-+    +-+-+-+-+-+-+-+-+             / / / _ _ _

(Ref: 10.4)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Accumulator   |   ROR A               |    6A   |    1    |    2     |

|  Zero Page     |   ROR Oper            |    66   |    2    |    5     |

|  Zero Page,X   |   ROR Oper,X          |    76   |    2    |    6     |

|  Absolute      |   ROR Oper            |    6E   |    3    |    6     |

|  Absolute,X    |   ROR Oper,X          |    7E   |    3    |    7     |

+----------------+-----------------------+---------+---------+----------+

Note: ROR instruction is available on MCS650X microprocessors after

June, 1976.

RTI                    RTI Return from interrupt                      RTI

N Z C I D V

Operation:  P fromS PC fromS                           From Stack

(Ref: 9.6)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   RTI                 |    4D   |    1    |    6     |

+----------------+-----------------------+---------+---------+----------+

RTS                    RTS Return from subroutine                     RTS

N Z C I D V

Operation:  PC fromS, PC + 1 -> PC                    _ _ _ _ _ _

(Ref: 8.2)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   RTS                 |    60   |    1    |    6     |

+----------------+-----------------------+---------+---------+----------+

SBC          SBC Subtract memory from accumulator with borrow         SBC

-

Operation:  A - M - C -> A                            N Z C I D V

-                                              / / / _ _ /

Note:C = Borrow             (Ref: 2.2.2)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Immediate     |   SBC #Oper           |    E9   |    2    |    2     |

|  Zero Page     |   SBC Oper            |    E5   |    2    |    3     |

|  Zero Page,X   |   SBC Oper,X          |    F5   |    2    |    4     |

|  Absolute      |   SBC Oper            |    ED   |    3    |    4     |

|  Absolute,X    |   SBC Oper,X          |    FD   |    3    |    4*    |

|  Absolute,Y    |   SBC Oper,Y          |    F9   |    3    |    4*    |

|  (Indirect,X)  |   SBC (Oper,X)        |    E1   |    2    |    6     |

|  (Indirect),Y  |   SBC (Oper),Y        |    F1   |    2    |    5     |

+----------------+-----------------------+---------+---------+----------+

* Add 1 when page boundary is crossed.

SEC                        SEC Set carry flag                         SEC

Operation:  1 -> C                                    N Z C I D V

_ _ 1 _ _ _

(Ref: 3.0.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   SEC                 |    38   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

SED                       SED Set decimal mode                        SED

N Z C I D V

Operation:  1 -> D                                    _ _ _ _ 1 _

(Ref: 3.3.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   SED                 |    F8   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

SEI                 SEI Set interrupt disable status                  SED

N Z C I D V

Operation:  1 -> I                                    _ _ _ 1 _ _

(Ref: 3.2.1)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   SEI                 |    78   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

STA                  STA Store accumulator in memory                  STA

Operation:  A -> M                                    N Z C I D V

_ _ _ _ _ _

(Ref: 2.1.2)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Zero Page     |   STA Oper            |    85   |    2    |    3     |

|  Zero Page,X   |   STA Oper,X          |    95   |    2    |    4     |

|  Absolute      |   STA Oper            |    80   |    3    |    4     |

|  Absolute,X    |   STA Oper,X          |    90   |    3    |    5     |

|  Absolute,Y    |   STA Oper, Y         |    99   |    3    |    5     |

|  (Indirect,X)  |   STA (Oper,X)        |    81   |    2    |    6     |

|  (Indirect),Y  |   STA (Oper),Y        |    91   |    2    |    6     |

+----------------+-----------------------+---------+---------+----------+

STX                    STX Store index X in memory                    STX

Operation: X -> M                                     N Z C I D V

_ _ _ _ _ _

(Ref: 7.2)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Zero Page     |   STX Oper            |    86   |    2    |    3     |

|  Zero Page,Y   |   STX Oper,Y          |    96   |    2    |    4     |

|  Absolute      |   STX Oper            |    8E   |    3    |    4     |

+----------------+-----------------------+---------+---------+----------+

STY                    STY Store index Y in memory                    STY

Operation: Y -> M                                     N Z C I D V

_ _ _ _ _ _

(Ref: 7.3)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Zero Page     |   STY Oper            |    84   |    2    |    3     |

|  Zero Page,X   |   STY Oper,X          |    94   |    2    |    4     |

|  Absolute      |   STY Oper            |    8C   |    3    |    4     |

+----------------+-----------------------+---------+---------+----------+

TAX                TAX Transfer accumulator to index X                TAX

Operation:  A -> X                                    N Z C I D V

/ / _ _ _ _

(Ref: 7.11)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   TAX                 |    AA   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

TAY                TAY Transfer accumulator to index Y                TAY

Operation:  A -> Y                                    N Z C I D V

/ / _ _ _ _

(Ref: 7.13)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   TAY                 |    A8   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

TSX              TSX Transfer stack pointer to index X                TSX

Operation:  S -> X                                    N Z C I D V

/ / _ _ _ _

(Ref: 8.9)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   TSX                 |    BA   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

TXA                TXA Transfer index X to accumulator                TXA

N Z C I D V

Operation:  X -> A                                    / / _ _ _ _

(Ref: 7.12)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   TXA                 |    8A   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

TXS              TXS Transfer index X to stack pointer                TXS

N Z C I D V

Operation:  X -> S                                    _ _ _ _ _ _

(Ref: 8.8)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   TXS                 |    9A   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

TYA                TYA Transfer index Y to accumulator                TYA

Operation:  Y -> A                                    N Z C I D V

/ / _ _ _ _

(Ref: 7.14)

+----------------+-----------------------+---------+---------+----------+

| Addressing Mode| Assembly Language Form| OP CODE |No. Bytes|No. Cycles|

+----------------+-----------------------+---------+---------+----------+

|  Implied       |   TYA                 |    98   |    1    |    2     |

+----------------+-----------------------+---------+---------+----------+

+------------------------------------------------------------------------

| INSTRUCTION ADDRESSING MODES AND RELATED EXECUTION TIMES

| (in clock cycles)

+------------------------------------------------------------------------

A   A   A   B   B   B   B   B   B   B   B   B   B   C

D   N   S   C   C   E   I   M   N   P   R   V   V   L

C   D   L   C   S   Q   T   I   E   L   K   C   S   C

Accumulator  |  .   .   2   .   .   .   .   .   .   .   .   .   .   .

Immediate    |  2   2       .   .   .   .   .   .   .   .   .   .   .

Zero Page    |  3   3   5   .   .   .   3   .   .   .   .   .   .   .

Zero Page,X  |  4   4   6   .   .   .   .   .   .   .   .   .   .   .

Zero Page,Y  |  .   .   .   .   .   .   .   .   .   .   .   .   .   .

Absolute     |  4   4   6   .   .   .   4   .   .   .   .   .   .   .

Absolute,X   |  4*  4*  7   .   .   .   .   .   .   .   .   .   .   .

Absolute,Y   |  4*  4*  .   .   .   .   .   .   .   .   .   .   .   .

Implied      |  .   .   .   .   .   .   .   .   .   .   .   .   .   2

Relative     |  .   .   .   2** 2** 2** .   2** 2** 2** 7   2** 2** .

(Indirect,X) |  6   6   .   .   .   .   .   .   .   .   .   .   .   .

(Indirect),Y |  5*  5*  .   .   .   .   .   .   .   .   .   .   .   .

Abs. Indirect|  .   .   .   .   .   .   .   .   .   .   .   .   .   .

+-----------------------------------------------------------

C   C   C   C   C   C   D   D   D   E   I   I   I   J

L   L   L   M   P   P   E   E   E   O   N   N   N   M

D   I   V   P   X   Y   C   X   Y   R   C   X   Y   P

Accumulator  |  .   .   .   .   .   .   .   .   .   .   .   .   .   .

Immediate    |  .   .   .   2   2   2   .   .   .   2   .   .   .   .

Zero Page    |  .   .   .   3   3   3   5   .   .   3   5   .   .   .

Zero Page,X  |  .   .   .   4   .   .   6   .   .   4   6   .   .   .

Zero Page,Y  |  .   .   .   .   .   .   .   .   .   .   .   .   .   .

Absolute     |  .   .   .   4   4   4   6   .   .   4   6   .   .   3

Absolute,X   |  .   .   .   4*  .   .   7   .   .   4*  7   .   .   .

Absolute,Y   |  .   .   .   4*  .   .   .   .   .   4*  .   .   .   .

Implied      |  2   2   2   .   .   .   .   2   2   .   .   2   2   .

Relative     |  .   .   .   .   .   .   .   .   .   .   .   .   .   .

(Indirect,X) |  .   .   .   6   .   .   .   .   .   6   .   .   .   .

(Indirect),Y |  .   .   .   5*  .   .   .   .   .   5*  .   .   .   .

Abs. Indirect|  .   .   .   .   .   .   .   .   .   .   .   .   .   5

+-----------------------------------------------------------

*  Add one cycle if indexing across page boundary

** Add one cycle if branch is taken, Add one additional if branching

operation crosses page boundary

------------------------------------------------------------------------+

INSTRUCTION ADDRESSING MODES AND RELATED EXECUTION TIMES              |

(in clock cycles)                                                     |

------------------------------------------------------------------------+

J   L   L   L   L   N   O   P   P   P   P   R   R   R

S   D   D   D   S   O   R   H   H   L   L   O   O   T

R   A   X   Y   R   P   A   A   P   A   P   L   R   I

Accumulator  |  .   .   .   .   2   .   .   .   .   .   .   2   2   .

Immediate    |  .   2   2   2   .   .   2   .   .   .   .   .   .   .

Zero Page    |  .   3   3   3   5   .   3   .   .   .   .   5   5   .

Zero Page,X  |  .   4   .   4   6   .   4   .   .   .   .   6   6   .

Zero Page,Y  |  .   .   4   .   .   .   .   .   .   .   .   .   .   .

Absolute     |  6   4   4   4   6   .   4   .   .   .   .   6   6   .

Absolute,X   |  .   4*  .   4*  7   .   4*  .   .   .   .   7   7   .

Absolute,Y   |  .   4*  4*  .   .   .   4*  .   .   .   .   .   .   .

Implied      |  .   .   .   .   .   2   .   3   3   4   4   .   .   6

Relative     |  .   .   .   .   .   .   .   .   .   .   .   .   .   .

(Indirect,X) |  .   6   .   .   .   .   6   .   .   .   .   .   .   .

(Indirect),Y |  .   5*  .   .   .   .   5*  .   .   .   .   .   .   .

Abs. Indirect|  .   .   .   .   .   .   .   .   .   .   .   .   .   .

+-----------------------------------------------------------

R   S   S   S   S   S   S   S   T   T   T   T   T   T

T   B   E   E   E   T   T   T   A   A   S   X   X   Y

S   C   C   D   I   A   X   Y   X   Y   X   A   S   A

Accumulator  |  .   .   .   .   .   .   .   .   .   .   .   .   .   .

Immediate    |  .   2   .   .   .   .   .   .   .   .   .   .   .   .

Zero Page    |  .   3   .   .   .   3   3   3   .   .   .   .   .   .

Zero Page,X  |  .   4   .   .   .   4   .   4   .   .   .   .   .   .

Zero Page,Y  |  .   .   .   .   .   .   4   .   .   .   .   .   .   .

Absolute     |  .   4   .   .   .   4   4   4   .   .   .   .   .   .

Absolute,X   |  .   4*  .   .   .   5   .   .   .   .   .   .   .   .

Absolute,Y   |  .   4*  .   .   .   5   .   .   .   .   .   .   .   .

Implied      |  6   .   2   2   2   .   .   .   2   2   2   2   2   2

Relative     |  .   .   .   .   .   .   .   .   .   .   .   .   .   .

(Indirect,X) |  .   6   .   .   .   6   .   .   .   .   .   .   .   .

(Indirect),Y |  .   5*  .   .   .   6   .   .   .   .   .   .   .   .

Abs. Indirect|  .   .   .   .   .   .   .   .   .   .   .   .   .   .

+-----------------------------------------------------------

*  Add one cycle if indexing across page boundary

** Add one cycle if branch is taken, Add one additional if branching

operation crosses page boundary

00 - BRK                        20 - JSR

01 - ORA - (Indirect,X)         21 - AND - (Indirect,X)

02 - Future Expansion           22 - Future Expansion

03 - Future Expansion           23 - Future Expansion

04 - Future Expansion           24 - BIT - Zero Page

05 - ORA - Zero Page            25 - AND - Zero Page

06 - ASL - Zero Page            26 - ROL - Zero Page

07 - Future Expansion           27 - Future Expansion

08 - PHP                        28 - PLP

09 - ORA - Immediate            29 - AND - Immediate

0A - ASL - Accumulator          2A - ROL - Accumulator

0B - Future Expansion           2B - Future Expansion

0C - Future Expansion           2C - BIT - Absolute

0D - ORA - Absolute             2D - AND - Absolute

0E - ASL - Absolute             2E - ROL - Absolute

0F - Future Expansion           2F - Future Expansion

10 - BPL                        30 - BMI

11 - ORA - (Indirect),Y         31 - AND - (Indirect),Y

12 - Future Expansion           32 - Future Expansion

13 - Future Expansion           33 - Future Expansion

14 - Future Expansion           34 - Future Expansion

15 - ORA - Zero Page,X          35 - AND - Zero Page,X

16 - ASL - Zero Page,X          36 - ROL - Zero Page,X

17 - Future Expansion           37 - Future Expansion

18 - CLC                        38 - SEC

19 - ORA - Absolute,Y           39 - AND - Absolute,Y

1A - Future Expansion           3A - Future Expansion

1B - Future Expansion           3B - Future Expansion

1C - Future Expansion           3C - Future Expansion

1D - ORA - Absolute,X           3D - AND - Absolute,X

1E - ASL - Absolute,X           3E - ROL - Absolute,X

1F - Future Expansion           3F - Future Expansion

40 - RTI                        60 - RTS

41 - EOR - (Indirect,X)         61 - ADC - (Indirect,X)

42 - Future Expansion           62 - Future Expansion

43 - Future Expansion           63 - Future Expansion

44 - Future Expansion           64 - Future Expansion

45 - EOR - Zero Page            65 - ADC - Zero Page

46 - LSR - Zero Page            66 - ROR - Zero Page

47 - Future Expansion           67 - Future Expansion

48 - PHA                        68 - PLA

49 - EOR - Immediate            69 - ADC - Immediate

4A - LSR - Accumulator          6A - ROR - Accumulator

4B - Future Expansion           6B - Future Expansion

4C - JMP - Absolute             6C - JMP - Indirect

4D - EOR - Absolute             6D - ADC - Absolute

4E - LSR - Absolute             6E - ROR - Absolute

4F - Future Expansion           6F - Future Expansion

50 - BVC                        70 - BVS

51 - EOR - (Indirect),Y         71 - ADC - (Indirect),Y

52 - Future Expansion           72 - Future Expansion

53 - Future Expansion           73 - Future Expansion

54 - Future Expansion           74 - Future Expansion

55 - EOR - Zero Page,X          75 - ADC - Zero Page,X

56 - LSR - Zero Page,X          76 - ROR - Zero Page,X

57 - Future Expansion           77 - Future Expansion

58 - CLI                        78 - SEI

59 - EOR - Absolute,Y           79 - ADC - Absolute,Y

5A - Future Expansion           7A - Future Expansion

5B - Future Expansion           7B - Future Expansion

5C - Future Expansion           7C - Future Expansion

50 - EOR - Absolute,X           70 - ADC - Absolute,X

5E - LSR - Absolute,X           7E - ROR - Absolute,X

5F - Future Expansion           7F - Future Expansion

80 - Future Expansion           A0 - LDY - Immediate

81 - STA - (Indirect,X)         A1 - LDA - (Indirect,X)

82 - Future Expansion           A2 - LDX - Immediate

83 - Future Expansion           A3 - Future Expansion

84 - STY - Zero Page            A4 - LDY - Zero Page

85 - STA - Zero Page            A5 - LDA - Zero Page

86 - STX - Zero Page            A6 - LDX - Zero Page

87 - Future Expansion           A7 - Future Expansion

88 - DEY                        A8 - TAY

89 - Future Expansion           A9 - LDA - Immediate

8A - TXA                        AA - TAX

8B - Future Expansion           AB - Future Expansion

8C - STY - Absolute             AC - LDY - Absolute

80 - STA - Absolute             AD - LDA - Absolute

8E - STX - Absolute             AE - LDX - Absolute

8F - Future Expansion           AF - Future Expansion

90 - BCC                        B0 - BCS

91 - STA - (Indirect),Y         B1 - LDA - (Indirect),Y

92 - Future Expansion           B2 - Future Expansion

93 - Future Expansion           B3 - Future Expansion

94 - STY - Zero Page,X          B4 - LDY - Zero Page,X

95 - STA - Zero Page,X          BS - LDA - Zero Page,X

96 - STX - Zero Page,Y          B6 - LDX - Zero Page,Y

97 - Future Expansion           B7 - Future Expansion

98 - TYA                        B8 - CLV

99 - STA - Absolute,Y           B9 - LDA - Absolute,Y

9A - TXS                        BA - TSX

9B - Future Expansion           BB - Future Expansion

9C - Future Expansion           BC - LDY - Absolute,X

90 - STA - Absolute,X           BD - LDA - Absolute,X

9E - Future Expansion           BE - LDX - Absolute,Y

9F - Future Expansion           BF - Future Expansion

C0 - Cpy - Immediate            E0 - CPX - Immediate

C1 - CMP - (Indirect,X)         E1 - SBC - (Indirect,X)

C2 - Future Expansion           E2 - Future Expansion

C3 - Future Expansion           E3 - Future Expansion

C4 - CPY - Zero Page            E4 - CPX - Zero Page

C5 - CMP - Zero Page            E5 - SBC - Zero Page

C6 - DEC - Zero Page            E6 - INC - Zero Page

C7 - Future Expansion           E7 - Future Expansion

C8 - INY                        E8 - INX

C9 - CMP - Immediate            E9 - SBC - Immediate

CA - DEX                        EA - NOP

CB - Future Expansion           EB - Future Expansion

CC - CPY - Absolute             EC - CPX - Absolute

CD - CMP - Absolute             ED - SBC - Absolute

CE - DEC - Absolute             EE - INC - Absolute

CF - Future Expansion           EF - Future Expansion

D0 - BNE                        F0 - BEQ

D1 - CMP   (Indirect@,Y         F1 - SBC - (Indirect),Y

D2 - Future Expansion           F2 - Future Expansion

D3 - Future Expansion           F3 - Future Expansion

D4 - Future Expansion           F4 - Future Expansion

D5 - CMP - Zero Page,X          F5 - SBC - Zero Page,X

D6 - DEC - Zero Page,X          F6 - INC - Zero Page,X

D7 - Future Expansion           F7 - Future Expansion

D8 - CLD                        F8 - SED

D9 - CMP - Absolute,Y           F9 - SBC - Absolute,Y

DA - Future Expansion           FA - Future Expansion

DB - Future Expansion           FB - Future Expansion

DC - Future Expansion           FC - Future Expansion

DD - CMP - Absolute,X           FD - SBC - Absolute,X

DE - DEC - Absolute,X           FE - INC - Absolute,X

DF - Future Expansion           FF - Future Expansion

INSTRUCTION OPERATION

The following code has been taken from VICE for the purposes of showing

how each instruction operates. No particular addressing mode is used since

we only wish to see the operation of the instruction itself.

src : the byte of data that is being addressed.

SET_SIGN : sets\resets the sign flag depending on bit 7.

SET_ZERO : sets\resets the zero flag depending on whether the result

is zero or not.

SET_CARRY(condition) : if the condition has a non-zero value then the

carry flag is set, else it is reset.

SET_OVERFLOW(condition) : if the condition is true then the overflow

flag is set, else it is reset.

SET_INTERRUPT :  }

SET_BREAK :      }  As for SET_CARRY and SET_OVERFLOW.

SET_DECIMAL :    }

REL_ADDR(PC, src) : returns the relative address obtained by adding

the displacement src to the PC.

SET_SR : set the Program Status Register to the value given.

GET_SR : get the value of the Program Status Register.

PULL : Pull a byte off the stack.

PUSH : Push a byte onto the stack.

LOAD : Get a byte from the memory address.

STORE : Store a byte in a memory address.

IF_CARRY, IF_OVERFLOW, IF_SIGN, IF_ZERO etc : Returns true if the

relevant flag is set, otherwise returns false.

clk : the number of cycles an instruction takes. This is shown below

in situations where the number of cycles changes depending

on the result of the instruction (eg. Branching instructions).

AC = Accumulator

XR = X register

YR = Y register

PC = Program Counter

SP = Stack Pointer

/* ADC */

unsigned int temp = src + AC + (IF_CARRY() ? 1 : 0);

SET_ZERO(temp & 0xff);	/* This is not valid in decimal mode */

if (IF_DECIMAL()) {

if (((AC & 0xf) + (src & 0xf) + (IF_CARRY() ? 1 : 0)) > 9) temp += 6;

SET_SIGN(temp);

SET_OVERFLOW(!((AC ^ src) & 0x80) && ((AC ^ temp) & 0x80));

if (temp > 0x99) temp += 96;

SET_CARRY(temp > 0x99);

} else {

SET_SIGN(temp);

SET_OVERFLOW(!((AC ^ src) & 0x80) && ((AC ^ temp) & 0x80));

SET_CARRY(temp > 0xff);

}

AC = ((BYTE) temp);

/* AND */

src &= AC;

SET_SIGN(src);

SET_ZERO(src);

AC = src;

/* ASL */

SET_CARRY(src & 0x80);

src <<= 1;

src &= 0xff;

SET_SIGN(src);

SET_ZERO(src);

STORE src in memory or accumulator depending on addressing mode.

/* BCC */

if (!IF_CARRY()) {

clk += ((PC & 0xFF00) != (REL_ADDR(PC, src) & 0xFF00) ? 2 : 1);

PC = REL_ADDR(PC, src);

}

/* BCS */

if (IF_CARRY()) {

clk += ((PC & 0xFF00) != (REL_ADDR(PC, src) & 0xFF00) ? 2 : 1);

PC = REL_ADDR(PC, src);

}

/* BEQ */

if (IF_ZERO()) {

clk += ((PC & 0xFF00) != (REL_ADDR(PC, src) & 0xFF00) ? 2 : 1);

PC = REL_ADDR(PC, src);

}

/* BIT */

SET_SIGN(src);

SET_OVERFLOW(0x40 & src);	/* Copy bit 6 to OVERFLOW flag. */

SET_ZERO(src & AC);

/* BMI */

if (IF_SIGN()) {

clk += ((PC & 0xFF00) != (REL_ADDR(PC, src) & 0xFF00) ? 2 : 1);

PC = REL_ADDR(PC, src);

}

/* BNE */

if (!IF_ZERO()) {

clk += ((PC & 0xFF00) != (REL_ADDR(PC, src) & 0xFF00) ? 2 : 1);

PC = REL_ADDR(PC, src);

}

/* BPL */

if (!IF_SIGN()) {

clk += ((PC & 0xFF00) != (REL_ADDR(PC, src) & 0xFF00) ? 2 : 1);

PC = REL_ADDR(PC, src);

}

/* BRK */

PC++;

PUSH((PC >> 8) & 0xff);	/* Push return address onto the stack. */

PUSH(PC & 0xff);

SET_BREAK((1));             /* Set BFlag before pushing */

PUSH(SR);

SET_INTERRUPT((1));

PC = (LOAD(0xFFFE) | (LOAD(0xFFFF) << 8));

/* BVC */

if (!IF_OVERFLOW()) {

clk += ((PC & 0xFF00) != (REL_ADDR(PC, src) & 0xFF00) ? 2 : 1);

PC = REL_ADDR(PC, src);

}

/* BVS */

if (IF_OVERFLOW()) {

clk += ((PC & 0xFF00) != (REL_ADDR(PC, src) & 0xFF00) ? 2 : 1);

PC = REL_ADDR(PC, src);

}

/* CLC */

SET_CARRY((0));

/* CLD */

SET_DECIMAL((0));

/* CLI */

SET_INTERRUPT((0));

/* CLV */

SET_OVERFLOW((0));

/* CMP */

src = AC - src;

SET_CARRY(src < 0x100);

SET_SIGN(src);

SET_ZERO(src &= 0xff);

/* CPX */

src = XR - src;

SET_CARRY(src < 0x100);

SET_SIGN(src);

SET_ZERO(src &= 0xff);

/* CPY */

src = YR - src;

SET_CARRY(src < 0x100);

SET_SIGN(src);

SET_ZERO(src &= 0xff);

/* DEC */

src = (src - 1) & 0xff;

SET_SIGN(src);

SET_ZERO(src);

STORE(address, (src));

/* DEX */

unsigned src = XR;

src = (src - 1) & 0xff;

SET_SIGN(src);

SET_ZERO(src);

XR = (src);

/* DEY */

unsigned src = YR;

src = (src - 1) & 0xff;

SET_SIGN(src);

SET_ZERO(src);

YR = (src);

/* EOR */

src ^= AC;

SET_SIGN(src);

SET_ZERO(src);

AC = src;

/* INC */

src = (src + 1) & 0xff;

SET_SIGN(src);

SET_ZERO(src);

STORE(address, (src));

/* INX */

unsigned src = XR;

src = (src + 1) & 0xff;

SET_SIGN(src);

SET_ZERO(src);

XR = (src);

/* INY */

unsigned src = YR;

src = (src + 1) & 0xff;

SET_SIGN(src);

SET_ZERO(src);

YR = (src);

/* JMP */

PC = (src);

/* JSR */

PC--;

PUSH((PC >> 8) & 0xff);	/* Push return address onto the stack. */

PUSH(PC & 0xff);

PC = (src);

/* LDA */

SET_SIGN(src);

SET_ZERO(src);

AC = (src);

/* LDX */

SET_SIGN(src);

SET_ZERO(src);

XR = (src);

/* LDY */

SET_SIGN(src);

SET_ZERO(src);

YR = (src);

/* LSR */

SET_CARRY(src & 0x01);

src >>= 1;

SET_SIGN(src);

SET_ZERO(src);

STORE src in memory or accumulator depending on addressing mode.

/* NOP */

Nothing.

/* ORA */

src |= AC;

SET_SIGN(src);

SET_ZERO(src);

AC = src;

/* PHA */

src = AC;

PUSH(src);

/* PHP */

src = GET_SR;

PUSH(src);

/* PLA */

src = PULL();

SET_SIGN(src);	/* Change sign and zero flag accordingly. */

SET_ZERO(src);

/* PLP */

src = PULL();

SET_SR((src));

/* ROL */

src <<= 1;

if (IF_CARRY()) src |= 0x1;

SET_CARRY(src > 0xff);

src &= 0xff;

SET_SIGN(src);

SET_ZERO(src);

STORE src in memory or accumulator depending on addressing mode.

/* ROR */

if (IF_CARRY()) src |= 0x100;

SET_CARRY(src & 0x01);

src >>= 1;

SET_SIGN(src);

SET_ZERO(src);

STORE src in memory or accumulator depending on addressing mode.

/* RTI */

src = PULL();

SET_SR(src);

src = PULL();

src |= (PULL() << 8);	/* Load return address from stack. */

PC = (src);

/* RTS */

src = PULL();

src += ((PULL()) << 8) + 1;	/* Load return address from stack and add 1. */

PC = (src);

/* SBC */

unsigned int temp = AC - src - (IF_CARRY() ? 0 : 1);

SET_SIGN(temp);

SET_ZERO(temp & 0xff);	/* Sign and Zero are invalid in decimal mode */

SET_OVERFLOW(((AC ^ temp) & 0x80) && ((AC ^ src) & 0x80));

if (IF_DECIMAL()) {

if ( ((AC & 0xf) - (IF_CARRY() ? 0 : 1)) < (src & 0xf)) /* EP */ temp -= 6;

if (temp > 0x99) temp -= 0x60;

}

SET_CARRY(temp < 0x100);

AC = (temp & 0xff);

/* SEC */

SET_CARRY((1));

/* SED */

SET_DECIMAL((1));

/* SEI */

SET_INTERRUPT((1));

/* STA */

STORE(address, (src));

/* STX */

STORE(address, (src));

/* STY */

STORE(address, (src));

/* TAX */

unsigned src = AC;

SET_SIGN(src);

SET_ZERO(src);

XR = (src);

/* TAY */

unsigned src = AC;

SET_SIGN(src);

SET_ZERO(src);

YR = (src);

/* TSX */

unsigned src = SP;

SET_SIGN(src);

SET_ZERO(src);

XR = (src);

/* TXA */

unsigned src = XR;

SET_SIGN(src);

SET_ZERO(src);

AC = (src);

/* TXS */

unsigned src = XR;

SP = (src);

/* TYA */

unsigned src = YR;

SET_SIGN(src);

SET_ZERO(src);

AC = (src);