This is VERY brief documentation on the terminal program.

1. In terminal mode, you can store a screen by using Shift-1 and giving

and address (I hope I remember this correctly!)

Shift-2 clears the screen. Shift-4 returns you to the menu.

2. Parameter setting: All the items are self explanatory except for

register addressed. You need to enter the 16-bit I/O address for

the registers. For my interface, the CTL reg is at 375 (117H) and

the data reg is at 119 (77H). For the TS2050, I think the CTL reg

is at 119 (77H) and the data reg is at 115 (73H), but I can't

guarantee it.

3. Data and program transfers do NOT use any protocol! These were

meant at the time to transfer between two ZX81's running this

program.

4. Capital letters are displayed in inverse video. I didn't have

any knowledge of Wilf's true high res algorithm at the time,

and I didn't really want to do it in pseudo.

5. Some special chars are available by setting "F" cursor mode,

I think. Sorry, I don't really remember anything about this.

6. This program is bug-free as far as I know. I used to use it

all night long every weekend on some of the BBSs that were

around then. I have made a few slight mods to this version to

make it a little more friendly, and I've checked those out.